3GPP TSG-RAN WG2 Meeting #131	R2-2506201
Bangalore, India, August 25th – 29th, 2025

Agenda Item:	9.1

Source: 		Vice Chairman (Samsung)

Title: 			Report from session on R18 SL, R18/19 MOB and R19 NES

Document for:	Approval

Time Schedule 
Please refer to the latest schedule in the RAN2 inbox on the public 3GPP servers.

## Status of Rel-19 work items

Rel-19 Mobility enhancement and network energy saving enhancement WIs are completed from RAN2 point of view. Thank you all for your hard working and efforts.

## List and Status of Offline/Email Discussions

POST Email discussion:

[POST131][108][NES] (Huawei)

Scope: Update NES 38.300 CR (including this meeting agreements also).

Intended outcome: 38.300 CR in R2-2506219 to be agreed.

Deadline: Short email discussion

[POST131][109][NES] (Apple)

Scope: Update NES 38.304 CR (including this meeting agreements also).

Intended outcome: 38.304 CR in R2-2506220 to be agreed.

Deadline: Short email discussion

[POST131][110][NES] (Ericsson)

Scope: Update NES 38.331 CR (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506221 to be agreed.

Deadline: Short email discussion

[POST131][111][NES] (InterDigital)

Scope: Update NES 38.321 CR (including this meeting agreements also).

Intended outcome: 38.321 CR in R2-2506222 to be agreed.

Deadline: Short email discussion

[POST131][112][NES] (ZTE)

Scope: Update NES UE capability CRs (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506223 and 38.306 CR in R2-2506224 to be endorsed.

Deadline: Short email discussion

[POST131][113][MOB] (Apple)

Scope: Update MOB 38.300 CR (including this meeting agreements also).

Intended outcome: 38.300 CR in R2-2506225 to be agreed.

Deadline: Short email discussion

[POST131][114][MOB] (China Telecom)

Scope: Update MOB 37.340 CR (including this meeting agreements also).

Intended outcome: 37.340 CR in R2-2506226 to be agreed.

Deadline: Short email discussion

[POST131][115][MOB] (CATT)

Scope: Update MOB UE capability CRs (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506227 and 38.306 CR in R2-2506228 to be endorsed.

Deadline: Short email discussion

[POST131][116][MOB] (Vivo)

Scope: Update MOB 38.321 CR (including this meeting agreements also).

Intended outcome: 38.321 CR in R2-2506229 to be agreed.

Deadline: Short email discussion

[POST131][117][MOB] (Ericsson)

Scope: Update MOB 38.331 CR (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506230 to be agreed.

Deadline: Short email discussion

AT Email discussion:

[AT131][101][NES] (Apple)

Scope: Discuss and conclude the following stage-3 issues:

- Size of OD-SSB configuration index? P1 in R2-2506050, P1 in R2-2506112

- Whether RRC activation supports implicit deactivation? P1 of R2-2505338, P2 in R2-2506089

- Any other new PEI configuration? P3 in R2-2505500

- Information structure of paging adaptation: P2a and P2b in R2-2505255

- TP of SSB adaptation: P4 in R2-2506051, P1 in R2-2506095

- Other clarifications/corrections: P7 of R2-2505315, P4 and P7 in R2-2506089, P2 in R2-2505506, P6 in R2-2505097, P3 in R2-2505338, P5 in R2-2505507, P4 in R2-2505253, P1 in R2-2505255, and P4 in R2-2505170

Intended outcome: Discussion summary in R2-2506211.

Deadline: Phase-1: Email discussion (until 8/26 Tuesday noon), Phase-2: F2F discussion (8/26 Tuesday afternoon, offline meeting room will be announced) => Completed.

[AT131][102][MOB] (Ericsson)

Scope: Discuss and conclude RRC-2, RRC-3, RRC-4, RRC-5, RRC-6, RRC-7, RRC-8 and RRC-10 (with the consideration of RRC CR rapporteur’s inputs). Also including P2a and P2b in R2-2505165 and P3 in R2-2505893.

Intended outcome: Discussion summary in R2-2506212.

Deadline: 1000 – 1100 (8/27 Wednesday, main session room) => Completed.

[AT131][103][MOB] (Vivo)

Scope: Discuss and conclude MAC-26, MAC-27, MAC-28, MAC-29, MAC-30, MAC-31, and P2 in R2-2505926.

Intended outcome: Discussion summary in R2-2506213.

Deadline: 1000 – 1100 (8/26 Tuesday, main session room) => Completed.

[AT131][104][MOB] (Nokia)

Scope: Prepare an agreeable TP on mTRP.

Intended outcome: TP in R2-2506214.

Deadline: Comeback in 8/27 Wednesday session => Completed.

[AT131][105][NES] (Xiaomi)

Scope: Provide and discuss complete TP for option1 (including signalling and UE behaviour)

Intended outcome: TP in R2-2506215.

Deadline: TP will be treated in Wednesday session => Completed.

[AT131][106][NES] (Samsung)

Scope: Provide and discuss LS.

Intended outcome: LS in R2-2506216.

Deadline: Email approval until end of Wednesday => Handled online => Completed.

[AT131][107][MOB] (CATT)

Scope: Discuss and decide the following questions.

- Difference between per UE capability and per band capability with a restriction to set same value for all bands?

- “cltm-ExecutionConditionL3-r19” and ”cltm-ExecutionConditionL1-r19” per UE capability or per band capability with the restriction?

- Do we need differentiation for intra-F C-LTM and inter-F C-LTM? Note LTM capability has different ones for intra-F and inter-F cases.

Intended outcome: Discussion summary in R2-2506217.

Deadline: Offline meeting room will be announced by CATT. Comeback either in Wednesday session or CB session. => Completed.

## Approved outgoing LSs

R2-2506216	LS on RAN2 agreement on SUL support for SIB1 request	LSout	To: RAN1	Rel-19	Netw_Energy_NR_enh-Core

#### 7.0.2.20	NR Sidelink evolution

(NR_SL_enh2-Core; leading WG: RAN1; REL-18; WID: RP-230077)

#### 7.0.2.22	Further NR mobility enhancements

(NR_Mob_enh2-Core; leading WG: RAN2; REL-18; WID:RP-233970)

R2-2505718	Corrections to Rel-18 LTM (LTM Recovery to the source cell)	Nokia	discussion	Rel-18	NR_Mob_enh2-Core	R2-2504136

[Lenovo]: Why we didn’t have the problem in the earlier release, e.g. HO case? [Nokia]: For LTM, we have a candidate configuration and it can be different than actual serving cell configuration that has been used, which is different than L3 based HO. [Qualcomm]: Understand it is a corner case. [Lenovo]: Assume it can be handled by network. [Samsung]: Assume network needs to reconfigure the associated candidate configuration also when it reconfigures the serving cell configuration. Assume it is corner case. [Nokia]: Do not think it is a corner case. [Huawei]: Understand if network wants to have recovery function, network should be responsible for configurations in order this not to happen. [ZTE]: Do not agree with Samsung’s comment. network can configure the candidate configuration and the serving configuration differently, but network can handle each case by implementation. If network configures two resource differently, network should be able to handle two resources together.

Not pursued.

R2-2505814	Correction on when the UE applies the SR configuration	Ericsson	CR	Rel-18	38.331	18.6.0	5442	-	F	NR_Mob_enh2-Core

[Samsung]: Understand it is not a correction. It proposes a new mechanism. If SR resource is directly used during LTM cell switch procedure, we need a mapping between SSB and SR resource. [Lenovo]: If the intention is to use SR after LTM cell switch completion, it’s ok. However, if the intention is to use SR during LTM cell switch, it’s not acceptable as a correction. [MediaTek]: Understand, based on the sentence “Before RACH-less LTM procedure completion, the UE shall not trigger random access procedure if it does not have a valid PUCCH resource for triggered SRs.” in 38.300, SR resource can be available during LTM cell switch for RACH-less LTM case. [Session chair]: Let’s have offline to check which understanding is correct. [Ericsson]: Will take the offline until either Wednesday session or CB session.

Comeback:

[Ericsson]: Discussed with other companies that had concerns and all companies are ok with the proposal now. We do not need to agree the CR, but it’s good to capture it in the minutes. [ZTE]: Wonder why it cannot be applied to C-LTM case? [Ericsson]: C-LTM is different than LTM since the target candidate cannot receive the selected beam information from the source. [Samsung]: Do not see any additional spec impact for LTM since it is already clarified in 38.300. [Apple]: Wonder if it can be also applicable to RACH-less HO? [Nokia]: Want to have more time to consider. [Session chair]: Any companies want to have more time to check it until next meeting? No company.

SR may be available during RACH-less LTM cell switch procedure if TA is valid.

R2-2506012	Correction on Power Ramping for early TA	Samsung	CR	Rel-18	38.321	18.6.0	2111	-	F	NR_Mob_enh2-Core

Agreed.

R2-2506073	Correction for RRC segment handling	Samsung	CR	Rel-18	38.331	18.6.0	5450	-	F	NR_Mob_enh2-Core

[Huawei]: Wonder why we don’t have such a sentence in HO case? [Samsung]: For LTM, MAC is used to cell switch command. For HO, RRC Reconfiguration will be sent after all segmented RRC messages sent. [Ericsson]: Understand, for LTM, the UE clears everything except the specified ones. Assume it is ok without this correction. [MediaTek]: Although the intention is correct, understand network can avoid this situation. [ZTE]: Understand the difference compared to L3 based HO. Support the proposal. Also note it is already captured in other places in the spec. [Apple]: Agree with MediaTek. It can be avoided by network implementation. Note it was discussed for L3 based HO in the past and that’s why it is not specified for the cases where network can handle. [Ericsson]: Agree with Apple. [ZTE]: For LTM, not sure what network option is good to avoid this issue. If cannot avoid, what’s the consequence?

Noted.

R2-2506119	Clarification on PDCCH order triggered early RACH	ZTE Corporation	CR	Rel-18	38.321	18.6.0	2115	-	F	NR_Mob_enh2-Core

[Samsung]: Understand the concerned scenario is not valid since PDCCH order triggered early RACH is a kind of one-shot procedure. [ZTE]: Consider it is valid that the NW can send multiple PDCCH orders with the different PCI, but if we follow the current spec, RACH for the new candidate will not be triggered. [Samsung]: Actually, note was added for the same serving cell although it is not included in the note clearly. Otherwise we need other corrections for other cases (not only for LTM). [ZTE]: Want to have some time to talk more with companies.

Revised in R2-2506421.

R2-2506421	Clarification on PDCCH order triggered early RACH	ZTE Corporation	CR	Rel-18	38.321	18.6.0	2115	1	F	NR_Mob_enh2-Core

[Huawei]: In cover page, Radio Access Network should be clicked.

Radio Access Network should be clicked in affects.

CR in R2-2506232 is agreed with the change above.

R2-2505813	Correction on capabilities for LTM	Ericsson	CR	Rel-18	38.306	18.6.0	1339	-	F	NR_Mob_enh2-Core	Withdrawn

## 8.5	Network Energy Saving Enh.

(Netw_Energy_NR_enh-Core; leading WG: RAN1; REL-19; WID: RP-242354)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.5.1	Organizational

Incoming LS, WI rapporteur inputs, CR rapporteur inputs (including post email discussion [POST130][107], [108], [109], [119], summary of identified stage-3 open issues that need online discussion and rapporteur’s suggestions if needed, details of UE capability discussion if needed).

WI is completed from RAN2 point of view.

R2-2505051	Reply LS on new servingCellMO of OD-SSB on SCell (R4-2508440; contact: Apple)	RAN4	LS in	Rel-19	Netw_Energy_NR_enh	To:RAN2	Cc:RAN1

Noted.

R2-2505564	Introduction of Network Energy Savings Enhancements	Huawei, HiSilicon	CR	Rel-19	38.300	18.6.0	1013	-	B	Netw_Energy_NR_enh-Core

[Session chair]: What about the following editor’s note?

“Editor’s note: FFS whether to merge OD-SSB and SSB adaptation into one section.”

Editor’s note needs to be removed in the revision.

Endorsed and considered as baseline for further discussion.

[POST131][108][NES] (Huawei)

Scope: Update NES 38.300 CR (including this meeting agreements also).

Intended outcome: 38.300 CR in R2-2506219 to be agreed.

Deadline: Short email discussion

R2-2505497	Summary report of [POST130][108][NES] 38.304 CR (Apple)	Apple (Rapporteur)	discussion	Rel-19	Netw_Energy_NR_enh-Core

Noted.

R2-2505496	Running 38.304 CR for network energy saving	Apple (Rapporteur)	CR	Rel-19	38.304	18.4.0	0442	-	B	Netw_Energy_NR_enh-Core

Endorsed and considered as baseline for further discussion.

[POST131][109][NES] (Apple)

Scope: Update NES 38.304 CR (including this meeting agreements also).

Intended outcome: 38.304 CR in R2-2506220 to be agreed.

Deadline: Short email discussion

R2-2505708	Report of [POST130][107][NES] (Ericsson)	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

[Ericsson]: Indicates there is one issue that CR rapporteur accepted during the email discussion, but realize we need more discussion. Will indicate that issue to session chair (including WI rapporteur), then we can the discussion back later. [Session chair]: In the email, Ericsson clarified that it was already included in the offline discussion (P5 in R2-2505507).

Noted.

R2-2505699	Introduction of enhancements for network energy efficiency 	Ericsson	CR	Rel-19	38.331	18.6.0	5428	-	B	Netw_Energy_NR_enh-Core

Endorsed and considered as baseline for further discussion.

[POST131][110][NES] (Ericsson)

Scope: Update NES 38.331 CR (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506221 to be agreed.

Deadline: Short email discussion

R2-2505791	Report of [POST130][109][NES] Comments to 38.321 CR for NES	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

Noted.

R2-2505792	Introduction of network energy saving enhancements to TS 38.321	InterDigital	CR	Rel-19	38.321	18.6.0	2110	-	B	Netw_Energy_NR_enh-Core

Endorsed and considered as baseline for further discussion.

[POST131][111][NES] (InterDigital)

Scope: Update NES 38.321 CR (including this meeting agreements also).

Intended outcome: 38.321 CR in R2-2506222 to be agreed.

Deadline: Short email discussion

R2-2505987	Introduction of UE capability for network energy saving enhancement in TS 38.306	ZTE Corporation, Sanechips	draftCR	Rel-18	38.306	18.6.0	F	Netw_Energy_NR_enh-Core

Endorsed.

R2-2505988	Introduction of UE capability for network energy saving enhancement in TS 38.331	ZTE Corporation, Sanechips	draftCR	Rel-18	38.331	18.6.0	A	Netw_Energy_NR_enh-Core

Endorsed.

[POST131][112][NES] (ZTE)

Scope: Update NES UE capability CRs (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506223 and 38.306 CR in R2-2506224 to be endorsed.

Deadline: Short email discussion

[AT131][101][NES] (Apple)

Scope: Discuss and conclude the following stage-3 issues:

- Size of OD-SSB configuration index? P1 in R2-2506050, P1 in R2-2506112

- Whether RRC activation supports implicit deactivation? P1 of R2-2505338, P2 in R2-2506089

- Any other new PEI configuration? P3 in R2-2505500

- Information structure of paging adaptation: P2a and P2b in R2-2505255

- TP of SSB adaptation: P4 in R2-2506051, P1 in R2-2506095

- Other clarifications/corrections: P7 of R2-2505315, P4 and P7 in R2-2506089, P2 in R2-2505506, P6 in R2-2505097, P3 in R2-2505338, P5 in R2-2505507, P4 in R2-2505253, P1 in R2-2505255, and P4 in R2-2505170

Intended outcome: Discussion summary in R2-2506211.

Deadline: Phase-1: Email discussion (until 8/26 Tuesday noon), Phase-2: F2F discussion (8/26 Tuesday afternoon, offline meeting room will be announced) => Completed.

R2-2506211	Summary report of [AT131][101][NES] (Apple)	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Agreeable proposals

Proposal 1 (13/15) maxNrofOD-SSB is set to 16.

Proposal 2 (13/15) RAN2 don’t pursue further optimization of MAC-CE format.

Proposal 3 (14/15) Following RAN1 conclusion, not support implicit deactivation with RRC-based activation. The following revisions to be reflected in RRC running CR:

•	od-ssb-ActivationStatus: Indicates the activation status of this OD-SSB pattern upon configuration. Only one OD-SSB pattern can be activated at one point of time.

•	od-ssb-nrofBurst: Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is activated. Network only configures this field when od-ssb-ActivationStatus is absent.

Proposal 4 (7/13) The following Rel-19 PEI configurations with the same value range (except pei-FrameOffset-r19) as Rel-17 PEI configuration need to be introduced.

o	po-NumPerPEI-r19

o	payloadSizeDCI-2-7-r19

o	pei-FrameOffset-r19: extend to 32 radio frame

Proposal 5 (15/15) Adopt the following information structure of paging adaptation

•	If network supports paging adaptation, both pagingAdaptation-NS-r19 and pagingAdaptationNAndPagingFrameOffset are configured.

•	pagingAdaptation-NS-r19,  pagingAdaptationNAndPagingFrameOffset and  firstPDCCH-MonitoringOccasionOfPO are grouped together in one IE. pagingAdaptation-NS-r19,  pagingAdaptationNAndPagingFrameOffset are mandatory in this IE. firstPDCCH-MonitoringOccasionOfPO-r19 is optionally configured in the IE.

Proposal 8: Revise the type of od-ssb-nrofBurst as ENUMERATED {5, 10, 15, 20, 25, 30, 40, 50, 75, 100, 150, 200}.

Proposal 9: OD-SSB SMTC adaptation apply to both inter and intra frequency measurement, and it should be clear which SMTC to use for the UE. It is up to RRC CR Rapporteur how to capture it in running CR.

Proposal 10: On od-SSB-periodicity, remove default value of 5ms and change the presence to mandatory.

Proposal 11: For Case#2, if the PBCH block power or SSB sub-carrier spacing or absolute frequency for OD-SSB is not configured, the UE applies the same value configured for the always-on SSB. RRC running CR to reflect this agreement in the field description.

Proposal 12: Adopt the change suggested by Proposal 5 of R2-2505507.

Proposal 13: Adopt the following revised correction on OD-SIB1:

5.2.2.4.2x             Actions upon reception of SIBxx

1> if the UE has (re)selected to a cell providing OD-SIB1, the UE stores the configuration for SIB1 request for this cell and considers it valid while camping in this cell:

2>	store the SIBxx;

2>	SIB1 request configuration in the SIBxx is valid for acquiring OD-SIB1 of this cell in accordance with clause 5.2.3.3.x;

2>	SIB1 request configuration of another cell in this stored SIBxx is valid for acquiring OD-SIB during reselection to that cell, and after reselection to that cell if the stored SIBxx is a valid version for that cell in accordance with clause 5.2.2.2.1;

Proposal 14: If firstPDCCH-MonitoringOccasionOfPO-r19 is not configured, UE does not determine POs based on starting PDCCH monitoring occasion number. The starting PDCCH monitoring occasion number of (i_s + 1)th PO is equal to i_s * S*X.

Proposal 15: When pagingAdaptationPEI-Config is signaled in system information, pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 should be configured.

All proposals above (proposal 1 - 5, and 8 - 15) are agreed.

Need online discussion:

Proposal 6: Discuss whether to adopt the SSB adaptation TP (from Huawei or Ericsson) as baseline.

[Apple]: Propose first to handle the TP on OD-SSB, then we can comeback to proposal 6. [Apple]: Propose to have same way as OD-SSB. [Ericsson]: Prefer keeping the current running RRC way to avoid changes in procedure text in the last meeting. [Apple]: Huawei’s TP already provides the procedure text. Xiaomi can merge it. [Ericsson]: Prefer to handle TP in separate for this case. Either option can work. [Xiaomi]: Although either option works, prefer to have same way as OD-SSB.

Comeback:

R2-2506231	TP for SSB adaptation	Huawei	discussion	Rel-19	Netw_Energy_NR_enh-Core

[Huawei]: The new parts are text in 5.5.2.10 and adding SMTC in Measurement object. [Ericsson]: Wonder if we need also some UE procedure (e.g. like something captured in TP on OD-SSB). [Session chair]: It is up to RRC CR rapporteur to see the existing legacy section and analyse whether we need some UE procedure change also or not.

TP in R2-2506231 is baseline.

### 8.5.2	On-demand SSB SCell operation

Remaining essential open issues including stage-3 identified open issues if needed, etc.

1. Information structure of OD-SSB-Config (including where to include servingCellMO)?

R2-2505498	Remaining issues on on-demand SSB for SCell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 3 (RRC open issue 3): Following RAN2#130 agreement, move new servingCellMO for OD-SSB under ServingCellConfig (i.e. servingCellMO is per SCell configuration rather than per OD-SSB configuration).

Proposal 4 (RRC open issue 3): Move SMTC for OD-SSB under MeasObjectNR and add procedure text.

Following RAN2#130 agreement, move new servingCellMO for OD-SSB under ServingCellConfig (i.e. servingCellMO is per SCell configuration rather than per OD-SSB configuration).

Move SMTC for OD-SSB under MeasObjectNR and add procedure text.

[AT131][105][NES] (Xiaomi)

Scope: Provide and discuss complete TP for option1 (including signalling and UE behaviour)

Intended outcome: TP in R2-2506215.

Deadline: TP will be treated in Wednesday session => Completed.

On proposal 3 and 4:

[Ericsson, Nokia]: If we agree with proposal 3, it needs more specification effort, e.g. SMTC list needs to be moved into measurement object and we should define an association between SMTC in the list and the corresponding OD-SSB configuration. Understand either option can work. [LG, Huawei, Xiaomi]: Support the proposal to keep the design of measurement configuration that has been for long time. [Session chair]: Agree with that either option can work. Suggest to follow majority companies’ preference.

- Option1: proposal 3 + proposal 4

  - Apple, Huawei, Xiaomi, Samsung, Google, Fujitsu, LG, Vivo, CATT

- Option2: keep the current information structure

  - Ericsson, Nokia, ZTE, OPPO, Qualcomm

Proposal 5 (RRC open issue 3): Move the following OD-SSB IEs which can’t be dynamically adapted via MAC-CE under SCellConfig (i.e. they can only be reconfigured by RRC rather than by MAC-CE)

•	od-ssb-sfn-Offset-r19

•	od-ssb-halfFrameIndex-r19

•	od-ssb-absoluteFrequency-r19

•	od-ssbSubcarrierSpacing-r19

•	od-ss-PBCH-BlockPower-r19

Agree with the intention of proposal 5.

How to implement it to RRC is left to RRC rapporteur.

On proposal 5:

[Ericsson]: Although it was not clear according to RAN1’s provided excel file, agree with intention. [LG, NTTDCM]: Understand od-ssb-sfn-Offset-r19 and od-ssb-halfFrameIndex-r19 can be also dynamically changed by MAC CE. [Apple]: It is not for OD-SSB. Understand there was similar discussion/decision for SSB adaptation. It is very clear what can be dynamically changed by MAC CE from RAN1 agreements.

R2-2506215	Text proposal for option 1	Xiaomi	discussion	Rel-19	Netw_Energy_NR_enh-Core

TP in R2-2506215 is baseline.

[Ericsson]: Proposed TP seems complicated and want to revert RAN2 agreement back. [Nokia]: We can first agree with three cases that Apple indicated during offline, then we may need further discussion how to implement the three cases. [Apple]: Understand Xiaomi’s TP already covers all three cases well. [Ericsson]: Understand this TP is also different compared to how to capture Rel-17 additional MO. [Xiaomi]: The difference comes from more cases for OD-SSB. [Session chair]: Suggest check companies’ opinions again after seeing the TP.

- Option1: Current running CR: Ericsson, Nokia, OPPO

- Option2: TP provided by Xiaomi: LG, Apple, Huawei, Xiaomi, Samsung

Comeback:

[Ericsson]: Wonder, when OD-SSB is activated, whether CSI measurement cannot be done. [LG]: Not understand the concern. The TP just specifies the relation between measurement and OD-SSB. [Nokia]: Understand the Ericsson’s concern is more RAN4 scope. [LG]: If concerned, we can just add legacy behaviours under the bullet “3> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO-OD:”

TP is updated that measurement on CSI-RS is allowed as legacy.

Under “3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO-OD:”, the following legacy UE behaviour is added.

4>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO:

4>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to csi-rs:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

“2> if the servingCellMO-OD is configured and OD-SSB transmission is activated:” is updated to “2> else if the servingCellMO-OD is configured and OD-SSB transmission is activated:”

2. Definition of intra-F measurement and inter-F measurement

R2-2505498	Remaining issues on on-demand SSB for SCell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 6: RAN2 discuss and down-select between the following two options on how to define intra-frequency and inter-frequency measurement when OD-SSB and AO-SSB are in different frequencies:

•	Option 1: When the center frequency of either AO-SSB or OD-SSB of one SCell is same as that of neighbor cell SSB, it is intra-frequency measurement. Otherwise, it is inter-frequency measurement.

•	Option 2: When the center frequency of the applied servingCellMO of one SCell is same as that of neighbor cell SSB, it is intra-frequency measurement. Otherwise, it is inter-frequency measurement.

o	Note: which servingCellMO is applied follows RAN2#130 agreement.

[Ericsson]: There are some description for intra-F and inter-F in the ASN.1 form, we can rename that if it confuses us. [LG]: Understand the current intra-F and inter-F measurement definition is based on the SSB frequency location and there is no reason to change it from RAN2 point of view. Since it may be helpful to RAN4’s further discussion, it is good to capture RAN2 agreement/understanding.

RAN2 understand the current definition of intra-F measurement ad inter-F measurement is kept even for OD-SSB case.

Proposal 7: RAN2 discuss and down-select between the following two options on how to handle measurement gap when OD-SSB and AO-SSB are in different frequencies:

•	Option 1: if any of the UE configured BWPs (other than initial BWP) do not contain the frequency domain resources of both AO-SSB and OD-SSB, a measurement gap configuration is always provided.

•	Option 2: if any of the UE configured BWPs (other than initial BWP) do not contain the frequency domain resources of SSB in the applied servingCellMO, a measurement gap configuration is always provided.

o	Note: which servingCellMO is applied follows RAN2#130 agreement.

Noted.

R2-2505097	Remaining open issues on OD-SSB SCell operation	LG Electronics France	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505113	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505168	Consideration on on-demand SSB SCell operation	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505254	On-demand SSB SCell Operation	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505276	Some details on OD-SSB for NES cell	Quectel	discussion

R2-2505315	Remaining open issues on OD-SSB	Xiaomi	discussion	Netw_Energy_NR_enh-Core

R2-2505338	Remaining issues of On-demand SSB SCell operation	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505498	Remaining issues on on-demand SSB for SCell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505506	Remaining issues on on-demand SSB SCell operation	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505526	Discussion on on-demand SSB SCell operation	NTT DOCOMO, INC.	discussion	Rel-19

R2-2505566	Remaining issues on OD-SSB	Sharp	discussion

R2-2505790	Remaining issues for On demand SSB	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505845	Discussion on remaining issues of on-demand SSB?	Qualcomm Incorporated	discussion

R2-2505943	Discussion on open issues of on-demand SSB for Scell	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505989	Remaining issues of on demand SSB SCell operation	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506050	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506089	On demand SSB handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506112	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

### 8.5.3	On-demand SIB1

Remaining essential open issues including whether to support SUL in OD-SIB1 cell, stage-3 identified open issues if needed, etc.

1. Whether to support SUL in OD-SIB1 cell?

R2-2505253	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1a: SIB 1 request is supported for both SUL and NUL.

Proposal 1b: RSRP threshold for UL carrier selection is included in SIB1 request configuration.

R2-2505990	Remaining issues of on demand SIB1	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: Follow RAN1 conclusion on whether to support SUL for on demand SIB1 request configuration, i.e. SupplementaryUL is not included in the UL-WUS configuration. LS should be sent to RAN1 for confirmation if RAN2 make different agreement.

[Samsung]: Understand the intention of “SupplementaryUL is not included in the UL-WUS configuration” was not really regarding whether SUL is supported or not for OD-SIB1. It was about whether SUL was included or not in BWP configuration of OD-SIB1 REQ configuration. [Apple]: Understand there was no real technical discussion regarding whether SUL is supported or not in RAN1, just due to lack of time, RAN1 feature leader didn’t include SUL without actual discussion. Understands RAN2 can make decision. [Nokia]: Understand the RAN1 situation at that moment, but do we really have time to ask it to RAN1 to support SUL. Better to forget SUL due to lack of time. [Xiaomi]: Understand there is no actual RAN1 impact, even though RAN2 agrees to support SUL.

SIB 1 request is supported for both SUL and NUL.

RSRP threshold for UL carrier selection is included in SIB1 request configuration.

[AT131][106][NES] (Samsung)

Scope: Provide and discuss LS.

Intended outcome: LS in R2-2506216.

Deadline: Email approval until end of Wednesday => Handled online => Completed.

R2-2506216	LS on RAN2 agreement on SUL support for SIB1 request	LSout	To: RAN1	Rel-19	Netw_Energy_NR_enh-Core

Approved.

2. Clarification on si-BroadcastStatus

R2-2505562	Discussion on remaining issues of on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: Clarify in the normative text that a UE supporting OD-SIB1 considers the si-BroadcastStatus in the stored SIB1 as the latest one if a cell is providing OD-SIB1.

“Since the normative text already captures the UE behaviour, it is optional to include it in si-BroadcastStatus. In our view it can be captured in both places for clarity of the si-BroadcastStatus behaviour.”

[Ericsson]: Agree with intention, but wants to avoid “a cell is providing OD-SIB1”. [Samsung]: Instead, we can use the existing conditions, e.g. something like “if ssbCarrierOffset in MIB indicates no SIB1 and the UE has the corresponding OD-SIB1 REQ configuration”. [Ericsson]: Agree with Samsung.

Agree with the intention, but the existing conditions that has same meaning as “a cell is providing OD-SIB1” will be used. It is only applicable to RRC connected state. How to implement is up to RRC rapporteur.

Agree to include the similar description in the field description of si-BroadcastStatus.

3. Clarification on SIB1 update upon reception of short message

R2-2505339	Remaining issues of On-demand SIB1	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: RAN2 to discuss how to understand the previous agreement ‘Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’:

-	Understanding 1: The UE assumes the NES cell will always transmits CD-SSB after sending SI change notification, the UE acquires SIB1 with the Coreset#0 and Searchspace#0 parameters in MIB as legacy;

-	Understanding 2: It is up to the NES cell to transmits CD-SSB or NCD-SSB after sending SI change notification. If the NES cell transmits CD-SSB, the UE acquires SIB1 with the Coreset#0 and Searchspace#0 parameters in MIB as legacy; and if the NES cell transmits NCD-SSB,

•	2a: the UE assumes the Coreset#0 and Searchspace#0 parameters in the stored OD-SIB1 configuration are still valid, and acquires the updated SIB1 with the stored Coreset#0 and Searchspace#0 parameters from the start of the of the next modification period;

•	2b: the UE assumes the Coreset#0 and Searchspace#0 parameters in the stored OD-SIB1 configuration are no longer valid, and performs cell reselection to other cells;

•	2c: the UE assumes the stored OD-SIB1 request configuration is still valid, and performs OD-SIB1 request as the current procedure in the running CR;

R2-2505169	Consideration on on-demand SIB1	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from network directly without sending OD-SIB1 request in next MP, i.e., the network will switch to normal mode with legacy K_SSB value and transmit the updated SIB1 in next MP.

Proposal 3a: In emergency cases, it is up to network implementation to broadcast the SIB1 periodically based on the valid OD-SIB1 request configuration stored by UE, no spec impact will be introduced.

Proposal 3b: Upon receiving the etwsAndCmasIndication in a Short Message on the NES cell, the UE shall immediately re-acquire SIB1 based on the CORESET information contained in the OD-SIB1 request information, without sending an OD-SIB1 request. i.e., the UE will follow the legacy procedure.

[Samsung]: Agree with proposal 2, and think we need some change accordingly. [Ericsson]: Do not agree with proposal 2. [Session chair]: Based on the discussion, let’s list possible options.

- Option1: Upon sending SI change notification, NW configure Kssb < 24 or <12 and the UE (including legacy UEs) receives the updated SIB (legacy way)

- Option2a: Upon sending SI change notification, NW may configure/keep Kssb >24 or > 12, and the NES UE performs OD-SIB1 request procedure to acquire the updated SIB

- Option2b: Upon sending SI change notification, NW may configure/keep Kssb>24 or >12, but the NES UE does not perform OD-SIB1 request procedure, and with the additional UE behaviour change, the NES UE receives the updated SIB (legacy UE cannot receive it)

[Session chair]: Seems many companies prefer NW may configure any Kssb upon SI change notification, but NES UE still acquires the updated SIB1 without performing OD-SIB1 REQ procedure (option1 or option2b). Understand option2a is not aligned with RAN2 previous agreement.

NW may configure any Kssb upon SI change notification (including ETWS/CMAS), but NES UE still acquires the updated SIB1 without performing OD-SIB1 REQ procedure.

RRC CR rapporteur will provide the TP in R2-2506218 until Wednesday session.

R2-2506218	TP on SIB1 update upon reception of short message	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

TP in R2-2506218 is baseline.

[Vivo]: In the concerned case, coreSetZero comes from OD-SIB1 REQ configuration, which is new. Think it may impact on RAN1 spec. We may need to send LS to RAN1. [Samsung, Apple]: This behaviour is specified in RAN2 spec. [Samsung]: Kssb is legacy value, and two cases (Kssb > 24/12, or not) are already specified in RAN1 spec. Do not see a need to send an LS to RAN1.

4. Co-existence with other legacy features

R2-2505110	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: RAN2 to discuss following options for co-existence of on-demand SIB1 and SDT:

•	Option 0: Do nothing. Reselection to an OD-SIB1 cell is not supported during SDT procedure.

•	Option 1: UE stops SDT before performing OD-SIB1 procedure.

•	Option 2: UE performs OD-SIB1 when SDT is ongoing.

[Apple, Samsung, Huawei, ZTE, Qualcomm]: Agree with option 0. [Samsung]: Understand with option1, there is no spec impact.

For co-existence of on-demand SIB1 and SDT, do nothing.

R2-2505766	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: RAN2 to agree on addition of a new value for RA purpose to accommodate for the case where RA procedure was used for request for SIB1.

Noted.

[IDC]: Understand, based on the RACH configuration/partitioning, the network can know this is used for OD-SIB1 REQ. [Samsung]: Understand in general, feature specific MDT/SON will be discussed in the next release. We don’t need to discuss it now. [ZTE, Huawei, Xiaomi, NEC]: Agree with Samsung. [Xiaomi]: It can be discussed as TEI19/20.

R2-2505157	Discussion on on-demand SIB1	Transsion Holdings	discussion	Rel-19

Proposal 3: RAN2 is kindly asked to confirm whether the OD-SIB1 cell can be target cell of HO procedure.

Proposal 4: The SIB1 of OD-SIB1 cell includes OD-SIB1 indication (e.g., in CellAccessRelatedInfo)

[LG]: Support the proposal 3 only for OD-SIB capable UE. [Apple, Xiaomi, CATT, Huawei, Samsung]: Think OD-SIB1 cell can be target cell of HO, but do not think anything additional is required. [Ericsson]: Wonder if OD-SIB1 UE capability for RRC connected is required. [Session chair]: Do we need this capability for RRC connected UE? Seems companies do not think any OD-SIB1 UE capability for RRC connected is required.

OD-SIB1 cell can be target cell of HO but no spec change is required.

5. Others

R2-2505990	Remaining issues of on demand SIB1	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: The OD-SIB1 capability is defined as optional with capability signaling to assist the dedicated reselection priority configuration at network side.

[NEC]: Support the proposal. [Huawei]: In RAN1 discussion, it was decided as optional without UE capability since it is applicable to RRC idle/inactive states. [Huawei, Nokia]: Ok with proposal 5. [Lenovo]: Understand the cells with OD-SIB1 REQ are more per cell level, not per frequency level. [Nokia]: It can be also frequency level.

The OD-SIB1 capability is defined as optional with capability signaling to assist the dedicated reselection priority configuration at network side.

[Ericsson]: Ok with the proposal. Wonder if we can also include this capability into paging container, so the network may skip sending paging to the cell that operates based on OD-SIB1 REQ and if the UE does not support this capability. [Session chair]: We didn’t bar the legacy UEs into NES cell. Any companies want to support putting this capability into paging container also? No company.

OD-SIB1 capability is not included into paging container.

R2-2505562	Discussion on remaining issues of on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: Once a NES UE acquires Cell A’s SIB1 and knows that it provides SIBxx, it can acquire Cell A’s SIBxx to obtain WUS configurations of neighbouring cells, even if the UE type is barred in SIB1.

Noted.

[Vivo]: Understand the current specification does not stop that UE behaviour. We can leave it to UE implementation. [LG]: Consider over-optimization and also network can send it when it supports all types of the UE. [Xiaomi]: Do not think it is essential issue.

R2-2505110	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505157	Discussion on on-demand SIB1	Transsion Holdings	discussion	Rel-19

R2-2505169	Consideration on on-demand SIB1	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505253	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505339	Remaining issues of On-demand SIB1	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505499	Remaining issues on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505507	Remaining issues on on-demand SIB1 procedure	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505509	Remaining open issues on OD-SIB1 request	Sharp	discussion	Rel-19

R2-2505527	Discussion on on-demand SIB1	NTT DOCOMO, INC.	discussion	Rel-19

R2-2505562	Discussion on remaining issues of on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505644	Discussion on the remaining open issues for OD-SIB1	ITRI, Acer Incorporated	discussion	Netw_Energy_NR_enh-Core

R2-2505656	On-demand SIB1 request on SUL	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505749	Consideration on on-demand SIB1	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505766	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505846	Discussion on remaining issues of on-demand SIB1	Qualcomm Incorporated	discussion

R2-2505944	Discussion on open issues of OD-SIB1	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505978	Remaining issues on OD-SIB1 operation	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505990	Remaining issues of on demand SIB1	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506008	Remaining essential open issues including whether to support SUL in OD-SIB1 cell.	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506063	Discussion on on-demand SIB1	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506090	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506170	Discussion on Additional Aspects of OD-SIB1	CEWiT	discussion

### 8.5.4	Adaptation of common signal/channel transmissions

Remaining essential open issues including down-selection of solutions on how to disable RACH adaptation for MSG1 repetition, stage-3 identified open issues if needed, etc.

1. Detailed format of firstPDCCH-MonitoringOccasionOfPO-r19

R2-2505500	Remaining issues on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1 (RRC open issue Q13/Samsung003/ LGE003/Apple003): As legacy, keep IE firstPDCCH-MonitoringOccasionOfPO-r19 with symbol level offset and without NW restriction on ssb-periodicityServingCell. Adopt the following ASN.1 format to support Ns=8 and N=T/32:

Proposal 2 (RRC open issue Q14/Samsung003/ LGE003/Apple003): As legacy, keep IE firstPDCCH-MonitoringOccasionOfPEI-O-r19 with symbol level offset. Adopt the following ASN.1 format to support up to 8 PEI-O per PF and 32 radio frames:

R2-2506095	Adaptation of common signal/channel transmissions for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 6: For Rel-19 NES UEs, firstPDCCH-MonitoringOccasionOfPO-r19 is configured at slot level.

R2-2505340	Remaining issues of common signal adaptation	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: For signaling overhead optimization of Rel-19 firstPDCCH-MonitoringOccasionOfPO, restrict the maximum value range of the following parameters not to exceed symbol amount corresponding to 16rf (radio frames) for different SCS corresponding to the Rel-19 nAndPagingFrameOffset (i.e. N):

-	Rel-19 firstPDCCH-MonitoringOccasionOfPO;

-	Rel-19 firstPDCCH-MonitoringOccasionOfPEI-O;

R2-2506091	Adaptation of common signals	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8: RAN2 to discuss how to efficiently transmit firstPDCCH-MonitoringOccasionOfPO for the 4 additional POs of a PF e.g. by reducing granularity or delta to previous PO signaling.

[Samsung]: firstPDCCH-MonitoringOccasionOfPO is not simple number of symbols or slots. It is logical number of paging occasions. [LG, Huawei, CATT, OPPO, Qualcomm, Samsung]: Support Apple’s proposal. [Vivo]: Ok, can accept Apple’s proposal. [Samsung]: From Apple’s proposal, let’s remove “offset” not to give any confusion.

As legacy, keep IE firstPDCCH-MonitoringOccasionOfPO-r19 with symbol level and without NW restriction on ssb-periodicityServingCell. Adopt the following ASN.1 format to support Ns=8 and N=T/32.

As legacy, keep IE firstPDCCH-MonitoringOccasionOfPEI-O-r19 with symbol level. Adopt the following ASN.1 format to support up to 8 PEI-O per PF and 32 radio frames:

2. Whether to support paging adaptation in non-initial BWP for redcap UE?

R2-2506051	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: firstPDCCHMonitoringOccasionOfPO-r19 is introduced both PCCH-Config and PDCCH-ConfigCommon.

Proposal 2: firstPDCCH-MonitoringOccasionOfPEI-O-r19 is only introduced in PDCCH-ConfigCommon, not in PCCH-Config.

R2-2505275	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: R2 not work on paging adaptation co-existence with (e)RedCap at Q3, i.e., remove the IE pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 implemented in PDCCH-ConfigCommon in the current RRC running CR for now, and discuss the cross-WI issues (if any) in the maintenance phase.

[Apple]: After checking, there is no RAN1/2 spec impact to support (e)RedCap. Also note the current running CR already supports (e)RedCap. [Vivo]: Agree with Apple. Understand if we exclude only (e)RedCap, it needs more specification efforts.

firstPDCCHMonitoringOccasionOfPO-r19 is introduced both PCCH-Config and PDCCH-ConfigCommon.

firstPDCCH-MonitoringOccasionOfPEI-O-r19 is only introduced in PDCCH-ConfigCommon, not in PCCH-Config.

3. Enhancement for the combination of paging adaptation and RACH adaptation

R2-2506097	Joint utilization of paging and RA resource adaptations 	Ericsson, Nokia, Nokia Shanghai Bell, InterDigital, NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1	Extend the paging message with a per-UE indicator that signals the availability of activated adaptive RA resources for the corresponding UE.

Proposal 2	Adopt the TP below in the running 38.331 CR.

[Samsung, Huawei, Apple, LG, Lenovo, Qualcomm, OPPO, CATT, Vivo, Fujitsu]: Do not think a real need of it. [Xiaomi]: Support the proposal since understand it does not impact other WGs. [Apple]: Understand paging adaptation is used when the load is low, then do not agree with the scenario. Also understand there may be cross-WG issues. Paging adaptation is handled by CU and RACH adaptation is handled by DU, then we may need some information exchange between CU and DU, impact to RAN3. [LG]: Understand the gain from the proposal is marginal since the responding UE is random. [Lenovo]: Lenovo submitted a contribution in the past to show the paging log information in big cities. Paging was very few in each DRX cycle. Do not see the need of this optimization. [Ericsson]: Agree that it cannot be accurate, but think to certain extension, it will be helpful for distribution. For inter-WG issues, we may rely on network implementation. [Vivo]: Network doesn’t know where the idle/inactive UEs locate.

We don’t consider any optimization for the combination of paging adaptation and RACH adaptation.

4. Additional RO configuration with CE exclusion

R2-2505789	Remaining issues for time domain adaptation of common signalling and channels	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: RAN2 to select between option 1b and 1c based on whether there is a need for the network to mix CE and non-CE RA in RACH-ConfigCommon. Consider TP in Annex if option 1b is agreed.

[Nokia]: Prefer option 1b. [Session chair]: All understand either option works and considering option1b can give some network flexibility, is option1b ok? Seems so.

Option 1b (a same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon.) is agreed.

TP in R2-2505789 is baseline for MAC update.

R2-2505099	Remaining issues on common signal and channel transmissions adaptation	LG Electronics Inc.	discussion	Rel-19

R2-2505170	Adaptation of Common signal channel transmissions	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505255	Adaptation of common signal channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505275	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505316	Remaining open issues on common channel adaptation	Xiaomi	discussion	Netw_Energy_NR_enh-Core

R2-2505340	Remaining issues of common signal adaptation	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505500	Remaining issues on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505508	Remaining issues on adaptation of common signal/channel	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505528	Discussion on adaptation of common signal and channel	NTT DOCOMO, INC.	discussion	Rel-19

R2-2505789	Remaining issues for time domain adaptation of common signalling and channels	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2505847	Discussion on remaining issues for RACH adaptation 	Qualcomm Incorporated	discussion

R2-2505991	Adaptation of common signal/channel transmissions	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506051	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506064	Discussion on adaptation of common signal/channel transmissions	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506091	Adaptation of common signals	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506095	Adaptation of common signal/channel transmissions for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2506097	Joint utilization of paging and RA resource adaptations 	Ericsson, Nokia, Nokia Shanghai Bell, InterDigital, NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

## 8.6	Mobility Enhancement Ph4

(NR_Mob_Ph4-Core; leading WG: RAN2; REL-19; WID: RP-242356)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.6.1	Organizational

Incoming LS, WI rapporteur inputs, CR rapporteur inputs (including post email discussion [POST130][110], [111], [120], summary of identified stage-3 open issues that need online discussion and rapporteur’s suggestions if needed).

WI is completed from RAN2 point of view.

R2-2505012	LS on frequency location of CSI-RS resources for CSI acquisition in LTM (R1-2504828; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN4	Cc:RAN2

Noted.

R2-2505065	Reply LS on security handling for inter-CU LTM in non-DC cases (S3-252398; contact: Huawei)	SA3	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN3	Cc:RAN2

Noted.

R2-2505453	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple Inc	CR	Rel-19	38.300	18.6.0	1011	-	B	NR_Mob_Ph4-Core

=> Revised in R2-2506195

R2-2506195	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple Inc	CR	Rel-19	38.300	18.6.0	1011	1	B	NR_Mob_Ph4-Core	R2-2505453

Revised in R2-2506415.

[Nokia, ZTE, Qualcomm]: Prefer removing “Beam characteristics could be different depending on whether they are configured for L3 or L1 mobility.” from NOTE 1. [Ericsson]: Ok to remove it, but in that case, the figure is only applicable to L3 measurement model. [Ericsson]: Understand, once the UE detects a cell, L1 beam(s) can be different (e.g. narrower/wider) than a L1 beam used for L3 measurement purpose. [Apple]: Want to have some offline to handle this issue.

Comeback:

R2-2506415	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple Inc	CR	Rel-19	38.300	18.6.0	1011	2	B	NR_Mob_Ph4-Core	R2-2505453

Endorsed and considered as baseline for further discussion.

[POST131][113][MOB] (Apple)

Scope: Update MOB 38.300 CR (including this meeting agreements also).

Intended outcome: 38.300 CR in R2-2506225 to be agreed.

Deadline: Short email discussion

R2-2505291	Introduction of NR mobility enhancements Phase 4 in TS 37.340	China Telecom	CR	Rel-19	37.340	18.6.0	0419	-	B	NR_Mob_Ph4-Core

Endorsed and considered as baseline for further discussion.

[POST131][114][MOB] (China Telecom)

Scope: Update MOB 37.340 CR (including this meeting agreements also).

Intended outcome: 37.340 CR in R2-2506226 to be agreed.

Deadline: Short email discussion

R2-2505164	Report of [POST130][120][MOB]	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: Define a per UE capability for the support of all the LTM events.

Agreed.

Proposal 2: No need to define a separate UE capability for the support of the following functions in L1 event triggered measurement reporting,

-	The new MR MAC CE.

-	The Truncated MR MAC CE.

-	The event triggered periodic MR MAC CE reporting

-	The reportOnLeave for LTM

-	The current beam reporting in the MR MAC CE

Agreed.

Proposal 3: Discuss whether to define a per UE capability for reporting the beam not satisfying the event condition in the MR MAC CE.

No UE capability for reporting the beam not satisfying the event condition in the MR MAC CE.

[Nokia]: Do not understand what additional UE capability means to support it. [Ericsson]: Even when network sets this configuration, the actual reporting depends on the availability of measurement. [Samsung, CATT]: Ok to not having this capability.

Proposal 4: When a UE indicates support for both conditional LTM and ltm-RACH-LessCG-r18, and at least one of cltm-EarlyTA-Indication-r19 and ue-TA-Measurement-r18, it implies that the UE supports RACH-less conditional LTM with a configured grant. This revises the previous agreement.

Agreed.

Proposal 5: For the new UE capability “cltm-ExecutionConditionL3-r19” and ”cltm-ExecutionConditionL1-r19”, Except for NTN bands, UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. For NTN case, the decision is up to NR NTN WI.

[Ericsson]: Prefer to have per UE capability since anyway same value should be set for all bands. [ZTE]: Per UE capability is ok, but we need differentiated bits between FR1 and FR2, probably between TDD and FDD. Also, think we need to define whether this capability needs to be set based on the serving cell or target cell. [MediaTek]: Sounds no real difference then, why not stick to the original proposal. [Samsung]: Wonder if cltm-ExecutionConditionL1-r19 means measurement related capability or not. Understand it’s not about measurement related one, then we don’t need per band differentiation. [Qualcomm]: Do not see the need of differentiation of FR1/FR2 and TDD/FDD in this release phase. [Ericsson]: Agree with Samsung and still think per UE capability should be enough.

[AT131][107][MOB] (CATT)

Scope: Discuss and decide the following questions.

- Difference between per UE capability and per band capability with a restriction to set same value for all bands?

- “cltm-ExecutionConditionL3-r19” and ”cltm-ExecutionConditionL1-r19” per UE capability or per band capability with the restriction?

- Do we need differentiation for intra-F C-LTM and inter-F C-LTM? Note LTM capability has different ones for intra-F and inter-F cases.

Intended outcome: Discussion summary in R2-2506217.

Deadline: Offline meeting room will be announced by CATT. Comeback either in Wednesday session or CB session. => Completed.

Proposal 6: Discuss whether to define a separate UE capability for CLTM fast recovery.

Treated based on company contribution.

Proposal 7: Discuss whether to define a per UE capability for inter-CU LTM recovery.

A per UE capability for inter-CU LTM recovery.

R2-2506217	Report of [AT131][107][MOB](CATT)	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1a: FRx or xDD differentiation is not needed for the UE capability“cltm-ExecutionConditionL3-r19”and “cltm-ExecutionConditionL1-r19”.

Proposal 1b: “cltm-ExecutionConditionL3-r19”and “cltm-ExecutionConditionL1-r19” are defined as per UE capabilities without FRx or xDD differentiation. Whether this capability is actually supported on given band depends on whether the L1/L3 measurement is supported on the corresponding band.

Proposal 2: No need to differentiate between intra-F CLTM and inter-F CLTM.

Proposal 1a, 1b and 2 are agreed.

R2-2505162	Draft 306 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.306	18.6.0	NR_Mob_Ph4-Core

Endorsed and considered as baseline for further discussion.

R2-2505163	Draft 331 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.331	18.6.0	NR_Mob_Ph4-Core

Endorsed and considered as baseline for further discussion.

[POST131][115][MOB] (CATT)

Scope: Update MOB UE capability CRs (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506227 and 38.306 CR in R2-2506228 to be endorsed.

Deadline: Short email discussion

R2-2505398	Discussion summary and list of MAC open issue for Mob Ph4	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Open issue MAC-20 (essential): How to design the MAC CE to activate/deactivate the SP CSI-RS resource for LTM CSI acquisition for candiate cells based on RAN1 LS

Proposal 1a: (12/14) Common SP CSI-RS Resource Set Activation/Deactivation MAC CE is used for both RRM measurement and early CSI acquisition purpose, as captured in the current MAC running CR.

Agreed.

Proposal 1b: RAN2 to discuss whether to ask RAN1 for confirmation:

-	whether CSI-RS resource set configured for early CSI acquisition can contain CSI-RSs from multiple candidates. If so, the UE is only required to measure the RSs of target cell and ignore other RSs.

-	whether the CSI-RS resources related to one LTM-CSI-ResourceConfigId can be applied to both RRM and CSI acquisition.

[Ericsson]: Understand there is no need to ask RAN1. Network configures CSI-RS resource set configuration in correct way anyway. [ZTE]: Want to support a case that “CSI-RS resource set configured for early CSI acquisition can contain CSI-RSs from multiple candidates. If so, the UE is only required to measure the RSs of target cell and ignore other RSs.” [LG]: Why not allow the UE performing measurement on other candidates than target cell after the reception of LTM CSC if capable? We do not need any restriction. [Samsung]: Agree with LG. [Vivo]: Even though UE performs measurement on other candidates for early CSI acquisition, it is not useful since early CSI acquisition is for one-shot for the target.

If a CSI-RS resource set configured for early CSI acquisition contains CSI-RSs from multiple candidates, the UE is expected to measure the RSs of target cell after reception of LTM CSC.

Proposal 2: (14/14) SP CSI-RS Resource Set Activation/Deactivation MAC CE could be also used to activate and deactivate the configured semi-persistent CSI-IM resource sets of a LTM candidate cell, as captured in the current MAC running CR.

Agreed.

[Ericsson]: RAN1 decided that IM resource and NZP CSI resource cannot be configured (in a given CSI-RS resource set) together. [Vivo]: Good to check with own RAN1 delegate if it is true.

Comeback:

[Vivo]: After checking further, IM resource and NZP CSI resource cannot be configured for a CSI-RS resource configuration id. [Samsung]: Do not understand it. In legacy, it was already allowed. [Apple]: Understand Samsung’s concern, but it was RAN1 decision. [CATT]: Prefer to re-design the MAC CE to enable to include two CSI-RS resource configuration id. [Ericsson]: Do not see a need. Nothing is broken. Two MAC CEs can be sent.

IM resource and NZP CSI resource cannot be configured for a CSI-RS resource configuration id.

Comeback session:

[Vivo]: Based on further offline discussion, all companies are ok with change of MAC CE format to handle both a NZP CSI resource configuration and IM resource configuration.

SP-CSI MAC CE includes up to two CSI-RS resource configuration ids.

Detailed change is left to MAC CR rapporteur.

Open issue MAC-21: Whether indicate the selected RS index to the lower layer for RACH-less CLTM.

Proposal 3: (14/14) MAC indicates the selected RS index to the lower layer for RACH-less CLTM, as captured in the current MAC running CR.

Agreed.

Open issue MAC-22: whether multiple MR should be triggered and whether only one or multiple L1 measurement report MAC CE should be generated in case multiple beams associated with an LTM report configuration satisfies triggering condition for TTT.

Proposal 4: (14/14) RAN2 confirms when multiple MR(s) associated with an LTM report configuration is triggered, only one L1 measurement report MAC CE will be generated, as captured in the current MAC running CR.

Agreed.

Open issue MAC-23: whether support CG resource associated with CSI-RS.

Proposal 5: (14/14) RAN2 assumes CG resource associated with CSI-RS is not supported.

Agreed.

Other open issues:

Proposal 6: RAN2 to consider the above open issues related to MAC for mobility enhancement in section 2.2. Rapp: MAC-27 to MAC-31 are all marked as “Not Essential” as they should not impact the completion of this WI, and are proposed by single company.

Noted.

R2-2505397	Introduction of NR mobility enhancements Phase 4 in MAC	vivo (Rapporteur)	CR	Rel-19	38.321	18.6.0	2098	-	B	NR_Mob_Ph4-Core

Endorsed and considered as baseline for further discussion.

[POST131][116][MOB] (Vivo)

Scope: Update MOB 38.321 CR (including this meeting agreements also).

Intended outcome: 38.321 CR in R2-2506229 to be agreed.

Deadline: Short email discussion

R2-2505815	Introduction of RRC changes for mobility enhancements phase 4	Ericsson	CR	Rel-19	38.331	18.6.0	5443	-	B	NR_Mob_Ph4-Core

Endorsed and considered as baseline for further discussion.

[POST131][117][MOB] (Ericsson)

Scope: Update MOB 38.331 CR (including this meeting agreements also).

Intended outcome: 38.331 CR in R2-2506230 to be agreed.

Deadline: Short email discussion

R2-2505816	List of open issues for mobility ph4	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Noted.

[AT131][102][MOB] (Ericsson)

Scope: Discuss and conclude RRC-2, RRC-3, RRC-4, RRC-5, RRC-6, RRC-7, RRC-8 and RRC-10 (with the consideration of RRC CR rapporteur’s inputs). Also including P2a and P2b in R2-2505165 and P3 in R2-2505893.

Intended outcome: Discussion summary in R2-2506212.

Deadline: 1000 – 1100 (8/27 Wednesday, main session room) => Completed.

[AT131][103][MOB] (Vivo)

Scope: Discuss and conclude MAC-26, MAC-27, MAC-28, MAC-29, MAC-30, MAC-31, and P2 in R2-2505926.

Intended outcome: Discussion summary in R2-2506213.

Deadline: 1000 – 1100 (8/26 Tuesday, main session room) => Completed.

R2-2506213	Discussion report on [AT131][103][MOB] MAC open issues (vivo)	Vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal: RAN2 to agree the below offline agreement:

Offline agreement for Event triggered L1 measurement:

- No impact on event triggered L1 MR procedure when beam failure is detected.

- RAN2 will keep the current definition of Type II in MAC, i.e. any beam.

Offline agreement for conditional intra-CU LTM:

- During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, UE always fall back to RACH-based LTM regardless whether UE based TA is valid or not.

- Stop the running ltm-Candidate-TimeAlignmentTimer for the corresponding candidate cell and release the stored TA value for the candidate cell if the corresponding CLTM candidate configuration is released.

- If RACH-less CLTM fallback to RACH-based CLTM due to PTAG expiration, RAN2 assumes it’s up to UE implementation to obtain the MAC PDU for 1st UL transmission. Captured as a Note in the MAC spec.

- For MAC-31, wait for RAN1. it is not essential issue, and does not impact the WI completion.

All proposals above are agreed.

[Lenovo]: If there is 2TA in a target, when fall-back RACH-based LTM is used, which TA is used? [Samsung]: It is not for Rel-19 LTM issue. When RACH-based LTM is used, 2TA is not considered.

R2-2506212	List of open issues for mobility phase4	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: In case of MCG failure recovery, network reconfigure the UE in case it wishes the UE to restart the evaluation of the LTM cell switch execution condtion.

Proposal 2: UE understands whether to send an MR or trigger a conditional LTM cell switch procedure based on the presence of the reporting content configuration. The related ASN.1 field is changed from mandatory to optional.

Proposal 3: The UE shall not perform secondary key change when intra-MN LTM is performed (take TP in R2-2505583 as baseline).

Proposal 4: The reference configuration can be signalled to UE via the ltm-Config and ltm-ConfigNRDC (current restrictions in RRC are removed).

Proposal 5: RRC-9, RRC-10, RRC-11, RRC-12 is handled during the post email discussion.

Proposal 6: RRC-5, RRC-7, and RRC-8 are not pursued.

All proposals above (proposal 1,2,3,4,5, and 6) are agreed.

[Ericsson]: If companies that raised RRC-10/11/12 provides a draft TP to RRC CR rapporteur, it will be good.

### 8.6.2	Inter-CU LTM

Remaining essential open issues including handling of target cell’s SP CSI-RS resources after cell switch, stage-3 identified open issues if needed, etc.

1. Handling of target cell’s SP CSI-RS resource after cell switch

R2-2505277	Remaining issues of inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: (MAC-25) SP CSI-RS resource for target cell is deactivated after cell switch.

R2-2505117	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: (MAC-25) The SP CSI-RS resource of the target cell remains activated after cell switch if the SP CSI-RS was activated before cell switch.

SP CSI-RS resource for target cell is deactivated after CSI reporting in the first UL transmission. How to capture it in MAC is up to MAC CR rapporteur.

R2-2505717	Final View on Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: For L1-RSRP measurement, the UE deactivates SP CSI-RS resources of all candidate cells, including the target cell, after the LTM cell switch.

Proposal 3: For early CSI acquisition, the UE deactivates SP CSI-RS resources of all candidate cells except the target cell, after the LTM cell switch.

Noted.

2. Inter-CU LTM Recovery to the target cell

R2-2505277	Remaining issues of inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: For inter-CU LTM failure recovery, even if the selected cell is the target cell, there is no security issue, and no specification change is needed in RRC CR for the case.

R2-2505893	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: If inter-CU LTM cell switch triggered by the enhanced LTM CSC MAC CE fails, LTM recovery is configured, and the selected cell is the same as the target cell, the UE performs RRC re-establishment.

[Huawei]: Think RRC reconfiguration complete message may be different at recovery. [Qualcomm]: Do not think RRC reconfiguration complete message can be different. [IDC]: Can other RRC message be sent even before RRC reconfiguration complete at recovery? [Nokia]: Guess not. [ZTE, CATT]: Support Xiaomi proposal and understand similar situation happens at CHO failure and we allow recovery for the same serving cell. [IDC]: After the second thought, support Xiaomi proposal.

For inter-CU LTM failure recovery, even if the selected cell is the target cell, there is no security issue, and no specification change is needed in RRC CR for the case.

3. Inter-CU LTM Recovery after L3 HO/CHO failure

R2-2505399	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 6: After L3 HO or CHO failure, the UE cannot perform inter-CU LTM fast recovery.

[Ericsson]: Assume there is no spec impact since it’s same as Rel-18 LTM. [Vivo]: We may have no spec impact, but first we need a decision on the issue.

After L3 HO or CHO failure, the UE cannot perform inter-CU LTM fast recovery.

4. Clarification for ltm-config and ltm-configNRDC

R2-2506139	Discussion on inter-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: RAN2 to confirm following principles for LTM:

	MCG LTM configuration is always provided within ltm-Config in an MN RRC message;

	Intra-SN or inter-SN SCG LTM with MCG configuration is provided within ltm-ConfigNRDC in an MN RRC message;

	Intra-SN SCG LTM without MCG configuration is provided within ltm-Config in an SN RRC message;

	For one UE, if inter-SN and intra-SN candidate cells are configured simultaneously, SCG LTM candidate configurations can only be provided within the ltm-ConfigNR-DC in MN RRC message(s).

Agreed.

Proposal 3: RAN2 to discuss where and how to capture the principles above, i.e. applicable scenarios of ltm-Config-r18 and ltm-ConfigNRDC-r19, in the current spec:

	Option 1: capture them in the RRC spec (see the TP to TS 38.331 above);

	Option 2: capture them in the stage-2 spec (see the TP to TS 37.340 above).

[Huawei]: Suggest not to use stage-3 IE names in stage-2 spec.

RRC CR will capture the principles.

5. UE capability for inter-CU LTM recovery

R2-2505158	UE capability for fast recovery for inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: New UE capability is defined to indicate the UE support for Rel-19 fast recovery procedure.

Noted.

R2-2505158	UE capability for fast recovery for inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505165	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505277	Remaining issues of inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505311	Remaining issues on inter-CU LTM	ETRI	discussion	Rel-19

R2-2505399	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505455	Remaining issues of Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505517	Discussion on open issues for inter-CU LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505546	Discussion on inter-CU LTM	Qualcomm Incorporated	discussion

R2-2505583	Remaining issues on inter-CU LTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505657	LTM CG Resource consumption for the target cells	Sony	discussion	Rel-19	NR_Mob_Ph4

R2-2505730	Remaining issues of Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2505786	Discussion on open issues of inter-CU LTM	Ofinno	discussion	Rel-19

R2-2505869	How to include the NCC value in the Enhanced Cell Switch Command MAC CE	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2505870	On remaining open issues for Inter-CU LTM and DC-LTM	Nokia	discussion

R2-2505893	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506022	Remaining issues and solutions on inter-CU LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506113	Discussion on SP CSI-RS for target cell	NTT DOCOMO, INC.	discussion	Rel-19

R2-2506139	Discussion on inter-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506158	Discussion on inter-CU LTM	DENSO CORPORATION	discussion	Rel-19	NR_Mob_Ph4-Core

### 8.6.3	L1 event triggered measurement reporting

Remaining essential open issues includingpost email discussion [POST130][117] and conclusion on the coexistence with mTRP, stage-3 identified open issues if needed, etc.

1. MAC-8

R2-2505719	Report from [POST130][117][MOB] (Nokia)	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: When mTRP is configured in the serving cell the UE uses the best beam (in terms of RSRP) of the two “current beams” for LTM event evaluation.

[MediaTek]: Ok if it does not bring additional UE requirements to compare two beams. [Samsung]: Understand the UE needs to measure both beams, which may provide more complexities. [Nokia]: During email discussion, think no company really expects additional UE procedure/requirements to derive the best beam. [Samsung]: Is proposal 1 applicable to single DCI based mTRP or multi-DCI based mTRP? [Apple]: Prefer leaving it to UE implementation considering all additional complexities.

When mTRP is configured in the serving cell the UE uses the best beam of the two “current beams” for LTM event evaluation, and it’s up to UE implementation to how to derive the best beam.

Proposal 2: RAN2 to discuss if the NW needs to know the exact TRP and thus MR MAC CE shall indicate the TCI state associated with the reported RSRP value for the serving/current beam.

No additional indication in MR MAC CE is needed.

Proposal 3: The coexistence between event-triggered L1 measurement reporting and mTRP for the source cell is supported in Release 19 LTM.

[Samsung]: Understand proposal 3 is applicable to C-LTM also if P3 is agreed. [Nokia]: Do not see any problem to apply it to C-LTM also.

The coexistence between event-triggered L1 measurement reporting and mTRP for the source cell is supported in Release 19 LTM.

[AT131][104][MOB] (Nokia)

Scope: Prepare an agreeable TP on mTRP.

Intended outcome: TP in R2-2506214.

Deadline: Comeback in 8/27 Wednesday session => Completed.

R2-2506214	Text Proposal for multi-TRP in Rel-19 LTM		Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

“best beam of the two current beams” is changed to “best beam of the current beams”

TP for stage-2 and MAC are baseline.

[MediaTek]: We may need to send a LS to RAN1 to inform RAN2 agreement. [ZTE, Apple]: Do not prefer sending a LS to RAN1. RAN2 agreement can be directly considered in RAN1 if need. [Apple]: Want to reword “best beam of the two current beams” to “best beam of the current beams”

R2-2505159	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: Option b) (up to the UE select the serving beams for L1 event evaluation) is adopted for event evaluation for LTM, if the coexistence with mTRP is agreed.

Noted.

2. MAC-24: Truncated MR MAC CE

R2-2506149	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion

Proposal 4: RAN2 to discuss below two options for reporting current beam measurements in LTM2

a.	UE reports the current beam measurements irrespective of reportCurrentBeam.

b.	Network always configures reportCurrentBeam in the LTM-EventTriggeredReportContent.

[ZTE]: Prefer option b. Simpler. [Nokia]: Prefer option a. [Session chair]: Where is current beam measured result located in MR for LTM2? [Vivo]: To the current running CR, it is located in the end as other LTM events. [LG]: Prefer option b with the consideration of where current beam measured result is located in MR for LTM2. [Apple, Ericsson]: Share the same understanding with Vivo. Understand either option works, but we need to consider which option brings less specification impact. In that point of view, support option b.

For LTM2, network always configures reportCurrentBeam in the LTM-EventTriggeredReportContent.

R2-2505518	Open issues for L1 event triggered measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5: Confirm the following working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

R2-2506140	Discussion on L1 event triggered measurement reporting	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: (MAC-24) RAN2 to confirm the working assumption with some modification: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, for Event LTM 3/4/5, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. While, for Event LTM2, minimum size of truncated MR MAC CE is the current serving beam. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

Serving beam measured result is only included at the end of a (truncated) MR MAC CE for LTM2.

[Nokia, Ofinno, Lenovo]: Prefer, if the grant is received only for triggered beam even when the network configures current serving beam reporting, UE still is able to send truncated MR MAC CE including only triggered beam. [Session chair]: Check how many companies are ok with previous working assumption? Many.

Confirm the following working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

3. UE capability for reporting type 11 beam(s)

R2-2505159	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: A per UE capability is introduced to report the beam not satisfying the event condition in the MR MAC CE.

Noted.

R2-2505117	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505135	Remaining issues of L1 event triggered measurement reporting	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505159	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505166	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505180	Discussion on L1 event triggered measurement reporting	Transsion Holdings	discussion	Rel-19

R2-2505348	Discussions on L1 event triggered measurement reporting	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505400	Discussion on LTM measurement event evaluation and reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505456	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505482	Remaining issues of LTM measurement	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505518	Open issues for L1 event triggered measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505544	Discussion on L1 event-triggered measurement reporting	Qualcomm Incorporated	discussion

R2-2505548	Discussion on L1 event triggered measurement reporting for LTM	KDDI Corporation	discussion	Rel-19

R2-2505717	Final View on Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505719	Report from [POST130][117][MOB] (Nokia)	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505731	Remaining issues of L1 event triggered measurement reporting 	Rakuten Mobile, Inc	discussion	Rel-19

R2-2505764	Discussion on triggering of MR MAC CE for leaving beam 	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

R2-2505787	Remaining issues for L1 event triggered measurement report	Ofinno	discussion	Rel-19

R2-2505868	Handling of SP CSI-RS resources of target cell after LTM cell switch	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2505926	Leftover for L1 measurement and report	Lenovo	discussion	NR_Mob_Ph4-Core

R2-2505966	Remaining issues of L1 event triggered measurement reporting	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505974	Remaining issues of event-triggered L1 measurement reporting for LTM 	Kyocera 	discussion	Rel-19

R2-2506023	Discussion on issues for supporting L1 event triggered measurement reporting	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506065	Discussion on measurement event evaluation and report	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506140	Discussion on L1 event triggered measurement reporting	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506149	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion

### 8.6.4	Conditional intra-CU LTM

Remaining essential open issues including stage-3 identified open issues if needed, etc.

1. How to handle if the number of stored TAs exceeds the UE capability?

R2-2505278	Remaining issues of conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7: When the number of CLTM TAs that the UE stores is up to cltm-EarlyTA-Indication-r19, to adding a new CLTM TA to be maintained by the UE, the following options can be considered.

•	Option 1: (Network-based method) Before sending the LTM Candidate Timing Advance Command to add a new CLTM TA, the network can remove one CLTM TA by setting the TAC field to “FFF” in LTM Candidate Timing Advance Command MAC CE.

•	Option 2: (UE-based method) Upon receiving the LTM Candidate Timing Advance Command MAC CE for adding a new CLTM TA, the UE shall remove the oldest CLTM TA and stop associated CLTM TAT.

[Ericsson]: It may not be easy network to track TAs all the time. If provided TA is beyond UE capability, it’s ok to leave it to UE implementation to decide which one is released. [Lenovo, NEC, Huawei, MediaTek]: Agree with Ericsson. And simpler. [ZTE, Vivo]: Prefer option 1. Note in RAN3, it was not decided whether the source forwards all remaining TAs for candidates to the target once C-LTM is completed.

For C-LTM, if the UE receives more TAs beyond its capability, it’s up to UE implementation to decide which one is released.

2. RRC-1

R2-2505360	Issue in Fast LTM Recovery after Conditional LTM Failure	Fujitsu, NTT DOCOMO, InterDigital	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: RAN2 to address the issue about the SN gap in fast LTM recovery after CLTM failure.

Proposal 2: RAN2 to choose the solution from the following three options.

1)	Place a bit in PDCP header of RRCReconfigurationComplete message which indicates whether the ongoing LTM is CLTM or fast LTM recovery.

2)	Attach a new MAC CE to RRCReconfigurationComplete message which indicates whether the ongoing LTM is CLTM or fast LTM recovery.

3)	Define a new RRC parameter which configures whether the UE performs fast LTM recovery after CLTM failure.

R2-2505167	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: It’s up to NW implementation to avoid the latency caused by the PDCP SN gap due to CLTM fast recovery.

[OPPO, Samsung, Ericsson, ZTE, Lenovo, Apple, Nokia, LG]: Support CATT proposal, i.e. leave it to network implementation to handle this issue. [NTTDCM]: Not sure if network can handle it well. [NTTDCM]: Ok with relying on network implementation, but prefer reflecting it as note in stage 2.

It’s up to NW implementation to avoid the latency caused by the PDCP SN gap due to CLTM fast recovery.

It will be added as a note into stage-2 spec. How to capture it is left to stage-2 spec CR rapporteur.

3. MAC-14

R2-2505483	Remaining issues of conditional LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: If UE receives the LTM CS command triggered by NW, UE shall follow the R18 behaviour to initiate LTM CS in the target cell regardless of whether the UE has the available C-TA value.

R2-2505312	Remaining issues on conditional LTM	ETRI	discussion	Rel-19

Proposal 3: When the UE receives an (Enhanced) LTM cell switch MAC CE with value “FFF” for the TA value requesting a cell switch to a CLTM candidate cell, the UE performs a RACH-less LTM cell switch with the indicated cell if it has a valid TA (i.e., CLTM TAT timer for the target cell is still running or the UE is configured with UE-based TA measurement). Otherwise, it performs a RACH-based LTM cell switch procedure.

[Huawei]: Do not understand why not network just sends the corresponding TA? Why in that case, the network will set “FFF”? [ZTE]: The concerned scenario is not for initial LTM case, it is for LTM after C-LTM switch. [Samsung, MediaTek, Nokia]: After C-LTM switch happened, if the new target decides for LTM, since the target doesn’t know which TA is valid for how long, target just set FFF, but if the UE maintains a valid TA for the candidate, it can be simply reused. See some benefit. [Qualcomm, Huawei, CATT, OPPO, LG, Ericsson]: Support Apple proposal, it just follows Rel-18 principle. [Ericsson]: Understand RAN3 will do their job, i.e. source forwards remaining TAs to the target upon C-LTM completion. Then we don’t need further optimization. [ZTE]: Ok with Apple proposal, but want to send a LS to RAN3 to ask their job. [Apple]: Prefer not to send a LS to RAN3, instead, we can capture RAN2 agreement/understanding in the session minutes.

If UE receives the LTM CS command triggered by NW, UE shall follow the R18 behaviour to initiate LTM CS in the target cell regardless of whether the UE has the available C-TA value.

RAN2 understands RAN3 handles the remaining TA information exchange from the source to the target.

4. UE capability for inter-CU LTM recovery

R2-2505278	Remaining issues of conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 9: No need to define a separate UE capability for CLTM fast recovery (i.e., reuse the UE capability for Rel-18 intra-CU LTM fast recovery).

Agreed.

5. Co-existence with HO/CHO/DAPS

R2-2505945	Discussion on open issue of conditional LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4: CLTM can coexist with network triggered L3 HO, CHO and network triggered PScell change, while the coexistence of CLTM and DAPS HO is not supported.

Agreed.

R2-2505894	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4: C-LTM supports all the MCG LTM DC scenarios that are supported in Rel-18. No impact to TS 38.331 or TS 38.321 is expected.

Noted.

[NEC, Apple, Samsung]: Think it is not aligned with WID scopes that RAN decided.

R2-2505098	Discussion on Conditional Intra CU LTM	Lekha Wireless Solutions	discussion	Rel-19

R2-2505104	Discussion on remaining open issues of conditional intra-CU LTM	Transsion Holdings	discussion

R2-2505160	Remaining issues in conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505167	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505278	Remaining issues of conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505312	Remaining issues on conditional LTM	ETRI	discussion	Rel-19

R2-2505349	Discussion on MAC open issues for Conditional intra-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505360	Issue in Fast LTM Recovery after Conditional LTM Failure	Fujitsu, NTT DOCOMO, InterDigital	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505401	Discussion on conditional LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505483	Remaining issues of conditional LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505519	Open issues for conditional LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505545	Discussion on conditional intra-CU LTM	Qualcomm Incorporated	discussion

R2-2505584	Remaining issues on CLTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505620	Remaining Open issues for CLTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505641	Discussion on early TA for conditional LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2505696	Remaining issues for CLTM	Lenovo	discussion	Rel-19

R2-2505729	Remaining issues of Conditional LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2505733	Introduction of Hybrid CHO/LTM Handover Mechanism	Jio Platforms	discussion	Rel-19

R2-2505788	Discussion on remaining issues of CLTM	Ofinno	discussion	Rel-19

R2-2505867	L2 Reset Coordination for Conditional LTM	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2505894	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2505945	Discussion on open issue of conditional LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506010	Remaining issues of Conditional intra-CU LTM 	Kyocera 	discussion

R2-2506024	Discussion on issues for supporting conditional LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506032	Discussion on remaining issue for C-LTM	ASUSTeK	discussion	Rel-19	38.321	NR_Mob_Ph4-Core

R2-2506066	Discussion on conditional LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2506132	Considerations on conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

R2-2506141	Discussion on conditional intra-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core