3GPP TSG-RAN WG2 #131	R2-2506218

Bangalore, India, August 25th– 29th, 2025

Agenda Item:	8.5.2

Source:	Ericsson

Title:	Discussion on on-demand SSB for NES

Document for:	Discussion, Decision

# 1	Introduction

3. Clarification on SIB1 update upon reception of short message

R2-2505339	Remaining issues of On-demand SIB1	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: RAN2 to discuss how to understand the previous agreement ‘Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’:

-	Understanding 1: The UE assumes the NES cell will always transmits CD-SSB after sending SI change notification, the UE acquires SIB1 with the Coreset#0 and Searchspace#0 parameters in MIB as legacy;

-	Understanding 2: It is up to the NES cell to transmits CD-SSB or NCD-SSB after sending SI change notification. If the NES cell transmits CD-SSB, the UE acquires SIB1 with the Coreset#0 and Searchspace#0 parameters in MIB as legacy; and if the NES cell transmits NCD-SSB,

•	2a: the UE assumes the Coreset#0 and Searchspace#0 parameters in the stored OD-SIB1 configuration are still valid, and acquires the updated SIB1 with the stored Coreset#0 and Searchspace#0 parameters from the start of the of the next modification period;

•	2b: the UE assumes the Coreset#0 and Searchspace#0 parameters in the stored OD-SIB1 configuration are no longer valid, and performs cell reselection to other cells;

•	2c: the UE assumes the stored OD-SIB1 request configuration is still valid, and performs OD-SIB1 request as the current procedure in the running CR;

R2-2505169	Consideration on on-demand SIB1	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from network directly without sending OD-SIB1 request in next MP, i.e., the network will switch to normal mode with legacy K_SSB value and transmit the updated SIB1 in next MP.

Proposal 3a: In emergency cases, it is up to network implementation to broadcast the SIB1 periodically based on the valid OD-SIB1 request configuration stored by UE, no spec impact will be introduced.

Proposal 3b: Upon receiving the etwsAndCmasIndication in a Short Message on the NES cell, the UE shall immediately re-acquire SIB1 based on the CORESET information contained in the OD-SIB1 request information, without sending an OD-SIB1 request. i.e., the UE will follow the legacy procedure.

[Samsung]: Agree with proposal 2, and think we need some change accordingly. [Ericsson]: Do not agree with proposal 2. [Session chair]: Based on the discussion, let’s list possible options.

- Option1: Upon sending SI change notification, NW configure Kssb < 24 or <12 and the UE (including legacy UEs) receives the updated SIB (legacy way)

- Option2a: Upon sending SI change notification, NW may configure/keep Kssb >24 or > 12, and the NES UE performs OD-SIB1 request procedure to acquire the updated SIB

- Option2b: Upon sending SI change notification, NW may configure/keep Kssb>24 or >12, but the NES UE does not perform OD-SIB1 request procedure, and with the additional UE behaviour change, the NES UE receives the updated SIB (legacy UE cannot receive it)

[Session chair]: Seems many companies prefer NW may configure any Kssb upon SI change notification, but NES UE still acquires the updated SIB1 without performing OD-SIB1 REQ procedure (option1 or option2b). Understand option2a is not aligned with RAN2 previous agreement.

NW may configure any Kssb upon SI change notification (including ETWS/CMAS), but NES UE still acquires the updated SIB1 without performing OD-SIB1 REQ procedure.

RRC CR rapporteur will provide the TP until Wednesday session.

# 2	TP

**********************************************start TP*****************************************************

##### 5.2.2.3.1	Acquisition of MIB and SIB1

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2>	if the UE is unable to acquire the MIB;

3>	perform the actions as specified in clause 5.2.2.5;

2>	else:

3>	perform the actions specified in clause 5.2.2.4.1.

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period; or

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1, and, the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, si-BroadcastStatus for the required SIB(s) or posSI-BroadcastStatus for the required posSIB(s) is set to notBroadcasting in acquired SIB1 in current modification period; or

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	if the UE is unable to acquire the SIB1:

4>	perform the actions as specified in clause 5.2.2.5;

3>	else:

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2.

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	if the UE has a stored valid version of od-sib1-Config for this cell as specified in clause 5.2.2.4.2x:

4>	if the SIB1 acquisition is upon receiving an indication that the system information has changed or upon receiving a PWS notification:

5>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

4>	else:

5>	perform the actions as specified in clause 5.2.2.3.3x;

4>	else:

5>	perform the actions as specified in clause 5.2.2.5.

NOTE 1:	The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e., the broadcast and unicast/MBS multicast beams are quasi co-located. The UE in RRC_INACTIVE state while T319a is running, is only required to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

NOTE 2:	UE in RRC_INACTIVE that does not support inactiveStateNTN-r17 enters RRC_IDLE upon cell reselection between TN cell and NTN cell, and initiates the NAS signalling connection recovery (see TS 24.501 [23]).

NOTE x:	It is up to UE 's implementation to check if SIB1 is currently being broadcasted for a cell before triggering SIB1 request procedure of that cell.

*********************************************end TP*******************************************************