3GPP TSG-RAN2 Meeting #131-RAN2#131	R2-2506282

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 5146 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UTC time offset in IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | From R17 IoT NTN, TLE-format ephemeris parameters have been introduced as neighbor satellite assistance information in SIB32(-NB) . TLE-format ephemeris data can provide mean values of the satellite orbital parameters and are critical input needed by the SGP4 model (Simplified General Perturbations Model 4). The combination of TLE-format ephemeris data and SGP4 model can make accurate prediction on satellite trajectory. Furthermore, in order to avoid too many bits for indicating the epoch time of TLE-format ephemeris data (the epoch time within legacy TLE is encoded as the last two digits of the Epoch year, and the Epoch day down to a granularity of ~1 microsecond and would need 44 bits), a new week-based UTC time offset has been introduced with less signaling overhead (21-bits). That is, in SIB32(-NB), the epochStar IE defines the epoch time as a time offset between the beginning of the current week (Monday 00:00:00 UTC) and the actual epoch time. Moreover, with reference to the definition of the epoch time for TLE-format ephemeris data, the service stop time of the serving satellite (t-Service-r17 in SIB3(-NB)), the service start time of neighbor satellites for both discontinuous coverage case (t-ServiceStart-r17 in SIB32(-NB)) and continuous coverage case (t-ServiceStartNeigh-r18 in SIB33(-NB)), and the time threshold for CondEvent T1 (t1-Threshold-r18 in ReportConfigEUTRA) are also defined as such week-based UTC time offset. However, for the cases where the UTC time offset is obtained through broadcast messages, due to that the SI window may accoss the bounary of week, the UE and eNB may have different assumptions on “current week”. Similarly, the different UEs which receive the same SI message carrying this time offset information at the different location within the SI window may also have different assumptions on “current week”, e.g., they may take the two consecutive weeks before and after as their respective current weeks. This issue can be explained by the following diagram. In order to aovid the above issue, it’s suggested to clarify that, for the case that the time offset information is received via system information block, the current week corresponds to the week where the SI window ends. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To clarifiy the epochStar in TLE-EphemerisParameters that, the current week refers to the week in which the SI window for transmitting the SI message carrying the epochStar ends. - To clarifiy the TimeOffsetUTC that, for the case that this time offset is received via system information, the week refers to the week in which the SI window for transmitting the SI message carrying this time offset ends. Impact Analysis Impacted functionality: The understanding for the time information that is defined as week-based UTC time offset. Inter-operability: If UE implements this change but NW does not, or vice versa, the UE and NW may have different understanding for the time information that is defined as week-based UTC time offset. Implementation of this CR from Rel-17 will not cause interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The time information that is defined as week-based UTC time offset (e.g., epochStar in SIB32(-NB)) cannot be correctly understood. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4, 6.3.6, Annex G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

6.3.4	Mobility control information elements

<Skip unrelated parts>

–	TLE-EphemerisParameters

The IE TLE-EphemerisParameters provides satellite ephemeris parameters based on the CCSDS orbit mean-elements message (OMM) format, see [111]. The reference frame for SGP4 propagator and SGP4 parameter generation is TEME as per the NORAD Space Track standard.

TLE-EphemerisParameters information element

-- ASN1START

TLE-EphemerisParameters-r17 ::= SEQUENCE {

inclination-r17					INTEGER (0..2097151),

argumentPerigee-r17				INTEGER (0..4194303),

rightAscension-r17					INTEGER (0..4194303),

meanAnomaly-r17					INTEGER (0..4194303),

eccentricity-r17					INTEGER (0..16777215),

meanMotion-r17						INTEGER (0..17179869183),

bStarDecimal-r17					INTEGER (-99999..99999),

bStarExponent-r17					INTEGER (-9..9),

epochStar-r17						INTEGER (-1048575..1048575)

}

-- ASN1STOP

TLE-EphemerisParameters field descriptions

argumentPerigee Argument of perigee, see [111] Table 4-3: OMM Data. Unit in degree. Step of 360 / 4194303 degree. Actual value = field value * (360 / 4194303).

bStarDecimal Decimal part of B*, see [111] Table 4-3: OMM Data. Unit in inverse Earth radii. Step of 0.00001 . Actual value = field value * 0.00001.

bStarExponent Exponent part of B*, see [111] Table 4-3: OMM Data.

eccentricity Eccentricity, see [111] Table 4-3: OMM Data. Step of 0.9999999 / 16777215. Actual value = field value * (0.9999999 / 16777215).

epochStar Time offset to the beginning of the current week (Monday 00:00:00 UTC) of the Epoch. Unit in second. The current week refers to the week in which the SI window for transmitting the SI message carrying the epochStar ends.

inclination Inclination, see [111] Table 4-3: OMM Data. Unit in degree. Step of 180 / 2097151 degree. Actual value = field value * (180 / 2097151).

meanAnomaly Mean anomaly at epoch time, see [111] Table 4-3: OMM Data. Unit in degree. Step of 360 / 4194303 degree. Actual value = field value * (360 / 4194303).

meanMotion Mean motion at epoch time, see [111] Table 4-3: OMM Data]. Unit in revolution/day. Step of 99.99999999 / 17179869183 rev/day. Actual value = field value * (99.99999999 / 17179869183).

rightAscension Right ascension of ascending node, see [111] Table 4-3: OMM Data. Unit in degree. Step of 360 / 4194303 degree. Actual value = field value * (360 / 4194303).

End of the change

Start of the change

6.3.6	Other information elements

<Skip unrelated parts>

#### –	TimeOffsetUTC

The IE TimeOffsetUTC provides the time offset to the beginning of week (Monday 00:00:00 UTC). Units in seconds. If this time offset is received via system information, the week refers to the week in which the SI window for transmitting the SI message carrying this time offset ends.

TimeOffsetUTC information element

-- ASN1START

TimeOffsetUTC-r17 ::=		INTEGER (0..1048575)

-- ASN1STOP

End of the change

Start of the change

Annex G (normative):	List of CRs Containing Early Implementable Features and Corrections

This annex lists the Change Requests (CRs) whose changes may be implemented by a UE of an earlier release than which the CR was approved in (i.e. CRs that contain on their coversheets the sentence "Implementation of this CR from Rel-N will not cause interoperability issues").

Table G-1: List of CRs Containing Early Implementable Features and Corrections

| TDoc Number (RP-xxxxxx): CR Title | CR Number(s) | CR Revision Number(s) | Earliest Implementable Release | Additional Information |
| --- | --- | --- | --- | --- |
| RP-181233: Successful acknowledgement of RRCConnectionRelease for BL and CE UE | 3324 | 1 | Release 13 | RRCConnectionRelease message, for which the poll bit is not set, can be considered succesfully acknowledged when UE has sent HARQ ACK feedback. |
| RP-182674: CR for T312 on LTE HetNet mobility | 3506 | 5 | Release 12 | Remove T312 in leaving condition for event trigger. |
| RP-182671: Corrections on paging monitoring and SI acquisition in RRC_CONNECTED for BL UEs and UEs in CE | 3647 | 2 | Release 13 |  |
| RP-190548: Update description of ack-NACK-NumRepetitions | 3899 | 2 | Release 13 |  |
| RP-190548: Corrections of NB-IoT Access Barring | 3900 | 2 | Release 13 |  |
| RP-191382: SI update notification and access barring in NB-IoT | 4020 | 2 | Release 13 |  |
| RP-192195 : Correction on handling of SCell(s) during Make Before Break handover | 3986 | 3 | Release 14 |  |
| RP-192940: Stop using redirectedCarrierOffsetDedicated after reselection to another frequency | 4144 | 1 | Release 14 |  |
| RP-200338: Corrections to T312 and Discovery Signals measurement | 4198 | 1 | Release 12 |  |
| RP-200367: Correction on H1 and H2 events | 4103 | 2 | Release 15 |  |
| RP-201166: Allowing PDCP version change without handover | 4262 | 2 | Release 15 |  |
| RP-201166: upperLayerIndication enhancements | 4266 | 3 | Release 15 |  |
| RP-201192: Relaxed serving cell measurement for UEs using WUS | 4344 | - | Release 15 |  |
| RP-202780: Corrections to the field descriptions for TDD/FDD capability differentiation, and to nMaxResource value range | 4389 | 5 | Release 12 | The CR corrects multiple UE capability field descriptions introduced in various releases, the changes are early implementable back to the release in which the corresponding capability was introduced. |
| RP-202789: Correction on uac-AC1-SelectAssistInfo | 4488 | 2 | Release 15 |  |
| RP-211481: Clarification on the initiation of RNA update | 4651 | 1 | Release 15 |  |
| RP-212596: Distinguishing support of extended band n77 | 4723 | 2 | Release 15 |  |
| RP-220472: Introduction of carrier specific NRSRP thresholds for NPRACH resource selection | 4777 | 1 | Release 14 |  |
| RP-221738: Distinguishing support of band n77 restrictions in Canada | 4799 | 2 | Release 15 |  |
| RP-232570: Addition of extended number range for NS value | 4917 | 6 | Release 16 |  |
| RP-233884: Correction to flightPathInfoAvailable when connected to 5GC | 4959 | 2 | Release 15 |  |
| RP-233883: Protection against improper reselection to GERAN/UTRAN [RESELECTION_TO GSM_AND_UTRAN] | 4971 | 1 | Release 15 |  |
| RP-233915: Introduction of measurements without gap with interruption | 4929 | 6 | Release 17 |  |
| RP-243230: Correction to NR Beam Reporting in LTE RRC | 5069 | 2 | Release 15 |  |
| RP-243233: Addition of reference to 38.304 for Qoffsettemp handling | 5080 | 1 | Release 15 |  |
| RP-243227: Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL] | 5083 | 1 | Release 16 |  |
| RP-250660: Clarification on the mapping of RSRP thresholds to CE levels | 5100 | - | Release 13 |  |
| RP-25xxxx: Clarification on UTC time offset in IoT NTN | 5146 | 1 | Release 17 |  |
| NOTE 1: In case a CR has mirror CR(s), the mirror CR(s) are not listed. NOTE 2: The Additional Information column briefly describes the content of a CR in cases where the CR title may not be descriptive enough. If the CR title is descriptive enough, then the Additional Information column may be left blank. |  |  |  |  |

End of the change