3GPP TSG-RAN WG2 Meeting #131	R2-2506413

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.1.3

Source:	InterDigital Inc. (Rapporteur)

Title:	Summary of [AT131][AI PHY] NW side data collection open issues

Document for:	Discussion

# 1. Introduction

This is the summary of the offline discussion on network side data collection:

RRC Open issues: Open issues that will be discussed first offline in order and then what’s left online

[AT131][004][AI PHY] NW sided OIs (InterDigital)

Intended outcome: agreeable proposals

Deadline:  Wednesday

# 2. Discussion

## RRC-19: Reporting assistance information related to logged measurements

R2-2505240	Remaining issues on NW side Data Collection	LG Electronics	discussion	Rel-19	NR_AIML_air-Core

Proposal 1. [RRC19] The otherConfig includes separate bits for low power, buffer full, and buffer threshold reached indications.

R2-2505504	Remaining issues on NW-sided data collection	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 4 (open issue RRC-19): Full buffer indication is enabled by default whenever LoggedDataCollectionAssistanceConfig is configured (i.e. no additional configuration bit is needed).

R2-2505675	Remaining issues of NW-sided data collection	Huawei, HiSilicon	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: (RRC-19) The indications reported by the UE as part of UAI for data collection purposes are controlled by the network in the following way:

- Full buffer indication is enabled by default when the UE is configured with

loggedDataCollectionAssistanceConfig

- Data threshold is enabled by including data threshold in the configuration

- Low power state indication is configurable with a dedicated parameter to enable it

Discussion:

Almost all companies wanted to have full buffer by default

There was some varying view regarding the low power indication, but there was no clear objection to having the low power indication also by default (e.g., Huawei indicated that the low power indication by default is acceptable under the assumption that we will keep the “only one time indication” for power limitation, Ericsson preferred to have the low power configurable, not by default).

Almost all companies agree that the buffer threshold indication is optional, and if it is included, it indicates the data threshold.

(RRC-19) If LoggedDataCollectionAssistanceConfig is configured, then full buffer and low power indications are supported by default (i.e., no additional fields/bits required to configure them). Data threshold is (optionally) configured by including the threshold in the loggedDataCollectionAssistanceConfig.

## RRC-21: Time related content of logged data

R2-2505839	NW-side data collection	Ericsson	discussion

Proposal 3 (RRC-21) The UE includes in the logged measurement report, an absolute time stamp (provided by the network at the time of configuring the UE with the NW-side data collection configuration), and for the first measurement entry and each measurement entry separated by a “gap” versus the immediate previous entry, a relative time stamp.

R2-2505075	Leftover Issue Discussion on NW side data collection	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: (RRC-21) Consecutive samples share the same record list, if there is a gap between two samples, another record list is used to include the consecutive samples after the gap. Optionally, the first sample of each record list is associated with a relative timestamp.

R2-2506118	Discussion on NW side data collection framework	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: For logged data attached with time information, the first logged data is attached with an absolute time stamp as long as UE starts the logging, and the following logged data is attached with relative time stamp until to the corresponding logging configuration is released.

Discussion:

There was no common view on how the gap information should be captured, and there were even few companies that questioned the need for it (e.g., Samsung, Qualcomm, Nokia). For the companies that supported to have the gap information, the options ranged from just one bit indication to indicate a gap, a relative time stamp, an absolute time stamp, or some optimization where the gap is from the last sample from the gap.

Issues were also raised (e.g., Nokia) that even absolute time stamp may not be sufficient (since the periodicity can be at a much finer granularity that can be captured in absolute time stamp).

Observation 1: Companies have different views regarding how to capture the gap information in the log and whether if there is even a need for it.

## RRC-28: Handling of data collection configuration and configuration to report data availability and low power state during RRCReestablishment, transition to RRC_INACTIVE etc

R2-2505150	Discussion on NW side data collection	Xiaomi	discussion

Proposal 4: (RRC-28) Logging and UAI configuration are released upon IDLE and RLF.

Proposal 5: (RRC-28) Logging and UAI configuration are kept upon INACTIVE.

R2-2505346	Discussion on NW side data collection	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 3 (RRC-28). Upon RRE initiation (including RLF detection), UE discards any logged data.

Proposal 4 (RRC-28). Upon RRE initiation, UE releases 1) the configuration of NW-side data collection and 2) UAI configuration for NW-side data collection.

Proposal 5 (RRC-28). When going to RRC_IDLE, UE releases 1) the configuration of NW-side data collection and 2) UAI configuration for NW-side data collection (i.e., No spec change is needed).

Proposal 6 (RRC-28). When going to RRC_INACTIVE, UE does not store the configuration of NW-side data collection in UE Inactive AS Context.

Proposal 7 (RRC-28). When going to RRC_INACTIVE, UE first stores UAI configuration for NW-side data collection in UE Inactive AS Context and it is released upon initiation of RRC connection resume.

Discussion:

Almost all companies wanted to have the data collection configuration and UAI to be released when the UE goes to IDLE or there is RLF/re-establishment. However, there were some varying views regarding the handling of the INACTIVE case, specially  the case of the low power indication. For example, Qualcomm indicated that if the UAI configuration is kept in INACTIVE, the UE, upon resuming can indicate the low power indication (implicitly telling the network that it doesn’t prefer to be configured for data collection).  But it was finally found this is an optimization and not that essential, and we can harmonize the UE behavior across all states transitions and release both the data collection configuration and the UAI configuration.

(RRC-28) Both the data collection configuration and the UAI configuration related to data collection are released when the UE transitions to IDLE/INACTIVE or initiates re-establishment (including RLF).

## RRC-29: Data Availability Indication and Low Power State

R2-2505650	Discussion on open issues on NW side data collection	Sony	discussion	Rel-19	NR_AIML_air-Core

Proposal 3 (RRC-29): Data availability indication is not sent when indicating low power state.

R2-2506104	Data Collection for Training of NW-side AI/ML Models	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: (RRC-29) If the UE is in a low power state AND it has data in its buffer, even below the threshold, UE can still indicate both low power state and buffer threshold met indications. That is, no additional cause or flag is required.

R2-2505471	Further Discussion on the Remaining RRC Issues for Network-Side Data Collection 	MediaTek Inc.	discussion

Proposal 3: When the UE enters a low power state, it sends a low power state indication together with the buffer status to the network.

Proposal 4: The buffer status indicator (bufferStatus-r19) is extended to include additional values, i.e. belowThreshold and empty.

Discussion:

Most companies agree that low power should not trigger a data availability indication trigger. In the rare case where the low power and the full buffer or data threshold triggers get fulfilled at the same time, the UE ofc course could indicate both in the UAI. And the low power UAI received without a full buffer or data threshold indication implicitly indicates that the data at the UE, if at all, is below the threshold.

(RRC-29) If the buffer is not full or the data threshold is configured and the amount of data is below the threshold, UE does not send data availability indication when it sends low power indication.

## RRC-31: Release of Logged AI/ML Data

R2-2505511	Discussion on remaining issues on NW side data collection	CMCC	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: In case the network does not retrieve logged AI/ML data, UE should store non-retrieved data for 48 hours after the release of data collection configuration. The UE will release all logged AI/ML data at switch off or de-registration, or receiving the explicit indication from the serving gNB.

R2-2506104	Data Collection for Training of NW-side AI/ML Models	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 6: (RRC-31) Data from the UE buffer will be released in the following scenarios:

- when data has been successfully transmitted using a UEInformationResponse message;

- the configuration for logging has been released and the flag to retain measurements after handover has not been set;

- the configuration for logging has been modified;

- All logged measurement configuration and the logged data shall be removed by the UE at switch off or detach.

R2-2505839	NW-side data collection	Ericsson	discussion

Proposal 7: (RRC-31) RAN2 does not specify further cases when the UE shall discard logged data for NW-side data collection, beyond already agreed cases.

Discussion:

The various options that were discussed regarding when the logged data is going to be released (in addition to the already agreed upon cases of transition to IDLE/INACTIVE and RLF) and the discussion outcomes are listed below:

  - Upon successful delivery of the logged data: (obvious)

  - Upon receiving new data collection configuration: (no consensus if this is needed, not a very likely scenarios that data collection configuration is updated within a data collection session)

  - Configuration release: (we shouldn’t tie data collection configuration and collected data, as the network can stop the data collection but may not have the resources to pull the data immediately).

  - 48hrs after data collection configuration is received (if not retrieved): (data collection is for CONNECTED state, and we have already agreed to release the data upon state transition and highly unlikely UE will stay hours, let alone minutes, in the CONNECTED state)

  - At switch off: implicitly covered in the handling of IDLE

  - At detach: like previous

  - Upon explicit indication to release: like above regarding not tying configuration handling with the collected data

(RRC-31) No additional handling of logged data to be specified (apart from the already agreed release during state transition and RLF and release upon successful delivery).

## RRC-34: Per use case vs common enhancements for NW-side data collection

R2-2505195	Remaining issues on NW side data collection	vivo	discussion	NR_AIML_air-Core

Proposal 6. [RRC-34] A common approach should be considered for enhancements for NW-side data collection, and the following enhancements are common for all use cases:

- AS layer memory;

- logged data availability;

- low-power state indication.

R2-2505075	Leftover Issue Discussion on NW side data collection	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 10: (RRC-34) AS layer memory and low power state indication should be defined per UE, i.e. common for all use cases, while data availability indicator should be sent per use case.

R2-2505687	Left issues for NW side data collection	Lenovo	discussion	Rel-19

Proposal 4: When UE indicates the data availability to NW, UE also indicates the list of data logging configuration ID for which data is available.

Proposal 5: When gNB requests logged data from UE via UEInformationRequest message, gNB can also include the interested data logging configuration ID.

Proposal 6: The cause value (i.e., buffer full, buffer threshold reached, low power state) of data availability indication remain generic for all data logging.

Discussion:

There was a consensus that since we have only one use case in rel-19 (BM), there is no need to discuss this further in rel-19 context.

(RRC-34) No further discussion is needed regarding this in re-19 as we have only one use case.

# 3. Conclusions

In this offline discussion, some of the RRC open issues on network side data collection were discussed and the following proposals and observations were made:

Observation 1: Companies have different views regarding how to capture the gap information in the log and whether if there is even a need for it.

Proposal 1: (RRC-19) If LoggedDataCollectionAssistanceConfig is configured, then full buffer and low power indications are supported by default (i.e., no additional fields/bits required to configure them). Data threshold is (optionally) configured by including the threshold in the loggedDataCollectionAssistanceConfig.

Proposal 2: (RRC-28) Both the data collection configuration and the UAI configuration related to data collection are released when the UE transitions to IDLE/INACTIVE or initiates re-establishment (including RLF).

Proposal 3:(RRC-29) If the buffer is not full or the data threshold is configured and the amount of data is below the threshold, UE does not send data availability indication when it sends low power indication.

Proposal 4:(RRC-31) No additional handling of logged data to be specified (apart from the already agreed release during state transition and RLF, and release upon successful delivery).

Proposal 5:(RRC-34) No further discussion is needed regarding this in re-19 as we have only one use case.