3GPP TSG RAN WG2#131	R2-2506432

Bengaluru, India, 25th - 29thAugust 2025

# 3GPP TSG-RAN WG4 Meeting #116	R4-2512161

# Bengaluru, India, August 25th – 29th, 2025

Title:	Reply LS on CSSF optimization for NR RRM Phase 5

Response to:	R2-2504728

Release:	Rel-19

Work Item:	NR_RRM_Ph5-Core

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact Person:

Name:	Jie Cui

E-mail Address:	Jie_cui@apple.com

Attachments:	N/A

# 1. Overall Description:

RAN4 would like to thank RAN2 for the LS on the CSSF optimization for NR RRM Phase 5. RAN4 had discussion on the alternatives from RAN2, and following conclusions have been made:

- About the network indication of a specific SCC per-band for measurement, RAN4 agreed to adopt an alternative 3 as following:

  - If UE support R19 CSSF solution 1 (measurement one CC per FR2-1 band) and network enables this feature,

    - Follow the new RRC network indication (if present) to measure the indicated SCC in an FR2-1 band, if

      - Neither PCC nor PSCC is in the same FR2-1 band, and

      - The indicated SCC to be measured shall be one of the SCCs on which UE is configured to report SSB based measurements in the same FR2-1 band.

    - For case that only MO(s) are configured on SCC(s) in an FR2-1 band, when neither PCC nor PSCC is in the same FR2-1 band, and there is no SCCs on which UE is configured to report SSB based measurements in this FR2-1 band, UE will select and perform serving and intra-frequency neighbour cell measurements on one SCC in that FR2-1 band. SCC selection in this FR2-1 band to measure is up to UE implementation.

    - RAN4 kindly requests RAN2 to implement RRC signalling enabling network to indicate which SCC the UE shall measure in an FR2-1 band.

# 2. Actions:

To RAN WG2 group.

ACTION: 	RAN4 kindly requests RAN2 to take the above agreement into consideration for future work.

# 3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #116bis	13 – 17 Oct 2025	Prague, Czech

TSG RAN WG4 Meeting #117	17 – 21 Nov 2025	Dallas, US