3GPP TSG-RAN WG2 Meeting #131			           		R2-2506439

Bangalore, India, August 25 - 29, 2025

Agenda item:	8.1.2.2

Source:	Samsung

Title:	Summary report of [AT131][030][AI PHY] Multicell v.s Single cell associate ID

Document for:	Discussion and Decision

# Introduction

RRC-46: Whether the associated ID is cell specific or multi-cell specific

R2-2505524	Support of associated ID	Samsung, Apple, Xiaomi, Qualcomm, Google, Interdigital, NTT DOCOMO, Nokia, LG Electronics, vivo	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: Both single cell and multi-cell associated ID can be supported based on NW implementation (i.e., the network may allocate an Associated ID to a single cell and/or to multiple cells).

Proposal 2: Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment.

R2-2505838	LCM for UE-side models for beam management	Ericsson	discussion

Proposal 7: (RRC-46) RAN2 to confirm that the associated ID is consistent within a cell, i.e. the same values can be reused in other cells, where they can correspond to different beam configurations.

R2-2505192	Discussion on LS on functionality activation and open issues on BM	vivo	discussion	NR_AIML_air-Core

Proposal 12: (RRC-46) It should be RAN1, not RAN2 to determine whether the associated ID is cell specific or multi-cell specific. RAN2 can assume multi-cell specific associated ID so far to define the value range before RAN1 does not reach the conclusion.

[AT131][030][AI PHY] Multicell v.s Single cell associate ID (Samsung)

Intended outcome: Agreable proposal for issue 46 and 41

Deadline:  Thursday

# Cell specific ID vs. Multi-cell specific ID

Cell specific ID: associated ID is unique within a cell.

- Expected operation for example…..

  - in each cell, gNB and UE needs to check applicability for the configured CSI-ReportConfig even if the associated ID is the same.

  - UE need to categorize different data set for data collected from each cell regardless of whether the configuration is the same or different.

Multi-cell specific ID: associated ID is unique within PLMN.

  - UE assumes that if associated ID is the same,  one same/similar beam deployment is assumed.

  - UE can categorize same data set for the data collection from multi-cells if associated ID and other configuration are the same.

-MTK, HW think that it may not need to be PLMN unique. It could be area specific.

# Discussion

Concern on cell specific ID

Example (R2-2505524): Observation 1: cell specific associated ID will require UE to train a huge number of models and consequently more complexity/burden in model training at UE side.

Concern on multi-cell specific ID

Example (R2-2505838):Some companies propose to define a multi-cell specific associated ID, where this associated ID could also be extended to be a global associated ID defined per PLMN. In our view, a global/multi-cell associated ID would require a solution for network vendors to coordinate with each other to split the associated ID range among each other, which is very complex. Alternatively, a central entity in the PLMN would have to split the associated ID range among network vendors and this is outside RAN2 scope.

Companies’ view:

- Xiaomi agree with observation. With multiple modles, loading of model can be also burden.

- LG: model training across cell would be very hard. It is very restrictive. RAN2 decide the

- Ericsson: mapping among multiple cells with same configuration may not be easy considering granularity. Associated ID is not only factor to train different modle.

- Nokia: allocating associate ID is NW implementation. IT is not scalable to have different model for each cell.

- CATT: share Ericsson that each cell may have different configuration.

- Interdigital: it seems feasible to allocate associate ID with multi-cell specific ID. Cell specific ID is infeasible to manage a lot of models.

- CMCC: global ID over whole PLMN is challenging. Also understand the burden of cell specific ID. We can consider tracking area. i.e. associated ID is unique within Tracking area.

- Vivo: can look at area specific ID.

- NTT DCM: support both with a long associate ID? Ericsson think we still need coordination.

- Xiaomi think that multi-cell specific ID provides good flexibility and NW can decide which option it uses considering own deployment and difficulty of categorization of NW configuration.

-HW think that RAN1 has not evaluate the benefit of multi-cell specif ID.

Potential proposals

- At least, preclude cell-specific only ID.

- Support both cell specific ID and multiple-cell specific ID but details are FFS.

- Object : Huawei, Ericsson

# Conclusion

Proposal 1: Support both cell specific ID and multiple-cell specific ID but detailed signaling is FFS.