3GPP TSG RAN WG2 Meeting #131                	                	                          R2-2506448

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.1.1

Source:	Xiaomi

Title:	Report of [AT131][028][AI PHY] UE capabilities (Xiaomi)

Document for:	Discussion and Decision

Introduction

During the previous online discussion, RAN2 has the following open issues need to be further discussed:

2 FFS on whether UE can support other memory sizes and indicate to network via optional capability signaling. 3 [CB if further updates are required] Introduce an optional UE capability to indicate supporting logged measurements of data collection for NW-side model, which includes the minimum AS layer memory size. FFS whether the following components are part of this capability or separate capabilities, and how: - periodic logging - event-based logging - measurement quantities and resources (Set A/B, maximum resources of Set A/B, etc) - provide assistance information (e.g., data is available, reason for trigger, low power indication) for NW-side data collection

Discussion

On UECap-2

During discussion yesterday, RAN2 agreed with the followings:

1	If LoggedDataCollectionAssistanceConfig is configured, then full buffer and low power indications are configured by default (i.e., no additional fields/bits required to configure them). Data threshold is (optionally) configured by including the threshold in the loggedDataCollectionAssistanceConfig.

It seems reasonable to keep reason for trigger (i.e., full buffer and low power indication) as part of the basic logged NW-side data collection capability. However, for data threshold, which can be optionally configured by network, this can be decided based UE capability whether UE supports it.

Question 1: Components included as the basic feature ‘loggedDataCollection-r19’:

- Minimum 64kB AS layer memory size

- Periodic logging

- Provide full buffer indication, low power indication

The granularity of this basic feature is per UE.

Companies’ view:

- QC thinks periodic loggidg is not basic, should be separate, UE supports either periodic or event-based. Vivo agree with QC, periodic and event-based are considered separately.

- ZTE thinks both periodic and event should be basic

- Oppo wonders if periodic logging is not supported, what should network do.

- HW thinks periodic is by default and simpler, to be considered as baseline. Nokia, E///, SS, CMCC, DCM agrees with Oppo and HW, since UE anyway needs to measure.

=> Introduce an optional per UE capability ‘loggedDataCollection-r19’ to indicate supporting logged measurements of data collection for NW-side model, which includes the following components:

- the minimum 64kB AS layer memory size

- periodic logging

- Provide full buffer indication, low power indication

Event-based logging is an optional per UE capability separate from basic. UE supporting this feature shall also indicate the support of basic NW data collection feature.

Question 2: Whether data threshold-based data availability indication is

- A separate optional per UE capability with signaling. If so, a UE supporting this feature shall also indicate support of the basic logged NW-side data collection.

- part of the basic feature

Companies’ view:

- HW, E///, ZTE thinks this should be basic feature. Nokia think it’s optional for NW to use it. E/// thinks this feature might be more useful.

- Xiaomi, SS, Apple, Oppo, QC, IDCC, vivo thinks this to be separate capability. Oppo think buffer full as basic is enough. QC wonders whether data threshold will still be configured if full buffer is configured? HW think it can be configured without full buffer.

- IDCC thinks Data threshold is not known by network, it’s limited by UE.

- Nokia thinks network knows the threshold based on minimum AS buffer. SS thinks periodic supported, this is not mandatory.

- Apple thinks the buffer can be supported more than minimum buffer.

- Vivo is not complementary for UE side.

- Data threshold-based data availability indication is an optional per UE capability with signaling. A UE supporting this feature shall also indicate support of the basic logged NW-side data collection.

RAN2 reached following agreement yesterday on NW-side data collection configuration:

3	The resource configuration does not have separate resources for Set A and Set B.

Only periodic CSI resources are used for NW sided data collection.  No need for new dynamic MAC CE mechanisms.

Looking at RAN1 feature list, following components are introduced for UE-side data collection:

Question 3: How to interpret CSI resource capability used for logged NW-side data collection：

- introduce a new ‘per band’ UE capability (e.g., ‘loggedDataCollection-CSI-Resource’) to indicate the supported resources used by logged NW-side data collection, e.g., the supported number of CSI resources, support of SSB, support of CSI-RS, etc.

  - similar components as defined for UE-side data collection (e.g., supported maximum number of resources, support of SSB, support of CSI-RS, etc) applies to this new UE capability (e.g., ‘loggedDataCollection-CSI-Resource’) without differentiating between Set A and Set B

- reuse MIMO CSI resource capability, i.e. network considers MIMO capabilities when configuring logged NW-side data collection. If so, RAN2 will not define new capability for CSI resource in logged NW-side data collection.

Companies’ view:

- QC, IDCC, Apple supports option 2. NW needs to consider UE capability as a whole for all features.

- Proposal 3: RAN2 will not introduce separate CSI resource capaiblity for logged NW-side data collection. Legecy capability will be used for logged NW-side data collection.

- Check with RAN1 on whether this is ok.

On UECap-1

Question 4: Discuss whether UE can support other memory sizes and indicate this to network via an optional capability signaling.

During the online discussion, this was further clarified that this additional memory size indicated by UE is also shared across all use cases (including R20).

Arguments from Proponent:

-	network flexibility for higher buffer threshold configuration

-	common AS layer buffer might need larger memory size to support other use cases as well

Arguments from Opponent:

-	for Rel-19 data collection, no need for additional memory

-	more memory size is up to UE implementation

-	network can retrieve data based on UE buffer full indication

Companies’ view:

- Xiaomi, ZTE, CMCC, Fujitsu, HW, E///, SS supports. Xiaomi thinks more memory may help network to configure larger data threshold. ZTE thinks for Rel-20, CSI two-sided model, this is important and necessary. CMCC thinks the network may configure operator deployed dedicated UE to perform NW-side data collection. HW thinks this will be helpful for network to fetch the data later.  SS

- QC, Apple, vivo, Oppo don’t support, logged MDT only has minimum AS buffer. There’s no separate NW behavior based on this capability. Apple thinks the motivation is not justified, and if needed, we can consider in Rel-20.

- UE can support other memory sizes and indicate this to network via an optional capability signaling.

On Option A/B capability

The following was agreed during Tuesday online session:

Introduce two conditional mandatory capabilities (with signaling) for AI/ML based BM Option A and Option B, if UE supports FG58-0-1 and/or FG58-1-2/3/4/5 (the details of those feature group depend on RAN1 progress).

Option A: Current draft CR implementation – By defining two conditional mandatory signalings, this means: if UE supports RAN1 feature group, UE shall also indicate the support of both Option A and Option B:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| applicabilityReportingCSI-r19 Indicates whether the UE supports applicability reporting based on inference configuration provided via CSI-ReportConfig, as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of ‘RAN1 feature group’. Editor’s Note: FFS on which RAN1 FG(s) UE is mandatory to support if UE supports this capability. | UE | CY | No | No |
| applicabilityReportingOther-r19 Indicates whether the UE supports applicability reporting based on inference configuration provided via OtherConfig, as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of ‘RAN1 feature group’. Editor’s Note: FFS on which RAN1 FG(s) UE is mandatory to support if UE supports this capability. | UE | CY | No | No |

Option B: based on Offline comments, this agreement can also be implemented by defining them as optional capability with dependency in the field description.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| applicabilityReportingCSI-r19 Indicates whether the UE supports applicability reporting based on inference configuration provided via CSI-ReportConfig, as specified in TS 38.331 [9]. Editor’s Note: FFS on which RAN1 FG(s) UE is mandatory to support if UE supports this capability. | UE | No | No | No |
| applicabilityReportingOther-r19 Indicates whether the UE supports applicability reporting based on inference configuration provided via OtherConfig, as specified in TS 38.331 [9]. Editor’s Note: FFS on which RAN1 FG(s) UE is mandatory to support if UE supports this capability. | UE | No | No | No |
| RAN1 feature group Xxx (field description of RAN1 FG) A UE supporting this features shall also indicate the support of at least one of applicabilityReportingCSI-r19 or applicabilityReportingOther-r19. |  |  |  |  |

Companies’ view:

Not discussed, to be continue in post email discussion on the detailed signaling design.

- Conclusion

Easy agreements

Proposal 1: Introduce an optional per UE capability ‘loggedDataCollection-r19’ to indicate supporting logged measurements of data collection for NW-side model, which includes the following components:

- the minimum 64kB AS layer memory size

- periodic logging

- Provide full buffer indication, low power indication

Proposal 2: Event-based logging is an optional per UE capability separate from ‘loggedDataCollection-r19’. UE supporting this feature shall also indicate the support of ‘loggedDataCollection-r19’.

Proposal 4: RAN2 will not introduce separate CSI resource capability for logged NW-side data collection. Legacy capability will be used for logged NW-side data collection. Check with RAN1 on whether this assumption is ok.

To be discussed

Proposal 3: (7 proponents, 4 opponents) Data threshold-based data availability indication is an optional per UE capability with signaling. A UE supporting this feature shall also indicate support of the basic logged NW-side data collection.

Proposal 5: (7 proponents, 4 opponents) UE can support other memory sizes and indicate this to network via an optional capability signaling.

Open issue list

Open issue UECap-1: whether UE can support other memory sizes and indicate to network via optional capability signaling

Issue Description:

This was discussed during the email discussion, and there are 5 companies think that UE can optionally indicate other AS layer memory sizes to network due to the following reasons:

-	network flexibility for higher buffer threshold configuration

-	common AS layer buffer might need larger memory size to support other use cases as well

4/9 companies don’t agree to indicate other memory size to network due to the following reasons:

-	for Rel-19 data collection, no need for additional memory

-	more memory size is up to UE implementation

-	network can retrieve data based on UE buffer full indication

Status in running CR:

Not captured

Proposed resolution:

companies to provide contributions to the following meeting.

Open issue UECap-2: How to define UE logged measurement of data collection for NW-side model

Issue Description:

This was discussed during the email discussion that how to capture UE logged measurement of data collection for NW-side model, including what components should be included, what other separate capabilities are needed. Following FFS is proposed by rapporteur:

Introduce an optional UE capability to indicate supporting logged measurements of data collection for NW-side model, which includes the minimum AS layer memory size. FFS whether the following components are part of this capability or separate capabilities, and how:

-	periodic logging

-	event-based logging

-	measurement quantities and resources (Set A/B, maximum resources of Set A/B, etc)

-	provide assistance information (e.g., data is available, reason for trigger, low power indication) for NW-side data collection

Status in running CR:

Not captured

Proposed resolution:

companies to provide contributions to the following meeting.

Open issue UECap-3: RAN1 feature group dependency

During the discussion, there are three RAN2 features having dependency with RAN1 feature group:

- 1) include RAN2 feature ‘UE can provide update of applicability reporting via UAI’ as part of RAN1 FGs (e.g., 58-0-1 and/or FG 58-1-2/3/4/5)

- 2) include RAN2 feature ‘providing UE preferred configuration for UE-side data collection’ as part of RAN1 FG58-1-7/FG58-3-4

- 3) introduce two conditional mandatory capabilities (with signaling) for AI/ML based BM Option A and Option B, if UE supports FG58-0-1 and/or FG58-1-2/3/4/5

Status in running CR

UECap-3-1 and UECap-3-2 are not captured, UECap-3-3 with editor notes in 38.306

Proposed resolution:

Wait for further updates of RAN1 feature list and implementation of those features in RAN2 UE capability mega CR.

Annex

Agreements

1	If UE supports NW side data collection, it is mandatory for UE to support the minimum AS layer memory size of 64kB for UE supporting AI/ML based beam management, which is shared across all use cases with NW-sided model.   The assumption is that this will be shared across all use cases (i.e. Rel-20 as well)

2	FFS on whether UE can support other memory sizes and indicate to network via optional capability signaling.

3	[CB if further updates are required] Introduce an optional UE capability to indicate supporting logged measurements of data collection for NW-side model, which includes the minimum AS layer memory size. FFS whether the following components are part of this capability or separate capabilities, and how:

-	periodic logging

-	event-based logging

-	measurement quantities and resources (Set A/B, maximum resources of Set A/B, etc)

-	provide assistance information (e.g., data is available, reason for trigger, low power indication) for NW-side data collection

4	Include RAN2 feature ‘UE can provide update of applicability reporting via UAI’ as part of RAN1 FGs (e.g., 58-0-1 and/or FG 58-1-2/3/4/5, the details of those feature group depend on RAN1 progress) once implemented.

5	Introduce two conditional mandatory capabilities (with signaling) for AI/ML based BM Option A and Option B, if UE supports FG58-0-1 and/or FG58-1-2/3/4/5 (the details of those feature group depend on RAN1 progress).

6	Include RAN2 feature ‘providing UE preferred configuration for UE-side data collection’ as part of RAN1 FG58-1-7/FG58-3-4 (once implemented).

7	UAI is mandatory for both Option A and B

Agreements on NW side data collection

1	RAN2 confirms that the network data logging is captured in a new clause (e.g. 5.5x) in the RRC specification.

2	A hysteresis should be configured and used (alongside threshold and timeToTrigger) for event-triggered logging for NW-side data collection.

3	The resource configuration does not have separate resources for Set A and Set B.

4	RAN2 to send an LS to RAN1 to inform about the RAN2 agreements on solution for network data logging

5	RAN2 to send an LS to RAN3 to inform about the RAN2 agreements on solution for network data logging

- keep event-triggered logging

- logging configuration is introduced as a new list of configurations under CSI-MeasConfig, based on TP1 in R2-2505860,

Agreements

- Multiplexing of legacy SON/MDT report and AIML logged data is not supported in the same UE information response message.  Up to the network to ensure that data is not requested at the same time

- The logging periodicity of a NW-side data collection configuration is configurable.

- No further indication/condition is specified (beyond already agreed ones) for the UE to inform source gNB about data availability before HO in Rel-19.

- The UE stores logged data for BM in a variable specific to L1 CSI related measurements.

- Only periodic CSI resources are used for NW sided data collection.  No need for new dynamic MAC CE mechanisms.

Agreements:

1	If LoggedDataCollectionAssistanceConfig is configured, then full buffer and low power indications are configured by default (i.e., no additional fields/bits required to configure them). Data threshold is (optionally) configured by including the threshold in the loggedDataCollectionAssistanceConfig.

2	Both the data collection configuration and the UAI configuration related to data collection are released when the UE transitions to IDLE/INACTIVE or initiates re-establishment (including RLF).

3	If the buffer is not full or the data threshold is configured and the amount of data is below the threshold, UE does not send data availability indication when it sends low power indication.

4	No additional handling of logged data to be specified (apart from the already agreed release during state transition and RLF, and release upon successful delivery).

5	No further discussion is needed on RRC issue 34 in re-19 as we have only one use case.

6	The UE will indicate the presence of a gap (i.e. there will be no indication on the length of gap or time instance, etc).  Rapporteur will suggest a way to implemented as part of the RRC review.