3GPP TSG-RAN WG2 Meeting #131bis	R2-2507792

# Prague, Czechia, 13th – 17th October 2025                        (was R2-2507099)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5510 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PC5 Relay RLC channel configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For L2 U2N relay UE, the PC5 Relay RLC channel configurations in sl-ConfigDedicatedNR in RRCReconfiguration needs to be configured accordingly based on the need of SRAP configuration included in sl-L2RelayUE-Config. This is because the PC5 relay RLC channel configurations included in sl-ConfigDedicatedNR are destination-agnostic and do not indicate which PC5 relay RLC channel is to be established towards which remote UE. If the relay UE blindly add all configured PC5 relay RLC channels towards all its remote UEs, that would waste PC5-RRC signaling overhead and result a lot of unused RLC channels. If not, then, based on the current specification, necessary PC5 Relay RLC channel(s) will not be setup if only SRAP configuration has been changed but sl-ConfigDedicatedNR remain unchanged. Because SRAP-config change alone will not trigger PC5-RRC procedure. Hence, the dependency on SRAP configuration needs to be clarified when L2 U2N relay UE adds a new PC5 relay RLC channel. For PC5 relay RLC channel modification/release cases, no change is needed because there is no ambiguity about which RLC channel is referred to which remote UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 5.8.9.7.2, added the sentence to describe L2 U2N relay UE only initiate PC5 relay RLC channel addition towards a remote UE if the remote UE is indicated to use this RLC channel in accordance with sl-SRAP-ConfigRelay. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality Layer 2 Sidelink UE-to-NW Relay Inter-operability: If the UE implements the CR but not the network, there is no inter-operability issue. If the network implements the CR but not the UE, there is no inter-operability issue. If one UE implements the CR but not the other UE, there is no inter-operability issue. Implementation of this CR from Rel-17 will not cause interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | L2 U2N relay UE will either unable to set up needed PC5 relay RLC channels towards its connected L2 U2N remote UEs or set up useless PC5 Relay RLC channels. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.7.2, Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The CR has been revised to Cat “F” with the magic sentence added. |  |  |  |  |  |  |  |  |

First change

##### 5.8.9.7.2	PC5 Relay RLC channel addition/modification

Upon PC5-RRC connection establishment between the L2 U2N Relay UE and L2 U2N Remote UE, the L2 U2N Relay UE shall:

1>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

1>	apply RLC specified configuration of SL-RLC0 as specified in clause 9.1.1.4:

1>	apply RLC default configuration of SL-RLC1 as defined in clause 9.2.4 if the L2 U2N Relay UE is in RRC_IDLE/INACTIVE state;

Upon PC5-RRC connection establishment between two UEs for L2 U2U relay operation, the L2 U2U Relay UE and the L2 U2U Remote UE shall:

1>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

1>	apply RLC specified configuration of SL-U2U-RLC as specified in clause 9.1.1.4;

A L2 U2N relay UE initiates a PC5 Relay RLC channel addition towards a L2 U2N Remote UE if the sl-RLC-ChannelID received in sl-ConfigDedicatedNR is indicated for this L2 U2N Remote UE in accordance with sl-SRAP-ConfigRelay, and a PC5 Relay RLC channel with the received sl-RLC-ChannelID was not configured before.

The UE shall:

1>	if the PC5 Relay RLC channel addition/modification was triggered due to the reception of the RRCReconfigurationSidelink message; or

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel addition/modification was triggered due to the configuration received within the sl-ConfigDedicatedNR; or

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel addition/modification was triggered based on the derivation of PC5 Relay RLC channel configuration(s) for end-to-end sidelink DRB(s) as specified in 5.8.9.7.0:

2>	if the current configuration contains a PC5 Relay RLC channel with the received sl-RLC-ChannelID or sl-RLC-ChannelID-PC5; or

2>	if the configuration in SIB12 or SidelinkPreconfigNR has updated, based on which the PC5 Relay RLC channel is derived:

3>	reconfigure the sidelink RLC entity in accordance with the received sl-RLC-Config or sl-RLC-ConfigPC5;

3>	reconfigure the sidelink MAC entity with a logical channel in accordance with the received sl-MAC-LogicalChannelConfig or sl-MAC-LogicalChannelConfigPC5;

2>	else (a PC5 Relay RLC channel with the received sl-RLC-ChannelID or sl-RLC-ChannelID-PC5 was not configured before):

3>	establish a sidelink RLC entity in accordance with the received sl-RLC-Config (in sl-ConfigDedicatedNR, or SIB12, or SidelinkPreconfigNR) or sl-RLC-ConfigPC5;

3>	configure the sidelink MAC entity with a logical channel in accordance with the received sl-MAC-LogicalChannelConfig or sl-MAC-LogicalChannelConfigPC5.

Next change

Annex C (normative):	List of CRs Containing Early Implementable Features and Corrections

This annex lists the Change Requests (CRs) whose changes may be implemented by a UE of an earlier release than which the CR was approved in (i.e. CRs that contain on their coversheets the sentence "Implementation of this CR from Rel-N will not cause interoperability issues").

Table C-1: List of CRs Containing Early Implementable Features and Corrections

| TDoc Number (RP-xxxxxx): CR Title | CR Number(s) | CR Revision Number(s) | Earliest Implementable Release | Additional Information |
| --- | --- | --- | --- | --- |
| RP-200335: Correction on usage of access category 2 for UAC for RNA update | 1141 | 2 | Release 15 |  |
| RP-201185: Introduction of signalling for high-speed train scenarios | 1464 | 5 | Release 15 |  |
| RP-201216: Release-16 UE capabilities based on RAN1, RAN4 feature lists and RAN2 | 1665 | 2 | Release 15 | Early implementation part is referring to the aspect covered by - R2-2006203: Extension of CSI-RS capabilities per codebook type - R2-2006360: Intraband EN_DC power class expansion for 29 dBm |
| RP-202768: UE behaviour when UL 7.5KHz shift is not supported | 2107 | 2 | Release 15 |  |
| RP-202790: Correction on uac-AccessCategory1-SelectionAssistanceInfo | 2130 | 1 | Release 15 |  |
| RP-211483: Clarification on the initiation of RNA update | 2581 | 1 | Release 15 |  |
| RP-201190: Introduction of eCall over IMS for NR | 1670 | - | Release 15 |  |
| RP-212598: Distinguishing support of extended band n77 | 2810 | 2 | Release 15 |  |
| RP-213342: Duty cycle signalling for power class 1.5 | 2817 | 1 | Release 15 |  |
| RP-213345: CR on 38.331 for introducing UE capability of txDiversity | 2859 | 1 | Release 15 |  |
| RP-220497: Introduction of function for RRM enhancements for Rel-17 NR FR1 HST | 2898 | 2 | Release 16 |  |
| RP-220838: Release-17 UE capabilities based on R1 and R4 feature lists (TS38.331) | 2901 | 1 | Release 15 | Early implementation part is referring to the aspect covered by: - R2-2203898: Introduction of BCS4 and BCS5 - R2-2203836: Introducing UE capability for power class 5 for FR2 FWA |
| RP-221721: CR on the CBM/IBM reporting-38331 | 2916 | 2 | Release 16 |  |
| RP-221736: Distinguishing support of band n77 restrictions in Canada [n77 Canada] | 3078 | 2 | Release 15 |  |
| RP-222527: Correction to additionalSpectrumEmission for UL CA in n77 for the US | 3476 | - | Release 15 |  |
| RP-222527: Correction to additionalSpectrumEmission for UL CA in n77 for Canada | 3478 | - | Release 15 |  |
| RP-232570: Addition of extended number range for NS value | 3900 | 6 | Release 16 |  |
| RP-233888: Introduction of FR2 FBG2 CA BW classes | 2867 | 6 | Release 15 |  |
| RP-233882: Enhancing SCell A2 event reporting | 4375 | 2 | Release 15 |  |
| RP-233890: PTM retransmission reception for multicast DRX with HARQ feedback disabled [PTM_ReTx_Mcast_HARQ_Disb] | 4504 | - | Release 17 |  |
| RP-240667: UE capability for Enhanced channel raster | 4445 | 3 | Release 16 |  |
| RP-240658: Introduction of TxDiversity for 2Tx capability | 4639 | 2 | Release 17 |  |
| RP-241543: Introduction of barring exemption for RedCap UEs for emergency calls | 4570 | 4 | Release 17 |  |
| RP-241543: Correction on Redcap 1 Rx and 2 Rx barring | 4632 | 5 | Release 17 |  |
| RP-241544: Correction on TRS in idle and inactive | 4755 | 3 | Release 17 |  |
| RP-241544: Clarification RLM/BFD relaxation and short DRX | 4771 | 2 | Release 17 |  |
| RP-233915: Introduction of further measurement gap enhancements | 4063 | 6 | Release 17 | Early implementation part is referring to all aspects other than effective measurement window. Early implementation also includes the corresponding capability of R2-2311897 (within CR 4510 of RP-233940). |
| RP-243227: Introduction of network signalling of maximum number of UL segments [Max-RRC-SegUL] | 5004 | 3 | Release 16 |  |
| RP-243233: Correction on deprioritisationTimer | 5170 | 2 | Release 15 |  |
| RP-252791: SSB position restrictions for less-than-5MHz SCells | 5249 | 3 | Release 18 |  |
| RP-252924: Introduction of 7MHz channel bandwidth | 5308 | 4 | Release 18 |  |
| RP-252788: Introduction of less than 5MHz in NTN | 5389 | 2 | Release 18 |  |
| RP-25xxxx: Correction on PC5 Relay RLC channel configuration | 5510 | 1 | Release 17 |  |

End of change