3GPP TSG-RAN WG2 Meeting #132	R2-2509111
Dallas, US, November 17th – 21st, 2025

Agenda Item:	9.1

Source: 		Vice Chairman (Samsung)

Title: 			Report from session on R18/19 MOB, R19 NES and R20 AI/ML MOB

Document for:	Approval

Time Schedule 
Please refer to the latest schedule in the RAN2 inbox on the public 3GPP servers.

## Comeback for main session:

[MOB][E055]:

R2-2508716	Remaining issues for LTM [E061, E055]	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: For the PDCP discarding on SRBs, the principle from Rel-18 is followed where the legacy RRC field discardOnPDCP is used. RAN2 to agree on the TP provided in the RIL.

[CATT]: The current text is aligned with RAN2 agreement (“For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.”). With relying on discardOnPDCP, NW cannot distinguish whether intra-CU or inter-CU will happen when NW configured LTM candidate configuration. Prefer sticking the current text. [Ericsson]: Do not see any issue since NW can handle it. [Ericsson]: With the current text, understand the subsequent LTM is not supported. [Huawei]: With the current text, Rel-19 UE implementing only Rel-18 LTM feature may have problem. [Vivo]: Understand Rel-19 UE implementing only Rel-18 LTM feature should follow Rel-18 spec for LTM function. [Ericsson]: No.

Offline (Ericsson)

Comeback in the main session Friday.

## List and Status of Offline/Email Discussions

POST Email discussion:

[POST132][104][NES] (Huawei)

Scope: Update 38.300 CR capturing the agreements made this meeting.

Intended outcome: 38.300 CR in R2-2509326 to be agreed.

Deadline: Short email discussion.

[POST132][105][NES] (IDC)

Scope: Update 38.321 CR capturing the agreements made this meeting.

Intended outcome: 38.300 CR in R2-2509327 to be agreed.

Deadline: Short email discussion.

[POST132][106][NES] (Ericsson)

Scope: Update 38.331 CR capturing the agreements made this meeting (also including an update of RIL conclusion).

Intended outcome: 38.331 CR in R2-2509329 to be agreed and updated RIL conclusion in R2-2509392.

Deadline: Short email discussion.

[POST132][108][MOB] (Vivo)

Scope: Update 38.321 CR capturing the agreements made this meeting.

Intended outcome: 38.321 CR in R2-2509336 to be agreed.

Deadline: Short email discussion.

[POST132][107][MOB] (Huawei)

Scope: Prepare LS to RAN1 according to P3 in R2-2509312.

Intended outcome: LS in R2-2509335 to be approved.

Deadline: Short email discussion

[POST132][109][MOB] (Ericsson)

Scope: Update 38.331 CR capturing the agreements made this meeting (also including an update of RIL conclusion).

Intended outcome: 38.331 CR in R2-2509337 to be agreed and updated RIL conclusion in R2-2509393.

Deadline: Short email discussion.

AT Email discussion:

[AT132][101][NES] (Ericsson)

Scope: Discuss and conclude remaining ToDo RILs (including S062, S063, other RILs that needs further discussion after conclusion on [POST131bis][301][NES], and new issues included in the contributions (P1 and P3 in R2-2508242). + J071, V504, V505

Intended outcome: Discussion summary in R2-2509311.

Deadline: Offline discussion 16:00 – 17:00, Tuesday, in the breakout room 1 => Completed.

[AT132][102][MOB] (Ericsson)

Scope: Discuss and conclude remaining PropReject [N141], [B123], [H160], [O602], [C166]

Intended outcome: Discussion summary in R2-2509312.

Deadline: Comeback in Wednesday CB session => Completed.

[AT132][103][MOB] (Vivo)

Scope: Discuss and conclude P1-P2 in R2-2508252, P1-P2 in R2-2508264, P1, P5 in R2-2508362, P1-P4 in R2-2509124, P1-P2 in R2-2508704, P1, P3 in R2-2508888. If still have time, can also handle remaining other MAC issues.

Intended outcome: Discussion summary in R2-2509313.

Deadline: 1000 – 1100AM Wednesday, in the main session => Completed.

## Approved outgoing LSs

R2-2509391	LS on per-band UE capabilities for LTM	LSout	To: RAN1	Rel-18	NR_Mob_enh2-Core

R2-2509338	LS on DCI 2_9 for SSB adaptation for deactivated SCells	LSout	To:RAN1, Cc:RAN4	Rel-19	Netw_Energy_NR_enh-Core

R2-2509331	LS on the signalling design of SCG LTM related parameters	LSout	To: RAN3	Rel-19	NR_Mob_Ph4-Core

R2-2509394	LS on support of LTM modification	LSout	To:RAN3	Rel-19	NR_Mob_Ph4-Core

### 7.0.2	Rel-18 corrections

### 7.0.0	In-principle agreed CRs

R2-2508088	Correction on the execution of SCG LTM	CATT	CR	Rel-18	38.331	18.7.0	5554	2	F	NR_Mob_enh2-Core	R2-2507712

R2-2508089	Correction on the execution of SCG LTM	CATT	CR	Rel-19	38.331	19.0.0	5558	2	F	NR_Mob_enh2-Core	R2-2507713

[Session chair]: Why does Rel-19 CR have different change than Rel-18 CR? [CATT]: It is intended and correct. [CATT]: Based on offline, Rel-19 CR cover page should be updated.

Rel-19 cover page will be updated in R2-2509314

Comeback in CB session

R2-2509314	Correction on the execution of SCG LTM	CATT	CR	Rel-19	38.331	19.0.0	5558	3	A	NR_Mob_enh2-Core	R2-2507713

CRs in R2-2508088 and R2-2509314 are agreed.

R2-2508886	Miscellaneous corrections on mobility enhancements	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.7.0	5540	2	F	NR_Mob_enh2-Core	R2-2507718

R2-2508887	Miscellaneous corrections on mobility enhancements	ZTE Corporation, Sanechips	CR	Rel-19	38.331	19.0.0	5541	2	A	NR_Mob_enh2-Core	R2-2507719

CRs in R2-2508886 and R2-2508887 are agreed.

R2-2509053	Corrections on validation of reported idle/inactive and reselection measurements	Samsung	CR	Rel-18	38.331	18.7.0	5519	2	F	NR_Mob_enh2-Core	R2-2507714

=> Revised in R2-2509129

R2-2509129	Corrections on validation of reported idle/inactive and reselection measurements	Samsung	CR	Rel-18	38.331	18.7.0	5519	3	F	NR_Mob_enh2-Core

R2-2509056	Corrections on validation of reported idle/inactive and reselection measurements	Samsung	CR	Rel-19	38.331	19.0.0	5564	1	A	NR_Mob_enh2-Core	R2-2507715

CRs in R2-2509129 and R2-2509056 are agreed.

R2-2509125	Corrections on Rel-18 UE capabilities for LTM	Huawei, HiSilicon	CR	Rel-18	38.306	18.7.0	1363	3	F	NR_Mob_enh2-Core

R2-2509126	Corrections on Rel-18 UE capabilities for LTM	Huawei, HiSilicon	CR	Rel-19	38.306	19.0.0	1364	3	A	NR_Mob_enh2-Core

CRs in R2-2509125 and R2-2509126 are agreed.

#### 7.0.2.22	Further NR mobility enhancements

(NR_Mob_enh2-Core; leading WG: RAN2; REL-18; WID:RP-233970)

R2-2508016	LS on per-band UE capabilities for LTM (R1-2508124; contact: Ericsson)	RAN1	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2

Noted.

R2-2508708	Correction on per-band UE capabilities for LTM	Ericsson	CR	Rel-18	38.306	18.7.0	1386	-	F	NR_Mob_enh2-Core

R2-2508709	Correction on per-band UE capabilities for LTM	Ericsson	CR	Rel-19	38.306	19.0.0	1387	-	A	NR_Mob_enh2-Core

[Huawei]: Would like to change it into “if the UE indicates this capability in one band of a FR, it indicates the same capability values for all supported bands in that FR”. [Nokia]: Want to understand what’s the problem with the current version, why should it be same capability values for all bands in a FR? [Ericsson]: It was a compromised in RAN1 discussion. [Huawei]: Cover page may need to be updated. There would be inter-operability issue if NW implements this CR but UE not.

Changes will be replaced by “if the UE indicates this capability in one band of a FR, it indicates the same capability values for all supported bands in that FR”

Cover page will be updated.

Comeback (R2-2509315 and R2-2509316)

R2-2509315	Correction on per-band UE capabilities for LTM	Ericsson	CR	Rel-18	38.306	18.7.0	1386	1	F	NR_Mob_enh2-Core

R2-2509316	Correction on per-band UE capabilities for LTM	Ericsson	CR	Rel-19	38.306	19.0.0	1387	1	A	NR_Mob_enh2-Core

CRs in R2-2509315 and R2-2509316 are agreed.

R2-2508361	On per-band UE capabilities for Rel-18 LTM	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

Noted.

R2-2508486	Clarification on LTM candidate TCI state deactivation upon reconfiguration	MediaTek Inc.	CR	Rel-18	38.321	18.7.0	2142	-	F	NR_Mob_enh2-Core

R2-2508487	Clarification on LTM candidate TCI state deactivation upon reconfiguration	MediaTek Inc.	CR	Rel-19	38.321	19.0.0	2143	-	A	NR_Mob_enh2-Core

[ZTE]: Do not consider it is essential correction. Note we have similar sentence for other MAC CE for non-LTM purpose, so if we change here, we may need to change all other places too. [MediaTek]: Cases are different, the other legacy MAC CE is all about the current serving cell while this MAC CE applies to candidate cells. Do not understand why legacy MAC CE also needs the similar change with this correction. [Vivo]: Understand the current wording has same intention (covering all cases with TCI state ID level). [Ericsson]: Agree with the proposal. The case when NW reconfigures one of TCI states was not covered in the current text. [Vivo]: Ok with the proposed change since we understand the current text aims same intention. [Huawei]: Understand it is same situation between the serving cell and candidate cell. Then do not understand why we need correction only for candidate cell case. [Ericsson]: The difference is for the candidate cell case, it is used for early sync, then if we start from scratch for all TCI states whenever a certain TCI state is reconfigured, it will make early sync less meaningful. [Samsung]: Considering this is functional change, ok not to agree with it. [Nokia]: With this change, we may also need to check RAN4. Understand for both serving cell and candidate cell cases, the requirement how long the UE needs to maintain the DL synchronization would be same. [Vivo]: It would be good to have some offline to have common understanding regarding what the current text intends for and which direction we intend for.

Offline (MediaTek) and discussion summary in R2-2509325 (if needed).

Comeback in CB session

R2-2509325	Scope of LTM candidate TCI state deactivation	MediaTek	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: Describe deactivation behavior for (i) reconfiguration and (ii) non-LTM mobility separately, with clear granularity for each scenario.

Describe deactivation behavior for (i) reconfiguration and (ii) non-LTM mobility separately, with clear granularity for each scenario.

Proposal 2: For case1 (upon reconfiguration), RAN2 to down select from the following options:

2a: the granularity of deactivation is at TCI state level. (Only the reconfigured TCI states are deactivated).

2b: the granularity of deactivation is at cell level. (TCI states of the candidate cell with reconfigured TCI states are deactivated).

[MediaTek]: Although most companies are ok with either 2a or 2b, would like to suggest 2b for safer way since some companies expressed concern on 2a complication.

For case1 (upon reconfiguration), the granularity of deactivation is at cell level. (TCI states of the candidate cell with reconfigured TCI states are deactivated).

Proposal 3: For case2 (upon non-LTM mobility), all candidates TCI states for all candidate cells are deactivated.

For case2 (upon non-LTM mobility), all candidates TCI states for all candidate cells are deactivated.

R2-2509333	Clarification on the scope of LTM candidate TCI state deactivation	MediaTek Inc.	CR	Rel-18	38.321	18.7.0	2142	1	F	NR_Mob_enh2-Core

R2-2509334	Clarification on the scope of LTM candidate TCI state deactivation	MediaTek Inc.	CR	Rel-19	38.321	19.0.0	2143	1	A	NR_Mob_enh2-Core

CRs in R2-2509333 and R2-2509334 are agreed.

R2-2508488	Corrections on NRDC UE capabilities for LTM	MediaTek Inc.	CR	Rel-18	38.306	18.7.0	1374	-	F	NR_Mob_enh2-Core

R2-2508489	Corrections on NRDC UE capabilities for LTM	MediaTek Inc.	CR	Rel-19	38.306	19.0.0	1375	-	A	NR_Mob_enh2-Core

[ZTE]: Intention is correct, but by removing “when LTM cell switch command MAC CE is received”, it can cover both cases. [MediaTek]: We discussed it and if we remove “when LTM cell switch command MAC CE is received”, that will also include C-LTM, which is not correct. [Ericsson, Nokia, CATT]: Ok with the correction. [MediaTek]: We may need similar change for some other capability. [OPPO]: The other capability that MediaTek mentioned is for Rel-19 UE capability, so it’s ok to agree this CR now. [MediaTek]: Work item code needs to be corrected.

Work item code should be corrected.

With the correction above, CRs in R2-2509320 and R2-2509321 are agreed.

R2-2508710	Misc corrections for LTM and SCPAC	Ericsson	CR	Rel-18	38.331	18.7.0	5591	-	F	NR_Mob_enh2-Core

R2-2508711	Misc corrections for LTM and SCPAC	Ericsson	CR	Rel-19	38.331	19.0.0	5592	-	A	NR_Mob_enh2-Core

[Ericsson]: We do not need a second change that was already covered in another CR.

Remove the second change from the cover page and correction

With the correction above, CRs in R2-2509322 and R2-2509323 are agreed.

R2-2508891	Correction on SR handling during RACH-less LTM and RACH-less HO	ZTE Corporation, Sanechips	CR	Rel-18	38.321	18.7.0	2148	-	F	TEI18, NR_Mob_enh2-Core

R2-2508892	Correction on SR handling during RACH-less LTM and RACH-less HO	ZTE Corporation, Sanechips	CR	Rel-19	38.321	19.0.0	2149	-	A	TEI18, NR_Mob_enh2-Core

[MediaTek, Apple]: Since it is functional change and consider it is a corner case to be covered, do not want to agree with it unless it is essentially needed.

Not pursued.

R2-2508712	Clarification on early RACH capabilities for LTM	Ericsson	CR	Rel-18	38.306	18.7.0	1399	-	F	NR_Mob_enh2-Core	Late

For the first change:

[MediaTek]: There are two cases, i) PRACH bandwidth is outside of any configured UL BWP and ii) PRACH bandwidth is inside of any configured UL BWP, with this change, it is not for i) specific anymore and it will become umbrella of i) and ii), which is not correct. [ZTE]: Agree with MediaTek. [MediaTek]: Understand for ii) case, the value will be set to zero. [Ericsson]: Ok with MediaTek’ proposal (i.e. setting the value to zero for case ii). [Session chair]: Can RAN2 make decision, or should we ask RAN4 for confirmation? [Ericsson, MediaTek, Vivo]: Ok to make decision in RAN2 since we already discussed with RAN4 on this aspect.

For the second change,

[MediaTek]: We need to be cautious on this change since it was not included in RAN1 LS. [Ericsson]: Suggest to send a LS to RAN1 to ask whether we also need similar change for other capabilities or not. [Huawei, Nokia]: Reluctant to send a LS to RAN1 asking other parameters unless we identify the problem of other parameters. [ZTE]: Support sending a LS to RAN1 including RAN4. [Session chair]: Since this request was done by RAN1 discussion, there would be no harm to check with RAN1 for other capabilities that have similar characteristics.

For the first change, the updated CR in R2-2509317 and R2-2509318 will be provided. Comeback in CB session.

Send a LS to RAN1/4 ask, on the other capabilities indicated by “the capability refers to the source band” in per band signalling, whether the current text is ok or whether we need similar change as the ones in the RAN1 LS. LS in R2-2509319 will be provided (Ericsson). Comeback in CB session.

R2-2509317	Clarification on early RACH capabilities for LTM	Ericsson	CR	Rel-18	38.306	18.7.0	1399	1	F	NR_Mob_enh2-Core

R2-2509318	Clarification on early RACH capabilities for LTM	Ericsson	CR	Rel-19	38.306	19.0.0	1400	-	A	NR_Mob_enh2-Core

[ZTE]: In pdcch-RACH-Switching-TargetBandTimeList-r18, “TS 38.133 [5], clause 6.2.2C.2.” should be correct to “TS 38.133 [5], clause 8.2.2.2.20.”

In pdcch-RACH-Switching-TargetBandTimeList-r18, “TS 38.133 [5], clause 6.2.2C.2.” should be correct to “TS 38.133 [5], clause 8.2.2.2.20.”

With the correction above, CRs in R2-2509339 and R2-2509340 are agreed.

R2-2509319	[Draft] LS on per-band UE capabilities for LTM	Ericsson	[Draft]LSout	To: RAN1	Rel-18	NR_Mob_enh2-Core

LS in R2-2509391 is approved.

[Nokia]: After checking with RAN1, three parameters were also discussed in RAN1 when RAN1 sent the related LS to RAN2. With that consideration, do not see a need to send a LS to RAN1. [Ericsson]: If RAN1 already made such decision, then RAN1 will easily respond that no change is needed for these three parameters.

## 8.5	Network Energy Saving Enh.

(Netw_Energy_NR_enh-Core; leading WG: RAN1; REL-19; WID: RP-242354)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 8.5.1	Organizational

Incoming LS, CR rapporteurs’ inputs (including in-principle agreed CRs), etc.

Endorsed/in-principle agreed CR:

R2-2508514	Network Energy Savings Enhancements miscellaneous stage-2 corrections	Huawei, HiSilicon	CR	Rel-19	38.300	19.0.0	1042	1	F	Netw_Energy_NR_enh-Core	R2-2507273

Endorsed.

[POST132][104][NES] (Huawei)

Scope: Update 38.300 CR capturing the agreements made this meeting.

Intended outcome: 38.300 CR in R2-2509326 to be agreed.

Deadline: Short email discussion.

R2-2508696	Miscellaneous MAC corrections for R19 NES	InterDigital	CR	Rel-19	38.321	19.0.0	2127	2	F	Netw_Energy_NR_enh-Core	R2-2507769

Endorsed.

[POST132][105][NES] (IDC)

Scope: Update 38.321 CR capturing the agreements made this meeting.

Intended outcome: 38.300 CR in R2-2509327 to be agreed.

Deadline: Short email discussion.

R2-2509089	NES Comments File	Ericsson	report	Rel-19	Netw_Energy_NR_enh-Core	Late

R2-2509090	Conclusions for NES RILs	Ericsson	report	Rel-19	Netw_Energy_NR_enh-Core	Late

Propose

PropAgree: S061, Z104, Z106 are agreed.

PropReject: J070, O008, Z105 are rejected.

ToDo that is not related to [Post131bis][301][NES]: S062, S063, F001, F002, O009

TODo that is related to [Post131bis][301][NES]: A103, E023, E024, E025, H131, J001, J005, N001, V503, X200, X201, Z101, Z102

[AT132][101][NES] (Ericsson)

Scope: Discuss and conclude remaining ToDo RILs (including S062, S063, other RILs that needs further discussion after conclusion on [POST131bis][301][NES], and new issues included in the contributions (P1 and P3 in R2-2508242). + J071, V504, V505

Intended outcome: Discussion summary in R2-2509311.

Deadline: Offline discussion 16:00 – 17:00, Tuesday, in the breakout room 1 => Completed.

R2-2509311	[AT132][101][NES] RRC open issues (Ericsson)	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: For OD-SSB, the association between ssb-ToMeasure and od-ssb-PositionsInBurst is up to network implementation.

Proposal 2: For SSB adaptation, the association between ssb-ToMeasure and od-ssb-PositionsInBurst is up to network implementation.

Proposal 3: For the description of condition ODssbAOssb;

- The field is mandatory present when absoluteFrequencySSB is absent.

- For the same frequency case, the field is absent. (i.e., absoluteFrequencySSB is present)

- For the different frequency case, the field is optionally present. (i.e., absoluteFrequencySSB is present)

Proposal 4: Rename cond Diff_Freq as MeasObjectOD and adopt the following TP: “This field is mandatory present if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell or if absoluteFrequencySSB is absent and OD-SSB is configured in IE CellGroupConfig. It is absent otherwise.”

Proposal 6: [V504] is agreed.

Proposal 7: [V505] is agreed

Proposal 8: [S062] is agreed.

Proposal 9: [S063] is agreed.

Proposal 10: Fallback from stmc7 should be possible only to smtc1 and smtc2.

Proposal 11: [X200], [X201], [A103], [N001], [J001], [J005], [E023], [E024], [E025], [H131], [Z101], [Z102] are rejected.

All proposals above are agreed.

[Nokia]: Ok with proposal 10, then we may need to consider the same approach for smtc6.

Whether/how to apply same approach for smtc6 can be handled as part of POST email discussion on RRC CR.

Proposal 5: [J071] is rejected, and the discussion is postponed to the next meeting.

[J071] is rejected.

[OPPO, Vivo]: We don’t need to postpone it to the next meeting.

R2-2509093	Corrections for Network Energy Saving	Ericsson	CR	Rel-19	38.331	19.0.0	5559	2	F	Netw_Energy_NR_enh-Core	R2-2507783	Late

[Ericsson]: Already captured PropAgree J071, S061, Z104, Z106.

Endorsed except the change of J071.

[POST132][106][NES] (Ericsson)

Scope: Update 38.331 CR capturing the agreements made this meeting (also including an update of RIL conclusion).

Intended outcome: 38.331 CR in R2-2509329 to be agreed and updated RIL conclusion in R2-2509392.

Deadline: Short email discussion.

R2-2508992	Correction on NES UE capability for paging adaption	ZTE Corporation, Sanechips, Ericsson	CR	Rel-19	38.306	19.0.0	1396			F	Netw_Energy_NR_enh-Core

Agreed.

### 8.5.2	Corrections

Remaining essential corrections (including the remaining issues related to RILs).

[X200/X201/A103/N001/J001/J005/E023/E024/E025/H131/V503/Z101/Z102]: ToDo

R2-2508480	Summary report of [POST131bis][301][NES] OD-SSB and CSI-RS measurements	Xiaomi	discussion	Netw_Energy_NR_enh-Core

Proposal 1: RAN2 agree to extend the number of OD-SSB specific SMTC from 6 to 16 with each OD-SSB configuration corresponding to one SMTC.

Agreed.

Proposal 2: RAN2 agree the procedure text on OD-SSB/CSI-RS measurement and SMTC setup for OD-SSB/SSB adaptation.

TP on the procedure text on OD-SSB/CSI-RS measurement and SMTC setup for OD-SSB/SSB adaptation is endorsed as the baseline.

We can discuss further wordings as part of offline discussion [101].

[V503]: ToDo

R2-2508086	Remaining issues of NES	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: Introduce two new UE capabilities to indicate the support of: 1) intra-band SSB-less SCell&OD-SSB on SCell; 2) inter-band SSB-less SCell&OD-SSB on SCell; respectively. TPs for TS 38.306/38.331 are provided in Annex 5.1/5.2.

[Ericsson]: What is difference compared to the UE sets both capabilities as true? [OPPO]: Although it is a capability for cross features, do not see the real need of a new combined capability. For example, for RedCap UE + paging bundlings, we do not have a separate new combined capability. [Apple, Huawei]: Agree with OPPO and Ericsson. Understand no real new functionality for the new combined capability is required compared to the case the UE sets both capabilities as true.

V503 is rejected.

[F001, F002]: ToDo

R2-2509130	Corrections to SIB1 request [F001] [F002]	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1 [F001]: Modify RRC specification so that the UE can determine whether the serving cell is configured with supplementary uplink upon SIB1 request, at least according to the supported band(s).

[OPPO]: Do not consider it is essential correction. [Apple]: Agree with OPPO and it seems only part of conditions were covered in the proposed TP. [Xiaomi, CATT]: Support the intention. [Samsung]: Understand all SUL carrier selection criteria are covered by RRC. [OPPO]: Understand the intention is technically correct, but it is questionable if this is essential change. [Vivo]: Bandwidth checking is also needed for OD-SIB1 REQ case. [Session chair]: It would be good to have offline discussion for better understanding and whether it is essential correction or we can survive without it.

Comeback in Tuesday CB session

Comeback:

[Fujitsu]: During offline discussion, companies considered it’s a valid issue. Also get impression that companies agreed with the need for correction. [Samsung]: Considering we also have similar condition for NUL, we would like to have more time to think how to correct both cases. [Apple]: Agree with the intention and agree with Samsung (want to have more time). [Apple]: Understand it’s ok to open and resolve it next meeting. [Ericsson]: Ok to handle it as part of POST email discussion, but prefer having it as a separate POST email discussion.

Intention is agreed.

How to implement will be further discussed and TP in R2-2509330 will comeback in CB session (Fujitsu).

Proposal 2 [F002]: Modify RRC specification so that the UE applies the first listed band which it supports for the selected uplink upon SIB1 request.

[Apple] [Xiaomi]: It is already covered by the corresponding field description, so it is not needed.

F002 is rejected.

Proposal 3 [F001/F002]: Adopt the TP as below:

R2-2509330	Report of [AT132][xxx][NES] Corrections to SIB1 request (Fujitsu)	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal (modified): The revised TP should be agreed as baseline. Further re-wordings can be done as part of POST email discussion on RRC CR.

The revised TP is agreed as baseline. Further re-wordings can be done as part of POST email discussion on RRC CR.

[O009]: ToDo

R2-2508116	Left Issues on On-Demand SSB and OD-SIB1 (O008, O009)	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal 2: [O009] Revise the R2#131bis agreement “servingcellMO is not configured for OD-SSB measurement in Case1 SCell and OD-SSB specific MO is used for both CSI-RS measurements and OD-SSB measurements after OD-SSB is activated.” as “servingcellMO can be configured for CSI-RS measurement only in Case1 SCell and OD-SSB specific MO is used for both CSI-RS measurements and OD-SSB measurements after OD-SSB is activated.”, and revise the spec accordingly.

[Xiaomi, Apple]: Since the CR already captured it correctly, we don’t need to agree any additional one this meeting. [OPPO]: It is missed in the field description, as shown in TP.

Agree that TP is the baseline. Detailed rewording can be further discussed as part of POST email discussion on RRC CR.

O009 is agreed.

[C184]: Rejected

R2-2508091	[C184]Impact of od-ssb-PositionsInBurst on ssb-ToMeasure	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: The ssb-ToMeasure should adapt dynamically to the changes in OD-SSB-PositionsInBurst. TP in annex is adopted.

[Ericsson]: Already included this issue into offline discussion.

Will be further discussed as part of offline discussion [AT132][101][NES].

[H130]: Rejected

R2-2508242	[N001][N003] Remaining issues on OD-SSB and SSB adaptation	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 6: Confirm that the UE can receive common DCI format 2_9 for SSB adaptation for a deactivated SCell but it shall not impact the SMTC for measurement for the deactivated SCell.

[OPPO]: Wonder if there is any RAN2 spec impact with this proposal. [Nokia]: We can clarify it in the corresponding field description. [Qualcomm]: Agree with the proposal but prefer no RAN2 spec impact. [Samsung]: Prefer changing “it shall not impact the SMTC for measurement for the deactivated SCell” to “the UE shall ignore it for the deactivated SCell”. [Nokia]: Ok with Samsung proposal, but think it should be captured somewhere in the spec. [Apple]: Agree with Nokia and we can consider capturing it into stage-2 spec. [Qualcomm]: Support Nokia.

Confirm that the UE can receive common DCI format 2_9 for SSB adaptation for a deactivated SCell but the UE shall ignore it for the deactivated SCell.

Will capture this clarification in stage 2 spec.

Send LS to RAN1 (Cc: RAN4) to inform this RAN2 agreement. LS in R2-2509328 (Nokia). Comeback in CB session.

R2-2509328	[DRAFT] LS on DCI 2_9 for SSB adaptation for deactivated SCells	Nokia	[Draft]LSout	To:RAN1, Cc:RAN4	Rel-19	Netw_Energy_NR_enh-Core

LS in R2-2509338 is approved.

Others:

R2-2508242	[N001][N003] Remaining issues on OD-SSB and SSB adaptation	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: Allow direct SSB adaptation activation with RRC configuration (similarly to SCell activation and on-demand SSB).

[Nokia]: Understand this proposal 4 will make sense since SSB adaption activation is not applied once the SCell is deactivated. [Apple]: Understand legacy direct SCell activation can cover this case. [Session chair]: What’s alternative way? W/o this proposal, we cannot survive? [Nokia]: W/o this proposal, NW can send SCell activation MAC CE then send SSB adaptation activation. Can survive, but not efficient. [Qualcomm]: Understand RAN1 didn’t agree RRC-based option for SSB adaptation. Prefer not having this one in this late phase.

Rejected.

R2-2508456	Remaining issues on Rel-19 NES	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8: To align with the agreement that legacy MO is applied when OD-SSB and AO-SSB are in same frequency, make the following correction on NOTE 2b of Running CR of TS 38.300:

NOTE 2b (modified):	The above measurement object associated with the serving cell refers to the serving cell measurement object for OD-SSB when SSB is absent and OD-SSB is activated, or when SSB and OD-SSB have different frequency and OD-SSB is activated. Ootherwise, it refers to the serving cell measurement object for SSB.

[OPPO]: Understand the sentence of “and OD-SSB is activated” is also needed when SSB and OD-SSB have different frequency.

TP is agreed and will be included in the CR.

[O008]: PropReject

R2-2508116	Left Issues on On-Demand SSB and OD-SIB1 (O008, O009)	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

Proposal 1: [O008] R2 clarify R19 OD-SIB1 capable UE perform SIB1 acquisition upon SIB change notification, regardless of od-sib1-Config availability, and revise the spec accordingly.

Covered by RIL conclusion.

MAC correction:

R2-2508996	Discussion on remaining issues in MAC for R19 NES	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: Clarify in the NR MAC specs that when both sib1-RA-AssociationPeriodIndex and sib1-RequestPeriod are configured, the index of association period shall be determined by sib1-RA-AssociationPeriodIndex.

Proposal 2: Align the NR MAC specs with the NR RRC specs by replacing ra-ssb-OccasionMaskIndexSib1 with sib1-RA-SSB-OccasionMaskIndex.

P1 and P2 are agreed and will be included into CR.

R2-2508283	[C184] Discussion on remaining RRC issues	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2508481	Remaining open issues for NES	Xiaomi	discussion	Netw_Energy_NR_enh-Core

R2-2508675	Maintenance for R19 NES 	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core	Late

R2-2508753	Discussion on remaining issues	Qualcomm Incorporated	discussion

R2-2508378	Remaining issues on Rel-19 NES	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core	Withdrawn

## 8.6	Mobility Enhancement Ph4

(NR_Mob_Ph4-Core; leading WG: RAN2; REL-19; WID: RP-252111)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

### 8.6.1	Organizational

Incoming LS, CR rapporteurs’ inputs (including in-principle agreed CRs), etc.

Endorsed/in-principle agreed CR:

R2-2508385	Miscellaneous corrections for stage-2 in Rel-19 Mobility Enhancements	Apple Inc	CR	Rel-19	38.300	19.0.0	1040	2	F	NR_Mob_Ph4-Core	R2-2507723

Agreed.

[Ericsson]: Figure for Inter-CU LTM has really poor quality. Want to enhance the quality. [Apple]: This figure is not part of this CR. But promise to enhance the quality in future. [Ericsson]: Note the figure should be also editable.

R2-2508251	Miscellaneous corrections on MAC for Mob Ph4	vivo (Rapporteur)	CR	Rel-19	38.321	19.0.0	2123	2	F	NR_Mob_Ph4-Core	R2-2507728

Endorsed.

[POST132][108][MOB] (Vivo)

Scope: Update 38.321 CR capturing the agreements made this meeting.

Intended outcome: 38.321 CR in R2-2509336 to be agreed.

Deadline: Short email discussion.

R2-2508713	Mobility Comments file	Ericsson	report	Rel-19	NR_Mob_Ph4-Core

=> Revised in R2-2509104

R2-2509104	Mobility Comments file	Ericsson	report	Rel-19	NR_Mob_Ph4-Core

R2-2508714	Mobility RILs conclusions	Ericsson	report	Rel-19	NR_Mob_Ph4-Core

=> Revised in R2-2509105

R2-2509105	Mobility RILs conclusions	Ericsson	report	Rel-19	NR_Mob_Ph4-Core

PropAgree (modified): Z166, X157, C167, X156, Z167, M205, O601, H154, H153, H157, H158, H156, C168, N142, S036 are agreed.

PropReject (modified): E054 is rejected.

ToDo: E061, E055, C157, H159, H155, Z165

[Ericsson]: For Z165, propose to handle it as part of post email discussion since it is related to RAN3 signaling and discussion. [ZTE]: We still need to discuss to make basic principle for the case when RAN3 does not include the indicated missing parameters. [

If Z165 is agreed, only missing parameters from RAN3 signaling will be included in INM.

Will send an LS to RAN3 to address Z165 and take it into account for RAN3 job. (ZTE, R2-2509324). LS comeback in Wednesday session.

R2-2509324	[Draft] LS on the signalling design of SCG LTM related parameters	ZTE	LSout	To: RAN3	Rel-19	NR_Mob_Ph4-Core

Meeting number should be corrected.

“signallings” should be changed “signalling”.

“However, if RAN3 decides not to do so, then RAN2 will add these IEs to inter-node message (i.e. CG-Config)” will be changed to “Note RAN2 has placed holders in INM and will take further actions dependent on RAN3.”.

With the corrections above, the LS in R2-2509331 is approved.

Z165 will be now implemented as part of POST email discussion.

[Nokia]: Prefer waiting for RAN3 response before taking any further change in RAN2 spec. [Ericsson, ZTE]: Prefer implementing Z165 now and see whether we need to take further action (dependent on RAN3 response) later.

[AT132][102][MOB] (Ericsson)

Scope: Discuss and conclude remaining PropReject [N141], [B123], [H160], [O602], [C166]

Intended outcome: Discussion summary in R2-2509312.

Deadline: Comeback in Wednesday CB session => Completed

R2-2509312	Summary of [AT132][102][MOB] (Ericsson)	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1	C166 is to be discussed online.

Proposal 2	O602 is Rejected. We rely on the network to set the header compression parameters 	for an LTM candidate configuration.

Proposal 3	For H160, we send an LS to RAN1 to ask whether the network expect to configure only one element within ltm-CSI-IM-ResourceToAddModList. H160 is marked as Agreed.

Proposal 4	N141 is Rejected. We do not support delta signalling for the CLTM execution conditions (no changes to current specification).

Proposal 5	B123 is Rejected. We do not optimize the case when the UE may start evaluating the CLTM execution conditions before an LTM cell switch execution is completed.

Proposal 2, 3, 4, and 5 above are agreed.

C166 is agreed.

[POST132][107][MOB] (Huawei)

Scope: Prepare LS to RAN1 according to P3 in R2-2509312.

Intended outcome: LS in R2-2509335 to be approved.

Deadline: Short email discussion

[AT132][103][MOB] (Vivo)

Scope: Discuss and conclude P1-P2 in R2-2508252, P1-P2 in R2-2508264, P1, P5 in R2-2508362, P1-P4 in R2-2509124, P1-P2 in R2-2508704, P1, P3 in R2-2508888. If still have time, can also handle remaining other MAC issues.

Intended outcome: Discussion summary in R2-2509313.

Deadline: 1000 – 1100AM Wednesday, in the main session => Completed.

R2-2509313	Discussion report on [AT132][103][MOB] MAC corrections (vivo)		Vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1 (modified): RAN2 to agree the below offline agreement:

	If an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed from or modified in the current UE configuration, the MAC entity shall cancel the triggered L1 measurement report associated with the ltm-CSI-ReportConfig.

	Update “i.e. the RS configured in the indicated TCI State of the SpCell,” to include both SSB and CSI-RS in the procedure and the description of serving RS for Event LTM2/3/5.

	Other editorial change in R2-2508264 will be fixed.

	The missing cases in P1 in R2-2508362 will be baseline, and further update will be included/discussed as part of POST email discussion on MAC CR.

	P1 in R2-2508704 will be taken as the baseline for MAC CR update.

	RAN2 to agree the deactivation of SP CSI-RS for L1-RSRP from the target cell after reconfiguration with sync.

	RAN2 to clarify that the CSI Resource Configuration ID2 is only used to indicate the CSI-IM resource set, i.e. remove “CSI-RS and” from the sentence “SP CSI-RS and CSI-IM resource set for the candidate cell(s) associated with the CSI Resource Configuration ID2 in the same octet”.

	RAN2 to clarify that the TCI-state indicated in the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE refers to TCI-state provided in the IE LTM-TCI-Info.

All proposals above are agreed.

Additional proposal: CR rapporteur will consider P2 in R2-2509124, P3 and P4 in R2-2509124 in the next round of CR update and review.

Agreed.

R2-2508715	Corrections on RRC for mobility enhancements Phase 4	Ericsson	CR	Rel-19	38.331	19.0.0	5593	-	F	NR_Mob_Ph4-Core

=> Revised in R2-2509107

R2-2509107	Corrections on RRC for mobility enhancements Phase 4	Ericsson	CR	Rel-19	38.331	19.0.0	5593	1	F	NR_Mob_Ph4-Core

Endorsed.

[POST132][109][MOB] (Ericsson)

Scope: Update 38.331 CR capturing the agreements made this meeting (also including an update of RIL conclusion).

Intended outcome: 38.331 CR in R2-2509337 to be agreed and updated RIL conclusion in R2-2509393.

Deadline: Short email discussion.

R2-2509106	Corrections on RRC for mobility enhancements Phase 4	Ericsson	CR	Rel-19	38.331	19.0.0	5625	-	F	NR_Mob_Ph4-Core	Withdrawn

### 8.6.2	Corrections

Remaining essential corrections (including the remaining issues related to RILs).

[C157]: ToDo

R2-2508090	[C157][C166] Discussion on RILs for Mob 	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: [C157] The ltm-TimeAlignmentTimerTag2-r19 in EarlyUL-SyncConfig-r18 is optionally present if tag2 is present in the SpCellConfig in ltm-CandidateConfig. Adopt the TP in Annex 1.

[OPPO]: Ok with the proposal, but we need a need code. [Ericsson]: Ok with the proposal.

C157 is agreed.

Need code is up to RRC CR rapporteur, which can be discussed as part of POST email discussion on RRC CR.

[E061][E055]: ToDo, ToDo

R2-2508716	Remaining issues for LTM [E061, E055]	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: RAN2 to send an LS to RAN3 to ask to implement the necessary signalling in order to support the proper modification of an LTM candidate (i.e., not relying on the LTM cancel procedure).

Agreed.

Proposal 2: For the PDCP discarding on SRBs, the principle from Rel-18 is followed where the legacy RRC field discardOnPDCP is used. RAN2 to agree on the TP provided in the RIL.

[CATT]: The current text is aligned with RAN2 agreement (“For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.”). With relying on discardOnPDCP, NW cannot distinguish whether intra-CU or inter-CU will happen when NW configured LTM candidate configuration. Prefer sticking the current text. [Ericsson]: Do not see any issue since NW can handle it. [Ericsson]: With the current text, understand the subsequent LTM is not supported. [Huawei]: With the current text, Rel-19 UE implementing only Rel-18 LTM feature may have problem. [Vivo]: Understand Rel-19 UE implementing only Rel-18 LTM feature should follow Rel-18 spec for LTM function. [Ericsson]: No.

Offline (Ericsson)

Comeback in the main session Friday.

R2-2509332	[Draft] LS on support of LTM modification	Ericsson	[Draft]LSout	To:RAN3	Rel-19	NR_Mob_Ph4-Core

[CATT] [Nokia]: Would like to remove “In fact, modifying LTM-related configuration means that candidate node shall be able to signal to serving node either a “toAddModList” field or “toReleaseList” field (or both) without having to release the whole LTM candidate configuration.” since it is related to RAN3 job. [Ericsson]: When it was discussed in RAN3, it seems they did not understand what AddModList and ReleaseList means. This sentence is key point to inform RAN3. [Samsung]: Agree with Ericsson. [Nokia]: Ok with the sentence “RAN2 understand that modifying LTM-related configuration means that candidate node shall be able to signal to serving node either a “toAddModList” field or “toReleaseList” field (or both) without having to release the whole LTM candidate configuration.” [Huawei]: Ok with Nokia’s suggestion.

“In fact, modifying LTM-related configuration means that candidate node shall be able to signal to serving node either a “toAddModList” field or “toReleaseList” field (or both) without having to release the whole LTM candidate configuration.” will be changed to ““RAN2 understand that modifying LTM-related configuration means that candidate node shall be able to signal to serving node either a “toAddModList” field or “toReleaseList” field (or both) without having to release the whole LTM candidate configuration.”

Remove “of initiating a cancellation procedure and” from “In this case, the candidate node shall be able to add, modify, or release any of the listed parameters and the ltm-CandidateConfig-r18 without the need of initiating a cancellation procedure and re-inititate the LTM preparation from start.”

With the changes above, LS in R2-2509394 is approved.

[H155][H159]: ToDo, ToDo

R2-2509124	LTM corrections	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5a: To allow including new fields in LTM-ReportContent-v1900 in a later release and omit reportQuantity-r19, make reportQuantity optional.

Agreed.

Proposal 5b: To avoid issues with Rel-19 UEs that only support Rel-18 LTM and will not understand reportQuantity-r19, remove the value ssb-index-RSRP from reportQuantity-r19 and capture that absence means that SSB index and RSRP are to be reported.

Remove the value ssb-index-RSRP from reportQuantity-r19 and set a default value as ssb-index-RSRP.

Proposal 7: Define EarlyCSI-ReportConfig in LTM-Candidate with the only 5 fields required for early CSI report, and remove the fields specific to early CSI report from LTM-CSI-ReportConfig (whose IE description does not cover early CSI report).

Offline (Huawei)

Comeback in CB session.

Comeback:

[Huawei]: Updated TP is provided in R2-2509383.

R2-2509383	LTM corrections	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

TP is agreed as baseline. Further detailed correction can be discussed as part of POST email discussion on RRC CR.

MAC Corrections:

R2-2508704	LTM MAC remaining issues 	Qualcomm Incorporated	discussion

Proposal 3:  RAN2 to agree replacing “A/D” to “IM” in Octet 2 for the explicit indication of the presence of CSI Resource Configuration ID2 and adopt the TP for clause 6.1.3.12a in Annex B.2.

[Session chair]: One bit A/D is enough? [Qualcomm]: Understand for CSI reporting purpose, we need both CSI resource configuration ID1 and ID2, thus single A/D should be enough and it can indicate activation/deactivation for both. [Session chair]: It would be good to clarify it in A/D description.

TP is agreed as baseline. Further correction/update can be done as part of POST email discussion on MAC CR.

R2-2508888	Discussion on LTM remaining issues	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5	RAN2 to discuss how to handle the case if a Type 1 or 2 beam is reported in the truncated MR MAC CE and then meets the leaving or entering condition, with the following options:

•	Option 1: The UE removes beam(s) reported in the truncated MR MAC CE from the corresponding beam list(s), i.e. to revise the previous agreement;

•	Option 2: If a Type 1 or 2 beam is reported in the truncated MR MAC CE and then meets the leaving or entering condition, the UE shall not cancel the triggered MR, and report the beam in the subsequent MR MAC CE.

Proposal 5a	If option 1 is adopted, the UE shall use the regular MR MAC CE if all triggered beams (i.e. Type 1 and 2 beam) can be included in one MAC CE (even if there are remaining Type 3 or 4 beam), instead of the truncated MR MAC CE. To adopt the TP in Annex 2-1.

Proposal 5b	If option 2 is adopted, if a Type 1 beam reported in the truncated MR MAC CE and then meets the leaving condition, the UE shall move it from BEAM_ENTERING_LIST to BEAM_LEAVING_LIST; while, if a Type 2 beam reported in the truncated MR MAC CE and then meets the entering condition, the UE shall move it from BEAM_LEAVING_LIST to BEAM_ENTERING_LIST. To adopt the TP in Annex 2-2.

Proposal 6: RAN2 confirms that the UE shall not initiate the L1 measurement reporting if the BEAM_ENTERING_LIST or BEAM_ LEAVING _LIST is empty, i.e. add the condition in clause 5.35.3.1 for triggering measurement reporting and remove Note 3 in clause 5.35.4. To adopt TP in Annex 3.

[Qualcomm, Apple, LG]: To use truncated MR is to handle a kind of corner case, but now design of truncated MR becomes very complicated. Prefer not to have more optimization. [LG]: Even in the concerned case, if NW doesn’t receive following full MR, then it can guess the situation. [Nokia, Ericsson]: Ok with option 1, but also understand Apple’s concern, so to do nothing is also acceptable if most companies want.

Not pursued.

R2-2508252	Discussion on open issues for R19 mobility	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2508264	Corrections to L1 event triggered measurement reporting	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2508362	Miscellaneous Release 19 LTM corrections	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2508664	LTM corrections	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

=> Revised in R2-2509124

R2-2509124	LTM corrections	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2508704	LTM MAC remaining issues 	Qualcomm Incorporated	discussion

R2-2508888	Discussion on LTM remaining issues	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Others:

R2-2508441	Collision between PUSCH for early CSI and measurement gap	NEC	discussion	Rel-19	NR_Mob_Ph4-Core	R2-2507304

Proposal 1. Send LS to RAN1 to ask whether the UE transmits CSI report to the candidate cell using PUSCH during the measurement gap or not.

[Vivo]: Understand the UE cannot send all CSI report (including early CSI report during LTM cell switch procedure) during MG. [OPPO]: During LTM cell switch procedure, does MG mean MG configuration from the source cell or target candidate cell? [NEC]: Understand in this case, target candidate cell’s MG. [Vivo]: Understand it means source cell’s MG. [Session chair]: Understand collision with MG is more like RAN2 scope, no need to send a LS to RAN1 now.

Noted.

R2-2509049	RRC corrections on Rel-19 Mobility	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core	Withdrawn

## 9.3	AI/ML for mobility

(NR_AIML_Mob, leading WG: RAN2; REL-20; WID: RP-252899)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdoc

### 9.3.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2508211	Work plan for AI mobility WID	OPPO,Interdigital	discussion	Rel-20	NR_AIML_Mob-Core

Noted.

[Huawei]: Since we did many things during SI phase, we may start CR from the next meeting. [Huawei]: Do we still need confirmation/agreements on the conclusions derived during SI phase? [OPPO]: Yes. We can simply confirm that. [Vivo]: Starting CR from the next meeting is too early. [Huawei]: Ok with OPPO’s confirmation, however would like to avoid whole repetitions that were already done during SI phase.

### 9.3.2	RRM measurement prediction

LCM functionality management for RRM measurement prediction, including UE capability, applicability, inference configuration/reporting etc.   Contributions should focus on UE sided RRM measurement prediction for this meeting.

NOTE: No contributions expected on data collection and performance monitoring for this meeting.

1. General:

R2-2508212	Discussion on functionality management for RRM measurement prediction	OPPO	discussion	Rel-20	NR_AIML_Mob-Core

Proposal 1: SSB is taken as starting point as reference signal type for normative work in RAN2.

SSB is taken as starting point as reference signal type for normative work in RAN2.

[Xiaomi, Samsung]: Support the proposal 1. [Samsung]: Agree with that CSI-RS will bring a lot of RAN4 works. [LG]: Agree with Samsung.

Proposal 2: Only RSRP is specified in R20 as measurement quantity i.e. RSRQ and SINR are not specified in R20

Only RSRP prediction is specified in R20 as measurement quantity i.e. RSRQ and SINR are not specified in R20 unless we receive RSRQ/SINR support from RAN4.

[MediaTek]: Is the intention only for RRM measurement prediction, so predicted RSRQ can be used for event prediction? [OPPO]: Matter is not about whether to include RSRQ in MR. Matter is that during SI phase, we only considered RSRP. [Ericsson, ZTE]: Understand from RAN2 point of view, no much spec impact from supporting predicted RSRQ, but we need to wait for RAN4 inputs on their works. [Apple, Vivo]: Agree with Ericsson’s comment, and it means the realistic option is RSRP only.

Proposal 3: For AI mobility, SA (and CA) and NR-DC are supported network architecture

SA is taken as starting point as reference architecture for normative work in RAN2. FFS on NR-DC.

[Samsung]: Would like to focus on SA first. Then once SA is reliable, we should consider whether/how to apply it to NR-DC. [Xiaomi, LG, Nokia]: Agree with Samsung.

Proposal 4: For NR-DC, MN and SN can configure inference configuration independently based on legacy coordination on radio capability on measurement. And it is up to UE’s implementation to coordinate the inference configurations from MN and/or SN.

[Xiaomi]: Doesn’t understand “up to UE’s implementation to coordinate the inference configurations from MN and/or SN” well. How does the UE do that? [LG]: Would like to understand what issues are if there is no coordination between MN and SN. [ZTE]: Understand HO performance is good enough for SA, so restriction to SA makes AI/ML MOB useless. Better HO performance based on AI/ML MOB is needed more for NR-DC, so want to support NR-DC. [MediaTek]: Understand there is no blocking issue to support NR-DC in NW side AI/ML MOB. [Lenovo]: If the UE is configured with NR-DC, but inference configuration is only configured for PCell, understand this case should be supported (no real difference compared to SA case). [OPPO]: Agree with Lenovo. [Ericsson]: Would like to support NR-DC and thinks NW can handle each inference configuration properly based on UE capability coordination. [Qualcomm]: Prefer removing “including a scenario the UE is configured with NR-DC but inference configuration is only configured from MCG” since it still assumes MN does that based on UE capability coordination with SN. [Nokia]: Agree with Qualcomm.

Proposal 5: The network architecture i.e. (NE)EN-DC and NE-DC are not in the scope in R20.

Agreements: general

- SSB is taken as starting point as reference signal type for normative work in RAN2.

- Only RSRP prediction is specified in R20 as measurement quantity i.e. RSRQ and SINR are not specified in R20 unless we receive RSRQ/SINR support from RAN4.

- SA is taken as starting point as reference architecture for normative work in RAN2. FFS on NR-DC.

2. Applicability reporting option(s)

R2-2508417	Discussion on RRM measurement prediction	ZTE Corporation	discussion	Rel-20	NR_AIML_Mob

Proposal 1: For applicability reporting in AI-Mob, both Option A and Option B mechanisms are supported.

	Option A: Network configures full inference configuration, and UE reports applicability status to the network via RRCReconfigurationComplete, RRCResumeComplete and UAI;

	Option B: Network configures partial inference configuration via OtherConfig, and UE reports applicability status to the network via RRCReconfigurationComplete and UAI, then the network provides full inference configuration based on the received applicability status.

R2-2508371	Measurement prediction	Apple	discussion	Rel-20	NR_AIML_Mob

Proposal 7: No need to support partial configuration (for applicability reporting).

[Apple]: Assume changing inference configuration, e.g. PW, is not often. [Vivo]: Support ZTE proposal. Option B also provides signaling overhead reduction. [NEC]: Think it’s too early to exclude option B. [OPPO]: According to WID, both options are included. We should follow WID.

For applicability reporting in AI-Mob, both Option A and Option B mechanisms are supported.

- Option A: Network configures full inference configuration, and UE reports applicability status to the network via RRCReconfigurationComplete, RRCResumeComplete and UAI;

- Option B: Network configures partial inference configuration via OtherConfig, and UE reports applicability status to the network via RRCReconfigurationComplete and UAI, then the network provides full inference configuration based on the received applicability status.

Agreements: applicability reporting option(s)

- For applicability reporting in AI-Mob, both Option A and Option B mechanisms are supported.

- Option A: Network configures full inference configuration, and UE reports applicability status to the network via RRCReconfigurationComplete, RRCResumeComplete and UAI

- Option B: Network configures partial inference configuration via OtherConfig, and UE reports applicability status to the network via RRCReconfigurationComplete and UAI, then the network provides full inference configuration based on the received applicability status.

3. Applicability reporting procedure

R2-2508212	Discussion on functionality management for RRM measurement prediction	OPPO	discussion	Rel-20	NR_AIML_Mob-Core

Proposal 6: The following text in Table 3-2 from TR [2] are agreed:

6.1, UE can be configured with a full inference configuration and/or a partial inference configuration with inference parameters defined in section 6.1.2.1.2 of TR [2] in a RRCReconfiguration message.

UE can be configured with a full inference configuration and/or a partial inference configuration with inference parameters defined in section 6.1.2.1.2 of TR [2] in a RRCReconfiguration message.

6.2, Upon receiving an inference configuration (either full or partial) via RRCReconfiguration message, UE reports whether it is applicable or inapplicable in initial applicability report via RRCReconfigurationComplete message.

Upon receiving an inference configuration (either full or partial) via RRCReconfiguration message, UE reports whether it is applicable or inapplicable in initial applicability report via RRCReconfigurationComplete message.

6.3 (modified), FFS on “If an inference configuration is inapplicable, UE-may include a flag to indicate its preference to NW. When UE indicates that an inference configuration is inapplicable, network is expected to release it i.e., autonomous release by UE is not supported.”

[Ericsson, Apple, Samsung]: Support the MediaTek proposal. [Qualcomm]: Does “Fallback to non-AI operation” mean legacy RRM measurement or disabling the inference configuration? [Nokia]: Understand in some cases, fallback to non-AI is not easy, e.g. for inter-F measurement, it is related to SMTC configuration also. Would like not to directly agree with the proposal now since AI/ML MOB can be different from AI/ML BM since it’s not an intra-CU procedure. [Apple]: For inter-F case, it is limited to co-located case, so it should be easier to support it. [ZTE]: If we agree with AI/ML MOB to fallback procedure, we also need to discuss when fallback to AI/ML MOB happens.

FFS on “If an inference configuration is inapplicable, UE-may include a flag to indicate its preference to release to NW. When UE indicates that an inference configuration is inapplicable, network is expected to release it i.e., autonomous release by UE is not supported.”

6.4, (modified) Applicability can be updated via UAI. FFS on RRC reconfiguration complete.

Applicability can be updated via UAI. FFS on RRC reconfiguration complete.

6.5, No prohibit timer needs to be introduced for prohibiting applicability reporting.

No prohibit timer needs to be introduced for prohibiting applicability reporting.

Proposal 8 (modified): Working assumption: Upon transition to RRC_INACTIVE state, UE keep inference configuration (full and/or partial inference configuration) without spec impact.

Working assumption: Upon transition to RRC_INACTIVE state, UE keep inference configuration (full and/or partial inference configuration) without spec impact.

R2-2508580	Discussion on LCM for RRM prediction and event prediction	MediaTek Inc.	discussion

Proposal 8: Same as AI-BM, the UE doesn’t autonomous release the inference configuration when it becomes inapplicable. But the UE can autonomously fallback to non-AI operation.

FFS on “the UE doesn’t autonomous release the inference configuration when it becomes inapplicable. But the UE can autonomously fallback to non-AI operation.”

R2-2508552	RRM measurement prediction for UE-side models	Qualcomm Incorporated	discussion	Rel-20	NR_AIML_Mob

Proposal 7: The following are a few agreements from Rel-19 AI/ML for beam management WI that apply for AI/ML for mobility.

- No enhancements are pursued for reporting applicability in RRCReestablishmentComplete.

No enhancements are pursued for reporting applicability in RRCReestablishmentComplete.

- Applicability reporting is added in RRCResumeComplete for inference configurations that exist at the UE based on legacy procedures (restored or received in RRCResume).

Working assumption: Applicability reporting is added in RRCResumeComplete for inference configurations.

3. RRCReconfigurationComplete shall include applicability/inapplicability status for:

a) All prediction configurations included in the immediately preceding RRCReconfiguration message.

RRCReconfigurationComplete shall include applicability/inapplicability status for:

a) All prediction configurations included in the immediately preceding RRCReconfiguration message.

b) Any previously configured prediction configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.

4. Applicability reporting via UAI is always enabled.

Applicability reporting via UAI is always enabled.

5. (modified) During mobility, the target gNB can check the applicability by including target cell inference configurations in the handover command message, i.e., RRCReconfiguration, and the UE can respond with RRCReconfigurationComplete.

During mobility, the target gNB can check the applicability by including target cell inference configurations in the handover command message, i.e., RRCReconfiguration, and the UE can respond with RRCReconfigurationComplete.

R2-2508212	Discussion on functionality management for RRM measurement prediction	OPPO	discussion	Rel-20	NR_AIML_Mob-Core

Proposal 7: When applicability status is changed from applicable/inapplicable to inapplicable/applicable, UE shall report the updated applicability status via UAI message only.

[Xiaomi, NEC, LG]: Support OPPO’s P7. [Samsung]: Understand there is a submitted contribution in AI/ML BM on this aspect. [LG]: Understand in change of non-applicable to applicable, there would be an issue on when to start inference configuration. Understand OPPO’s P7 is simpler in that case. [Ericsson]: Anyway the signaling is already supported for initial applicability in RRC reconfiguration complete, then there will be no additional spec impact to support case b). [Session chair]: If we use UAI in the case b), what should be an issue? [Qualcomm]: For actual applicable inference configuration, it will take more time and more signaling overhead. [Huawei]: Although consider the case b) is a kind of corner case, if it is applied to AI/ML BM, ok to reuse it also for AI/ML MOB.

FFS whether we also apply applicability reporting in RRC reconfiguration complete when applicability status is changed from applicable/inapplicable to inapplicable/applicable.

Agreements: applicability reporting procedure

- UE can be configured with a full inference configuration and/or a partial inference configuration with inference parameters defined in section 6.1.2.1.2 of TR [2] in a RRCReconfiguration message.

- Upon receiving an inference configuration (either full or partial) via RRCReconfiguration message, UE reports whether it is applicable or inapplicable in initial applicability report via RRCReconfigurationComplete message.

- FFS on “If an inference configuration is inapplicable, UE-may include a flag to indicate its preference to release to NW. When UE indicates that an inference configuration is inapplicable, network is expected to release it i.e., autonomous release by UE is not supported.”

- Applicability can be updated via UAI. FFS on RRC reconfiguration complete.

- No prohibit timer needs to be introduced for prohibiting applicability reporting.

- Working assumption: Upon transition to RRC_INACTIVE state, UE keep inference configuration (full and/or partial inference configuration) without spec impact.

- FFS on “the UE doesn’t autonomous release the inference configuration when it becomes inapplicable. But the UE can autonomously fallback to non-AI operation.”

- No enhancements are pursued for reporting applicability in RRCReestablishmentComplete.

- Working assumption: Applicability reporting is added in RRCResumeComplete for inference configurations.

- RRCReconfigurationComplete shall include applicability/inapplicability status for: a) All prediction configurations included in the immediately preceding RRCReconfiguration message.

- Applicability reporting via UAI is always enabled.

- During mobility, the target gNB can check the applicability by including target cell inference configurations in the handover command message, i.e., RRCReconfiguration, and the UE can respond with RRCReconfigurationComplete.

- FFS whether we also apply applicability reporting in RRC reconfiguration complete when applicability status is changed from applicable/inapplicable to inapplicable/applicable.

4. Associate ID

R2-2508057	Discussion on RRM measurement prediction	Xiaomi	discussion

Proposal 1 (partially captured): Apply following agreements from AI air for AI mobility

-	Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar mobility deployment.

-	The Associated ID is 24 bits.

-	If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

Proposal 2: Neighbour cell’s associated ID can be provided to UE for applicability checking and data collection.

Proposal 3: The associated ID can be provided per cell/multiple cells/frequency.

R2-2508744	Discussion on RRM Measurement Prediction	Ericsson	discussion	Rel-20	NR_AIML_Mob

Proposal 4: It is up to UE implementation to ensure the consistency between the training and the inference. No associated ID is needed for AI for Mobility use case.

5. Inference configuration framework

R2-2508426	Discussion on RRM measurement prediction	Samsung	discussion	Rel-19	NR_AIML_Mob-Core

Proposal. 5: For inference configuration, reuse the existing IE (i.e., MeasObjectNR, MeasId, ReportConfigNR).

Proposal. 6: For inference configuration, introduce new fields to configure inference parameters in MeasObjectNR and/or ReportConfigNR IE.

R2-2509060	On RRM measurement prediction	Nokia, Nokia Shanghai Bell	discussion	Rel-20

Proposal 6: RAN2 to take the measurement prediction configuration framework in Figure 2.1.4-1 as a starting point for measurement prediction configuration. Details related to configuration of measurement predictions and prediction reporting are FFS.

R2-2508580	Discussion on LCM for RRM prediction and event prediction	MediaTek Inc.	discussion

Proposal 4: NW provide full inference configuration to UE via MeasConfig, which contains

	Measurement object (via MeasObject): Describes the information of measurement/observed target.

	Report configuration (via ReportConfig): Describes the triggering condition and report quantity for the measurement report.

	Predicted object (via PredObject): Describes the information of inference/predicted target (can reuse MeasObject if applicable).

	Inference identity (via InferenceId): Refers to an identity (can reuse MeasId if applicable) that provides the mapping of a measurement object, a predicted object, and a report configuration.

6. Inference configuration for temporal domain prediction case A

R2-2508707	Functionality management for UE sided model for RRM measurement prediction	InterDigital Pennsylvania	discussion	Rel-20

Proposal 5: RAN2 to confirm that the UE can be configured with the following regarding RRM measurement inference for temporal case A:

-	Prediction window length

-	(optional) list of cells for which network expects inference report and their corresponding associated IDs

R2-2508426	Discussion on RRM measurement prediction	Samsung	discussion	Rel-19	NR_AIML_Mob-Core

Proposal. 9: For PW length configuration in temporal domain Case A, RAN2 discuss the following two options:

-	Option 1 (time duration): NW configures the length of PW as a single time duration value (e.g., xx msec).

-	Option 2 (# of instances and time interval): NW configures the number of time instances to predict within PW and the time interval between consecutive time instances (similar to R19 BM case).

Proposal. 10: For list of target cells configuration in temporal domain Case A, RAN2 discuss the following two options:

-	Option 1: Reuse the existing list (i.e., allowedCellsToAddModList).

-	Option 2: Introduce a new list (e.g., predictedCellsToAddModList).

7. Inference configuration for temporal domain prediction case B

R2-2508212	Discussion on functionality management for RRM measurement prediction	OPPO	discussion	Rel-20	NR_AIML_Mob-Core

Proposal 22: In R20 only solution for implementation case B-1 will be specified and current SMTC parameter can be reused.

R2-2508245	Discussion on UE-side RRM measurement prediction 	NEC	discussion	Rel-20	NR_AIML_Mob

Proposal-3 (partially captured): the inference configuration for temporary domain case B should consider the following parameters:

- Timing information of SSB configuration, e.g., a separate SMTC dedicated for AI/ML

- Cell information, used to indicate which cell should be used for inference

8. Inference configuration for inter-F prediction

R2-2508101	Functionality management for RRM measurement prediction	CATT, Turkcell	discussion	Rel-20	NR_AIML_Mob

Proposal 7: RAN2 confirms that following inference parameters for inter frequency domain prediction can be configured to UE for inference and assessing applicability:

-	Measured frequency carrier and predicted frequency carrier information for inter-frequency prediction.

R2-2508245	Discussion on UE-side RRM measurement prediction 	NEC	discussion	Rel-20	NR_AIML_Mob

Proposal-4 (partially captured): the inference configuration for frequency domain prediction should consider the following parameters:

- FFS Cell information under predicted frequency

- FFS inference periodicity

9. Inference report

R2-2508057	Discussion on RRM measurement prediction	Xiaomi	discussion

Proposal 8: Legacy measurement report trigger can be reused.

Proposal 9: For temporal case A, following inference result can be reported based on NW configuration:

-	predicted measurement result on each timing instance within PW

-	average of predicted measurement results within PW

Proposal 10: For temporal case B and inter-frequency prediction, the latest measurement results are reported, regardless of actual results or predicted results.

R2-2508417	Discussion on RRM measurement prediction	ZTE Corporation	discussion	Rel-20	NR_AIML_Mob

Proposal 13: For temporal case A, to introduce a separate list of predicted measurement results in MeasResults IE. RAN2 to discuss whether to also indicate the time info for each reported result

10. Inference control

R2-2508580	Discussion on LCM for RRM prediction and event prediction	MediaTek Inc.	discussion

Proposal 11: For RRM Temporal domain case B and frequency domain prediction, when to perform inference is up to UE implementation. The UE needs to meet the report requirements determined by RAN4.

R2-2508057	Discussion on RRM measurement prediction	Xiaomi	discussion

Proposal 11: For temporal case A, NW can configure the radio condition for inference. UE is not required to perform inference if the radio event is not fulfilled.

R2-2508057	Discussion on RRM measurement prediction	Xiaomi	discussion

R2-2508062	Discussion on RRM measurement prediction	vivo	discussion	Rel-20	NR_AIML_Mob-Core

R2-2508095	Discussion on RRM measurement prediction	NTT DOCOMO, INC.	discussion

R2-2508101	Functionality management for RRM measurement prediction	CATT, Turkcell	discussion	Rel-20	NR_AIML_Mob

R2-2508181	"	 RRM measurement prediction"	LG Electronics Inc.	discussion	Rel-20	NR_AIML_Mob

R2-2508212	Discussion on functionality management for RRM measurement prediction	OPPO	discussion	Rel-20	NR_AIML_Mob-Core

R2-2508228	Discussion on RRM measurement prediction	Transsion Holdings	discussion

R2-2508245	Discussion on UE-side RRM measurement prediction 	NEC	discussion	Rel-20	NR_AIML_Mob

R2-2508266	Discussion on RRM measurement prediction	Fujitsu	discussion	Rel-20	NR_AIML_Mob

R2-2508336	RRM measurement prediction	Lenovo	discussion

R2-2508371	Measurement prediction	Apple	discussion	Rel-20	NR_AIML_Mob

R2-2508417	Discussion on RRM measurement prediction	ZTE Corporation	discussion	Rel-20	NR_AIML_Mob

R2-2508420	Discussion on RRM measurement prediction	Tejas Network Limited	discussion

R2-2508426	Discussion on RRM measurement prediction	Samsung	discussion	Rel-19	NR_AIML_Mob-Core

R2-2508479	Measurement gap relaxation 	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2508496	LCM for RRM measurement prediction	China Telecom	discussion	Rel-20	NR_AIML_Mob

R2-2508532	Functionality management for RRM measurement prediction	TURKCELL, CATT	discussion	Rel-20

R2-2508552	RRM measurement prediction for UE-side models	Qualcomm Incorporated	discussion	Rel-20	NR_AIML_Mob

R2-2508580	Discussion on LCM for RRM prediction and event prediction	MediaTek Inc.	discussion

R2-2508601	Discussion on RRM measurement prediction	HONOR	discussion	Rel-20	NR_AIML_Mob

R2-2508701	Discussion on RRM measurement prediction	Huawei, HiSilicon	discussion	Rel-20	NR_AIML_Mob

R2-2508707	Functionality management for UE sided model for RRM measurement prediction	InterDigital Pennsylvania	discussion	Rel-20

R2-2508744	Discussion on RRM Measurement Prediction	Ericsson	discussion	Rel-20	NR_AIML_Mob

R2-2508817	Discussion on RRM Measurement Prediction Functionality Management	Sharp	discussion

R2-2508818	Discussion on RRM Measurement Inference Reporting Configuration	Sharp	discussion

R2-2508842	Discussion on UE sided RRM measurement prediction	CMCC	discussion	Rel-20	NR_AIML_Mob

R2-2508931	Discussion on LCM for RRM prediction	Spreadtrum, UNISOC	discussion	Rel-20

R2-2508958	Discussion on RRM prediction	ASUSTeK	discussion	Rel-19	NR_AIML_Mob

R2-2508981	Discussion on RRM measurement prediction	KDDI Corporation	discussion	Rel-20

R2-2508982	Discussion on inference configuration and report for RRM measurement prediction for UE sided model	KT Corp.	discussion	NR_AIML_Mob

R2-2509015	Discussion on functionality management for RRM measurement prediction	BYD	discussion

R2-2509060	On RRM measurement prediction	Nokia, Nokia Shanghai Bell	discussion	Rel-20

### 9.3.3	RRM measurement event prediction

LCM functionality management for RRM measurement event prediction, including UE capability, applicability, inference configuration/reporting etc.

NOTE: No contributions expected on data collection and performance monitoring.

R2-2508058	Discussion on RRM measurement event prediction	Xiaomi	discussion

R2-2508063	Discussion on RRM measurement event prediction	vivo	discussion	Rel-20	NR_AIML_Mob-Core

R2-2508096	Discussion on RRM measurement event prediction	NTT DOCOMO, INC.	discussion

R2-2508102	Functionality management for RRM measurement event prediction	CATT, Turkcell, CBN	discussion	Rel-20	NR_AIML_Mob

R2-2508182	RRM measurement event prediction	LG Electronics Inc.	discussion	Rel-20	NR_AIML_Mob

R2-2508213	Discussion on functionality management for measurement event prediction	OPPO	discussion	Rel-20	NR_AIML_Mob-Core

R2-2508246	Discussion on UE-side event prediction 	NEC	discussion	Rel-20	NR_AIML_Mob

R2-2508267	Discussion on RRM measurement event prediction	Fujitsu	discussion	Rel-20	NR_AIML_Mob

R2-2508272	Discussion on RRM measurement event prediction	Transsion Holdings	discussion

R2-2508370	Event prediction	Apple	discussion	Rel-20	NR_AIML_Mob

R2-2508418	Discussion on RRM measurement event prediction	ZTE Corporation	discussion	Rel-20	NR_AIML_Mob

R2-2508427	Discussion RRM measurement event prediction	Samsung	discussion	Rel-19	NR_AIML_Mob-Core

R2-2508497	LCM for measurement event prediction	China Telecom	discussion	Rel-20	NR_AIML_Mob

R2-2508526	Discussion on Event Predictions	Ericsson	discussion	Rel-20	NR_AIML_Mob

R2-2508553	Measurement Event prediction	Qualcomm Incorporated	discussion	Rel-20	NR_AIML_Mob

R2-2508654	Discussion on LCM for RRM measurement event prediction	Lenovo	discussion	Rel-20

R2-2508702	Discussion on RRM measurement event prediction	Huawei, HiSilicon	discussion	Rel-20	NR_AIML_Mob

R2-2508722	Functionality management for UE sided model for RRM measurement event prediction	InterDigital Pennsylvania	discussion	Rel-20	NR_AIML_Mob-Core

R2-2508844	Discussion on RRM measurement event prediction	CMCC	discussion	Rel-20	NR_AIML_Mob

R2-2508932	Discussion on LCM for RRM event prediction	Spreadtrum, UNISOC	discussion	Rel-20

R2-2508959	Discussion on measurement event prediction	ASUSTeK	discussion	Rel-19	NR_AIML_Mob

R2-2509062	Views on Measurement Event Prediction	Nokia	discussion	Rel-20