3GPP TSG-RAN2 Meeting #132	R2-2509364

# Dallas, USA, 17th – 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR |  | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Delta configuration for CB-Msg3-EDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, for NB-IoT, most of parameters in CB-Msg3-Config-NB are mandatory. Considering that CB-Msg3-EDT resources can also be provided on non-anchor carriers, and it is highly possible that several or most parameters for a certain CE level on a non-anchor carrier may be identical to that of the corresponding CE level on the anchor carrier, it is necessary to consider allowing delta configuration for CB-Msg3-EDT resources on non-anchor carriers, enabling them to refere to the configuration of the corresponding CE level on the anchor carrier. This is beneficial for reducing signaling overhead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To allow per-parameter delta configuation for CB-Msg3-EDT resources on non-anchor carriers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary siganling overhead |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

6.7.3	NB-IoT information elements

<Skip unrelated parts>

6.7.3.2	NB-IoT Radio resource control information elements

<Skip unrelated parts>

–	CB-Msg3-ConfigSIB-NB

The IE CB-Msg3-ConfigSIB-NB is used to specify the CB-Msg3-EDT configuration.

CB-Msg3-ConfigSIB-NB information element

-- ASN1START

CB-Msg3-ConfigSIB-NB-r19 ::=		SEQUENCE {

cb-Msg3-MinRSRP-Threshold-NB-r19		NRSRP-Range-NB-r14				OPTIONAL,	--Need OR

cb-Msg3-RSRP-CE-Levels-NB-r19			CB-Msg3-RSRP-CE-Levels-NB-r19	OPTIONAL,	--Need OR

cb-Msg3-ConfigList-NB-r19				CB-Msg3-ConfigList-NB-r19,

powerRampingParameters-NB-r19			PowerRampingParameters-NB-r19

}

CB-Msg3-ConfigList-NB-r19 ::=		SEQUENCE (SIZE (1.. maxCE-Level-CB-Msg3-NB-r19)) OF

CB-Msg3-Config-NB-r19

CB-Msg3-Config-NB-r19 ::=			SEQUENCE {

cb-Msg3-ConfigSIB-CEL-Container-r19		SEQUENCE {

cb-Msg3-TBS-NB-r19						ENUMERATED {b144, b328, b408, b504, b584, b680, b808,

b936} OPTIONAL,	-- Need OP

cb-Msg3-NumOfReplicas-NB-r19				INTEGER (1..4) OPTIONAL,	-- Need OP

cb-Msg3-TimeResource-NB-r19		SEQUENCE {

npusch-Periodicity-r19					ENUMERATED {ms40, ms80, ms160, ms240,

ms320, ms640, ms1280, ms2560},

npusch-StartSFN-r19						INTEGER (0..1023),

npusch-StartSubframe-r19				INTEGER (0..9)

} OPTIONAL,	-- Need OP

cb-Msg3-PhysicalConfig-r19		SEQUENCE {

npusch-NumRUsIndex-r19					INTEGER (0..7) OPTIONAL,	-- Need OP

npusch-NumRepetitionsIndex-r19			INTEGER (0..7) OPTIONAL,	-- Need OP

npusch-SubCarrierSetList-r19		CHOICE {

npusch-SubCarrierSetList-khz15		SEQUENCE (SIZE(1..12)) OF INTEGER (0..18),

npusch-SubCarrierSetList-khz3dot75	SEQUENCE (SIZE(1..48)) OF INTEGER (0..47)

} OPTIONAL,	-- Need OP

npusch-MCS-r19						CHOICE {

singleTone							INTEGER (0..10),

multiTone							INTEGER (0..13)

} OPTIONAL,	-- Need OP

ack-NumRepetitions-NB-r19			ACK-NACK-NumRepetitions-NB-r13 OPTIONAL,	-- Need OP

p0-UE-NPUSCH-r19						INTEGER (-8..7) OPTIONAL,	-- Need OP

alpha-NB-r19							ENUMERATED {al0, al04, al05, al06,

al07, al08, al09, al1} OPTIONAL,	-- Need OP

npdcch-CarrierIndex-r19					INTEGER (1..maxNonAnchorCarriers-NB-r14)

OPTIONAL,	-- Need OP

npdcch-NumRepetitions-r19				ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1} OPTIONAL,	-- Need OP

npdcch-StartSF-CSS-r19					ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64} OPTIONAL,	-- Need OP

npdcch-Offset-CSS-r19					ENUMERATED {zero, oneEighth, oneFourth, threeEighth} OPTIONAL,	-- Need OP

},

cb-Msg3-TxWindow-NB-r19			SEQUENCE {

windowSize-NB-r19					INTEGER {3..10},

windowPeriodicity-NB-r19			ENUMERATED { n16, n32, n48, n64, n128, n256, n384

n512，n768, n1024, n1280, n1536, n1792,

n2048, n2560, n3072 }

}																	OPTIONAL,	--Need OP

cb-Msg3-ResponseWindow-NB-r19			ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32,

pp64} OPTIONAL,	-- Need OP

cb-Msg3-MaxAttemptNum-NB-r19				ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n10} 																			OPTIONAL,	--Need OP

...

}	OPTIONAL	-- Need OR

}

CB-Msg3-RSRP-CE-Levels-NB-r19 ::=	SEQUENCE (SIZE(1..2)) OF RSRP-Range

NPUSCH-SubCarrierSet-r19 ::=		CHOICE {

khz15								INTEGER (0..18),

khz3dot75							INTEGER (0..47)

}

PowerRampingParameters-NB-r19 ::=			SEQUENCE {

powerRampingStep-NB-r19						ENUMERATED {dB0, dB2, dB4, dB6},

cb-Msg3-InitialReceivedTargetPower-NB-r19	ENUMERATED {

dBm-130, dBm-128, dBm-126, dBm-124, dBm-122,

dBm-120, dBm-118, dBm-116, dBm-114, dBm-112,

dBm-110, dBm-108, dBm-106, dBm-104, dBm-102,

dBm-100, dBm-98, dBm-96, dBm-94, dBm-92, dBm-90,

dBm-88, dBm-86, dBm-84,dBm-82, dBm-80}

}

-- ASN1STOP

CB-Msg3-ConfigSIB-NB field descriptions

ack-NumRepetitions-NB Number of repetitions for the ACK resource unit carrying HARQ response to NPDSCH, see TS 36.213 [23], clause 16.4.2. See NOTE.

alpha-NB Parameter: αc(3). See TS 36.213 [23], clause 16.2.1.1.1. See NOTE.

cb-Msg3-ConfigList-NB CB-Msg3-EDT configuration for each CE level applicable to a UE performing CB-Msg3-EDT. The first entry in the list is the CB-Msg3-EDT configuration for CE level 0, the second entry in the list is the CB-Msg3-EDT configuration for CE level 1, and so on. For the CB-Msg3-ConfigList-NB in SystemInformationBlockType22-NB, E-UTRAN includes the same number of entries, and listed in the same order, as in CB-Msg3-ConfigList-NB in SystemInformationBlockType2-NB.

cb-Msg3-InitialReceivedTargetPower-NB Initial power for CB-Msg3 transmission as specified in TS 36.321 [6]. Value in dBm. Value dBm-130 corresponds to -130 dBm, dBm-128 corresponds to -128 dBm and so on.

cb-Msg3-MaxAttemptNum-NB The maximum number of attempts of CB-Msg3-EDT within this CE level. If the field is absent, the UE shall assume no re-attempt.

cb-Msg3-MinRSRP-Threshold-NB Indicates the minimum RSRP threshold for initiating CB-Msg3-EDT.

cb-Msg3-NumOfReplicas-NB Indicates the number of replicas that UE should send within one attempt of CB-Msg3-EDT. See NOTE.

cb-Msg3-ResponseWindow-NB NPDCCH search space window duration. See TS 36.321 [6] and TS 36.213 [23]. Value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on. The value considered by the UE is: cb-Msg3-ResponseWindow-NB = Min (signaled value x PDCCH period, 10.24s). See NOTE.

cb-Msg3-RSRP-CE-Levels-NB RSRP thresholds for determing which configuration is used for CB-Msg3-EDT.

cb-Msg3-TBS-NB Indicates the TB size threshold for initiating CB-Msg3-EDT. Value b144 corresponds to 144 bits, value b328 corresponds to 328 bits and so on. See TS 36.213 [23]. See NOTE.

cb-Msg3-TimeResource-NB NPUSCH resource configuration for CB-Msg3 transmission. See NOTE.

cb-Msg3-TxWindow-NB CB-Msg3 transmission window configuration. The start time of the CB-Msg3 transmission window is aligned with the NPUSCH start time indicated by npusch-StartSFN-r19 and npusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-NB-r19 equals 1, this field is absent. For windowSize, value 3 corresponds to 4 PUSCH periods, 4 corresponds to 8 PUSCH periods and so on. For windowPeriodicity, value n16 corresponds to 160ms, n32 corresponds to 320ms and so on.

npdcch-CarrierIndex Indicates the non-anchor carrier for receiving CB-Msg4. If this field is absent, UE receives CB-Msg4 on the anchor carrier.

npdcch-NumRepetitions Maximum number of repetitions for NPDCCH common search space (CSS) for receiving CB-Msg4. See NOTE.

npdcch-Offset-CSS Fractional period offset of starting subframe for an NPDCCH common search space, see TS 36.213 [23], clause 16.6. See NOTE.

npdcch-StartSF-CSS Starting subframe configuration for an NPDCCH common search space, see TS 36.213 [23], clause 16.6. Value v1dot5 corresponds to 1.5, value 2 corresponds to 2 and so on. See NOTE.

npusch-MCS Index to tables specified in TS 36.213 [23], Table 16.5.1.2-1 and Table 16.5.1.2-2 for single tone and multi tone respectively, that defines modulation and TBS index for NPUSCH for CB-Msg3-EDT. See NOTE.

npusch-NumRepetitionsIndex Index to a table specified in TS 36.213 [23], Table 16.5.1.1-3, that defines number of repetitions for NPUSCH for CB-Msg3-EDT. See NOTE.

npusch-NumRUsIndex Index to a table specified in TS 36.213 [23], Table 16.5.1.1-2, that defines number of resource units for NPUSCH for CB-Msg3-EDT. See NOTE.

npusch-SubCarrierSetList For NPUSCH transmission with subcarrier spacing 3.75 kHz, indicates the subcarrier used for CB-Msg3-EDT, as specified in TS 36.213 [23]. For NPUSCH transmission with subcarrier spacing 15 kHz, indicates the index to Table 16.5.1.1-1 specified in TS 36.213 [23], which defines the set of subcarriers for NPUSCH for CB-Msg3-EDT. See NOTE.

p0-UE-NPUSCH Parameter: P0_UE_PUSCH,c (3). See TS 36.213 [23], clause 16.2.1.1.1, unit dB. See NOTE.

powerRampingStep-NB Power ramping factor in TS 36.321 [6]. Value in dB. Value dB0 corresponds to 0 dB, dB2 corresponds to 2 dB and so on.

NOTE:

-	The field is mandatory present in all the entries of cb-Msg3-ConfigList-NB if it’s configured in SystemInformationBlockType2-NB.

-	If the field is absent in an entry of cb-Msg3-ConfigList-NB in SystemInformationBlockType22-NB, the value of the same field in the corresponding entry of cb-Msg3-ConfigList-NB in SystemInformationBlockType2-NB applies. If cb-Msg3-ConfigList-NB in SystemInformationBlockType2-NB is not configured, the field is mandatory present.

End of the change