3GPP TSG RAN WG2#131bis	R2-2506724

Prague, Czech Republic, 13th - 17th October 2025

3GPP TSG-RAN WG3 Meeting #129	R3-255896

Bengaluru, India, 25 – 29 August, 2025

Title:	Reply LS on MBS Communication Service Type

Response to:	S4-251096/R3-255028

Release:	Release 18

Work Item:	NR_QoE_enh-Core

Source:	RAN3

To:	SA4

Cc:	RAN2, SA5

Contact Person:

#### Name:	Håkon Helmers

E-mail Address:	hakon.helmers@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks SA4 for their reply LS on MBS Communication Service Type in S4-251096/R3-255028. RAN3 has several follow up questions.

Related to the definition of the value ‘all’ of @communicationServiceType, SA4 indicates in their answer 1: “Option 1 is the right interpretation.  It refers to all communication service types (not limited to MBS communication services).” (Option 1: Value “all” means that the UE shall conduct QoE measurements on broadcast, multicast and unicast.)

Question A: In the CR to TS 26.247 agreed by SA4 in S4-251095, in the description of the @communicationServiceType attribute, it is stated that “When present, this attribute indicates a list of communication service type(s) for which the QoE collection and reporting of QoE metrics is requested and shall contain one or more of the following values:”. Regarding the “one or more” part, RAN3 would like to point out that, for a QMC configuration, only one communication service type is possible at a time. This effectively means that the “or more” should be deleted from the Description field of the attribute. RAN3 kindly asks SA4 to confirm this.

Question B: In line with the above, a given QoE Reference can, in RAN3 specifications, only point to a single communication service type, and a combination of two or more communication service types in a single QMC configuration is therefore not supported in RAN3 specifications. This means that the value “all” should be deleted from the @communicationServiceType attribute. RAN3 kindly asks SA4 to confirm this.

RAN3 observes in SA4’s answer 2 that ““OD” is revised into “O” in TS 26.247”. However, the CR to TS 26.247 agreed by SA4 in S4-251095 states: “When absent, quality metrics collection is requested for all communication service types.”

Question C: Whether the @communicationServiceType can have a default value even though the Use is “O”? If the answer is “no”, RAN3 kindly asks SA4 to remove the following sentence from the Description field of the attribute: “When absent, quality metrics collection is requested for all communication service types.”. If the answer is “yes”, RAN3 kindly asks SA4 to confirm whether the default value can be “unicast” because the RAN3 specifications do not support combinations of two or more communication service types in a single QMC configuration.

RAN3 assumes that the Rel-17 UE can perform QoE measurement collection and reporting only for unicast communication service.

2. Actions:

To SA4: 	RAN3 kindly asks SA4 to take the above into account, provide answers to the questions above, and, if feasible update their specifications accordingly.

3. Dates of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting #129-bis	13 - 17 October 2025	Prague, CZ

TSG-RAN WG3 Meeting #130	17 - 21 November 2025	Dallas, TX, US