3GPP TSG-RAN WG3#129 	R3-255139

Bengaluru, India, 25-29 August 2025

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	TP (BL CR TS 38.423) Inter-CU LTM with Dual Connectivity

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-250339 NR mobility enhancements Phase 4), including the following related objectives and scope impacting RAN3:

• Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] o Prioritize the case when CU is acting as MN when DC is not configured o When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:  As secondary priority, support the case where CU is acting as SN and MN is unchanged  As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released o Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM  Coordination with SA3 needed with respect to security key handling o Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support • Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] o Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM o Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] o Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] o Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This paper discusses the remaining FFS from RAN3#128 and issues related to inter-CU SCG LTM, as well as some key aspects related to Inter-CU MCG LTM, which were not yet treated in RAN3.

# 2	Inter-CU LTM with Dual Connectivity

At the last RAN3 meetings, there was progress with respect to Inter-CU SCG LTM. However, some aspects related to inter-CU SCG LTM are still open. Furthermore, Inter-CU MCG LTM has still to be discussed in RAN3.

## 2.1 Inter-CU SCG LTM Issues

Issue 1: Interworking of fast MCG link failure recovery with inter-CU SCG LTM

Support for fast MCG link recovery when inter-CU SCG LTM is configured is possible with some minor enhancements to Xn signalling. Specifically, by introducing an interaction between procedures as follows. For Dual Connectivity LTM scenarios, If MCG FAILURE INDICATION message is received and is being processed at source MN prior to the reception of CELL SWITCH NOTIFICATION message, the source MN shall determine if the cell switch notification is for an inter-CU SCG LTM and if so, the source MN sends the RRCReconfigurationWithSync or RRCRelease message via the new serving SN.

Apart from the behaviour described above, we foresee no additional specification impact.

Proposal 1: RAN3 to discuss solutions to support fast MCG link recovery when Inter-CU SCG LTM is configured taking the TP in Annex as baseline.

## 2.2 Inter-CU SCG LTM – Open issues

Issue 2: LTM Modification/Cancel

The Following FFS was left open at the last RAN3 meeting. Likewise, during offline discussion two related scenarios were discussed.

FFS on whether LTM modification/cancel related procedures are needed.

<< Excerpt from offline discussion >> Issue 9: LTM modification/cancel Case 1: candidate SN can decide to update or release some of the prepared candidate PSCells. Case 2: source SN decides to update or release some of the suggested candidate PSCell - Case 2-1: the source SN decides to add or release a candidate PSCell prepared at the source SN - Case 2-2: the source SN decides to update the suggested candidate PSCells based on the latest L3 measurement results, or to update the upper limited number of PSCells of the candidate SN. Moderator suggest that companies consider above use cases and continue to discuss the issue in next meeting.

Regarding the relevance of these scenarios, we consider them relevant to be support. However, for the use case definition in Case-2, in our view for Case-2 it is more appropriate to replace “source SN” to “Serving SN”. The reason being that after Inter-SN PSCell change, one of the Candidate SN will become the new Serving SN, while the Source SN may become a Candidate SN when subsequent LTM is configured. Hence, a Source SN will take role of Candidate SN.

Proposal 2: RAN3 to derive a solution to support LTM Modification/Cancel triggered by the Candidate SN (Case-1) and by the Serving SN (Case-2).

## 2.2 Inter-CU MCG LTM Issues

The WI scope includes Inter-CU MCG LTM with SN release.

In this section we discuss a set of open issues that may be relevant to RAN3, while drawing attention to some of the agreements related to Inter-CU MCG LTM that RAN2 has made:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). - RAN2 confirms to support the coexistence of Inter-MN LTM and intra-SN LTM. - RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

Issue 1 - Candidate MN prepares a different candidate PSCell at source SN

In Inter-CU MCG LTM with unchanged SN, it is possible that a candidate MN may prepare a different PSCell even if it retains the source SN. Thus, both the below listed scenarios should be considered, and handling would vary as follows.

Scenario 1: Candidate MN retains the source PSCell: Even though the PSCell remains unchanged after Inter-CU MCG LTM execution, the UE must perform reconfiguration with sync as SN security key changes due to MN change.

Scenario 2: Candidate MN prepares a different PSCell: Intra-CU PSCell HO is triggered, as a result of Inter-CU MCG LTM PCell change.

In scenario 2 above, given that the Intra-CU PSCell change (or Intra-CU SCG LTM) may result from Inter-CU MCG LTM execution, the network must ensure it considers the latest PSCell measurements while triggering PSCell HO.

Proposal 3: For Inter-CU MCG LTM where a Candidate MN prepares a different PSCell resulting in an Intra-CU PSCell Handover, the PSCell change must be triggered based on latest UE reported radio measurements.

Issue 2 - Avoiding blind SCG or PSCell addition

An SCG Addition or a PSCell Addition during subsequent Inter-CU MCG LTM execution (after SCG release during earlier Inter-CU MCG execution) may result into a potential blind PSCell addition if the network does not consider latest PSCell measurements to identify the suitable PSCell at the time of inter-CU MCG LTM execution. Further, if there is more than one candidate target PSCells prepared associated with the Inter-CU candidate target PCell, then considering the latest UE reported radio measurements for candidate PSCells is essential to identify the suitable candidate PSCell and the associated LTM Candidate Configuration ID to avoid user experience and throughput degradation after SCG addition.

Proposal 4: In the case of Inter-CU MCG LTM involving SCG Addition (after SCG release), the PSCell addition is based on latest UE reported radio measurements for candidate PSCells.

## 2.3	Other Issues

Issue 4 - Configuration Mismatch during LTM Cell Switch

During initial preparation of candidate target node, the UE’s source cell configuration is used to prepare the target cell configuration for the prepared candidate target nodes. Given that the configuration at the UE after the initial preparation may vary, by the time of LTM cell switch or HO execution, the candidate target cell configuration prepared during the initial LTM preparation may not be aligned with the latest configuration at the UE. The handling of such a problem may require reconfiguring the UE at the time of cell switch or HO execution. However, to support subsequent LTM, the UE reconfiguration after cell change must be avoided. We propose RAN3 to discuss this issue and consider signalling enhancements for handling mismatch in the PDU session and DRB configuration between the serving and the target cells.

Proposal 5: RAN3 to derive a solution to address configuration mismatch between the serving and the target cells after initial preparation, and at the time of HO execution to support subsequent LTM cell change.

# 3	Conclusions

Inter-CU SCG LTM

Proposal 1: RAN3 to discuss solutions to support fast MCG link recovery when Inter-CU SCG LTM is configured taking the TP in Annex as baseline.

Proposal 2: RAN3 to derive a solution to support LTM Modification/Cancel triggered by the Candidate SN (Case-1) and by the Serving SN (Case-2).

Inter-CU MCG LTM

Proposal 3: For Inter-CU MCG LTM where a Candidate MN prepares a different PSCell resulting in an Intra-CU PSCell Handover, the PSCell change must be triggered based on latest UE reported radio measurements.

Proposal 4: In the case of Inter-CU MCG LTM involving SCG Addition (after SCG release), the PSCell addition is based on latest UE reported radio measurements for candidate PSCells.

Other Issues

Proposal 5: RAN3 to derive a solution to address configuration mismatch between the serving and the target cells after initial preparation, and at the time of HO execution to support subsequent LTM cell change.

# Annex: Text Proposal for TS 38.423

### 8.y.1	Cell Switch Notification

Editor’s note: Details need to be continued.

#### 8.y.1.1	General

The purpose of the Cell Switch Notification procedure is to enable the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the source NG-RAN node to the target NG-RAN node. The procedure uses UE-associated signalling.

#### 8.y.1.2	Successful Operation

Figure 8.y.1.2-1: Cell Switch Notification procedure. Successful operation.

The source NG-RAN node initiates the procedure by sending a CELL SWITCH NOTIFICATION message.

Upon reception of the CELL SWITCH NOTIFICATION message, the target NG-RAN node shall, if supported, consider that a cell switch command was sent to the UE towards one cell of the target NG-RAN node indicated by the included Target Cell Global ID IE.

Interactions with other procedures

For Dual Connectivity scenarios, If MCG FAILURE INDICATION message is received and is being processed at source MN prior to the reception of CELL SWITCH NOTIFICATION message, the source MN shall determine if the cell switch notification is for an inter-CU SCG LTM and if so, the source MN sends the RRCReconfigurationWithSync or RRCRelease message via the new serving SN.