3GPP TSG-RAN WG3 Meeting #129                                                                    R3-255244

Bengaluru, India, 25th – 29th, August 2025

Source:	CATT

Title:	On resource coordination for WAB

Agenda Item:	12.2

Document for:	Approval

1 Introduction

In RAN3#127bis [1] and RAN3#128 [2] meeting, following agreements were achieved and some FFSs are leftover.

RAN#127bis: Include a WAB-MT Identifier in the XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK e.g. for colocation discovery for resource multiplexing or for WAB node indication. It is understood that reception of the WAB-MT Identifier indicates that the sender is a WAB node. The receiver will behave accordingly. When access and backhaul links of a WAB node are operated out-of-band, there is no need for WAB resource coordination. RAN3 assumes that WAB deployments use out-of-band operation in case access and backhaul use different PLMNs. In this release, In-band WAB operation is only considered for intra PLMN scenarios where BH gNB is upgraded with WAB-specific enhancements. For WAB resource coordination, introduce a new class-1 Xn procedure between WAB-gNB and BH gNB. The message content can be based on relevant parts of the XnAP IEs defined in clauses 9.2.2.94-97 of TS 38.423. FFS whether to reuse the F1AP NB-DU RESOURCE CONFIGURATION procedure between BH gNB-CU and BH gNB-DU. After RAN3 has finished specifying WAB resource coordination, RAN3 to send an LS to inform RAN1 and RAN2 about conclusions. RAN3#128: Adopt the following principles for WAB resource coordination: - The specifications shall not define any priority between the WAB-gNB or the BH-gNB on how to split resources. - It needs to be further discussed if time domain and/or frequency domain coordination is supported - It needs to be further discussed if indication of soft resources (the “S” in HSNA) is supported. - It needs to be further discussed if only the WAB-gNB should be able to indicate the hard/not available resource allocation. - It is FFS whether to send an LS to RAN1/RAN2 on the above “to be continued” points

In this contribution, we will further discuss the topic of resource coordination.

2 Discussion

In the last meeting [2], companies have not converged on supporting resource coordination by specification in Rel-19. However, this meeting is the last meeting before R19 frozen, thus it seems impossible to complete the specification work within one meeting. RAN3 should decide whether to support resource coordination this meeting and how to complete it if support. Following are potential solutions:

Solution 1: Support resource coordination in R19

It requires RAN3 to complete resource coordination within maintenance of R19, otherwise, RAN3 need to request extending the work item period.

Solution 2: Support resource coordination in R19 TEI

Resource coordination is not in R19 but in R19 TEI, in which case RAN3 do not need to extend the WI period.

Solution 3: Not support resource coordination in R19/TEI

Supporting resource coordination is one of the objectives for R19 WAB topic from very beginning of the work item. Several agreements have been achieved during the last two meetings, and some of them were captured in XnAP BLCR already. Thus, if there is no concrete reason to give up it we tend to support.

RAN3 can do the best effort to complete resource coordination functionality for WAB in Rel-19 (e.g., request for extending WI period). Otherwise, this topic is done during R19 TEI.

Proposal 1: RAN3 can try best to complete resource coordination for WAB in Rel-19, otherwise it is done within R19 TEI.

In last meeting, the agreement is made that the WAB-gNB or the BH-gNB have the same priority on resource coordination. But some open issues were leftover and it’s not clear if it sending LS to RAN1 is needed.

- Whether time/frequency domain coordination is supported

Some companies believe that the frequency domain coordination would impact RAN1, while some companies think that RAN1 will anyway be involved no matter which domain is supported. Other companies believed we should not restrict the domain, because RAN1 has worked extensively on resource coordination for time and frequency domain in Rel16/17 for IAB, reopening the identical topic in RAN1 is not necessary, thus WAB can copy the framework of IAB to support both domains.

In our view, as it’s somehow RAN1 scope issue although WAB is essentially identical as IAB, if RAN3 cannot reach consensus, it can be left to RAN1’s decision.

If RAN3 decide to do resource coordination within Rel-19, RAN3 should ask for RAN1’s opinion on this issue by sending LS as early as possible. Without the conclusion on this issue, RAN3 cannot start the stage-3 specification work.

- Whether soft symbol is supported

The common understanding is that the Hard/NA resources indication can be supported. But there was objection in last meeting on the soft resource indication for various reasons, e.g., due to the complexity to realize it, or considering this indication is not necessary in case the WAB-gNB and BH-gNB have the same priority.

Our perspective is that the soft indication can provide some flexibility but also increases the complexity (requiring the new DCI design) including the implementation at WAB-node side and the impact to RAN1 spec. On the other hand, resource coordination can work even without this soft indication, e.g., WAB-gNB can consider the resource except those indicated by H/NA indication as “soft” and decide whether it can be used by implementation.

If RAN3 decides to send LS to RAN1, it’s important to inform RAN1 that the soft indication is not supported to get rid of DCI design effort.

- Whether WAB-gNB is able to send Hard/NA resource indication

It was not discussed in last meeting. In our point of view, the WAB-gNB should also be able to send Hard/NA resource indication to BH-gNB, that is because the WAB-gNB and the BH-gNB have the same priority in resource coordination, which is essentially different to IAB.

If RAN3 decides to send LS to RAN1, RAN3 inform RAN1 that WAB-gNB and BH-gNB have the same priority, which is different to IAB. It’s important for RAN1 to understand why WAB-gNB can also send H/NA resource indication and other aspects different with IAB, e.g., timing of WAB-gNB is independent to BH-gNB.

In summary, we believe it’s important to send RAN1 LS including following content:

- Ask the which domain to support,

- The difference between WAB and IAB (such as the same priority between WAB-gNB and BH-gNB),

- WAB-gNB and BH-gNB can send H/NA resource indication to each other

- Not supporting S resource indication

Proposal 2: If RAN3 decide to support resource coordination in Rel-19, RAN3 send LS to RAN1 as early as possible.

Proposal 3: The LS at least includes following contents:

- The decision to support resource coordination for WAB in Rel-19

- Describe the difference between IAB and WAB (e.g., the same priority between WAB-gNB and BH-gNB)

- WAB-gNB and BH-gNB can send H/NA resource indication to each other

- The decision of not supporting S resource indication

- Ask RAN1 whether time/frequency domain coordination is supported

3 Conclusion

In the previous sections we made the following proposals:

Proposal 1: RAN3 can try best to complete resource coordination for WAB in Rel-19, otherwise it is done within R19 TEI.

Proposal 2: If RAN3 decide to support resource coordination in Rel-19, RAN3 send LS to RAN1 as early as possible.

Proposal 3: The LS at least includes following contents:

- The decision to support resource coordination for WAB in Rel-19

- Describe the difference between IAB and WAB (e.g., the same priority between WAB-gNB and BH-gNB)

- WAB-gNB and BH-gNB can send H/NA resource indication to each other

- The decision of not supporting S resource indication

- Ask RAN1 whether time/frequency domain coordination is supported

4 Reference

- RAN3#127bis Minutes, RAN3 Chair

- RAN3#128 Minutes, RAN3 Chair