3GPP TSG-RAN WG3 Meeting #129                                                                                R3-255247

Bengaluru, India, 25th – 29th, August 2025

Source:	CATT

Title:	(TP to BLCRs for TS 38.473) On remaining issue of OD-SSB

Agenda Item:	17.2

Document for:	Discussion and Approval

1 Introduction

For NES on-demand SSB SCell operation topic, the WID [1] content is as follows:

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we will discuss the remaining issue of on-demand SSB SCell operation, our views will be given as well.

2 Discussion

2.1 Interaction over F1 interface

The OD-SSB transmissions are SSB transmissions different to always-on SSB (AO-SSB), that OD-SSB is not always transmitted but only activated as needed (e.g., used for SCell activation or L3 measurement). Due to the OD-SSB transmission, always-on SSB may not be needed, an OD-SSB cell would not transmit always-on SSB or would transmit spares always-on SSB. There are two cases of OD-SSB SCell:

Case #1: No AO-SSB on the cell

Case #2: AO-SSB is periodically transmitted on the cell, where the AO-SSB can be cell-defining SSB (CD-SSB) on synchronization raster or non-cell-defining SSB (NCD-SSB).

To facilitate UE to use OD-SSB, gNB-DU can configure the UE with OD-SSB via RRC message, activate or deactivate the OD-SSB transmission by RRC or MAC CE. OD-SSB operation including the OD-SSB transmission, no AO-SSB and spares AO-SSB transmission are all configured by gNB-DU based on cell condition and energy saving requirement.

Observation 1: The OD-SSB operation including OD-SSB transmission, no AO-SSB or spares AO-SSB transmission is configured by gNB-DU.

In the RAN3#127 meeting [2], the issue about whether CU indicating the allowed cells for OD-SSB to DU was discussed and the potential WF is provided:

RAN3#127: For the allowing On-demand SSB SCell, we discussed the two way-forward as follows: - WF1: No new List from the gNB-CU to the gNB-DU for “allowing OD SSB SCell” is needed, instead we can reuse, if needed, the Rel-18 existing “Cells Allowed to be Deactivated List” (CU to DU). Potential rewording of the procedure may be needed to make the “list” more general. - WF2: It is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”. But the above is not agreed, there are two concerns raised during the discussion: - One concern is that gNB-DU does not always need the “CU to DU” assistance in order to go to the OD-SSB SCell operation. - Another concern is how and whether to handle the transition from PCell normal operation to OD-SSB SCell only operation is unclear. To be continued at the next meeting on the above basis.

RAN3#127bis: The CU reuses the existing IE (the Cells Allowed to be Deactivated List IE) to indicate the allowed cells for on-demand SSB operation to the DU. The details of stage2/3 impact can be further discussed.

However, since the OD-SSB is only transmitted as needed, the change of OD-SSB transmissions status will not impact the cell coverage. So, gNB-CU would not care about whether OD-SSB is configured and the OD-SSB transmission status. Hence it makes no sense for gNB-DU to inform the OD-SSB transmission to gNB-CU or gNB-CU to indicate allowance on OD-SSB transmissions to gNB-DU.

Observation 2: It’s not necessary for gNB-DU to inform gNB-CU the OD-SSB transmission status, also not needed for gNB-CU to send allowance of OD-SSB transmissions to gNB-DU.

As well known, the AO-SSB (i.e., legacy SSB) deactivation can impact the cell coverage. When OD-SSB is applied, the transition from normal cell to OD-SSB SCell may lead to AO-SSB deactivation. There are three consequences after a normal cell switching to OD-SSB operation:

- Case a): No AO-SSB transmission

- Case b): With AO-SSB of only NCD-SSB

- Case c): With AO-SSB of CD-SSB on synch-raster

For the former two cases, the legacy CD-SSB transmissions are deactivated, which results in the NES cell coverage reduction and sole SCell operation. However, there must be coverage of normal cell(s) in the same DU overlapping with the NES cell (otherwise this NES cell cannot operate as OD-SSB SCell). Therefore, although the always-on CD-SSB is deactivated for OD-SSB SCell, the overall coverage of the DU is not impacted. The gNB-DU does not need the permission of gNB-CU on CD-SSB deactivation for OD-SSB SCell.

Observation 3: gNB-DU does not need the permission from gNB-CU on CD-SSB deactivation for OD-SSB SCell.

In the last RAN2 meeting [3], some agreements were made for OD-SSB based L3 measurement:

Agreements on OD-SSB

- (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

…

- In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation:

• The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation.

• The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

Spec impact can be further discussed in running CR preparation.

- When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

- RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

- RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.

- When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

- RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

Per these agreements, we can observe that:

For Case 1, i.e., the SCell does not have AO-SSB, UE starts L3 measurement after OD-SSB is activated. The measurement objective for OD-SSB is configured to UE via legacy servingCellMO and SMTC is configured via legacy SMTC.

For Case 2, i.e., the SCell has AO-SSB, there are two conditions:

- When AO-SSB and OD-SSB have the same frequency, UE performs L3 measurement still towards the legacy measurement objective after the OD-SSB is activated, but with the OD-SSB specific SMTC. The OD-SSB specific SMTC is configured via a new SMTC IE.

- When AO-SSB and OD-SSB have different frequency, UE performs L3 measurement towards the OD-SSB specific measurement objective after OD-SSB is activated with OD-SSB specific SMTC. The OD-SSB specific measurement objective is configured via a new servingCellMO IE. And the OD-SSB specific SMTC is configured via the legacy SMTC IE.

Based on these agreements, RRC running CR in RAN2 [4] has captured the new servingCellMO IE in OD-SSB configuration information to support the condition AO-SSB and OD-SSB have different frequency, and has captured the new SMTC IE to support the condition AO-SSB and OD-SSB have the same frequency:

#### OD-SSB-Config

The IE OD-SSB-Config is used to configure the OD-SSB activated by a MAC CE see TS 38.321 [3], clause XXX

OD-SSB-Config information element

-- ASN1START

-- TAG-OD-SSB-CONFIG-START

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-ConfigId-r19                    OD-SSB-ConfigId-r19,

od-ssb-ActivationStatus-r19            ENUMERATED {activated},

od-ssb-Periodicity-r19                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL, -- Cond ODssbAOssb

od-ssb-sfn-Offset-r19                   INTEGER (0..15)                                                         OPTIONAL, -- Cond ODssbAOssb

od-ssb-halfFrameIndex-r19              ENUMERATED {zero, one}                                                  OPTIONAL, -- Cond ODssbAOssb

od-ssb-absoluteFrequency-r19           ARFCN-ValueNR                                                           OPTIONAL, -- Cond ODssbAOssb

od-ssb-PositionsInBurst-r19            CHOICE {

shortBitmap                            BIT STRING (SIZE (4)),

mediumBitmap                           BIT STRING (SIZE (8)),

longBitmap                             BIT STRING (SIZE (64))

}                                                                                                              OPTIONAL, -- Cond ODssbAOssb

od-ssbSubcarrierSpacing-r19            SubcarrierSpacing                                                       OPTIONAL, -- Cond ODssbOnly

od-ss-PBCH-BlockPower-r19              INTEGER (-60..50)                                                       OPTIONAL, -- Cond ODssbOnly

od-ssb-nrofBurst-r19                   INTEGER (1..8)                                                          OPTIONAL, -- Need R

servingCellMO-r19                      MeasObjectId                                                            OPTIONAL, -- Cond InterFreq

smtc1-r19                            SSB-MTC                                                                 OPTIONAL, -- Cond IntraFreq

...

}

During the F1 setup procedure, the gNB-DU includes the information of measurement frequency(ies) and timing for reference signal in the MeasurementTimingConfiguration IE for each cell in the F1AP F1 Setup Request message. The gNB-CU can configure the legacy measurement objective and legacy SMTC to UE based on the received MeasurementTimingConfiguration IE for each UE’s serving cell. The content of MeasurementTimingConfiguration IE is defined by an inter-node RRC message in TS 38.331. After that the gNB-CU can indicate the gNB-DU the measurement objective ID (servingCellMO) configured for each serving cell via F1AP UE Context Setup/modification Request message, since the servingCellMO needs to be configured to UE in CellGroupConfig IE which is generated by gNB-DU in RRC message.

In Case 1, since the legacy servingCellMO and legacy SMTC are used to configure the OD-SSB based L3 measurement, we believe the legacy mechanism above can be reused.

For Case 2, if AO-SSB and OD-SSB have the same frequency, the OD-SSB based L3 measurement is configured by the legacy servingCellMO and new SMTC IE. Since the legacy servingCellMO is used and the new SMTC IE is sent in the OD-SSB configuration which is generated by DU, the legacy mechanism above can be reused.

For Case 2, if AO-SSB and OD-SSB have different frequency, the OD-SSB specific measurement objective is configured via the new servingCellMO, and the OD-SSB specific SMTC is configured via legacy SMTC IE. The gNB-CU is in charge of configuring the new measurement objective, thus it should indicate the objective ID (new servingCellMO) to gNB-DU, so that gNB-DU can include the new servingCellMO in the OD-SSB configuration. Therefore, a new servingCellMO IE for OD-SSB should be added to the SCell To Be Setup List IE of F1AP UE Context Setup/Modification Request message.

Observation 4: For Case 2 AO-SSB and OD-SSB have different frequency, a new servingCellMO IE for OD-SSB should be introduced to the SCell To Be Setup List IE of F1AP UE Context Setup/Modification Request message.

Proposal 1: A new servingCellMO IE for OD-SSB should be introduced to the SCell To Be Setup List IE of F1AP UE Context Setup/Modification Request message.

A TP to BLCR for F1AP is provided in Annex.

Furthermore, in this case gNB-CU at least should know the OD-SSB measurement frequency so as to configure the new servingCellMO for OD-SSB, but gNB-CU cannot discriminate whether a measurement timing configuration received from gNB-DU is for OD-SSB or not through existing MeasurementTimingConfiguration IE (i.e., inter-node RRC message). Thus, this inter-node RRC message should also be enhanced. However, all the inter-node RRC messages are defined in RRC spec, which is not in RAN3 scope. RAN3 can send a LS to RAN2 to ask for the enhancement to the inter-node RRC message MeasurementTimingConfiguration.

Observation 5: For Case 2 AO-SSB and OD-SSB have different frequency, gNB-CU should be able to discriminate whether a measurement timing configuration received from gNB-DU is for OD-SSB or not through MeasurementTimingConfiguration IE. RAN3 can send a LS to RAN2 for the enhancement to inter-node RRC message.

Proposal 2: RAN3 can send a LS to RAN2 for the enhancement to MeasurementTimingConfiguration RRC message, so that gNB-CU can discriminate whether a measurement timing configuration received from gNB-DU is for OD-SSB or not.

3 Conclusion

According to the analysis in section 2, we have following observations and proposals:

Observation 1: The OD-SSB operation including OD-SSB transmission, no AO-SSB or spares AO-SSB transmission is configured by gNB-DU.

Observation 2: It’s not necessary for gNB-DU to inform gNB-CU the OD-SSB transmission status, also not needed for gNB-CU to send allowance of OD-SSB transmissions to gNB-DU.

Observation 3: gNB-DU does not need the permission from gNB-CU on CD-SSB deactivation for OD-SSB SCell.

Observation 4: For Case 2 AO-SSB and OD-SSB have different frequency, a new servingCellMO IE for OD-SSB should be introduced to the SCell To Be Setup List IE of F1AP UE Context Setup/Modification Request message.

Proposal 1: A new servingCellMO IE for OD-SSB should be introduced to the SCell To Be Setup List IE of F1AP UE Context Setup/Modification Request message.

Observation 5: For Case 2 AO-SSB and OD-SSB have different frequency, gNB-CU should be able to discriminate whether a measurement timing configuration received from gNB-DU is for OD-SSB or not through MeasurementTimingConfiguration IE. RAN3 can send a LS to RAN2 for the enhancement to inter-node RRC message.

Proposal 2: RAN3 can send a LS to RAN2 for the enhancement to MeasurementTimingConfiguration RRC message, so that gNB-CU can discriminate whether a measurement timing configuration received from gNB-DU is for OD-SSB or not.

4 Reference

- RP-242354, Revised WID: Enhancements of network energy savings for NR, RAN#105, Ericsson

- RAN3#127 Minutes, RAN3 chair

- draft_MeetingReport_RAN2#130

- R2-250xxxx, Running RRC CR for enhancements for network energy efficiency, Ericsson, RAN2#131

Annex: TP for BLCR to TS 38.473

Start of TP

8.3.1	UE Context Setup

8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

If the servingCellMO List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, select servingCellMO after determining the list of BWPs for the UE and include the list of servingCellMOs that have been encoded in CellGroupConfig IE as ServingCellMO-encoded-in-CGC List IE in the UE CONTEXT SETUP RESPONSE message.

If the Configured BWP List IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, take it into account when requesting the gNB-DU for generating preconfigured measurement GAP for the indicated BWPs.

If the SpCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly.

If the SCell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider it as a list of candidate SCells to be set up. If the SCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly. If the servingCellMO for OD-SSB IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for OD-SSB for the indicated SCell accordingly.

Next Change

8.3.4	UE Context Modification (gNB-CU initiated)

8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

If the SpCell ID IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace any previously received value and regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the ServCellIndex IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take this into account for the indicated SpCell. If the SpCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly. If the servingCellMO List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, configure servingCellMO after determining the list of BWPs for the UE and include the list of servingCellMOs that have been encoded in CellGroupConfig IE as ServingCellMO-encoded-in-CGC List IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the Configured BWP List IE is included in the UE CONTEXT MODIFICATION RESPONSE message the gNB-CU shall, if supported, take it into account when requesting the gNB-DU for generating preconfigured measurement GAP for the indicated BWPs.

If the Preconfigured Measurement GAP Request IE is present in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATON REQUEST message, the gNB-DU shall, if supported, consider that the content of the previous CellGroupConfig IE was not sent to the UE and generate the pre-configured measurement GAP for the indicated BWPs in the MeasConfig IE. If the gNB-DU successfully generates pre-configured measurement GAP for the indicated BWPs, the gNB-DU shall update the CellGroupConfig IE with the content of the previous CellGroupConfig IE and the preconfigured measurement GAP configuration in the UE CONTEXT MODIFICATION RESPONSE message.

If the SCell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a list of candidate SCells to be set up. If the SCell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message and the indicated SCell(s) are already setup, the gNB-DU shall replace any previously received value. If the SCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly. If the servingCellMO for OD-SSB IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for OD-SSB for the indicated SCell accordingly.

Next Change

9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31, ...) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| >> servingCellMO for OD-SSB | O |  | INTEGER (1..64, ...) | This IE indicates the measurement objective for OD-SSB when the OD-SSB and always-on SSB for the SCell have different centre frequencies. | YES | ignore |
| ***************** Unchanged parts are omitted ******************** |  |  |  |  |  |  |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N or U2U relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofLTMgNBDUs | Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

Next Change

9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31, ...) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| >> servingCellMO for OD-SSB | O |  | INTEGER (1..64, ...) | This IE indicates the measurement objective for OD-SSB when the OD-SSB and always-on SSB for the SCell have different centre frequencies. | YES | jgnore |
| ***************Unchanged parts are omitted********************* |  |  |  |  |  |  |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying or L2 N3C relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channel allowed for L2 U2N or U2U relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofSLdestinations | Maximum number of destination for NR sidelink communication, the maximum value is 32 |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

| Condition | Explanation |
| --- | --- |
| ifCHOcancel | This IE may be present if the CHO Trigger IE is present and set to "CHO-cancel". |

End of TP