3GPP TSG-RAN WG3 #129	          R3-255292

# Bengaluru, India, 25 – 29 August 2025

Agenda item:	14.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP to BL CR for TS 38.300) Clarification on the OAM requirements

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses the missing OAM requirement.

# 2	OAM configuration for NTN Payload

The gNB needs to be configured with the Ephemeris information for the hosting NTN payload and also the neighbouring NTN payload(s) for following reasons:

- For ISL. A regenerative NTN payload may terminate one or more inter-satellite links toward other regenerative payloads. ISL usually uses laser communication, and requires Pointing, Acquisition, and Tracking (PAT). To support PAT, a gNB/satellite need to be configured with the ephemeris information of the peer NTN payload.

- The gNB need the neighbour satellite ephemeris information to generate NTN-NeighCellConfig in SIB19

- The neighbour satellite ephemeris information is also needed to configure SMTC and measurement gaps.

- The neighbour satellite ephemeris information is also needed for configuring the measurement (e.g. reportConfigNR and MeasObjectNR) including target cell reference location.

- In addition, in IoT NTN, the neighbour satellite ephemeris information is also needed to generate SIB32 and SIB33.

- SIB32: The IE SystemInformationBlockType32 contains satellite assistance information for prediction of discontinuous coverage.

- SIB33: The IE SystemInformationBlockType33 contains satellite assistance information for neighbour cells.

Observation 1: the NTN gNB/eNB need to be configured with the satellite ephemeris information of neighbouring NTN payload(s).

TS 38.300 defines following OAM requirements related to the NTN payload:

### 16.14.7 O&M Requirements The following NTN related parameters shall be provided by O&M to the gNB providing NTN access: - Ephemeris information describing the orbital trajectory information or coordinates for the NTN payload. This information is provided on a regular basis or upon demand to the gNB;

Current Stage-2 uses “NTN payload”. It is is unclear on whether the gNB is only configured with the NTN payload which hosts the gNB, or also configured with the NTN payloads which hosts the neighbour gNB. We propose to clarify this in the Stage-2.

Proposal 1: clarify the gNB also need to be configured with the satellite ephemeris information of neighbouring gNB/NTN Payload.

An example for the possible change to Stage-2 TP is:

### 16.14.7 O&M Requirements The following NTN related parameters shall be provided by O&M to the gNB providing NTN access: - Ephemeris information describing the orbital trajectory information or coordinates for the hosting NTN payload of the serving gNB, and optionally for the neighbouring NTN payload(s). This information is provided on a regular basis or upon demand to the gNB;

Draft Stage-2 TP can be found in Annex A – TP to BL CR for TS 38.300

# 3	Conclusion

In this contribution, we discussed the OAM requirement to support NR NTN. Our proposals are:

Observation 1: the NTN gNB/eNB need to be configured with the satellite ephemeris information of neighbouring NTN payload(s).

Proposal 1: clarify the gNB also need to be configured with the satellite ephemeris information of neighbouring gNB/NTN Payload.

Draft Stage-2 TP can be found in Annex A – TP to BL CR for TS 38.300

# References

- TS 38.300

# Annex A – TP to BL CR for TS 38.300

-----------------Start of the Changes-------------------

### 16.14.7	O&M Requirements

The following NTN related parameters shall be provided by O&M to the gNB providing NTN access:

-	Ephemeris information describing the orbital trajectory information or coordinates for the hosting NTN payload of the serving gNB, and optionally for the neighbouring NTN payload(s). This information is provided on a regular basis or upon demand to the gNB;

-	Two different sets of ephemeris format shall be supported:

-	Set 1: NTN payload position and velocity state vectors:

-	Position;

-	Velocity.

-	Set 2: At least the following parameters in orbital parameter ephemeris format, as specified in NIMA TR 8350.2 [51]:

-	Semi-major axis;

-	Eccentricity;

-	Argument of periapsis;

-	Longitude of ascending node;

-	Inclination;

-	Mean anomaly at epoch time.

-	The explicit epoch time associated to ephemeris data;

-	The location of the NTN Gateways;

NOTE 1:	The ephemeris of the NTN payloads and the location of the NTN Gateways, are used at least for the Uplink timing and frequency synchronization. It may also be used for the random access and the mobility management purposes.

-	Additional information to enable gNB operation for feeder/service link switch overs.

NOTE 2:	The NTN related parameters provided by O&M to the gNB may depend on the type of supported service links, i.e., Earth-fixed, quasi-Earth-fixed, or Earth-moving.

-----------------End of the Changes-------------------