3GPP TSG-RAN WG3 Meeting #129	R3-255368

Bengaluru, India, 25th – 29th August, 2025

Title: 	Remaining open issues on split architecture support for Rel-18 “AI/ML for NG-RAN” use cases

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to split architecture support for Rel-18 use cases.

This contribution further analyzes the technical enablers for specifying support of split architecture for Rel-18 use cases, and provides proposals for discussion based on the outcome of the last RAN3 meeting.

# 2. Discussion

In the last meeting RAN3 discussed the split architecture support for Rel-18 RAN AI/ML use cases and agreed the following [4]:

Introduce new IE to include DL and UL Packet Loss Rate at PDCP Level in R19 refer to SA5 spec DL/UL UE Throughput Measurement will not be supported in split architecture in R19. Introduce the start and stop indication over F1 for Delay measurement. Introduce Data Collection ID in UE associated messages over E1 Turn WA to agreement: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface. Assign new BL CR for TS38.425 Nokia

The agreements above have been reflected in E1AP and F1AP BLCRs in [5] and [6], respectively, but there are still some FFS that need to be solved.

## 2.1 On UE performance metrics

The scenario to consider for UE performance metrics collection and reporting is illustrated in Figure 1: the measurement collection and reporting is triggered e.g. after an AI/ML-based mobility decision (i.e., handover) taken by the source gNB towards a split target gNB is successfully executed. The collection and reporting of UE performance over Xn from target split gNB to the source gNB rely on the Data Collection Reporting (Initiation) procedures as specified in Rel-18.

Figure 1 - Scenario for reporting UE performance from split target gNB to source gNB.

In RAN3#127bis meeting, and based on the agreements from RAN3#127 meeting [18] concerning the packet delay DL/UL measurements for which a “gNB-CU-UP-centric” approach is considered, RAN3 agreed to send the LS to SA5 in [6] to check the feasibility to specify UE throughput DL/UL and packet loss rate DL/UL measured at PDCP (hence in the gNB-CU-UP). In this way all UE performance metrics could be transferred over the E1 interface, by means of new Data Collection Reporting (Initiation) procedures agreed in RAN3#127bis meeting, without impacting the F1 interface [8]. Below an excerpt of the reply LS from SA5 in [9] received during the last meeting:

# 1 Overall description SA5 would like to thank RAN3 for their LS in R3-252492 on clarification on UE performance measurement at PDCP level. SA5 has discussed the LS from RAN3 and would like to provide the following clarifications: DL UE Throughput Measurement at PDCP Level SA5 would like to note that while the measurement ‘Average DL UE Buffered Throughput per DRB,’ defined in TS 28.552 clause 5.1.1.3.7, provides useful information, we would further like to inform RAN3 about the following information: *** skip non-relevant text *** - While the measurement provides a means to estimate PDCP-layer throughput based on the volume of PDCP SDUs buffered and the achievable DRB throughput (as inferred from DDDS), it does not directly measure over-the-air data transmission. - The precision of the measurement may vary depending on implementation-specific aspects such as: *** skip non-relevant text *** Average DL UE Buffered Throughput per DRB measurement is not a direct or precise representation of total PDCP-level throughput per UE. UL UE Throughput Measurement at PDCP Level SA5 would like to confirm that uplink PDCP throughput measurements are not defined in TS 28.552. While UE Data Volume counters are defined in TS 28.558 clause 6.3.1.5 (per UE), these represent cumulative data volume, not throughput. SA5 notes that throughput requires both volume and time components. Therefore, although TS 28.558 provides a useful basis, it is not sufficient by itself to define an accurate PDCP-layer UL throughput measurement. Packet Loss Rate at PDCP Level (UL and DL) SA5 recognises the importance of monitoring packet loss at the PDCP level in both UL and DL directions. The following measurements are already defined: - UL PDCP SDU Loss Rate – TS 28.552 clause 5.1.3.1.1 - DL PDCP SDU Drop Rate in gNB-CU-UP – TS 28.552 clause 5.1.3.2.1 These metrics allow packet loss monitoring in UL and packet drop monitoring in DL for PDCP-layer traffic. *** skip non-relevant text ***

DL/UL Average UE Throughput

From the above excerpt of the SA5 feedback, RAN3 reached the agreement in the last meeting that DL/UL UE Throughput measurement at PDCP is not supported in split architecture in Rel-19 [4]:

DL/UL UE Throughput Measurement will not be supported in split architecture in R19.

Our understanding of the above agreement is that it is the throughput measured at PDCP which is not supported, not the measurement per se. In other words, we think that per-UE DL/UL UE Throughout measurements measured at RLC (and hence in target gNB-DU) specified in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 [10], respectively, could still be made available at the target gNB-CU-CP over the F1 interface, even if we acknowledge that these measurements do not strictly reflect the UE performance as experienced at service level, for which the availability of PDCP-level measurements would have been preferred.

Observation 1: Per-UE RLC-level DL/UL Throughput measurements as specified in TS 28.558 could be used as UE performance in split architecture.

Referring to RLC-level measurements in TS 28.558 would also be aligned with the UE performance framework in XnAP as specified in Rel-18 RAN AI/ML [11], which only considered non-split architecture:

#### 9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4; Corresponds to Average UE Throughput DL per-UE as specified in TS 28.558 [58] clause 6.3.1.4.1.. Average UE Throughput UL; O; Bit Rate 9.2.3.4; Corresponds to Average UE Throughput UL per-UE as specified in TS 28.558 [58] clause 6.3.1.4.2.. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11. s

In such manner, the feature parity between non-split and split architectures would be ensured.

Observation 2: Referring to per-UE RLC-level DL/UL Throughput measurements as specified in TS 28.558 ensures alignment with the UE performance framework as specified in Rel-18 for the non-split architecture.

Based on the above, we propose the following:

Proposal 1: RAN3 to agree to refer to Average UE Throughput DL and Average UE Throughput UL measurements specified in TS 28.558 and sent from the gNB-DU to the gNB-CU-CP over F1.

Another aspect that should be discussed concerns the granularity of reported throughput measurements from the gNB-DU to the gNB-CU-CP. Following the agreement from RAN3#127bis meeting [8] that packet delay measurements are sent from gNB-CU-UP to gNB-CU-CP with a per-DRB per-UE granularity, and considering that SA5 clarified in [12] that DL/UL Average UE Throughput metrics as specified in TS 28.558 originate from per-DRB per-UE measurements (meaning that per-DRB per-UE measurements are anyway performed and hance made available at the gNB-DU), we think the same should also be valid for the reporting over F1. Hence we propose:

Proposal 2: RAN3 to agree to report Average UE Throughput DL and Average UE Throughput UL measurements from the gNB-DU to the gNB-CU-CP over F1 with a per-DRB per-UE granularity.

For the procedures to be used over F1 in order to request and report the throughput measurements from the gNB-DU, we think we could enhance the Resource Status Reporting (Initiation) procedures, as already agreed for the request and report of the gNB-DU’s measured EC [4]:

Turn WA to agreement: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

Proposal 3: RAN3 to agree to enhance the existing Resource Status Reporting (Initiation) procedures over F1 to request and report the DL/UL Average UE Throughput from gNB-DU to gNB-CU-CP.

DL/UL Packet Loss

For the DL/UL Packet Loss measurements, instead, and based on the reply LS from SA5 in [9], RAN3 agreed in the last meeting to refer to measurements as “DL PDCP SDU Drop Rate in gNB-CU-UP” and “UL PDCP SDU Loss Rate” as specified in TS 28.552 clauses 5.1.3.2.1 and 5.1.3.1.1, respectively [4]:

Introduce new IE to include DL and UL Packet Loss Rate at PDCP Level in R19 refer to SA5 spec

and such agreement has been reflected in the latest version of the E1AP BLCR in [5], with an Editor’s Note stating that the semantics description of Average Packet Loss IE could be further checked based on the potential LS from SA5.

#### 9.3.1.aa UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average Packet Delay; O; 9.3.1.bb; Corresponds to Average Packet Delay per-UE as specified in TS 28.558 [XX] clause 6.3.1.1.. Average Packet Loss DL; O; 9.3.1.cc; Corresponds to DL PDCP SDU Drop Rate in gNB-CU-UP as specified in TS 28.552 [XX1] clause 5.1.3.2.1. Average Packet Loss UL; O; 9.3.1.cc; Corresponds to UL PDCP SDU Loss Rate as specified in TS 28.552 [XX1] clause 5.1.3.1.1. Editor’s note: the semantics description could be further refined. <<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>> #### 9.3.1.cc Average Packet Loss This IE indicates the Average Packet Loss in the UL and DL directions. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Average Packet Loss; M; INTEGER (0..10000000, ...). Editor’s note: the semantics description of Average Packet Loss IE to be further checked based on potential LS from SA5.

As already explained in our paper in [13] pertaining slice-based UE performance, referencing to TS 28.552 for the PDCP-level measurements as suggested by SA5 to reflect DL/UL Average Packet Loss metrics is not appropriate because:

•	TS 28.552 does not list UE level measurements (as TS 28.558 does) but it encompasses NG-RAN level performance measurements, with a per-DRB granularity and, optionally, per-DRB per supported S-NSSAI (i.e., per-DRB per-slice);

•	the PDCP-level measurements mentioned in the SA5 reply LS are valid only for the split gNB scenario. Hence they are not applicable in case of non-split gNB, and as a consequence the feature parity between split and non-split architectures in not achieved.

Observation 3: Referencing to TS 28.552 for the PDCP-level measurements to reflect DL/UL Average Packet Loss metrics is not appropriate because TS 28.552 does not specify UE level measurements. Moreover, the PDCP-level measurements suggested by SA5 are valid only for the split gNB-scenario, hence feature parity with non-split architecture is not achieved.

To cope with the above concerns, RAN3 agreed in the last meeting to include a question in the reply LS to SA5 in [13] to ask whether packet loss measurements could be specified with a per-DRB per-UE granularity in the UL and DL at PDCP layer, that could also be aggregated to reflect a per-UE metric:

1. Overall Description: RAN3 thanks SA5 for their reply LS on per-UE UE performance metrics. RAN3 would further like to ask for clarifications about the granularity of Average Packet Delay UL/DL and Average Packet Loss UL/DL. RAN3 assumes that UE level performance is derived from DRB level measurements of the concerned UE. *** skip non-relevant text *** Average packet loss measurements: In RAN3#127-bis, RAN3 sent an LS to SA5 (R3-252436) to ask SA5 if it is feasible to measure Packet Loss Rate in UL and DL at PDCP level for both split and non-split architecture and, if feasible, RAN3 asked SA5 to specify these measurements for both split and non-split architecture. In the LS from SA5 R3-253824, SA5 has confirmed that the following metrics can be used for UE Packet Loss: - UL PDCP SDU Loss Rate – TS 28.552 clause 5.1.3.1.1 - DL PDCP SDU Drop Rate in gNB-CU-UP – TS 28.552 clause 5.1.3.2.1 RAN3 would like to further ask whether it is feasible to specify the above Packet Loss Rate metrics with a per-DRB per-UE granularity in the UL and DL at PDCP layer, which may also be aggregated to reflect a per-UE Packet Loss Rate metric. If the above is feasible then RAN3 would like to ask SA5 to carry out specification work accordingly. *** skip non-relevant text ***

Based on the above we think that, for solving the open point on the semantics description of the DL/UL Average Packet Loss IE, the SA5 input is essential.

Proposal 4: To remove the Editor’s Notes on the semantics description of the DL/UL Average Packet Loss, RAN3 to wait for the reply LS from SA5 on the feasibility of specifying PDCP-level packet loss metrics in DL and UL with a per-DRB per-UE granularity, that could be aggregated to reflect a per-UE metric, and valid for both split and non-split architectures.

In the latest version of the E1AP BLCR for Rel-19 RAN AI/ML in [5] the DL/UL Average Packet Loss metrics are encoded as integers within a range from 0 to 107, but in the semantics description important information needed to represent an actual measured value of a packet loss is missing, e.g., the measurement granularity. In our understanding, the same conclusion as currently captured in the XnAP BLCR for Rel-19 RAN AI/ML in [15] on the range and semantics description of the DL/UL Average Slice Packet Loss also applies to the range and semantics description of the DL/UL Average Packet Loss IEs in E1AP BLCR.

Observation 4: The same conclusion as currently captured in the XnAP BLCR for Rel-19 RAN AI/ML on the range and semantics description of the DL/UL Average Slice Packet Loss also applies to the range and semantics description of the DL/UL Average Packet Loss IEs in E1AP BLCR.

The encoding in both XnAP and E1AP BLCRs originates from the contribution in [16], where the shortcomings of the Rel-18 encoding of the Average Packet Loss DL IE in XnAP are detailed, and by considering the RAN3 agreement from RAN3#127bis [8]:

A Rel18 correction to the Average Packet Loss DL IE may be made when agreements on whether and how to specify Average Packet Loss in Rel19 are taken.

We think that the encoding currently captured in the E1AP BLCR can be kept if it is completed by also clarifying the granularity associated to the signalled packet loss values, so to derive the actual packet loss experienced by the concerned UE after an AI/ML-driven mobility event (i.e., handover). Still in [16], it is proposed to adopt a granularity of 10-8, and for us this makes sense in order to provide accurate and reliable packet loss measurements reflecting the actual measured packet loss.

Proposal 5: RAN3 to agree to keep the range associated to the “IE type and reference” of the Average Packet Loss DL/UL as currently captured in the E1AP BLCR, and to indicate the measurement granularity of 10-8 in the semantics description.

Considering that this is the last meeting for the Rel-19 RAN AI/ML WI, in case SA5 would not be able to send a reply LS during the meeting week, RAN3 should also discuss whether considering RLC-level packet loss currently specified in TS 28.558 for the DL only (clause 6.3.1.2.1) with a per-DRB per-UE granularity is acceptable in order to conclude the WI on time. If agreed, such metric could be measured by gNB-DU and reported to the gNB-CU-CP over F1 with a per-DRB per-UE granularity and by enhancing the legacy Resource Status Reporting (Initiation) procedures.

Proposal 6: RAN3 to discuss whether considering the RLC-level packet loss currently specified in TS 28.558 for the DL only is acceptable in order to conclude the WI on time.

Proposal 6bis: If Proposal 6 is agreed, RAN3 to agree to enhance the existing Resource Status Reporting (Initiation) procedures over F1 to request and report the DL Average Packet Loss from gNB-DU to gNB-CU-CP with a per-DRB per-UE granularity.

DL/UL Packet Delay

DL/UL Packet Delay measurements comprise delay components measured at both gNB-DU and gNB-CU-UP, as specified in TS 38.314 [17]. RAN3 already agreed in previous meetings [8][18] that the packet delay components measured at gNB-DU and gNB-CU-UP are expected to be aggregated at the gNB-CU-UP level and then the aggregated result is sent to the gNB-CU-CP over the E1 interface. Concerning the packet delay measurement at gNB-DU level, and based on TS 38.425 [19], the UL Delay DL Result and DL Delay DU Result IEs are measured per-DRB per-UE and are provided from the gNB-DU to the gNB-CU-UP over F1-U within the ASSISTANCE INFORMATION DATA (PDU Type 2) frame.

F1AP considerations

Once the UL Delay DU Result and DL Delay DU Result measurements are made available at the gNB-CU-UP level, the latter will aggregate them with the corresponding UL and DL delay components measured at the gNB-CU-UP level – that is, average delay over F1-U, average delay DL in CU-CP and average PDCP re-ordering delay (all defined per-DRB per-UE) – in order to derive the overall DL/UL packet delay measurements to be sent over E1 to the gNB-CU-CP as per the agreement from RAN3#127bis meeting [8].

Still referring to Figure 1, the target gNB-CU-CP receives over Xn the UE performance collection time duration and reporting periodicity (i.e., UE Performance Collection Configuration IE and Reporting Periodicity for Data Collection IE, respectively) within the DATA COLLECTION REQUEST message from the source gNB, hence the gNB-CU-CP has full control of the measurements collection process towards the gNB-DU. And as captured in the latest version of the F1AP BLCR [5], the Performance Delay Monitoring IE includes the Performance Delay Monitoring Request IE having the “UL and DL” and the “stop” codepoints as indications to ‘start’ and ‘stop’ the measurement collection in the gNB-DU, similar to the existing QoS Monitoring framework. Note that the “UL and DL” codepoint is considered because, as part of the request for UE performance reporting over Xn, a single bit (6th bit) in the Report Characteristics for Data Collection IE of the DATA COLLECTION REQUEST message is used to request for reporting of both DL and UL average packet delay simultaneously. Therefore, in alignment with the XnAP, also over F1 the reporting of both DL and UL average packet delay measurements is always considered in the UE performance reporting framework.

Observation 5: It is the target gNB-CU-CP which controls the collection of information to be reported by the gNB-DU over F1 by means of the Performance Delay Monitoring Request IE included in the Performance Delay Monitoring IE.

An aspect that requires further discussion is the need to indicate the reporting periodicity to the gNB-DU, which is FFS in the F1AP BLCR [5]. In fact, the Performance Delay Monitoring IE includes the Performance Delay Monitoring Reporting Periodicity IE whose codepoints are aligned with the values already specified for the XnAP Reporting Periodicity for Data Collection IE of the DATA COLLECTION REQUEST message. Some companies, in fact, have concerns on the actual need of this IE and prefer to keep the same approach used in the legacy QoS framework, where the reporting periodicity is not specified and it is up to implementation.

In our view, the reporting of delay measurements from the gNB-DU should not be left to implementation but should be done consistently with the measurements collection pertaining the gNB-CU-UP. This would also follow the same approach as already specified for the non-split architecture for the reporting of UE performance over Xn, for which the periodicity is clearly indicated by the source gNB.

Proposal 7: RAN3 to agree to introduce the Performance Delay Monitoring Reporting Periodicity IE within the Performance Delay Monitoring IE, i.e., remove the FFS in the F1AP BLCR.

As captured in the F1AP BLCR [5], the existing F1AP UE Context Setup and UE Context Modification (gNB-CU initiated) procedures are enhanced to include the new Performance Delay Monitoring IE. However, it is still FFS whether the new IE should also be included in the DRB to Be Setup List IE of the UE CONTEXT MODIFICATION REQUEST message. In our view, this is needed because it may happen that, during the collection window, new DRBs are set up for the UE for which a context has already been established at the gNB-DU, and for these DRBs there could be the need to collect the corresponding UE performance metrics.

Proposal 8: RAN3 to agree to introduce the Performance Delay Monitoring IE within the DRB to Be Setup List IE of the UE CONTEXT MODIFICATION REQUEST message, i.e., remove the FFS in the F1AP BLCR.

A TP for the F1AP BLCR reflecting proposals above for F1 is provided in [20].

E1AP considerations

Similar considerations as previously provided for the configuration to collect (and also report) DL/UL packet delay measurements at the gNB-DU apply. Hence, we think that a new Performance Delay Monitoring IE similar to the one already agreed for the F1AP  should also be introduced in E1, with the only difference with respect to the new F1AP IE is that the reporting periodicity does need to be included because the new E1AP IE is needed just to trigger the collection of measurements at the gNB-CU-UP, since the reporting to the gNB-CU-CP is only applicable to the overall aggregated DL/UL packet delay measurements.

Proposal 9: RAN3 to agree to introduce a new Performance Delay Monitoring IE which includes the codepoints ‘DL and UL’ and ‘stop’ to start and stop the collection at gNB-CU-UP of both DL and UL delay measurements pertaining to the gNB-CU-UP, respectively.

Concerning the E1 procedure(s) to be used to provide the DL/UL delay measurements collection configuration, we think it is reasonable to reuse the existing E1AP Bearer Context Setup and Bearer Context Modification (gNB-CU-CP initiated) procedures, which are enhanced to include the new Performance Delay Monitoring IE as proposed in Proposal 9. The presence of the new IE triggers the gNB-CU-UP to collect DL and UL packet delay measurements, which are then aggregated with the gNB-DU delay measurements and eventually reported to the gNB-CU-CP over E1.

Proposal 10: RAN3 to agree that the existing E1AP Bearer Context Setup and Bearer Context Modification (gNB-CU-CP initiated) procedures are enhanced to include the new Performance Delay Monitoring IE for the collection configuration of DL/UL delay measurements at gNB-CU-UP within the BEARER CONTEXT SETUP REQUEST and BEARER CONTEXT MODIFICATION REQUEST messages.

We now need to discuss how to configure the reporting over E1 of the overall aggregated DL/UL packet delay measurements. Still taking into account the scenario as in Figure 1 of a target split gNB being involved in XnAP Data Collection Reporting (Initiation) procedures from a source gNB, we should follow an approach similar to the one used since Rel-18:

- rely on an initiation procedure (target gNB-CU-CP  target gNB-CU-UP) to provide the configuration to be used by the target gNB-CU-UP to configure the reporting of the overall (aggregated) DL/UL packet delay measurements, including the reporting periodicity;

- identify procedure(s) which triggers the reporting of measurements, which is(are) properly linked with the initiation procedure as in step a. via a Data Collection ID IE;

- identify a procedure for the actual reporting of the overall DL/UL packet delay measurements.

For aspects a. and c., respectively, RAN3 already agreed in RAN3#127bis meeting to introduce new Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 [8]:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

The latest E1AP BLCR in [5] includes a bit in the Report Characteristics for Data Collection IE (1st bit) dedicated to the request of the overall DL/UL packet delay (i.e., the aggregated value) and the Reporting Periodicity for Data Collection IE as part of the DATA COLLECTION REQUEST message. Our understanding is that the reporting periodicity of the overall DL/UL packet delay measurements should be set according to the value as configured by the source gNB as part of the XnAP DATA COLLECTION REQUEST message.

Proposal 11: RAN3 to agree that the reporting periodicity of the overall DL/UL packet delay measurements over E1 is set by the gNB-CU-CP according to the value as received from the source gNB as part of the XnAP DATA COLLECTION REQUEST message.

Finally, for aspect b., we think that, in addition to the Bearer Context Setup already agreed in the last meeting, the existing Bearer Context Modification (gNB-CU-CP initiated) procedure could be enhanced to include the Data Collection ID in order to trigger the reporting of the overall DL/UL packet delay measurements from gNB-CU-UP to gNB-CU-CP which is eventually performed via the E1AP DATA COLLECTION UPDATE message based on the collection configuration provided via the E1AP DATA COLLECTION REQUEST message.

Proposal 12: RAN3 to agree that the existing E1AP Bearer Context Modification (gNB-CU-CP initiated) procedure is enhanced to include the Data Collection ID IE associated to the Data Collection Reporting Initiation over E1 used to configure the collection and reporting of the overall aggregated DL/UL delay measurements from gNB-CU-UP to gNB-CU-CP.

### 2.1.1 Other aspects

Same as for the XnAP Data Collection Reporting Initiation procedure, we think that partial reporting should be supported also as part of the new Data Collection Reporting Initiation procedure over E1AP. For reporting the list of measurements failed to be reported by the gNB-CU-UP, and considering that UE performance metrics are defined at node level, the DATA COLLECTION RESPONSE message in E1AP needs include the Node Measurement Initiation Result List IE which contains a bitmap indicating the measurements failed to be reported and an appropriate cause value.

Proposal 13: RAN3 to agree to include the Node Measurement Initiation Result List IE which lists the measurements failed to be reported by the gNB-CU-UP with an appropriate cause value.

A TP for the E1AP BLCR reflecting proposals above for E1 is provided in [21].

Another aspect that should be discussed concerns the granularity of overall DL/UL packet delay measurements once made available at the gNB-CU-CP, which need to be reported over the Xn interface to the source gNB. For Rel-18, the granularity of UE performance (including packet delay) has been specified to be per-UE, but the granularity of the overall DL/UL packet delay measurements at the gNB-CU-CP was agreed to be per-DRB per-UE. Pending the SA5 feedback on whether per-DRB per-UE packet delay measurements could be aggregated to reflect a per-UE metric [13], we think that eventually it will be up to the target gNB-CU-CP to derive the corresponding per-UE DL/UL Average Packet Delay measurements and report over Xn to the source gNB.

Proposal 14: RAN3 to agree that it will be up to the gNB-CU-CP to derive the corresponding per-UE DL/UL Average Packet Delay measurements originating from per-DRB per-UE measurements sent over E1 by the gNB-CU-UP.

## 2.2 On gNB-DU Energy Cost (EC) reporting over F1

In the last meeting RAN3 agreed to exchange the gNB-DU’s EC by enhancing the existing Resource Status Reporting (Initiation) procedures over F1, and this was also reflected in the F1AP BLCR in [5]. Moreover, RAN3 also agreed a Stage 2 TP for the TS 38.401 BLCR in [22] for listing the OAM requirements needed to enable the configuration of the “energy consumption – to – energy cost” mapping rule (which is defined by the Operator and it is provided by the OAM to the gNB-DU), as well as the recommended time window which should be considered by the gNB-DU itself when averaging the energy consumption measurements.

In our view RAN3 should make SA5 aware of the latest status of the discussion concerning the OAM support to enable the reporting of the gNB-DU’s EC, so that SA5 could introduce changes in their relevant specifications (if needed) to enable such OAM configuration, in addition to those ones already introduced by SA5 and communicated via the LS in [23] (which is the reply LS to the LS in [24] sent by RAN3).

Proposal 15: RAN3 to agree to send a LS to SA5 (reply LS to R3-251529) to make SA5 aware of the status of the RAN3 discussion pertaining the OAM support for enabling the reporting of the gNB-DU’s measured EC over F1.

A draft Reply LS to SA5 is provided in [25].

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals:

On reporting measured UE performance – DL/UL Average UE Throughput

Observation 1: Per-UE RLC-level DL/UL Throughput measurements as specified in TS 28.558 could be used as UE performance in split architecture.

Observation 2: Referring to per-UE RLC-level DL/UL Throughput measurements as specified in TS 28.558 ensures alignment with the UE performance framework as specified in Rel-18 for the non-split architecture.

Proposal 1: RAN3 to agree to refer to Average UE Throughput DL and Average UE Throughput UL measurements specified in TS 28.558 and sent from the gNB-DU to the gNB-CU-CP over F1.

Proposal 2: RAN3 to agree to report Average UE Throughput DL and Average UE Throughput UL measurements from the gNB-DU to the gNB-CU-CP over F1 with a per-DRB per-UE granularity.

Proposal 3: RAN3 to agree to enhance the existing Resource Status Reporting (Initiation) procedures over F1 to request and report the DL/UL Average UE Throughput from gNB-DU to gNB-CU-CP.

On reporting measured UE performance – DL/UL Packet Loss

Observation 3: Referencing to TS 28.552 for the PDCP-level measurements to reflect DL/UL Average Packet Loss metrics is not appropriate because TS 28.552 does not specify UE level measurements. Moreover, the PDCP-level measurements suggested by SA5 are valid only for the split gNB-scenario, hence feature parity with non-split architecture is not achieved.

Proposal 4: To remove the Editor’s Notes on the semantics description of the DL/UL Average Packet Loss, RAN3 to wait for the reply LS from SA5 on the feasibility of specifying PDCP-level packet loss metrics in DL and UL with a per-DRB per-UE granularity, that could be aggregated to reflect a per-UE metric, and valid for both split and non-split architectures.

Observation 4: The same conclusion as currently captured in the XnAP BLCR for Rel-19 RAN AI/ML on the range and semantics description of the DL/UL Average Slice Packet Loss also applies to the range and semantics description of the DL/UL Average Packet Loss IEs in E1AP BLCR.

Proposal 5: RAN3 to agree to keep the range associated to the “IE type and reference” of the Average Packet Loss DL/UL as currently captured in the E1AP BLCR, and to indicate the measurement granularity of 10-8 in the semantics description.

Proposal 6: RAN3 to discuss whether considering the RLC-level packet loss currently specified in TS 28.558 for the DL only is acceptable in order to conclude the WI on time.

Proposal 6bis: If Proposal 6 is agreed, RAN3 to agree to enhance the existing Resource Status Reporting (Initiation) procedures over F1 to request and report the DL Average Packet Loss from gNB-DU to gNB-CU-CP with a per-DRB per-UE granularity.

On reporting measured UE performance – DL/UL Packet Delay

Observation 5: It is the target gNB-CU-CP which controls the collection of information to be reported by the gNB-DU over F1 by means of the Performance Delay Monitoring Request IE included in the Performance Delay Monitoring IE.

Proposal 7: RAN3 to agree to introduce the Performance Delay Monitoring Reporting Periodicity IE within the Performance Delay Monitoring IE, i.e., remove the FFS in the F1AP BLCR.

Proposal 8: RAN3 to agree to introduce the Performance Delay Monitoring IE within the DRB to Be Setup List IE of the UE CONTEXT MODIFICATION REQUEST message, i.e., remove the FFS in the F1AP BLCR.

A TP for the F1AP BLCR reflecting proposals above for F1 is provided in [20].

Proposal 9: RAN3 to agree to introduce a new Performance Delay Monitoring IE which includes the codepoints ‘DL and UL’ and ‘stop’ to start and stop the collection at gNB-CU-UP of both DL and UL delay measurements pertaining to the gNB-CU-UP, respectively.

Proposal 10: RAN3 to agree that the existing E1AP Bearer Context Setup and Bearer Context Modification (gNB-CU-CP initiated) procedures are enhanced to include the new Performance Delay Monitoring IE for the collection configuration of DL/UL delay measurements at gNB-CU-UP within the BEARER CONTEXT SETUP REQUEST and BEARER CONTEXT MODIFICATION REQUEST messages.

Proposal 11: RAN3 to agree that the reporting periodicity of the overall DL/UL packet delay measurements over E1 is set by the gNB-CU-CP according to the value as received from the source gNB as part of the XnAP DATA COLLECTION REQUEST message.

Proposal 12: RAN3 to agree that the existing E1AP Bearer Context Modification (gNB-CU-CP initiated) procedure is enhanced to include the Data Collection ID IE associated to the Data Collection Reporting Initiation over E1 used to configure the collection and reporting of the overall aggregated DL/UL delay measurements from gNB-CU-UP to gNB-CU-CP.

On reporting measured UE performance – Other aspects

Proposal 13: RAN3 to agree to include the Node Measurement Initiation Result List IE which lists the measurements failed to be reported by the gNB-CU-UP with an appropriate cause value.

A TP for the E1AP BLCR reflecting proposals above for E1 is provided in [21].

Proposal 14: RAN3 to agree that it will be up to the gNB-CU-CP to derive the corresponding per-UE DL/UL Average Packet Delay measurements originating from per-DRB per-UE measurements sent over E1 by the gNB-CU-UP.

On gNB-DU Energy Cost (EC) reporting over F1

Proposal 15: RAN3 to agree to send a LS to SA5 (reply LS to R3-251529) to make SA5 aware of the status of the RAN3 discussion pertaining the OAM support for enabling the reporting of the gNB-DU’s measured EC over F1.

A draft Reply LS to SA5 is provided in [25].

# 4. Reference

- RP-251245, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#108

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#128 Chairman’s notes (RAN3_128_agenda_20250523_1530_EOM.doc)

- R3-255042, (BL CR to 37.483) Support of enhancements on AI/ML for NG-RAN, Samsung, ZTE Corporation, Nokia

- R3-255046, (BL CR to 38.473) Support of enhancements on AI/ML for NG-RAN, Ericsson (Rapporteur), ZTE, NEC, Nokia, Huawei, Ofinno

- R3-252492, LS on clarification on UE performance measurement at PDCP level, Rel-19, RAN3 (ZTE)

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- R3-253824 (S5-252839), Reply LS on clarification on UE performance measurement at PDCP level (Response to R3-252492), Rel-19, SA5 (NEC, Intel, Ericsson)

- 3GPP TS 28.558, Management and orchestration; UE level measurements for 5G system, Rel-19, V19.4.0

- 3GPP TS 38.423, “NG-RAN; Xn Application Protocol (XnAP)”, Rel-18, V18.6.0

- R3-253023, Reply LS on per-UE UE performance metrics (Response to R3-247775 / S5-251324), Rel-19, SA5 (NEC, Intel, Ericsson),

- R3-255357, Further discussion on per-UE per-slice UE performance in AI/ML-based Network Slicing, Huawei, Jio Platforms, FiberCop, BT

- R3-253816, Reply LS on per-UE UE performance metrics (Response to R3-253023 / S5-252056), Rel-18, RAN3 (Nokia)

- R3-255044, (BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange, FiberCop, InterDigital, Ofinno

- R3-253431, On packet loss metric in UE performance, Ericsson, Jio Platforms, Deutsche Telekom, InterDigital

- 3GPP TS 38.314, NR; Layer 2 measurements, Rel-18, V18.0.0

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- 3GPP TS 38.425, “NG-RAN; NR user plane protocol”, Rel-18, V18.1.0

- R3-255370, (TP for AI/ML BLCR to TS 38.473) F1AP enhancements for split architecture support for Rel-18 “AI/ML for NG-RAN” use cases, Huawei

- R3-255371, (TP for AI/ML BLCR to TS 37.483) E1AP enhancements for split architecture support for Rel-18 “AI/ML for NG-RAN” use cases, Huawei

- R3-253949, (TP for BLCR to TS 38.401) OAM Requirements for Energy Saving for split architecture, CMCC, Samsung, Huawei, NEC, CATT, ZTE, Nokia, Ericsson, Deutsche Telekom, FiberCop, Jio Platforms

- R3-251529, LS reply to LS on energy saving for split gNB (Response to R3-247907 / SA5-250288), SA5 (Ericsson)

- R3-247907, LS on energy saving for split gNB, RAN3 (CATT)

- R3-255369, [Draft] LS reply to LS on energy saving for split gNB, Huawei