3GPP TSG-RAN WG3 Meeting #129	R3-255444

Bengaluru, IN, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1294 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inventory completion indication (via Inventory Report Transfer) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Jio Platforms Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_solutions-Core |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add a BOOLEAN IE InventoryCompleteIndicator inside Inventory Report Transfer so NG‑RAN can mark the final Inventory Report when no follow‑on Command is expected. INVENTORY REPORT remains NG‑RAN → AMF; AMF forwards to AIOTF. No new EP is introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a BOOLEAN IE InventoryCompleteIndicator inside Inventory Report Transfer so NG‑RAN can mark the final Inventory Report when no follow‑on Command is expected. INVENTORY REPORT remains NG‑RAN → AMF; AMF forwards to AIOTF. No new EP is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Provide an explicit, low‑overhead end‑of‑inventory signal for inventory‑only scenarios; avoids timers/ambiguity and minimizes protocol churn. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.xx.2, 9.3.1, 9.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.xx.1	Inventory Request

-RefNote – Baseline CR- R3-253795

#### 8.xx.1.1	General

The purpose of the Inventory Request procedure is to request the NG-RAN node to perform inventory over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB.

#### 8.xx.1.2	Successful Operation

Figure 8.xx.1.2-1: Inventory Request, successful operation.

The A-IoT CN node triggers the procedure by sending an INVENTORY REQUEST message to the NG-RAN node. If the NG-RAN node accepts the request, the NG-RAN node responds with the INVENTORY RESPONSE message.

The NG-RAN node shall, include the same values of the AIOTF Identifier IE and A-IoT Correlation Identifier IE in the INVENTORY RESPONSE message as received from the INVENTORY REQUEST message.

If the Inventory Assistance Information IE is included in the INVENTORY REQUEST message, the NG-RAN node may take the information into account as defined in TS 38.300 [8].

Interaction with Inventory Report procedure

When starting the requested inventory, the NG-RAN node shall, include the same values of the AIOTF Identifier IE and A-IoT Correlation Identifier IE in the INVENTORY REPORT message(s) as received from the INVENTORY REQUEST message

If the Follow on Command Indication IE set to "true" is included in the Inventory Request Transfer IE in the INVENTORY REQUEST message, the NG-RAN node shall consider that there will be command to be transmitted to the device(s), and shall allocate and include a RAN A-IoT Device NGAP ID IE in each Device Report Item IE within the Inventory Report Transfer IE in the INVENTORY REPORT message(s).

When the NG-RAN node sends the final INVENTORY REPORT for an A-IoT session and no follow-on Command is expected,

It shall include InventoryCompleteIndicator set to 'true' in the Inventory Report Transfer IE of that last message.

If multiple INVENTORY REPORT messages are sent for the same session, only the final message shall carry

InventoryCompleteIndicator = true. The AMF shall forward this indication to the AIOTF.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 9.3	Information Element Definitions

### 9.3.1	Radio Network Layer Related IEs

#### 9.3.1.z InventoryCompleteIndicator (new)

The InventoryCompleteIndicator IE indicates that the carrying INVENTORY REPORT is the final report for the session and that no follow-on Command is expected.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| InventoryCompleteIndicator | O | — | BOOLEAN (9.4.3: InventoryCompleteIndicator) | Set by NG-RAN to true only on the final INVENTORY REPORT of a session when no follow-on Command is expected; otherwise absent (or false). AMF forwards to AIOTF. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.3	Elementary Procedure Definitions

----Skpi unchanged part---

InventoryCompleteIndicator ::= BOOLEAN

InventoryReportTransfer ::= SEQUENCE {

...,

inventoryCompleteIndicator  InventoryCompleteIndicator OPTIONAL,

...

}

-- IE registration:

-- id-InventoryCompleteIndicator (ProtocolIE-ID to be assigned), Criticality: ignore, Presence: optional

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>