3GPP TSG-RAN WG3 Meeting #129	R3-255591

Bengaluru, India, 25th – 29th August 2025

Agenda Item:	12.2

Source: 				LG Electronics Inc.

Title:			Discussion on remaining issues for WAB

Document for:		Discussion and Decision

1.	Introduction

In RAN3#128 meeting, RAN3 had agreed that it is possible to establish an Xn connection between two WAB-gNBs. To reflect the RAN3 agreement, additional information is needed to be exchanged between WAB-gNBs. Also, the Additional ULI to indicate the location information of WAB-node is currently provided to the UE’s AMF by using the non UE-associated signalling and UE-associated signalling. Therefore, it is still FFS on whether to keep the additional ULI in the UE-associated signalling.Based on the SA2 reply LS [1], RAN3 agreed to support two-logical gNB solution for the UE’s AMF change. However, whether to support the single-gNB solution with two logical cells is still FFS. In this contribution, we will discuss the remaining open issues (e.g., for additional ULI, the Xn setup between WAB-gNB, and the mobility), and provide our views on it.

2.	Discussion

In RAN3#127 meeting, it was agreed to include the Additional ULI IE (which contains a CGI and a TAI) into the User Location Information IE in the BL CR to TS 38.413 [2]. Therefore, this IE is provided to the UE’s AMF by using the non UE-associated signalling. In RAN3#127bis meeting, however, it was also agreed that the WAB-gNB reports the Additional ULI to the network within the NG Setup and RAN Configuration Update procedures in order to provide the Additional ULI for the case where there are no UEs in CONNECTED state. As a result, the Additional ULI to indicate the location information of WAB-node is currently provided to the UE’s AMF by using the non UE-associated signalling and UE-associated signalling.

We think that the Additional ULI needs to be removed from the UE-associated signalling. As soon as the WAB-gNB is initialized (e.g., OAM configuration), it establishes the NG connection(s) toward the AMF(s). Since the Additional ULI is already included in the NG SETUP REQUEST message, the AMF can determine the current location of the WAB-node. When the additional ULI has changed (e.g. due to WAB-node movement), the WAB-gNB derives the new Additional ULI and reports it to the AMF by the RAN CONFIGURATION UPDATE message. Therefore, although the Additional ULI is removed from the UE-associated signallings, there is no issue to determine the current location of the WAB-node in the AMF.

Proposal 1: The Additional ULI is removed from the UE-associated signallings.

In TS 23.501, SA2 already captured that the Additional ULI IE is provided to the AMF by the UE-associated signallings. To reflect the RAN3 agreements to the SA2 specification, therefore, it is needed to inform the SA2 of the removal of the Additional ULI IE in the UE-associated signallings.

Proposal 2: It is proposed to inform the SA2 of the removal of the Additional ULI in the UE-associated signallings (a draft Reply LS in Appendix).

It is still FFS whether to introduce a “WAB-gNB” indication in the NG SETUP REQUEST message. Since the Additional ULI in the NG SETUP REQUEST message enables the AMF to discover that the transmitting NG-RAN node is a WAB-gNB, there is no need to introduce another new indication in this NGAP message.

Proposal 3: Upon the reception of the Additional ULI in the NG SETUP REQUEST message, the AMF discovers that the transmitting NG-RAN node is a WAB-gNB.

Regarding the Xn connection between WAB-gNBs, it was agreed in RAN3 that it is possible to establish an Xn connection between two WAB-gNBs. It is also possible to prevent establishment of such connections.

Observation 1: The Xn interface between WAB-gNBs can be established.

As shown in Figure X.1-1 of [3], after the WAB-gNB setup, the WAB-gNB can initiate the establishment of the Xn connection(s) towards the BH-gNB and/or other NG-RAN node(s) by sending the Xn SETUP REQUEST message. According to previous RAN3 agreement, the WAB-gNB includes the information (i.e., the WAB-MT Identifier IE) related to the WAB-MT to the BH-gNB to indicate the co-location of the WAB-MT and the WAB-gNB. However, since the Xn interface between WAB-gNBs can be established, we think that the WAB-gNB can contain additional assistance information (e.g., the expected mobility (i.e., stationary/mobile) and the expected moving trajectory of the WAB-node, and so on) related to the WAB mobility in the Xn SETUP REQUEST message. If the receiving NG-RAN node is another WAB-gNB, it decides whether to establish the Xn interface with the WAB-gNB based on this assistance information. For example, if both WAB-gNBs have a similar moving characteristics/route, or either or both of WAB-gNBs are stationary, the receiving WAB-gNB can respond with the Xn SETUP RESPONSE message. In addition, it is possible for the receiving WAB-gNB to include additional assistance information related to the WAB mobility in the Xn SETUP RESPONSE message. The initiating WAB-gNB can take this information into account to manage the Xn interface between WAB-gNBs.

If the Xn setup between WAB-gNBs is not allowed or deemed not appropriate, the receiving WAB-gNB should reject the Xn setup with the initiating WAB-gNB and reply with the Xn SETUP FAILURE message. When the Xn SETUP FAILURE message contains appropriate information (e.g., WAB-gNB indication, new cause value “Target not allowed due to WAB”), the initiating node does not reinitiate the Xn Setup procedure towards the same receiving WAB-gNB. In addition, the receiving WAB-gNB may provide additional assistance information related to the WAB mobility to enable the initiating WAB-gNB to determine whether and when to reinitiate the Xn Setup procedure towards the same receiving node.

Proposal 4: The WAB-gNB can contain additional assistance information related to the WAB mobility during the Xn Setup procedure.

Proposal 5: The receiving WAB-gNB replies with the Xn SETUP FAILURE message including appropriate information to prevent re-initiation of the Xn Setup procedure towards the same receiving WAB-gNB.

In last RAN3 meeting, it was agreed that the “WAB-MT ID” sent from the WAB-gNB to the BH-gNB consists of the WAB-MT’s C-RNTI assigned by the BH-gNB and the cell ID of BH-gNB´s cell serving the WAB MT. However, this RAN3 agreement is not yet captured into the BL CR to TS 38.423 [4].

Proposal 6: The WAB-MT Identifier IE is defined as the combination of the WAB-MT’s C-RNTI assigned by the BH-gNB and the cell ID of BH-gNB´s cell serving the WAB MT in the BL CR to TS 38.423 [4].

According to SA2 reply LS in [1], two CRs [5][6] are endorsed to support the two logical gNB solutions. However, there is no SA2 consensus on single gNB solutions suggested by the RAN3. Based on the SA2, RAN3 in [7] agreed to support two-logical gNB solution for the UE’s AMF change. However, whether to support the single-gNB solution with two logical cells is still FFS. Considering the limited time for this WI, it is proposed to only support the two logical gNB solution in this release.

Proposal 7: In this release, the two logical gNB solution is only supported for UE’s AMF change during WAB-gNB mobility.

In order to capture the above observation and proposals, the following proposal is also suggested to RAN3:

Proposal 8: It is proposed to agree the corresponding TPs in [8].

3.	Conclusion

In this contribution, we focused on the remaining open issues (e.g., for additional ULI, the Xn setup between WAB-gNB, and the mobility), and provided our views on it. The following proposals are kindly suggested to RAN3:

Proposal 1: The Additional ULI is removed from the UE-associated signallings.

Proposal 2: It is proposed to inform the SA2 of the removal of the Additional ULI in the UE-associated signallings (a draft Reply LS in Appendix).

Proposal 3: Upon the reception of the Additional ULI in the NG SETUP REQUEST message, the AMF discovers that the transmitting NG-RAN node is a WAB-gNB.

Proposal 4: The WAB-gNB can contain additional assistance information related to the WAB mobility during the Xn Setup procedure.

Proposal 5: The receiving WAB-gNB replies with the Xn SETUP FAILURE message including appropriate information to prevent re-initiation of the Xn Setup procedure towards the same receiving WAB-gNB.

Proposal 6: The WAB-MT Identifier IE is defined as the combination of the WAB-MT’s C-RNTI assigned by the BH-gNB and the cell ID of BH-gNB´s cell serving the WAB MT in the BL CR to TS 38.423 [4].

Proposal 7: In this release, the two logical gNB solution is only supported for UE’s AMF change during WAB-gNB mobility.

Proposal 8: It is proposed to agree the corresponding TPs in [8].

4.	Reference

- R3-250073/S2-2501336, “Reply LS on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility),” SA2, January, 2025.

- R3-255052, “(BL CR to 38.413 for WAB) Support for Wireless Access Backhaul”, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, China Telecom, ZTE, Qualcomm, Samsung, CATT, Jio Platforms (JPL), Lenovo, August, 2025.

- R3-255050, “(BL CR to 38.401 for WAB) Support for Wireless Access Backhaul,” Ericsson, ZTE, Nokia, Nokia Shanghai Bell, Huawei, Samsung, Lenovo, Qualcomm, Jio Platforms, August, 2025.

- R3-255053, “(BL CR to 38.423 for WAB) Support for Wireless Access Backhaul,” Nokia, Nokia Shanghai Bell, Ericsson, ZTE, LG Electronics, Qualcomm, Huawei, China Telecom, Samsung, Lenovo, August, 2025.

- S2-2501319, “On MWAB mobility,” Nokia, January, 2025.

- S2-2501320, “MWAB Mobility handling clarification,” Ericsson, January, 2025.

- R3-250898, “(TP for WAB BL CR for TS 38.401) WAB mobility”, CATT, China Telecom, Ericsson, Nokia, Nokia Shanghai Bell, Lenovo, Huawei, Samsung, Qualcomm, NTTDOCOMO, LGE, Jio, February, 2025.

- R3-255599, “(TP to TS 38.401, 38.413 and 38.423) TP for WAB support,” LG Electronics, August, 2025.

- R3-253013/S2-2504110, “Reply LS on FS_VMR_Ph2 solution impacts to RAN (Additional ULI),” SA2, May, 2025.

6.	Appendix: Reply LS for S2-2504110 [9]

This appendix provides the draft reply LS to SA2 based on the proposals of this contribution.

----------------Start of the First Change---------------

3GPP TSG-RAN WG3 #129 R3-25xxxx Bengaluru, India, 25th – 29th August 2025 # Title: [Draft] Reply LS on FS_VMR_Ph2 solution impacts to RAN (Additional ULI) # Response to: R3-253013/S2-2504110 # Release: Rel-19 # Work Item: VMR_Ph2, NR_WAB_5GFemto-Core Source: RAN3 To: SA2 Cc: RAN2 Contact Person: #### Name: Seokjung Kim #### E-mail Address: seokjung.kim@lge.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org # Attachments: R3-255599 # 1 Overall description RAN3 thanks SA2 for the Reply LS on FS_VMR_Ph2 solution impacts to RAN (Additional ULI). RAN3 would like to further provide the following feedback to SA2. With respect to Additional ULI, RAN3 agreed that the MWAB-gNB reports the additional ULI to the network within the non UE-associated procedures (i.e., NG Setup and RAN Configuration Update procedures) instead of the NGAP UE-associated procedures. Therefore, the Additional ULI is removed from the NGAP UE-associated procedures. Meanwhile, RAN3 notices that clause 5.49.4 of TS 23.501 states the usage of Additional ULI in the NGAP UE-associated procedures. The TP to the 38.413 BL CR (R3-255599) capturing the above RAN3 agreements is provided in the attachment. 2. Actions: To SA2: ACTION: RAN3 would like to call for SA2 to take the above RAN3 agreements on additional ULI into account and update their specification(s). 3. Date of Next RAN3 Meetings: 3GPP TSG RAN3#129bis 13 – 17 October 2025 Prague, CZ 3GPP TSG RAN3#130 17 – 21 November 2025 Dallas, US

-----------------End of the First Change---------------