3GPP TSG-RAN WG3 Meeting #129	R3-255734

Bengaluru, India, 25 – 29 August 2025

Agenda Item:       10.2

Source:     ZTE Corporation

Title: 	(TP for SON BLCR for 38.473) LTM Ping-Pong left Issues

Document for:	Discussion and Approval

1	Introduction

This contribution provides  further discussion on Ping-Pong Avoidance, and corresponding TP for TS38.473 for SON BL CR.

2	Discussion

The latest progress from the RAN3#128 meeting:

UHI and ping-pong issue:

CU informs relevant information to the target DU via CU-DU Cell Switch Notification.

Down selection of following solutions:

1a: CU provide full UHI list with L1/L3 type to the DU (maximum item number is 16).

1b: CU provide full UHI list (except PScell) with L1/L3 type to the DU (maximum item number is 16).

2a: CU provide last consecutive L1 entries to the DU.

2b: CU provide FFS number of last consecutive L1 entries to the DU.

No stage2, work in stage3 with 1a.

If the above 1a information is included in F1AP message from CU to DU, the DU may consider the potential mobility issue detected including ping-pong.

FFS on the extra indication for ping-pong case.

During the latest RAN3 meeting, it was agreed that the CU could send UHI information to the target DU via the CU-DU Cell Switch Notification. However, it remains unclear whether the UHI must always be sent to the DU during  each LTM cell switch, or if it is only optionally sent when ping pong is detected by the CU.

We prefer for always sending the UHI, for the following reasons:

-  This ensures that the target DU has access to relevant historical mobility data, enabling it to make more better real-time decisions for future LTM cell switch, not only to prevent ping pong but also to optimize target cell selection; Furthermore, as local AI is expected to play a significant role in 5GA and 6G, UHI will be essential for AI/ML based LTM.

-  In legacy L3 handover, UHI is always sent to the target node via XnAP/NGAP. For example, it is a mandatory IE in the XnAP Handover Request message and in the NGAP Handover Required message.

Proposal 1: The CU shall mandatorily include UHI information in the F1AP CU-DU Cell Switch Notification message.

For optimizing LTM ping pong, it may seem unnecessary to include L3 HOs in the UHI. However, filtering out L3 HOs from the UHI creates gaps in the UHI, leading to misinterpretation of source-target cell pairs. To address this issue, options 1a/1b propose introducing a handover type indicator (L1/L3) , options 2a/2b propose including only one(last) or a limited number of consecutive L1 entries in UHI.

After reconsidering options 2a and 2b, we believe option 2b (allowing multiple consecutive L1 entries in the UHI)  introduces unnecessary complexity to the IE structure.However, Option 2a (limiting to only the last consecutive L1 entries) can still effectively support LTM ping pong optimization. Therefore, we now prefer option 2a.

Proposal 2: CU provide last consecutive L1 entries in the UHI  to the DU.

L3 measurement based LTM is already supported in Rel-18. Regarding the option for the DU to trigger LTM execution, RAN3 has agreed that the CU may provide L3 measurement results and assistance information to the DU, DU will then make the final decision on triggering the LTM cell switch. Therefore, it is suggested that RAN3 discuss whether an "L1/L3 measurement based LTM" indicator should be included in the UHI to distinguish between these two cases.

Proposal 3: RAN3 discuss whether an "L1/L3 measurement based LTM" indicator should be included in the UHI.

- 2 Conclusion:

Proposal 1: The CU shall mandatorily include UHI information in the F1AP CU-DU Cell Switch Notification message.

Proposal 2: CU provide last consecutive L1 entries in the UHI  to the DU.

Proposal 3: RAN3 discuss whether an "L1/L3 measurement based LTM" indicator should be included in the UHI.

Annex : TP for BLCR for TS38.473 for LTM Ping-Pong Avoidance

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

8.3.10	CU-DU Cell Switch Notification

8.3.10.1	General

The purpose of the CU-DU Cell Switch Notification procedure is to enable the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

8.3.10.2	Successful Operation

Figure 8.3.10.2-1: CU-DU Cell Switch Notification procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU CELL SWITCH NOTIFICATION message.

Upon reception of the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

If the LTM Cell Switch Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the TA Information List IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the Tag ID Pointer IE is also included, the gNB-DU shall, if supported, consider it to determine the TAG corresponding to the received TA value.

If the LTM Mobility Information List IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it for optimizing LTM cell switch.

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| LTM Mobility Information List |  | 1 |  | This IE includes information about the last visited consecutive cells associated with LTM cell switch. Most recent cell related information is added to the top of the list. | YES | ignore |
| >LTM Mobility Information List Item |  | 1..<maxnoofCellsinLTMMobilityInfo> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Time UE Stayed in Cell | M |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |
| maxnoofCellsinLTMMobilityInfo | Maximum no. of cells in the LTM mobility information, the maximum value is 16. |

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

LTMInformation-Setup,

LTMConfigurationIDMappingList,

LTMInformation-Modify,

LTMCells-ToBeReleased-List,

LTMCFRAResourceConfig-List,

LTMConfiguration,

LTMMobilityInformation-List,

EarlySyncInformation-Request,

EarlySyncInformation,

<<< NEXT CHANGE >>>

FROM F1AP-Containers

id-candidatePSCellsToCancel,

id-ValidityAreaSpecificSRSInformationExtended,

id-LTMMobilityInformation-List,

<<< NEXT CHANGE >>>

maxnoAggregatedPosSRSCombinations,

maxnoofCandidateCells,

maxnoofSSBIndices,

maxnoofCellsinLTMMobilityInfo

<<< NEXT CHANGE >>>

-- **************************************************************

--

##### -- CU-DU Cell Switch Notification

--

-- **************************************************************

CUDUCellSwitchNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDUCellSwitchNotificationIEs}},

...

}

CUDUCellSwitchNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-LTMCellSwitchInformation	 CRITICALITY ignore	TYPE LTMCellSwitchInformation	PRESENCE optional	}|

{ ID id-TAInformation-List			CRITICALITY ignore	TYPE TAInformation-List			PRESENCE optional		}|

{ ID id-LTMMobilityInformation-List	CRITICALITY ignore	TYPE LTMMobilityInformation-List		PRESENCE mandatory		},

...

}

<<< NEXT CHANGE >>>

9.4.5	Information Element Definitions

#### -- L

LTMMobilityInformation-List	::= SEQUENCE (SIZE(1.. maxnoofCellsinLTMMobilityInfo))	OF LTMMobilityInformation-List-Item

LTMMobilityInformation-List-Item ::= SEQUENCE {

nRCGI		NRCGI,

timeStay	INTEGER (0..40950),

iE-Extensions				ProtocolExtensionContainer { { LTMMobilityInformation-List-Item-ExtIEs} }	OPTIONAL,

...

}

LTMMobilityInformation-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoAggregatedPosSRSCombinations		INTEGER ::= 32

maxnoofCandidateCells					INTEGER ::= 8

maxnoofSSBIndices						INTEGER ::= 64

maxnoofCellsinLTMMobilityInfo			INTEGER ::= 16

<<< NEXT CHANGE >>>

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-candidatePSCellsToCancel							ProtocolIE-ID ::= 858

id-MobilityInitiation								ProtocolIE-ID ::= 859

id-ValidityAreaSpecificSRSInformationExtended		ProtocolIE-ID ::= 860

id-PLMNIndexNRAssistanceInfoForNetShar						ProtocolIE-ID ::= 861

id-LTMMobilityInformation-List					ProtocolIE-ID ::= ddd

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>