3GPP TSG-RAN3 Meeting #129-bis	R3-256529

# Prague, CZ, 13 - 17 Oct 2025

Agenda item:	13.3

Source:	Xiaomi

Title:	Discussion on Sensing procedures and singalling

Document for:	Other

1	Introduction

In this contribution, we’d like to discuss the sensing procedures and signalling of R20 5GA ISAC according to the following SID in [1].

Study the procedures, signaling between RAN and CN to support ISAC [RAN3]

Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3]

- Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts.

- No inter-gNB coordination will be studied.

- Coordination with SA2 as necessary.

2	Discussion

2.1 	General Sensing procedures

In this section, we provide a general sensing call flow for the RAN3 study. Before diving into the detailed procedures, we have the following clarifications regarding the call flow:

Since the architecture options are still under SA2 discussion, we use the general term “sensing CN” in the procedure descriptions. This term may refer to the sensing function in a direct architecture, or to both the sensing function and the AMF in an indirect architecture.

Based on SA2 discussions, from sensing service perspective, the sensing report mode can be one-time, periodic, or event-triggered (aperiodic). This implies that following the sensing request procedure, the sensing RAN node may generate one or multiple sensing reports.

With the above understandings, the following figure shows a general call flow between sensing RAN node and sensing CN:

- The sensing CN obtains the supported sensing information of the sensing RAN node(s) via OAM or signalling.

- The sensing CN sends a Sensing Request message to the sensing RAN node.

- The sensing RAN node sends a Sensing Response message to the sensing CN, if one-shot sensing is requested, the Sensing Response message may include the sensing results.

NOTE 1, the sensing RAN node may instead send a Sensing Failure message to the sensing CN indicating that the sensing procedure could not be performed.

- If more than one sensing are requested, the sensing RAN node performs sensing as requested and sends one or more sensing reports towards the sensing CN including the sensing result(s).

- After the sensing is completed, a sensing termination procedure may be performed to release the sensing resource in sensing RAN node.

In general, introducing a class 1 sensing procedure for sensing request/response/failure message is needed, for periodic or event-triggered sensing, a class 2 sensing report procedure is also needed. And similar to positioning, a gNB or CN initiated measurement termination can be considered, the details can be further discussed in the next few meetings.

Proposal 1, RAN3 agrees the following procedures to support sensing:

- a class 1 sensing procedure including sensing request/response/failure message

- a class 2 sensing report procedure including sensing report message.

- gNB/CN initiated sensing termination procedure(s), FFS on the details.

Proposal 2, RAN3 agrees the TP capturing the call flow of sensing to TR in the Annex.

2.2 	Sensing related information

Information in sensing request

In general, we think the following information is needed in sensing request:

| Information in sensing request | Motivation |
| --- | --- |
| Sensing measurement ID | In the NRPPa protocol, an LMF Measurement ID is used within the measurement procedure and measurement report procedure to uniquely identify a specific measurement. Similarly, for sensing, we propose introducing a dedicated Sensing Measurement ID to serve the same purpose. |
| Report characteristics | As discussed in most of the SA2 solutions in [2], different reporting modes (e.g., one-shot, periodic, or event-triggered) are considered for a sensing service, the selected sensing RAN node also needs to know this information to generate and send reports accordingly. Further discussions may be needed to define the assistance information associated with each reporting mode, which can be addressed in upcoming RAN3 meetings. |
| Requested sensing information (target sensing area or requested sensing TRP) | A sensing RAN node may contain multiple sensing TRPs that collectively cover a wide area. If the target sensing area is smaller than the supported sensing area of a sensing RAN node, not all the TRPs need to perform sensing. Then, how to select sensing TRP needs to be discussed. The following options are proposed for further discussion in RAN3: Option 1: The sensing RAN node selects the sensing TRP based on the sensing area information provided in the sensing request. This implies that the target sensing area information should be included in the sensing request message. Option 2: The sensing CN selects the sensing TRP based on the sensing area information from the sensing service request and the supported sensing capabilities of each TRP, which are configured via OAM or signaling. In this case, the requested sensing TRP information should be included in the sensing request message. |

Proposal 3, RAN3 agrees the following information can be included in a sensing request:

- Sensing measurement ID

- Report characteristics

- Requested sensing information

Proposal 4, RAN3 to discuss which entity is responsible for sensing TRP selection (sensing RAN node or sensing CN).

Information in sensing response/report

The following information is needed in sensing response/report.

| Information in sensing response/report | Motivation |
| --- | --- |
| Sensing Measurement ID | Same as positioning, the measurement ID should be included in the sensing response/report to identify a specific sensing measurement. |
| Sensing Measurement Results | As RAN1 is still discussing the types of sensing measurements, the specific measurement formats and content are yet to be determined. Therefore, RAN3 can await the outcome of RAN1’s discussions. |

Proposal 5, RAN3 agrees that the sensing measurement ID and sensing measurement results should be included in both the sensing response and sensing report messages.

Proposal 6, The detailed structure and content of the sensing measurement results will be defined based on the conclusions from RAN1.

3	Conclusion

In this contribution, we discussed sensing procedures and signalling, and have the following proposals.

General Sensing procedures

Proposal 1, RAN3 agrees the following procedures to support sensing:

- a class 1 sensing procedure including sensing request/response/failure message

- a class 2 sensing report procedure including sensing report message.

- gNB/CN initiated sensing termination procedure(s), FFS on the details.

Proposal 2, RAN3 agrees the TP capturing the call flow of sensing to TR in the Annex.

Sensing related information

Proposal 3, RAN3 agrees the following information can be included in a sensing request:

- Sensing measurement ID

- Report characteristics

- Requested sensing information

Proposal 4, RAN3 to discuss which entity is responsible for sensing TRP selection (sensing RAN node or sensing CN).

Proposal 5, RAN3 agrees that the sensing measurement ID and sensing measurement results should be included in both the sensing response and sensing report messages.

Proposal 6, The detailed structure and content of the sensing measurement results will be defined based on the conclusions from RAN1.

4	Reference

[1] RP-252819, “Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, China Telecom

[2] 3GPP TR 23.700-14, “Study on Integrated Sensing and Communication”, Sep. 2025.

5	Annex (TP for TR)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

X.1	Candidate procedures for sensing

Figure X.1-1: Message flow for sensing

- The sensing CN obtains the supported sensing information of the sensing RAN node(s) via OAM or signalling.

- The sensing CN sends a Sensing Request message to the sensing RAN node.

- The sensing RAN node sends a Sensing Response message to the sensing CN, if one-shot sensing is requested, the Sensing Response message may include the sensing results.

NOTE 1, the sensing RAN node may instead send a Sensing Failure message to the sensing CN indicating that the sensing procedure could not be performed.

- If more than one sensing are requested, the sensing RAN node performs sensing as requested and sends one or more sensing reports towards the sensing CN including the sensing result(s).

- After the sensing is completed, a sensing termination procedure may be performed to release the sensing resource in sensing RAN node.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>