3GPP TSG-RAN3 Meeting #129-bis	R3-256608

Prague, Czech Republic, 13 – 17 Oct 2025

Agenda Item:	11.2.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on MRO enhancements for Inter-CU LTM

Document for:	Discussion/Decision

# 1 Introduction

In this contribution, we discuss the details on supporting MRO enhancements for Inter-CU LTM to support the following objective in WID of Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5 (RP-252560):

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

If needed, co-operate with RAN1, SA5.

# 2 Discussion

## 2.1 LTM failure scenarios

Inter-CU LTM is an extended version of Intra-CU LTM. While, Intra-CU LTM handles cell switch within the same CU, Inter-CU LTM is for mobility between cells that are under different gNBs/CUs. Inter-CU LTM consists of the same basic steps as in Intra-CU LTM like preparing LTM candidate cells, Early sync procedure, and execution of cell switch. These Inter-CU LTM steps involve additional complexity due to the coordination between different CUs.

Observation 1: Inter-CU LTM preparation, early sync and execution procedures are an extension of the Intra-CU LTM procedures for multiple candidate cells belonging to different gNBs.

In Rel-19, the below LTM failure scenarios are defined for Intra-CU LTM:

- Too Late LTM cell switch

- Too Early LTM cell switch

- LTM cell switch to Wrong Cell

- RACH-based and RACH-less LTM failure

As the procedures for Inter-CU LTM and Intra-CU LTM are similar, we think that the LTM failure scenarios should be extended to cover Inter-CU LTM failures as well.

Proposal 1: Intra-CU LTM failure scenarios defined in R19 (Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell) should now be extended to inter-CU LTM as well (both RACH-based and RACH-less).

## 2.2 Baseline MRO scenarios

In Rel-19, the below scenarios are considered for MRO enhancements for LTM to help the network recover from or prevent LTM failure.

- Beam Failure Recovery (BFR) shortly after a successful RACH-less LTM cell switch

- RACH-less LTM cell switch failure followed by successful re-establish/recovery in a different beam in the same target cell

- RACH-less LTM cell switch failure followed by successful re-establish/recovery in the same beam or different beam in the same cell, but a different TA.

Similar MRO scenarios can happen in case of Inter-CU LTM, as UE performs RACH-less LTM on the target cell which belongs to a different CU. Thus, we think that the MRO scenarios for LTM defined in Rel-19 should be extended for Inter-CU LTM as well.

Proposal 2: RAN3 should support extending the MRO for intra-CU LTM scenarios to Inter-CU LTM and support the following scenarios:

- Beam Failure Recovery (BFR) shortly after a successful inter-CU RACH-less LTM cell switch

- Inter-CU RACH-less LTM cell switch failure followed by successful re-establish/recovery in a different beam in the same target cell

- Inter-CU RACH-less LTM cell switch failure followed by successful re-establish/recovery in the same beam or different beam in the same cell, but with a different TA.

## 2.3 New MRO scenarios for consideration

### 2.3.1 RACH-based LTM due to early UL sync failure

In RACH-less LTM, the source DU selects SSB for early UL synchronization based on measurements and signaling exchanged between the source and candidate CUs. However, if the chosen SSB is unsuitable, the early UL sync may fail. This prevents the target cell from providing the necessary TA, forcing the source cell to fall back to a RACH-based LTM procedure to a candidate cell, as shown in below figure. This fallback introduces delay and increases the risk of LTM failure and can happen in both Intra-CU LTM and Inter-CU LTM. Therefore, this scenario should be considered for MRO, as it highlights the need for improved SSB selection strategies and proactive detection of early UL sync issues.

Figure 1: RACH-based LTM due to early UL sync failure on an unsuitable SSB

Observation 2: The source cell may trigger RACH-based LTM to a candidate cell if early uplink sync fails or didn't happen. This can happen due to an unsuitable SSB indicated to the UE for early UL sync and thereby preventing the candidate cell from providing the TA to the source cell.

Proposal 3: RAN3 to consider detection of RACH-based LTM execution due to early UL sync failure for both Intra-CU LTM and Inter-CU LTM MRO enhancements.

### 2.3.2 Beam switching after LTM cell switch

In some scenarios, UE may successfully perform LTM cell switch initial access via SSB1 and subsequently switch to SSB2 for data transmission based on the TCI state indication from the target cell. However, this transition may lead to beam failure on SSB2, necessitating recovery via SSB3, as shown in figure below.

Figure 2: Beam Failure Recovery on a different beam than LTM Initial access

Such dynamic beam switching and failure recovery highlight the issues in early UL sync beam selection and beam management in LTM. Detecting and analysing this beam switching is crucial for optimizing RACH-less LTM procedures, as it enables the network to adjust beam configurations and improve reliability of LTM.

Observation 3: The UE may perform LTM cell switch via SSB1 and switch to SSB2 for data based on TCI state indication provided by Target cell. There could be beam failure shortly on SSB2 after successful LTM cell switch and recovery over SSB3.

Proposal 4: RAN3 should consider the scenario where BFR happens shortly after successful LTM cell switch (Intra-CU/Inter-CU LTM) on a beam different from the one used for LTM cell switch.

### 2.3.3 L3-based vs L1-based LTM

In Rel-19, MRO enhancements for Intra-CU LTM considered L1 measurements for mobility decisions. However, both Rel-18 and Rel-19 have introduced support for L3 measurements-based LTM for Intra-CU and Inter-CU scenarios. This means LTM failure can occur due to issues in either L1-based or L3-based mobility decisions. Without distinguishing the source of failure, it becomes challenging to optimize mobility robustness and recovery strategies. Thus, we propose to introduce MRO support to distinguish whether an LTM failure occurred due to L1-based or L3-based mobility decisions.

Observation 4: Rel-19 MRO for Intra-CU LTM only considered L1 measurements for Intra-CU LTM.

Observation 5: In Rel-18 and Rel-19, L3 measurements-based LTM is also introduced for Intra-CU LTM, Inter-CU LTM. Hence, it is beneficial to distinguish for the LTM failure scenarios if the failure occurred for the case of L3 based LTM or L1 based LTM

Proposal 5: Support distinguishing LTM failure is for L3 based LTM or L1 based LTM for both Intra-CU LTM and Inter-CU LTM.

Observation 6: Improper L1 measurement configuration may lead to RACH-based LTM or unnecessary L1 measurement reporting or improper LTM execution, e.g., even when the current beam is sufficient. This can happen if the UE fails to report suitable beams due to restrictive settings, causing the gNB to miss valid candidates or select suboptimal beams for LTM.

Proposal 6: Support enhancements to optimize L1 measurement configuration

# 3 Summary

Below are the observations and proposals discussed in this document.

Observation 1: Inter-CU LTM preparation, early sync and execution procedures are an extension of the Intra-CU LTM procedures for multiple candidate cells belonging to different gNBs.

Proposal 1: Intra-CU LTM failure scenarios defined in R19 (Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell) should now be extended to inter-CU LTM as well (both RACH-based and RACH-less).

Proposal 2: RAN3 should support extending the MRO for intra-CU LTM scenarios to Inter-CU LTM

- Beam Failure Recovery (BFR) shortly after a successful inter-CU RACH-less LTM cell switch

- Inter-CU RACH-less LTM cell switch failure followed by successful re-establish/recovery in a different beam in the same target cell

- Inter-CU RACH-less LTM cell switch failure followed by successful re-establish/recovery in the same beam or different beam in the same cell, but a different TA.

Observation 2: The source cell may trigger RACH-based LTM to a candidate cell if early uplink sync fails or didn't happen. This can happen due to an unsuitable SSB indicated to the UE for early UL sync and thereby preventing the candidate cell from providing the TA to the source cell.

Proposal 3: RAN3 to consider detection of RACH-based LTM execution due to early UL sync failure for both Intra-CU LTM and Inter-CU LTM MRO enhancements.

Observation 3: The UE may perform LTM cell switch via SSB1 and switch to SSB2 for data based on TCI state indication provided by Target cell. There could be beam failure shortly on SSB2 after successful LTM cell switch and recovery over SSB3.

Proposal 4: RAN3 should consider the scenario where BFR happens shortly after successful LTM cell switch (Intra-CU/Inter-CU LTM) on a beam different from the one used for LTM cell switch.

Observation 4: Rel-19 MRO for Intra-CU LTM only considered L1 measurements for Intra-CU LTM.

Observation 5: In Rel-18 and Rel-19, L3 measurements-based LTM is also introduced for Intra-CU LTM, Inter-CU LTM. Hence, it is beneficial to distinguish for the LTM failure scenarios if the failure occurred for the case of L3 based LTM or L1 based LTM

Proposal 5: Support distinguishing LTM failure is for L3 based LTM or L1 based LTM for both Intra-CU LTM and Inter-CU LTM.

Observation 6: Improper L1 measurement configuration may lead to RACH-based LTM or unnecessary L1 measurement reporting or improper LTM execution, e.g., even when the current beam is sufficient. This can happen if the UE fails to report suitable beams due to restrictive settings, causing the gNB to miss valid candidates or select suboptimal beams for LTM.

Proposal 6: Support enhancements to optimize L1 measurement configuration