3GPP TSG-RAN WG3 Meeting #129-bis	R3- 256687

Prague, CZ, 13 - 17 October 2025

Agenda item	: 10.2.1

Source	: Tejas Networks

Title	: High-level principles and requirements for 6G Architecture

Document for	: Discussion & Decision

# Introduction

RAN#108 agreed 6G Study Item for RAN WGs and revised in RAN#109 [1]. In this contribution, we provide a few high-level issues we believe 6G standardization should focus on.

# Deployment Scenarios

Most of the network deployments can be fit into one of the three scenarios:

Figure 1: Different deployment options

Deployment option D1 allows virtualization of hardware resources to support operations that occur in real-time. This is especially useful when the operator intends dense deployment in a no-so-large geographical area. The distance between RDC and cell site is critical in that propagation delays have to be kept within a limit.

Deployment option D2 will host DU and RU software at the cell site and in this case hardware resource sharing is not possible.

Deployment option D3 refers to the tried and tested deployment where the BBU and RU software all will be hosted at the cell site.

We believe that 6G standardization should support these three deployment options.

Proposal 1: 6GR standardization should support all the three deployment options D1, D2, and D3

## 2.1	Architecture

5G standardized two deployment architectures: aggregated (or, monolithic) architecture and split (i.e., dis-aggregated) architecture. In the split architecture, the RAN node is split into gNB-DU and gNB-CU and connecting them through the F1 interface. gNB-CU is further split into the control plane part gNB-CU-CP and the user plane part gNB-CU-CP, where gNB-DU connects gNB-CU-CP through the control plane interface F1-C and gNB-CU-UP through F1-U interface.

The motivation for the split architecture is twofold: (i) allow non-Real Time (nRT) functions like RRC to be hosted in cloud for economies of scale and real-time (RT) functions in DU, and (ii) allow control plane and user plane functionalities scale independently as per the operator needs. Going by real network deployments, many operators have moved CU functionality into the cloud. Going forward, we would like to continue the dis-aggregated architecture in 6G, along with monolithic architecture, to continue to enable cloud deployment of nRT functions.

Proposal 1: 6GR architecture should continue support for dis-aggregated architecture

## 2.2 Radio Protocol Architecture

Given the plethora of services like eMBB, FWA, URLCC, IoT, V2V, etc. that 3GPP network architecture is striving to offer, the idea ``one-size-fits-all” may not hold anymore. Further, an operator may not deploy network to offer all services possible with 3GPP standardized network architecture. Thus, an operator should be able to tailor network as per the particular services intended to offer. This ``tailoring” should be applied to network as well as UE design in terms of protocol/procedures design right from the initial access to the network.

Proposal 2: Network architecture/protocols/procedures should be tailored to specific services the operator wishes to offer in the deployment.

## 2.3 Interoperability between Network Functions

An important guiding principle in the design of RAN network architecture has been design of network interfaces that offer interoperability. Prior to 5G era, the only interface internal to RAN was X2 which connected two eNBs. With 5G, two more interfaces F1 and E1 were defined, in addition to, Xn interface to connect gNB-DU, gNB-CU-CP, and gNB-CU-UP.

We believe 6G should focus to make these interfaces truly interoperable. For example, certain PDU types defined on the F1-U interface are short of offering true interoperability

Proposal 3: The RAN interfaces should be made truly interoperable.

## 2,4 Spectrum Sharing

5G considered spectrum sharing between 5G and 4G not as an afterthought but as an integral part of the specification process right from the first release Rel-15. Subsequent improvements were introduced in later releases Rel-18. Though considerable progress was made, nevertheless, we feel more needs to be done in this space.

In the scenario in which the operator procures 6G network equipment from a vendor different from the vendor who provided 5G network gear, it is still a challenge to achieve coordination between 5G and 6G if one were to continue the same design that was used for spectrum sharing between 4G and 5G.

Though X2-C provides messages for control signalling for spectrum sharing for 4G-5G spectrum sharing, we believe lot more needs to be done in this space. For example, 5G and 6G DUs need to communicate on 1ms time scale about resource coordination (both data and PDCCH resources) and this requires definition of new messages. We believe 6G should focus on RAN architecture that provides a comprehensive network interface for better resource coordination between 5G and 6G DUs.

Proposal 4: The study item should consider a new interface for spectrum sharing between 5G and 6G.

# 3 Conclusion

Proposal 1: 6GR standardization should support all the three deployment options D1, D2, and D3

Proposal 2: 6GR architecture should continue support for dis-aggregated architecture

Proposal 3: Network architecture/protocols/procedures should be tailored to specific services the operator wishes to offer in the deployment.

Proposal 4: The RAN interfaces should be made truly interoperable.

Proposal 5: The study item should consider a new interface for spectrum sharing between 5G and 6G.