3GPP TSG-RAN3 Meeting #129bis	R3-256703

Prague, Czech, Oct 13 - 17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1530 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on OD-SIB1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TS 38.423 specification, the class 1 OD-SIB1 CONFIGURATION PROVISION REQUEST message is defined as following, an optional Cell A IE may be included, with the requested OD-SIB1 configuration identified by an NES cell ID. Accordingly, the receiving node responds with either an OD-SIB1 CONFIGURATION PROVISION RESPONSE or an OD-SIB1 CONFIGURATION PROVISION FAILURE message, where an optional Cell A IE may also be included. When the above REQUEST message does not contain a Cell A ID, the behavior of both the sender and the receiver is clear: the sender requests the transmission of the OD-SIB1 configuration, and the receiving gNB, based on its OAM configured Cell A(s),if supported, broadcasts received configuration across all such Cell A(s). However, if only one Cell A is included in the REQUEST message, the behavior of both the sender and the receiver becomes ambiguous in some cases. For example, assume that an NES gNB intends to have the Assisting gNB broadcast OD-SIB1 configuration 1 in Cell A1 and Cell A2. The Assisting gNB is also aware of its own Cell A(s), i.e., Cell A1 and Cell A2 for this OD-SIB1 configuration identified by an NES cell ID, and the Cell A(s) in both the NES gNB and the Assisting gNB are coordinated and configured by OAM. There are some ambiguities in following cases: - If the REQUEST message contains Cell A1, should the Assisting gNB broadcast only in Cell A1, or, according to the OAM configuration, in both Cell A1 and Cell A2? - If the REQUEST message does not contain any Cell A, can the Assisting gNB respond with Cell A1? Should such a response be interpreted as indicating that the Assisting gNB has only one Cell A, i.e., Cell A1, and that Cell A1 accepts the provision request, or should it be understood that the Assisting gNB has multiple Cell A(s) but only Cell A1 accepts the provision request? We consider it is necessary to include additional description in the OD-SIB1 provision procedure in order to avoid the above ambiguities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a description in the OD-SIB1 provision procedure to clarify that, if the REQUEST message contains Cell A1, the Assisting gNB broadcasts the OD-SIB1 configuration only in Cell A1, and not in all OAM-configured Cell A(s). - Add a description in the OD-SIB1 provision procedure to clarify that, if the REQUEST message does not contain any Cell A, the Assisting gNB broadcasts the OD-SIB1 configuration in all OAM-configured Cell A(s), and shall not include any Cell A in the response message. Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has isolated impact with the previous version of the specification (same release) because it only impacts the OD-SIB1 function in Rel-19. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The behavior of the NES gNB and the Assisting gNB becomes ambiguous during the OD-SIB1 configuration provision procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.17.2, 8.4.17.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

8.4.17	OD-SIB1 Configuration Provision

8.4.17.1	General

The purpose of the OD-SIB1 Configuration Provision procedure is to enable an NG-RAN node1 to provide OD-SIB1 configuration information to an NG-RAN node2 and to request the NG-RAN node2 to transmit the provided OD-SIB1 configuration information.

The procedure is also used to enable the NG-RAN node1 to request the NG-RAN node2 to stop transmission of the OD-SIB1 configuration.

The procedure uses non-UE -associated signaling.

8.4.17.2	Successful Operation

Figure 8.4.17.2-1: OD-SIB1 Configuration Provision procedure, successful operation

The NG-RAN node1 initiates the procedure by sending the OD-SIB1 CONFIGURATION PROVISION REQUEST message to the NG-RAN node2.

If the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message is set to "start", the NG-RAN node2 shall, if supported, transmit the provided OD-SIB1 Configuration in SIBx and reply with the OD-SIB1 CONFIGURATION PROVISION RESPONSE message. The NG-RAN node2 stores the OD-SIB1 Configuration information:

-	If the Cell A ID IE is included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, transmit the OD-SIB1 Configuration only in the cell indicated by the Cell A ID IE and includes the Cell A ID IE indicating the same cell in the OD-SIB1 CONFIGURATION PROVISION RESPONSE message.

-	If the Cell A ID IE is not included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, transmit the OD-SIB1 Configuration in the pre-configured cell(s), and shall not include the Cell A ID IE in the OD-SIB1 CONFIGURATION PROVISION RESPONSE message.

If the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message is set to "start" and the NES Cell ID IE indicates a cell for which the NG-RAN node2 is already transmitting the OD-SIB1 Configuration:

-	If the request is for a cell already transmitting the OD-SIB1 configuration for the cell identified by the NES Cell ID IE, the NG-RAN node2 shall, if supported, stop the ongoing transmission and start transmission of the newly provided OD-SIB1 Configuration in SIBx. The NG-RAN node2 replaces the stored OD-SIB1 Configuration information with the newly received OD-SIB1 Configuration information.

-	If the request is for a new cell indicated by the Cell A ID IE, the NG-RAN node2 shall, if supported, start transmitting the provided OD-SIB1 Configuration in SIBx in the cell indicated by Cell A ID IE.

If the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, is set to "stop" the NG-RAN node2 shall stop broadcasting OD-SIB1Configuration for the cell indicated by NES Cell ID IEin SIBx. The NG-RAN node2 removes the stored OD-SIB1 Configuration.

8.4.17.3	Unsuccessful Operation

Figure 8.4.17.3-1: OD-SIB1 Configuration Provision procedure, unsuccessful operation

If the NG-RAN node2 cannot transmit the provided OD-SIB1 Configuration in SIBx, the NG-RAN node2 shall send the OD-SIB1 CONFIGURATION PROVISION FAILURE message. The message shall contain the Cause IE with an appropriate value.

Only if the Cell A ID IE is included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message and the NG-RAN node2 cannot transmit the provided OD-SIB1 Configuration in SIBx of the cell indicated by the Cell A ID IE, the NG-RAN node2 shall send the OD-SIB1 CONFIGURATION PROVISION FAILURE message and include the Cell A ID IE in the OD-SIB1 CONFIGURATION PROVISION FAILURE message.

8.4.17.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>