3GPP TSG-RAN WG3 Meeting #129bis	R3-256717

# Prague, CZ, 13 – 17 Oct 2025

Agenda item:		10.3.1

Source:	T-Mobile USA, Qualcomm

Title:	AI Native Architecture for 6G

Document for:		Discussion and Approval

# Introduction

The transition from 5G to 6G is expected to be characterized by the native integration of Artificial Intelligence (AI) into network architecture. While in 5G, AI has been primarily supplementary—applied to functions such as coverage optimization or mobility prediction—6G requires an AI-native design where AI/ML capabilities are embedded as fundamental system functions rather than implemented as overlays.

A key gap in current 3GPP specifications is the absence of standardized AI-native interfaces within the RAN, and between RAN and CN. For 6G, such interfaces are essential to enable:

- Intelligent resource allocation

- Distributed and federated learning

- Closed-loop automation

- Network slicing across heterogeneous infrastructures

- Real-time policy and QoS control

This contribution highlights the importance of AI-native support in 6G RAN design and proposes that RAN3 study architectural enhancements to enable AI-native capabilities for 6G.

# Discussion

## Background

The design of 6G RAN is driven by the tight integration of AI and communication systems, manifested as two complementary paradigms:

- AI for RAN: Embedding AI into RAN functions (e.g., AI models, training, inference, signaling, and compute-resource management) to enable automation and optimization.

- RAN for AI: Designing RAN capabilities to serve AI applications and services (e.g., exposing RAN intelligence, orchestrating compute/communication resources, and supporting AI-driven verticals).

6G RAN requires a common architecture supporting joint communication, computing resources, computing-related control, management and orchestration, AI models, training/inference workflows and RAN capability exposure.

To support these paradigms, 6G RAN will require:

- Hierarchical control and scheduling across multiple timescales with interfaces for telemetry, analytics, and multi-dimensional resource control

- Multi-tier compute fabric enabling policy-driven workload placement across UE, RAN, edge, and central

- Edge steering and QoS-aware slicing (aligned with 5QI and service-level objectives to make RAN service aware in real time)

- Strong exposure, observability, and energy-aware operation

- Multi-dimensional resource discovery and placement that considers communication, computation, and data services

Moreover, 6G should distribute AI components (data collection, preprocessing, inference) across domains and protocol layers (PHY, MAC, etc.) in an on-demand and scalable manner. Combining centralized/hierarchical control and scheduling mechanisms with distributed intelligence (i.e., flexible placement of inference workloads across domains) will improve overall network efficiency and responsiveness.

A service-based design is critical for flexibility and interoperability. Service-based RAN intelligence exposure allows decoupling between data producers and consumers, enabling scalable evolution, multi-vendor ecosystems, and discoverable/versioned AI services. The current point-to-point-based RAN may introduce limitations in scalability, interoperability and lifecycle management of AI services. 6G should study architectural trade-offs between service-based architecture (SBA) vs. point-to-point (PTP) architecture in AI-RAN integration.

Observation 1: 6G RAN requires a new AI and communication design paradigm (AI for RAN and RAN for AI) with tight coupling between communication and computing resources

Proposal 1: 6G shall support both AI for RAN and RAN for AI

Observation 2: 6G RAN requires a centralized control mechanism combined with distributed intelligence

Proposal 2: 6G shall adopt centralized control architecture with distributed intelligence to balance scalability and responsiveness

Proposal 3: 6G shall study SBA vs. PTP architecture for various interfaces

# Proposal

Based on the above discussion, the following is proposed for RAN3 study:

Proposal 1: 6G shall support both AI for RAN and RAN for AI

Proposal 2: 6G shall adopt centralized control architecture with distributed intelligence to balance scalability and responsiveness

Proposal 3: 6G shall study SBA vs. PTP architecture for various interfaces

# References

[1] RP-251984, T-Mobile USA, Nokia, Ericsson, SK Telecom, Telstra, “Requirements for 6G RAN for AI”