3GPP TSG-RAN3 Meeting #129-bis	R3-256759

# Prague, Czech Republic, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1332 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on sercurity support in A-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS 33.369, there are two types of security information needs to be included in the inventory request transfer IE from AIOTF to NG-RAN node. - Authorization information according to 5.2.2 in TS 33.369. the authorization info RANDAIOT_n needs to be transferred from AIOTF to RAN, and then to the A-IoT device. The A-IoT device may provided the RESAIOT and RANDAIOT_d to the RAN, which are in the A-IoT NAS container from reader perspective. 1. ADM shall generate RANDAIOT_n. AIOTF shall retrieve RANDAIOT_n from ADM. 2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN. 3. NG-RAN shall include RANDAIOT_n in the paging request message to the AIoT device in addition to other device identification information. <omit the irrelevant parts> 4. Upon receiving the paging request message, if the device determines it needs to respond based on the device identification information, AIoT device shall generate RANDAIOT_d, calculate RESAIOT using KAIoT_root and RANDAIOT_n (see Annex A.2) for network authenticating AIoT Device.. - T-ID handling information according to 5.4.3 in TS 33.369. The T-ID handling formation includes the following: - - ID type, {concealed type, stored type} - - Concealed type base, {stored T-ID, permanent ID} - - Stored type update method {with command, without command} For the protection of AIoT device permanent identifier during the inventory procedure with AIoT device identifier described in clause 5.2.2, the following changes shall apply: - In step 1, AIOTF shall retrieve a T-ID in addition to the RANDAIOT_n from ADM. The ADM shall, based on T-ID type, either fetch the stored T-ID in the AIoT device profile or generate the T-ID as specified in Annex B.1. - In step 2, 3 and 4, the T-ID shall be used as a device identification information. - In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message. - In step 4, the AIoT device, based on the T-ID handling indication in the paging message, generates the T-ID in the same way as the ADM did in step 1. The AIoT device determines it needs to reply to the NG-RAN if the generated T-ID matches with the received T-ID. In case the stored T-ID update shall be done without a command, the AIoT Device generates a new Temp_ID_n+1 as specified in Annex B.1 and stores the new Temp ID_n+1. - In step 5 and 6, a device identification information is not included in the D2R message and Inventory Report message. - In step 7, the AIoT device permanent identifier is used as a device identification information. AIOTF requests the ADM to derive a new T-ID as specified in Annex B.1 and to store it in the AIoT Device profile.. - Based on the above, a new Security Information including RAND info and T-ID handling info needs to be added in inventory request message |
| Summary of change: |  | Add a new A-IoT Security Information IE in Inventory Request Transfer IE. The A-IoT Security Information includes - RAND - A-IoT T-ID Handling Information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The security cannot be supported in inventory service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.20.1.2, 9.3.3.x1, 9.2.3.x2, 9.3.6.1, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.300 CR |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change start>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

<skip the unchanged parts>

[60]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[x]	3GPP TS 33.369:"Security aspects of Ambient IoT service for isolated private networks; Stage 2".

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

8.20.1.2	Successful Operation

Figure 8.20.1.2-1: Inventory Request, successful operation.

The A-IoT CN node triggers the procedure by sending an INVENTORY REQUEST message to the NG-RAN node. If the NG-RAN node accepts the request, the NG-RAN node responds with the INVENTORY RESPONSE message.

The NG-RAN node shall, include the same values of the AIOTF Identifier IE and A-IoT Correlation Identifier IE in the INVENTORY RESPONSE message as received from the INVENTORY REQUEST message.

Upon receiving the Inventory Assistance Information IE contained in the Inventory Request Transfer IE in the INVENTORY REQUEST message, the NG-RAN node may take the information into account as defined in TS 38.300 [8].

Upon receiving the A-IoT Security Information IE contained in the Inventory Request Transfer IE in the INVENTORY REQUEST message, the NG-RAN node shall use it as defined in TS 33.369 [x].

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.3.x1	A-IoT Security Information

This IE includes the security related information for the A-IoT Inventory Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAND | O |  | BIT STRING (SIZE(128)) | This IE indicates the RANDAIOT_n defined in TS 33.369 [x] |
| A-IoT T-ID Handling Information | O |  | 9.3.3.x2 |  |

9.3.3.x2	A-IoT T-ID Handling Information

This IE includes the A-IoT T-ID Handling Information for the A-IoT Inventory Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE T-ID Type Handling | M |  |  |  |
| >Concealed Type Handling |  |  |  |  |
| >>Concealed Type Handling | M |  | ENUMERATED (stored, permanent, ...) | The concealed type basis as specified in TS 33.369 [x]. |
| >Stored Type Handling |  |  |  |  |
| >>Stored Type Handling | M |  | ENUMERATED (with-command, without-command, ...) | The update method of stored type as specified in TS 33.369 [x]. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.6.1	InvenAtory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.69 |  | YES | reject |
| A-IoT Device Identification Requested | M |  | 9.3.3.73 |  | YES | reject |
| Requested Service Area Information | M |  | 9.3.3.71 |  | YES | reject |
| Inventory Assistance Information | M |  | 9.3.3.72 |  | YES | reject |
| Follow on Command Indication | O |  | ENUMERATED (true, ...) | This IE indicates that there is command(s) to be transmitted. | YES | reject |
| A-IoT Security Information | O |  | 9.3.3.x1 |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-SCGActivationTime,

<skip the unchanged parts>

id-Aerial-UE-FlightInformationReporting,

id-Aerial-UE-FlightInformationReportingControl,

id-AIoT-SecurityInformation,

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

AIoT-RequestedAIoTAreaList ::= SEQUENCE (SIZE(1..maxnoofAIoTAreas)) OF AIoTAreaID

AIoT-RequestedServiceAreaInformation ::= SEQUENCE {

requestedReaderList				AIoT-RequestedReaderList					OPTIONAL,

requestedAIoTAreaList			AIoT-RequestedAIoTAreaList					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-RequestedServiceAreaInformation-ExtIEs} } OPTIONAL,

...

}

AIoT-RequestedServiceAreaInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-SecurityInformation ::= SEQUENCE {

rAND								BIT STRING (SIZE(128))				OPTIONAL,

a-IoT-t-ID-HandlingInfo					A-IoT-T-ID-HandlingInfo					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-SecurityInformation-ExtIEs} } OPTIONAL,

...

}

AIoT-SecurityInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-T-ID-HandlingInfo::= CHOICE {

concealedTypeHandling					ENUMERATED {stored, permanent, ...},

storedTypeHandling						ENUMERATED {with-command, without-command, ...},

choice-Extensions		ProtocolIE-SingleContainer { { AIoT-T-ID-HandlingInfo-ExtIEs} }

}

AIoT-T-ID-HandlingInfo-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

InventoryRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIoT-CorrelationIdentifier				CRITICALITY reject	TYPE AIoT-CorrelationIdentifier							PRESENCE	mandatory	}|

{ ID id-AIoT-DeviceIdentificationRequested		CRITICALITY reject	TYPE AIoT-DeviceIdentificationRequested				PRESENCE	mandatory	}|

{ ID id-AIoT-RequestedServiceAreaInformation	CRITICALITY reject	TYPE AIoT-RequestedServiceAreaInformation			PRESENCE mandatory	}|

{ ID id-AIoT-InventoryAssistanceInformation		CRITICALITY reject	TYPE AIoT-InventoryAssistanceInformation				PRESENCE	mandatory	}|

{ ID id-AIoT-FollowonCommandIndication			CRITICALITY reject	TYPE AIoT-FollowonCommandIndication					PRESENCE	optional		}|

{ ID id-AIoT-SecurityInformation				CRITICALITY reject	TYPE AIoT-SecurityInformation							PRESENCE	optional		},

...

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<skip the unchanged parts>

id-SCGActivationTime									ProtocolIE-ID ::= 492

id-Aerial-UE-FlightInformationReporting					ProtocolIE-ID ::= 493

id-Aerial-UE-FlightInformationReportingControl			ProtocolIE-ID ::= 494

id-AIoT-SecurityInformation								ProtocolIE-ID ::= xx1

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change end >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>