3GPP TSG-RAN WG3 Meeting #129bis                                                             R3-256772

Prague, Czech, 13rd – 17th, October 2025

Source:	CATT

Title:	RAN-CN procedures and signaling on ISAC

Agenda Item:	13.3

Document for:	Approval

1 Introduction

In the RAN#108 meeting held in June 2025, a new study item on Integrated Sensing and Communication (ISAC) for 5G was approved. This topic only supports the gNB-based mono-static sensing for UAV target use cases.

The objectives of Integrated Sensing and Communication (ISAC) study are described in SID [1] as follows:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

It is noted that SA has approved the study on functionality enhancement, end to end service and procedures for 5G to support ISAC in SA#107 (March 2025), with the scope aligned with RAN SI (focusing on gNB-based sensing for UAV detection and tracking). The following aspects captured in SID [2] were discussed during the last two SA2 meetings:

- WT-1: Architecture and function enhancements to support sensing. - WT-2: Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer. - WT-3: Discovery and selection of sensing entities based on service requirements triggered by the service request and capability of the sensing entities. - WT-4: Sensing data and the associated information collection and transport mechanisms for result calculation. - WT-5: Mechanisms for providing sensing associated information and result (including sensing result and contextual information) exposure, for one time, periodic or event based reporting. - WT-6: Configuration parameters/policy provisioning for the support of ISAC services.

Per RAN SID [1], RAN3 will study the procedure, signaling between CN and RAN as well as the network architecture for sensing. In this paper, we will discuss the procedures and signaling between RAN and CN. Section 2 gives the details discussion and followed by Section 3 that summarizes the conclusion of this paper.

2 Discussion

2.1 General

According to SA2’s TR 23.700-14 [3], sensing entity (SE) performs the sensing operation, and sensing function (SF) supports the sensing service and locates in core network.

TR 23.700-14 ## 3.1 Terms For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1]. Sensing Service Consumer: The entity that may consume the Sensing Result. The Sensing Service Consumer may also request the Sensing Result. Sensing Entity: The Sensing Entity referring to a Sensing Transmitter and/or to a Sensing Receiver. NOTE 1: In this release, the Sensing Entity is only referring to gNB. Sensing Function: Indicating the logical function which is involved to support Sensing Service. NOTE 2: The Sensing Function cannot be a Sensing Entity. Sensing Service: Capability to collect and provide information about object and/or characteristics of the environment using radio signals. gNB-based Sensing: it indicates gNB acting as Sensing Entity. This doesn’t imply any assumptions for the location of the sensing result generation.

Based on the study aspects captured in SA SID [2] and TR 23.700-14 [3] which have captured the candidate solutions for end-to-end procedure to support sensing, we observe that the SF can at least support the following functions:

- Sensing service authorization

- SEs discovery and selection based on sensing service required

- Configure and control the sensing measurements

- Sensing results exposure.

The gNB should support functionalities of SE. In wireless sensing, the reflections of the sensing signal sent from the sensing transmitter are received by the sensing receiver, and the signal is measured and processed to obtain characteristics of the sensing target. For gNB-based mono-static sensing, the gNB should receive the sensing signal which is transmitted by itself. Sensing result related to target detection and trajectory (e.g. target location) can be obtained after a series of processing on the raw data of the received sensing signal. Our contribution [4] on architecture analyzes the function of sensing results generation, it is proposed that RAN can generate the sensing results.

Based on above RAN-CN function split, the functionalities regarding the interactions between the RAN and SF include SE discovery and selection, configuring and control the sensing measurements and the sensing results reporting and collection.

Observation 1: The functionalities regarding RAN (SE) and SF interaction include:

- SE discovery and selection,

- Configures and controls sensing measurements,

- Sends the sensing report to SF.

According to SA2 end-to-end solutions, the SF may receive the sensing service request from NEF or sensing service consumer. If a sensing service is authorized, the SF should select the appropriate RAN nodes and trigger the sensing measurement at the RAN side. After the sensing measurement, the RAN nodes can send the sensing report to SF and the SF exposes the sensing results to NEF/service consumer.

2.2 RAN capability for SE discovery and selection

The SE selection should be based on the capability of RAN nodes and the requirements of sensing service.

The service requirements would include the target sensing area and sensing QoS. In addition, NES or sensing service consumer may also send requirement of sensing results exposure. Thus, the sensing capability of a SE is assumed to include these aspects.

- Sensing QoS

The sensing QoS means the sensing performance, which is evaluated by the key metrics defined in TS 22.137, such as accuracy, false alarm rate, missed detection probability, sensing resolution, sensing latency and so on. The computing capacity may impact the sensing performance. But the performance mainly depends on the radio resource used for sensing measurement given the computing capacity is enough, thus this capability mainly subjects to the radio resources allocation for sensing on gNB, including, for example, the frequency, bandwidth and time resource, also parameters of sensing reference signal.

SF should select the gNBs based on the QoS requirement of sensing service and the sensing QoS supported by gNB, which means the sensing QoS supported by the selected gNBs should fulfill the QoS requirement of sensing service.

- Sensing report capability

gNBs may support reporting the sensing results proactively or reporting at requested. For proactive reporting, gNB can send the sensing report periodically with time interval between successive reports. The gNB may support one or multiple intervals.

SF can select the gNBs based on the sensing report capability of gNB and the requirement of the sensing result exposure. For example, the SF can select the gNB supporting the reporting interval which adapts to the requirement of the time interval for exposing sensing results to NEF/service consumer.

- Sensing area

It means the largest area that the gNB supports to detect the targets. SF should select the gNB based on the area required by the sensing service and the sensing area supported by the gNB.

Per SID, the gNB mono-static sensing refers to single TRP with co-located sensing transmitter and receiver, the sensing reference signal is transmitted, received and measured by TRP, therefore the gNB’s sensing area is made up with the coverage of its TRPs who participate in the sensing performance. But TRPs should not be visible to SF.SF cannot select the TRP(s) under the gNB, thus SF does not need to obtain the area supported by a specific TRP.

Observation 2: SE selection should be based on the capability of gNB and the requirements of sensing service.

Proposal 1: The capability of gNB includes sensing QoS, sensing report capability and sensing area.

Proposal 2: TRPs under the gNB are not visible to SF and SF should not select TRPs under a gNB.

SF should be aware of the capability of gNB. There are two ways for SF to know that information:

Option 1: SF is pre-configured with the capability of gNBs

Option 1 requires no signaling interaction regarding capability between gNB and SF. A SF may be configured with capability of different gNBs depending on its serving area.

Option 2: gNB supporting sensing can report its capability to 5GC

The sensing capability of gNB can be directly sent to SF if the direct connection is applied between SF and RAN. If a gNB connects with multiple SFs taking care of different areas, it should send the capability to those SFs. Conversely, if the indirect connection is applied, the reported capability should be transferred by AMF, and the signaling may be transparent to the AMF or be treated by the AMF. Our paper [4] discusses the direct and indirect connection between SF and RAN.

Option 1 is simpler. However, Option 2 would be useful for supporting the change of gNB capability if needed. In practical applications, balance of sensing and communication will result in change of gNB capability. For example, when the traffic load is high, the gNB can reduce the radio resource allocated for sensing and reduce the TRPs participating in sensing, then the sensing QoS or sensing area of gNB will be decreased accordingly, which can be followed by an update of gNB capability to SF. RAN3 can discuss the two options.

Proposal 3: RAN3 discuss the ways for SF to know gNB capability including:

- SF is pre-configured with the capability of gNB,

- gNB supporting sensing can report its capability to 5GC.

2.3 Configure and control sensing measurements

If a gNB is selected, the SF should initiate a procedure to trigger sensing measurement on the gNB, this procedure is used for configuring the parameters, as required by the Key Issue#6 in TR 23.700-14. The configured parameters are determined by SF. The configuration signaling can be directly sent to gNB if the direct signal connection is applied between SF and RAN. Otherwise, if the indirect signal connection is applied, the configuration signaling should be transferred by AMF, and the signaling may be transparent to the AMF or be treated by the AMF.

A gNB may be configured by multiple SFs taking care of different areas. As a SF may receive the requests for different sensing service from NEF or service consumer, it may initiate multiple configuration procedures to RAN. That would lead to paralleled on-going sensing procedures between the gNB and the SF. RAN3 adopt that a gNB supports parallel sensing procedures with the same or different SFs.

Proposal 4: A gNB may support paralleled sensing procedures with same or different SFs.

Proposal 5: SF can initiate a signaling procedure to trigger sensing measurement at RAN side, used for configuring parameters.

The parameters configuration should be determined by SF based on the requirement of sensing service which is requested by NEF or service consumer, may include:

- Requirement of sensing QoS

This parameter includes the information on sensing performance required by the sensing service, such as accuracy of position, accuracy of velocity, false alarm rate, missed detection probability, sensing resolution, sensing latency and so on. The gNB can use this information for allocating the sensing radio resources, since the sensing performance depends on the radio resource used for sensing measurement.

- Sensing report configuration

- The configuration indicates how gNB sending the sensing report. gNB may be configured to report proactively with a specific time interval, or to report upon requested. SF can determine the sensing report configuration based on the requirement on sensing result exposure for the service.

- Sensing area

SF can indicate an area where the gNB performs sensing, such that the gNB uses this information for determining the relevant TRP(s). SF cannot indicate gNB explicitly the TRP(s) for performing sensing, since TRPs are not visible to SF.

SF can configure the area to gNB based on the area requirement of the sensing service.

Observation 3: SF determines the configuration parameters for sensing measurement, the parameters may include requirement of sensing QoS, Sensing report configuration, sensing area.

In addition, according to the key Issue #2 in TR 23.700-14, the end-to-end procedure should support revocation of an on-going sensing service, which means the SF can revoke an on-going sensing measurement. So, SF should be able to initiate a procedure to gNB for stopping the on-going sensing measurement.

Proposal 6: SF can initiate a procedure to stop the on-going sensing measurement on gNB.

2.4 Sending sensing report

The gNB can send the sensing report at the request of SF, or it can report proactively.

On reporting at requested, the SF can initiate a procedure to trigger RAN nodes to report the sensing results, for example, at the request of retrieving sensing result from the NEF or service consumer. SF may request the sensing report periodically or only once for a sensing service. SF can send the request for sensing report using the signal procedure for configuring parameters to RAN or using a separate procedure. If a gNB reports proactively, gNB can be configured with a time interval of successive reports, after which the gNB sends the sensing report periodically.

RAN3 can discuss above two ways of sensing reporting. In addition, the size of sensing report may determine whether it is transmitted through the control plane or the user plane. If the size of data is large, the gNB needs to transmit it through the user plane, otherwise it is not necessary.

Proposal 7: RAN3 discuss the two ways for gNB sending the sensing reporting, i.e., reporting upon requested and reporting proactively.

3 Conclusion

In the previous sections we made the following proposals:

General

Observation 1: The functionalities regarding RAN (SE) and SF interaction include:

- SE discovery and selection,

- Configures and controls sensing measurements,

- Sends the sensing report to SF.

RAN capability for SE discovery and selection

Observation 2: SE selection should be based on the capability of gNB and the requirements of sensing service.

Proposal 1: The capability of SE includes sensing QoS, sensing report capability and sensing area.

Proposal 2: TRPs under the gNB are not visible to SF and SF should not select TRPs under a gNB.

Proposal 3: RAN3 discuss the ways for SF to know gNB capability including:

- SF is pre-configured with the capability of gNB,

- gNB supporting sensing can report its capability to 5GC.

Configure and control sensing measurements

Proposal 4: A gNB may support paralleled sensing procedures with same or different SFs.

Proposal 5: SF can initiate a signaling procedure to trigger sensing measurement at RAN side, used for configuring parameters.

Observation 3: SF determines the configuration parameters for sensing measurement, the parameters may include requirement of sensing QoS, Sensing report configuration, sensing area.

Proposal 6: SF can initiate a procedure to stop the on-going sensing measurement on gNB.

Sending sensing report

Proposal 7: RAN3 discuss the two ways for gNB sending the sensing reporting, i.e., reporting upon requested and reporting proactively.

4 Reference

- RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR, AT&T (Moderator), RAN#108.

- SP-250401, New SID on Study on Stage 2 for Integrated Sensing and Communication

- 3GPP TR 23.700-14 V1.0.0 (2025-09)

- R3-256772, Discussion on network architecture for ISAC, CATT, RAN#129bis