3GPP TSG RAN3 meeting #129bis   	         	                                R3-256775

Prague, Czech Republic, 13 – 17 October 2025

Agenda item: 	    11.2.1

Source: 	China Unicom

Title: 	Discussion on MRO enhancements for inter-CU LTM

Document for:	Discussion

1	Introduction

At RAN#108, the new Work Item on Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5 was agreed and the WID was revised at RAN#109 in[1] introducing the following objectives in the core part:

The objective of this work item is to specify data collection enhancement in NR standalone for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] If needed, co-operate with RAN1, SA5.

This contribution provides our views on the scenarios of MRO enhancements which can be supported in Rel-20.

2	Discussion

2.1	MRO for inter-CU LTM

In Rel-19, inter-gNB(e.g.inter-CU) LTM is supported with signalling procedure shown below.

LTM as a new mobility function was introduced in Rel-18 and MRO for intra-CU LTM was supported in Rel-19.

In Rel-19 LTM supports intra-gNB mobility, and inter-gNB mobility.

LTM supports both intra-frequency and inter-frequency mobility, including mobility to inter-frequency cell that is not a current serving cell. LTM is supported only for licensed spectrum. The following scenarios are supported:

-	PCell change in non-CA scenario and non-DC scenario;

-	PCell and SCell(s) change in CA scenario;

-	Dual connectivity scenarios:

-	PCell change together with MCG SCells(s) change and intra-SN PSCell change;

-	PSCell change together with SCG SCell(s) change with or without MN involvement.

In Rel-19, MRO enhancements for intra-CU LTM supports to detect and optimize failure cases and near failure cases, which can be further considered in Rel-20.

Proposal 1: Both failure case and near failure case should be considered for MRO enhancements to inter-CU LTM.

MRO for intra-CU LTM failure, we defined the failure cases of Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell. The agreed LTM failure scenarios are listed below.

For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting): - - Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure. - - Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell. - - Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell. For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting): - - Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell. - - Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure. - - Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell. LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting): - - Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the cell which is different from the source or target one. - - Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure. - - Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

We can takes Rel-19 MRO for LTM scenarios as baseline for further study on inter-CU LTM failure scenarios.

Proposal 2: Rel-19 MRO for LTM scenarios can be considered as baseline for further study on inter-CU LTM failure scenarios.

The existing connection failure definition of LTM failure case can be further considered for inter-CU LTM.

Proposal 3: The existing connection failure definition of LTM failure case can be further considered for inter-CU LTM.

3	Conclusion

Proposal 1: Both failure case and near failure case should be considered for MRO enhancements to inter-CU LTM.

Proposal 2: Rel-19 MRO for LTM scenarios can be considered as baseline for further study on inter-CU LTM failure scenarios.

Proposal 3: The existing connection failure definition of LTM failure case can be further considered for inter-CU LTM.

4   References

- RP-234038: Revised WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5; TSG RAN Meeting #109, Beijing, China, Sep 15-18, 2025