3GPP TSG-RAN WG3 Meeting #129bis	R3-256782

Prague, Czech Republic, Oct. 13th-17th, 2025

Source:				NEC

Agenda item: 	11.2.1 Inter-CU LTM

Title: 	MRO for inter-CU LTM

Document for:				Discussion and decision

- Introduction

For Rel-20 Data collection for SON, the solution or the following objective is to be discussed and decided:

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

In this paper, we would like to provide our initial thinking on the MRO enhancement for inter-CU LTM.

- Discussion

In handover procedure, when the connection failure happens, the node to which the UE recovers or re-establishes connection transmits the failure information including RLF report to the gNB last serving the UE, and the last serving gNB should perform the root cause analysis based on the information received. For inter-CU LTM, one difference is that the initiating gNB which decides to configure LTM for the UE including preparing the candidate cell list may be not the source gNB during a subsequent LTM cell switch, and RAN3 should discussion on whether to forward the LTM failure information to the initiating gNB of the LTM cell switch procedure.

Proposal 1. RAN3 discusses the forwarding of LTM failure information (e.g., RLF report) for subsequent inter-CU LTM cell switch.

In Rel-19 MRO for intra-CU LTM, the target gNB-DU can identify the BFR shortly after a successful LTM cell switch, and send the recovery beam information to the source gNB-DU via the gNB-CU. For inter-CU LTM, the same case exists, and the same mechanism can be used.

Proposal 2. Support MRO enhancement over Xn interface for BFR happened in the UE shortly after a successful inter-CU LTM cell switch.

In Rel-19 MRO for intra-CU LTM, the target gNB-DU can identify that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell, and send the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to the source gNB-DU via the gNB-CU. For inter-CU LTM, the same case exists, and the same mechanism can be used.

Proposal 3. Support MRO enhancement over Xn interface for the case that UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell after inter-CU LTM cell switch failure.

In Rel-19 MRO of intra-CU LTM, the ping-pong issue of LTM cell switch was discussed and solved by sending from CU to DU the UE history information with a ping-pong indicator using CELL SWITCH NOTIFICATION message. For Rel-20 MRO of inter-CU LTM, the ping-pong issue of subsequent inter-CU LTM should also be discussed, because the UHI in handover request message from the source gNB to the candidate gNB is useless for the ping-pong happened during subsequent inter-CU LTM, solution should be considered and specified. RAN3 should discuss on using which message to sends the UHI information or ping-pong information, and when to send it between the gNBs.

Proposal 4. Support MRO enhancement over Xn interface for optimization of ping-pong issue during inter-CU LTM cell switch.

# 3. Conclusion

In this contribution we made the following proposals:

Proposal 1. RAN3 discusses the forwarding of LTM failure information (e.g., RLF report) for subsequent inter-CU LTM cell switch.

Proposal 2. Support MRO enhancement over Xn interface for BFR happened in the UE shortly after a successful inter-CU LTM cell switch.

Proposal 3. Support MRO enhancement over Xn interface for the case that UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell after inter-CU LTM cell switch failure.

Proposal 4. Support MRO enhancement over Xn interface for optimization of ping-pong issue during inter-CU LTM cell switch.