3GPP TSG-RAN WG3 Meeting #129bis	R3-256801

Prague, Czech Republic, Oct. 13th-17th

Source:				NEC

Agenda item: 	10.4. RAN internal functional split and interfaces

Title: 	Discussion on RAN internal functional split and interfaces

Document for:				Discussion and decision

- Introduction

In the study scope of the new SI on 6G Radio [1], the following items have RAN3 involved:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this paper, we would like to provide our initial thinking on the RAN internal functional split and interfaces.

- Discussion

  - General

The split architecture in 5G gNB i.e. gNB-CU/gNB-DU and gNB-CU-CP/gNB-CU-UP, is designed based on the concept of virtualization of logical nodes, which will be able to have flexibility and scalability of deployment possibility.

Figure 1. 5G Overall architecture for separation of gNB-CU-CP and gNB-CU-UP
(Quoted from TS 38.401 [2])

Considering the current trend of base station development leveraging virtualization and cloud-based implementations, such flexibility should be preserved in the design of the 6G RAN architecture.
Furthermore, as cellular networks become more widespread and increasingly critical as infrastructure, enhanced fault tolerance must be taken into account in anticipation of further virtualization and functional consolidation. This will be a key consideration in the architectural discussions for 6G.

  - CU-DU split

During the introduction of 5G, multiple options for the CU-DU split architecture were considered. As a result, it was agreed that the split between the Central Unit (CU) and Distributed Unit (DU) would be placed between the PDCP and RLC layers.

Figure 2: Function Split between central and distributed unit (Quoted from TR 38.801 [3])

In the discussions on the CU-DU split in 6G, the following three options are assumed:

- Option 1: Do not introduce CU-DU split.

- Option 2: Introduce CU-DU split, with reconsideration of the split point.

- Option 3: Introduce CU-DU split, with the split point placed between PDCP and RLC layers, as in 5G.

As previously mentioned, CU-DU split was introduced in 5G, and virtualization-based base station development has already progressed significantly. Extensive studies have been conducted, and operators are considering deployments based on these specifications. Given that no critical issues have been reported, there appears to be no need to discontinue the flexibility provided by the CU-DU split at this stage.

If CU-DU split is to be introduced in 6G, unless RAN2 agrees to a major change in the protocol stack, there seems to be no need to revisit the discussions held in 5G (E-UTRA) based on the current protocol stack.

Based on the above two points, it is recommended that the CU-DU split be maintained as Option 3, with the split point between the PDCP and RLC layers.

Based on the above two points, it is considered appropriate to continue adopting Option 3, in which the CU-DU split is placed between the PDCP and RLC layers.

Proposal 1:	Continue in 6G to adopt the CU-DU split between the PDCP and RLC layers.

When introducing the CU-DU split in 6G, the following options for CU-DU connectivity are considered:

- Option a: A DU is allowed to connect to multiple CUs.

- Option b: A DU is allowed to connect to only one CU.

In 5G, a gNB-CU (specifically gNB-CU-CP) can connect to multiple gNB-DUs, but a gNB-DU is not assumed to connect to multiple gNB-CUs (i.e., Option b). If this configuration is inherited, the DU is not shared among multiple CUs, allowing each CU to have full visibility and control over the resources within the DU. This enables the CU and DU to behave as a tightly coupled unit. However, even in this case, when the DU is shared via RAN sharing, the CU may not always have full visibility into the DU’s resource status.

On the other hand, allowing a DU to connect to multiple CUs, as in Option a, can enhance fault tolerance and provide greater flexibility in network deployment. In the current protocol stack, each cell is associated with a specific DU, meaning that the CU’s control over cells is physically constrained by the DU it connects to. Relaxing this constraint and enabling more flexible connectivity would be a significant advantage, especially in virtualization-based base station development.

Proposal 2: To study the possibility that a DU is connected to multiple CUs.

  - CP-UP split

At RAN#109, the following item was endorsed:

The 6G RAN architecture shall allow for control plane and user plane separation.

In accordance with this endorsement, and to maintain the flexibility introduced in 5G, the separation of CU-CP and CU-UP nodes should be continued.

Proposal 3:	the separation of CU-CP and CU-UP nodes should be continued.

When introducing CP-UP split, the following options for CU-CP/CU-UP connectivity are considered:

- Option a: A CU-UP is allowed to connect to multiple CU-CPs.

- Option b: A CU-UP is allowed to connect to only one CU-CP.

In 5G, a gNB-CU-CP can connect to multiple gNB-CU-UPs, but a gNB-CU-UP is not assumed to connect to multiple gNB-CU-CPs (i.e., Option b). Unlike the CU-DU split, the CU-UP can connect to multiple DUs and acts as a pure computational resource without being constrained by specific cells. Therefore, it is preferable to maintain a simple configuration without introducing unnecessary complexity.

Proposal 4:	A CU-UP should be allowed to connect to only one CU-CP.

# 3. Conclusion

In this contribution we made the following proposals:

Proposal 1:	Continue in 6G to adopt the CU-DU split between the PDCP and RLC layers.

Proposal 2:	To study the possibility that a DU is connected to multiple CUs.

Proposal 3:	the separation of CU-CP and CU-UP nodes should be continued.

Proposal 4:	A CU-UP should be allowed to connect to only one CU-CP.

# 4. Reference

[1] RP-252912, New SI: Study on 6G Radio

[2] TS 38.401, v19.0.0 , NG-RAN; Architecture description

[3] TR 38.801, v14.0.0, Study on new radio access technology: Radio access architecture and interfaces