3GPP TSG-RAN WG3 Meeting #129bis	             R3-256852

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	10.3.1

Source:	CMCC

Title:	Discussion on general principles and requirements for RAN-CN interface

Document for:	Discussion

# Introduction

A new study item [1] was agreed at RAN#108 that aims to develop a one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study in TR38.914 [2].

One of the objectives of the study is:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

This contribution discusses the general principles and requirements for interface between 6G RAN and 6G CN, establishing the foundation for subsequent interface design decisions.

# Discussion

In 5G, the NG interface as a logical interface between the gNB and the CN is supported to exchange of signalling information between the NG-RAN and 5GC. Following are the general aspects for NG Interface, as specified in TS 38.410[3]:

## 4.2 NG interface general principles The general principles for the specification of the NG interface are as follows: - the NG interface is open; - the NG interface supports the exchange of signalling information between the NG-RAN and 5GC; - from a logical standpoint, the NG is a point-to-point interface between an NG-RAN node and a 5GC node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the NG-RAN and 5GC; - the NG interface supports control plane and user plane separation; - the NG interface separates Radio Network Layer and Transport Network Layer; - the NG interface is future proof to fulfil different new requirements and support of new services and new functions; - the NG interface is decoupled with the possible NG-RAN deployment variants; - the NG Application Protocol supports modular procedures design and uses a syntax allowing optimized encoding /decoding efficiency.

As a point-to-point interface, the NG interface is capable of supporting legacy communication functions as well as the majority of service requirements introduced in 5G. The introduction of NG interface enables a clear separation of control-plane and user-plane functions through NG-C and NG-U respectively. This separation provides enhanced flexibility for network deployment, allowing independent scaling and optimized placement of control and user-plane functions. Since the NG interface has been well designed in 5G, the 6G RAN architecture shall follow the point-to-point interface design and support NG like interface between 6G RAN and 6G CN. Meanwhile, some of NG interface principles in 5G can be further inherited in the evolution for NG like interface in 6G systems.

Some of NG interface principles in 5G can be further inherited in the evolution for NG like interface in 6G systems.

The 6G RAN architecture shall follow the point-to-point interface design and support NG like interface between 6G RAN and 6G CN.

Followed the above general principles for NG interface in 5G, it is assumed that the following principle can be considered as baseline for NG like interface:

-	the NG like interface is open;

-	the NG like interface supports the exchange of signalling information between the NG-RAN and 5GC;

-	from a logical standpoint, the NG like is a point-to-point interface between an NG-RAN node and a 5GC node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the NG-RAN and 5GC;

-	the NG like interface supports control plane and user plane separation;

-	the NG like interface separates Radio Network Layer and Transport Network Layer;

The following principle can be considered as baseline for NG like interface:

- the NG like interface is open;

- the NG like interface supports the exchange of signalling information between the NG-RAN and 5GC;

- from a logical standpoint, the NG like is a point-to-point interface between an NG-RAN node and a 5GC node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the NG-RAN and 5GC;

- the NG like interface supports control plane and user plane separation;

- the NG like interface separates Radio Network Layer and Transport Network Layer;

With the advent of 6G, future 6G services such as AI and ISAC introduce fundamentally different requirements. As discussed in R3-256853, the requirements from new 6G services cannot be efficiently addressed by legacy fixed point-to-point interface. In 5G, the adoption of a service-based architecture for the core network represents a fundamental shift, enabling independent deployment, flexible scaling, and evolutionary development of network functions according to service requirements. Based on SBA, differentiated network slices can be flexibly designed to generate dedicated networks for various industries as needed. Looking ahead to 6G, both RAN and CN are expected to evolve to act as either a producer or a consumer of services, where CN network functions and RAN functions can communicate directly without the intermediate network function. To this end, a new interface is required to support the emerging 6G services efficiently. The introduction of service-based interface can fulfil different new requirements and support of new services and new functions in 6G.

The requirements from new 6G services cannot be efficiently addressed by legacy fixed point-to-point interface.

The introduction of service-based interface can fulfil different new requirements and support of new services and new functions in 6G.

Additionally, the RAN architecture should support service exposure to enable the RAN to expose service information to other core network functions, or OAM, or third-party applications through a service-based interface by standardized API framework. At the same time, this service exposure mechanism also allows retrieval of the service data from these entities. To realize this capability, two options may be considered:

- Exposing the RAN service information via the NEF;

- Exposing the service information directly without NEF involvement;

In our understanding, both two options should be further studied in the future meeting in RAN3.

The RAN architecture should support service exposure to enable the RAN to expose service information to other core network functions, or OAM, or third-party applications through a service-based interface by standardized API framework.

# Conclusion

This contribution has discussed the general principles and requirements for interface between 6G RAN and 6G CN. Since the NG interface has been well designed in 5G, some of its interface principles can be further retained in the evolution towards 6G systems. In addition, the architectural principles proven in 5G core network SBA provide valuable guidance for new service-based interface design in 6G. The following observations and proposals are provided:

- Some of NG interface principles in 5G can be further inherited in the evolution for NG like interface in 6G systems.

- The requirements from new 6G services cannot be efficiently addressed by legacy fixed point-to-point interface.

- The 6G RAN architecture shall follow the point-to-point interface design and support NG like interface between 6G RAN and 6G CN.

The following principle can be considered as baseline for NG like interface:

- the NG like interface is open;

- the NG like interface supports the exchange of signalling information between the NG-RAN and 5GC;

- from a logical standpoint, the NG like is a point-to-point interface between an NG-RAN node and a 5GC node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the NG-RAN and 5GC;

- the NG like interface supports control plane and user plane separation;

- the NG like interface separates Radio Network Layer and Transport Network Layer;

The introduction of service-based interface can fulfil different new requirements and support of new services and new functions in 6G.

The RAN architecture should support service exposure to enable the RAN to expose service information to other core network functions, or OAM, or third-party applications through a service-based interface by standardized API framework.