3GPP TSG-RAN WG3 #129bis                                                 R3-256868

Prague, CZ, 13th-17th Oct, 2025

Agenda Item:	8.1

Source:	CATT

Title:	Discussion on UE level performance

Document for:	Discussion

- Introduction

During the previous meetings, RAN3 has discussed how to properly derive UE-level UE performance which is transferred between gNBs to support AI/ML for NG-RAN function, and a clear conclusion is still missing. RAN3 has send a LS to SA5 to request clarification on the technical interpretation of the "Average UE Throughput DL" and "Average UE Throughput UL" performance metrics as defined in TS 28.558.

In this contribution, we further discuss the topic in light of the reply LS from SA5 in [1].

- Discussion

For Average packet delay measurements:

For Average packet delay measurements, the reply from SA is as follows [1]:

SA5 would like to thank RAN3 for the reply LS on per-UE UE performance metrics. SA5 has been asked to check the feasibility of the principles agreed by RAN3, to specify a solution that reflects such principles (if feasible) and to provide feedback to RAN3. RAN3: For Average packet delay measurements: RAN3 would like to ask SA5 whether the measurements are aggregated per-DRB per-UE to reflect corresponding per-UE metrics. If such per UE aggregation is not supported and if such UE aggregation is feasible, RAN3 would like to ask SA5 to specify these measurements. SA5’s answer: SA5 confirms that RAN3’s assumption is correct and the measurements defined in TS 28.558 clause 6.3.1.1 are aggregated per-DRB per-UE to reflect corresponding per-UE metrics.

During previous RAN3 meeting, in RAN3 side, we ever had a common understanding that UE level performance is derived as the average value of per-DRB level measurements of the UE performance metrics. So, clearly, there are two options on how to derive UE level performance.

One opinion is to define them as simple average, i.e. sum of each result per-DRB average delay divided by number of radio bearers:

DelayperUE=eachDRB_ForTheUEDelayperDRBeachDRB_ForTheUE1

Whereas another opinion is to define them as average weighted by number of packets, i.e. sum of delay of each packet divided by the total number of packets, regardless of the DRB used to deliver:

DelayperUE=eachDRB_ForTheUEDelayperDRBeachPacketInTheDRB1eachDRB_ForTheUEeachPacketInTheDRB1=eachPacketForTheUEDelayperPacketeachPacketForTheUE1

Observation 1: For Average packet delay measurements, there are two major opinions for the definition of UE level performance: simple average or average weighted by number of packets.

In the above LS, SA5 confirmed that measurements per DRB and UE are aggregated to reflect corresponding UE-level metrics (TS 28.558, 6.3.1.1). However, the aggregation method for delay metrics across multiple DRBs per UE remains unspecified.

Observation 2: SA5 confirmed that measurements per DRB and UE are aggregated to reflect corresponding UE-level metrics, but the aggregation method for delay metrics across multiple DRBs per UE remains unspecified.

The previous reply of SA5 did not resolve the above problem, despite multiple LS exchanges. And RAN3 has already discussed the above issue in multiple meetings and presented two candidate aggregation methods i.e. Simple Average and Weighted Average. We suggest that RAN3 need to discuss first which working group should define the method for packet delay aggregation i.e. the definition of Average packet delay measurements. For example, RAN3 autonomously defines the aggregation method or SA5 standardizes the method which request SA5 further clarify which need continue sending LS to SA5. And we slightly prefer RAN3 to define the aggregation methods of packet delay.

Proposal 1: RAN3 need to discuss which working group should define the method for packet delay aggregation i.e. the definition of Average packet delay measurements:

- Option 1: RAN3 autonomously defines the aggregation method, which we slightly prefer.

- Option 2：SA5 standardizes the method which needs to send LS to request further clarify.

For Average packet loss measurements:

For Average packet loss measurements, the reply from SA is as follows:

For Average packet loss measurements: RAN3 would like to further ask whether it is feasible to specify the above Packet Loss Rate metrics with a per-DRB per-UE granularity in the UL and DL at PDCP layer, which may also be aggregated to reflect a per-UE Packet Loss Rate metric. SA5’s answer: SA5 confirms that it is feasible to specify the packet loss/drop measurements with a per-DRB per-UE granularity. SA5 has specified the corresponding packet loss and drop rate measurements in TS 28.558.

Based on the above LS, SA5 has specified UL PDCP SDU Loss Rate in TS 28.558 for both split and non-split architecture. R18 AI/ML only introduced Average Packet Loss DL IE in Data Collection Request/Response message for UE performance feedback. UL Packet Loss Rate metric is also critical for 5G performance evaluation due to bandwidth constraints, interference patterns, and mobility effects, necessitating its introduction alongside DL metrics for comprehensive UE assessment. And for the Slicing use case, per slice per UE level UL Packet Loss has already introduced. Therefore, in our opinion, it is reasonable to introduce Average Packet Loss UL IE per UE for both split and non-split architecture in R18/R19 UE performance feedback mechanisms.

Corresponding R18 /R19 CR for TS 38.423 is provided in [4][5].And corresponding CR for TS 37.483 is provided in [6]. RAN3 is requested to discuss and agree the CRs on support of UE performance feedback.

Proposal 2: It is proposed to introduce Average Packet Loss UL IE per UE for both split and non-split architecture in R18/R19 UE performance feedback mechanisms.

Proposal 3: Discuss and agree the corresponding R18 /R19 CRs for TS 38.423 for UE performance feedback.

Proposal 4: Discuss and agree the CR for TS 37.483 for UE performance feedback.

And in LS reply, SA5 has specified DL PDCP SDU Drop Rate rather than DL PDCP SDU Loss Rate in TS 28.558. In TS 28.552, packet drop rate and packet loss rate are defined as follows:

Packet drop rate: share of packets that were not sent to the target due to high traffic load or traffic management and should be seen as a part of the packet loss rate. Packet loss rate: share of packets that could not be received by the target, including packets dropped, packets lost in transmission and packets received in wrong format.

The definition of packet drop rate and packet loss rate are obviously different. The packet drop rate corresponds only to packets that were never sent to the target and therefore were dropped. But the packet loss rate includes not only the packet drop rate but also Air interface packet loss and F1-U transmission packet loss.

Observation 3: The definition of packet drop rate and packet loss rate are obviously different.

In the previous meeting, we introduced a new IE, Slice Based UE Performance IE. In the current specifications, the "Average Slice Packet Loss DL" is defined as the DL PDCP SDU Drop Rate (only accounting for packets not transmitted), while the UL counterpart, "UL PDCP SDU Loss Rate," includes all loss scenarios (e.g., transmission failures, timeouts) [3]. The DL PDCP SDU Loss Rate covers losses across: Air interface (e.g., HARQ failures), gNB-CU processing errors、F1-U transmission issues. In general, the scope of DL PDCP SDU Loss Rate is end-to-end measurement. And the scope of DL PDCP SDU Drop Rate is local measurement limited to CU-UP node, excluding packets transmitted on F1/ Uu interfaces.

Observation 4: The scope of DL PDCP SDU Loss Rate is end-to-end measurement, including Air interface (e.g., HARQ failures), gNB-CU processing errors and F1-U transmission issues. The scope of DL PDCP SDU Drop Rate is local measurement limited to CU-UP node, excluding packets transmitted on F1/ Uu interfaces.

To address the above issues, we propose two solutions. In clause 5.3 of TS 38.323, the SDU discard mechanism is defined as follows:

## 5.3 SDU discard When the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall: - if pdu-SetDiscard is configured: - discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs; NOTE 1: PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set. - else: - discard the PDCP SDU along with the corresponding PDCP Data PDU.

The Discard Timer Mechanism is that unacknowledged SDUs are discarded after timeout to avoid infinite waiting, triggering retransmissions. The DL PDCP SDU Loss Rate calculated via existing discardTimer mechanism, provides a feasible metric, covering air interface, gNB-CU, and F1-U losses. Based on the above description of SDU discard, number of SDUs discarded due to discardTimer expiration within the observation window (T1,T2) should be counted within the extened window (T1+ discardTimer, T2+ discardTimer), as shown in Figure1.

Figure1. SDUs discarded due to discardTimer expiration

Therefore, Solution#1: the DL PDCP SDU Loss Rate within the observation window (T1, T2) could be calculated as follows:

DL PDCP SDU Loss Rat=Discarded SDUs due to discardTimer expirationTotal DL SDUs

Where:

Discarded SDUs: The number of SDUs discarded due to discardTimer expiration within the time window (T1+ discardTimer, T2+ discardTimer)

Total DL SDUs: Total number of DL SDUs receiver from upper layers within the observation window (T1, T2), including both successful delivered and lost packets.

Whereas another solution is to use DL Packet Loss Rate of RLC layer which has already defined in TS 28.558 and report to PDCP layer via DDDS. In our opinion, when air interface packet loss (e.g., HARQ failures) is the primary concern, and PDCP layer discards (e.g., buffer overflow) have minimal impact on service, RLC layer loss rate can directly replace PDCP layer loss rate. In conclusion, there are two solutions on how to calculate DL Packet Loss Rate:

- Solution#1: Request SA5 define DL PDCP Loss Rate, e.g., calculate the DL PDCP SDU Loss Rate by dividing the number of SDUs discarded due to discardTimer expiration by the total number of DL SDUs receiver from upper layers.

- Solution# 2: RAN3 Use DL Packet Loss Rate of RLC layer via DDDS.

We also suggest that RAN3 need to discuss first which working group RAN3 or SA5 should define the DL PDCP SDU Loss Rate. And if SA5 is required to define the DL PDCP SDU Loss Rate, RAN3 needs to send a reply LS to SA5 and suggests to adopt the calculation method of Solution#1. The draft LS is provided in appendix 5.

Proposal 5: There are two candidate solutions on how to calculate DL Packet Loss Rate:

- Solution#1: Request SA5 define DL PDCP Loss Rate, e.g., calculate the DL PDCP SDU Loss Rate by dividing the number of SDUs discarded due to discardTimer expiration by the total number of DL SDUs receiver from upper layers.

- Solution# 2: Using DL Packet Loss Rate of RLC layer via DDDS..

Proposal 6: If SA5 is required to define the DL PDCP SDU Loss Rate, RAN3 needs to send a reply LS to SA5 and suggests to adopt the calculation method of Solution#1. The draft LS is provided in appendix 5.

- Conclusions

Observation 1: For Average packet delay measurements, there are two major opinions for the definition of UE level performance: simple average or average weighted by number of packets.

Observation 2: SA5 confirmed that measurements per DRB and UE are aggregated to reflect corresponding UE-level metrics, but the aggregation method for delay metrics across multiple DRBs per UE remains unspecified.

Proposal 1: RAN3 need to discuss first which working group should define the method for packet delay aggregation i.e. the definition of Average packet delay measurements:

- RAN3 autonomously defines the aggregation method, which we slightly prefer.

- SA5 standardizes the method which needs to send LS to request further clarify.

Proposal 2: It is proposed to introduce Average Packet Loss UL IE per UE for both split and non-split architecture in R18/R19 UE performance feedback mechanisms.

Proposal 3: Discuss and agree the corresponding R18 /R19 CRs for TS 38.423 for UE performance feedback.

Proposal 4: Discuss and agree the CR for TS 37.483 for UE performance feedback.

Observation 3: The definition of packet drop rate and packet loss rate are obviously different.

Observation 4: The scope of DL PDCP SDU Loss Rate is end-to-end measurement, including Air interface (e.g., HARQ failures), gNB-CU processing errors and F1-U transmission issues. The scope of DL PDCP SDU Drop Rate is local measurement limited to CU-UP node, excluding packets transmitted on F1/ Uu interfaces.

Proposal 5: There are two candidate solutions on how to calculate Packet Loss DL:

- Solution#1: Request SA5 define DL PDCP Loss Rate, e.g., calculate the DL PDCP SDU Loss Rate by dividing the number of SDUs discarded due to discardTimer expiration by the total number of DL SDUs receiver from upper layers.

- Solution# 2: Using DL Packet Loss Rate of RLC layer via DDDS..

Proposal 6: If SA5 is required to define the DL PDCP SDU Loss Rate, RAN3 needs to send a reply LS to SA5 and suggests to adopt the calculation method of Solution#1. The draft LS is provided in appendix.

- Reference

- S5-253854 Reply LS on per-UE UE performance metrics

- RAN3_129_agenda_20250828_1900.docx

- R3-255983 (CR to TS 38.423) Support of enhancements on AIML for NG-RAN

- R3-256869 CR to TS 38.423 R18

- R3-256870 CR to TS 38.423 R19

- R3-256867 CR to TS 37.483

- Draft Reply LS to SA5 (S5-253854)

Title:	[DRAFT] Reply LS on per-UE UE performance metrics

Response to:	LS (S5-253854) on < Reply LS on per-UE UE performance metrics >

Release:	Rel-19

Work Item:	NR_AIML_NGRAN-Core

Source:	CATT [to become RAN3]

To:	SA5

Cc:

Contact Person:	Jiaying Sun

sunjiaying@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks SA5 for the reply LS on per-UE UE performance metrics. RAN3 needs to clarify the following issues:

The definition of packet drop rate and packet loss rate are obviously different. The packet drop rate corresponds only to packets that were never sent to the target and therefore were dropped. But the packet loss rate includes not only the packet drop rate but also Air interface packet loss and F1-U transmission packet loss.

And RAN3 has discussed the candidate solution which calculates the DL PDCP SDU Loss Rate by dividing the number of SDUs discarded due to discardTimer expiration by the total number of DL SDUs receiver from upper layers. SA5 could refer to the above solution.

Therefore, RAN3 would like to request SA5 to introduce a Packet Loss DL measurement measuring DL PDCP SDU Loss Rate at PDCP layer per S-NSSAI per UE.

2. Actions:

To SA5 group:

ACTION: 	RAN3 would like to ask SA5 the following:

•	To introduce, if feasible, a Packet Loss DL measurement measuring DL PDCP SDU Loss Rate at PDCP layer per UE and per S-NSSAI per UE and provide feedback to RAN3, if needed.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN WG3 Meeting #130			17 - 21 	November 2025						Dallas, US

TSG-RAN WG3 Meeting #131			9 – 13 	February 2026					Stor-Göteborg, SE