3GPP TSG-RAN WG3 Meeting #129-bis	R3-256966

Prague, Czech Republic, 13th ~17th Oct, 2025

Title:	(TP for BLCR for 38.413) Inter-CU LTM

Agenda Item:	11.2.1

Source:	Huawei

Document for:	other

# 1	Introduction

The work on MRO for inter-CU LTM is captured in the WID [RP-251869] as follows:

The objective of this work item is to specify data collection enhancement in NR standalone for SON/MDT purpose. The specific objectives of this work item are:

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3]

# 2	Discussion

Regarding the scope for inter-CU LTM, a reasonable assumption is that the scope is limited to enhancing the framework to support similar failure cases that were agreed for Rel-19 for the intra-CU case. In case any extension to this scope is proposed, this would need to be carefully evaluated before being agreed into the work item scope.

The scope of Rel-20 for inter-CU LTM is to extend existing failure cases for intra-CU LTM from Rel-19.

The mechanisms for intra-CU LTM can be divided into the following sub-sections:

- Enhanced RLF reporting, forwarded to the source DU

- Failure reporting without RLF report

- Recover/re-establishment information sent from target DU to source DU

- Information of last visited LTM cells sent from CU to the serving DU

In the following sections we analyse the potential impacts on all these mechanisms when the inter-CU scenario is to be supported.

## 2.1	Enhanced RLF reporting, forwarded to the source DU

In Rel19, it was agreed that the CU that received the RLF report could perform a failure analysis and send the RLF report to the source DU together with a failure indication. Note that the receiving node may in some be the last serving DU or the DU that was serving the UE before a successful handover. In the latter case, the CU would also include the C-RNTI that was used in the source cell, since this C-RNTI is not included in the RLF report.

In general, we assume that the legacy mechanism for forwarding the RLF report shall be re-used as much as possible for the inter -CU LTM case. There are two scenarios where forwarding may be needed:

- Forwarding of RLF report from the CU managing the re-establishment cell (or the cell where the network retrieves the RLF report) to CU managing the last serving cell. This could most likely be solved by re-using the Failure Indication message.

- Forwarding of the RLF report from the CU managing the re-establishment cell to the CU managing the source cell in case of failure shortly after successful HO. This could most likely be solved by re-using the Handover Report message.

As a first step, we propose to agree on the following:

RAN3 will define a solution for forwarding the RLF report from the CU managing the re-establishment cell (or the cell where the network retrieves the RLF report) to CU managing the last serving cell.

RAN3 will define a solution for forwarding the RLF report from the CU managing the re-establishment cell (or the cell where the network retrieves the RLF report) to the CU managing the source cell.

In a next step, we think that it would be wise consider whether it is possible to re-use existing messages, and further analyse whether additional information is required to support the intended functionality.

## 2.2	Failure reporting without RLF report

Although there is an agreement to solve this scenario, there is no final agreement on what the detailed solution would look like. Therefore, we would like to propose to put this part on hold for one more meeting until the Rel-19 solution is firmly in place.

Wait one meeting with the analysis for the failure reporting without RLF report.

## 2.3	Recover/re-establishment information sent from target DU to source DU

In the Rel-19 functionality, the information is forwarded from target DU to source DU via the CU. With the inter-LTM scenario, one impact is that we need additional forwarding between the two CUs. The assumption in the Rel-19 solution is that the UE is identified by the F1 APID of the receiver node. The same principle should apply also for the forwarding over Xn.

Forwarding over Xn seems quite straightforward. The only potential issue to be discussed is how to manage the UE identity. Over F1AP, the F1AP IDs are used. Therefore, it seems most straightforward to use XnAP IDs over Xn, more specifically the XnAP ID of the receiving node (the source CU). It would be possible to re-use either Failure Indication or Access and mobility indication to transfer the information. We think this could be further discussed in future meetings. Therefore, we propose:

RAN3 will define a solution for forwarding the MRO information from the target DU to the source DU over Xn, between the target CU and the source CU.

## 2.4	Information of last visited LTM cells sent from CU to the serving DU

It was agreed that the CN can provide the list of consecutive visited cells with LTM cell switch. In order to support this in the Inter-CU scenario, the source CU need to include information on which of the visited cells the mobility was based on LTM cell switch.

Agree to include an indicator for LTM cell switch in the Legacy IE for Visited Cells.

Agree the TP included in the annex.

# 3	Conclusion

This document proposes:

Proposal 1:	The scope of Rel-20 for inter-CU LTM is to extend existing failure cases for intra-CU LTM from Rel-19.

Proposal 2:	RAN3 will define a solution for forwarding the RLF report from the CU managing the re-establishment cell (or the cell where the network retrieves the RLF report) to CU managing the last serving cell.

Proposal 3:	RAN3 will define a solution for forwarding the RLF report from the CU managing the re-establishment cell (or the cell where the network retrieves the RLF report) to the CU managing the source cell.

Proposal 4:	Wait one meeting with the analysis for the failure reporting without RLF report.

Proposal 5:	RAN3 will define a solution for forwarding the MRO information from the target DU to the source DU over Xn, between the target CU and the source CU.

Proposal 6:	Agree to include an indicator for LTM cell switch in the Legacy IE for Visited Cells.

Proposal 7:	Agree the TP included in the annex.

# Annex1	TP to 38.413

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Cell Type | M |  | 9.3.1.98 |  | - |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | - |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. | - |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.3.1.235 | The PSCell related information. | - |  |
| LTM cell switch indicator | O |  | ENUMERATED (true,...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 8. |

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

[snip]

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-UserLocationInformationN3IWF-without-PortNumber,

id-ExtendedBackupAMFName,

id-LTMCellSwitchIndicator,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

[snip]

LastVisitedNGRANCellInformation::= SEQUENCE {

globalCellID								NGRAN-CGI,

cellType									CellType,

timeUEStayedInCell							TimeUEStayedInCell,

timeUEStayedInCellEnhancedGranularity		TimeUEStayedInCellEnhancedGranularity 			OPTIONAL,

hOCauseValue								Cause		 									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedNGRANCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedNGRANCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LastVisitedPSCellList	CRITICALITY ignore	EXTENSION LastVisitedPSCellList	PRESENCE optional }|

{ ID id-LTMCellSwitchIndicator	CRITICALITY ignore	EXTENSION LTMCellSwitchIndicator	PRESENCE optional },

...

}

LTMCellSwitchIndicator ::= ENUMERATED {

true,

...

}

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

[snip]

id-ExtendedBackupAMFName    							ProtocolIE-ID ::= 442

id-ExtendedOldAMF			    						ProtocolIE-ID ::= 443

id-LTMCellSwitchIndicator	    						ProtocolIE-ID ::= 999 – to be assigned by MCC