3GPP TSG-RAN WG3 Meeting #129bis	R3-257027

Prague, Czech, 13th – 17th October 2025

Agenda Item:	14.2

Source: 				LG Electronics Inc.

Title:			Discussion on support of Topology 2

Document for:		Discussion and Decision

1.	Introduction

New Rel-20 WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2 was agreed at RAN #108 [1]. This WID was updated in RAN #109 to reflect the SA decision on Topology 2 in [2]. One of the objectives in this WID is to specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader as follows:

… - Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: RAN1 - The objective includes specification of necessary power control and collision handling procedures between intermediate UE and BS. - No physical layer impact for signaling A-IoT resources to the intermediate UE. - No additional specification change on R2D and D2R for Device 1 in Rel-20 RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling. RAN3 - Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility …

In this contribution, we will discuss the RAN3 impacts on mechanisms to support the Topology 2, and provide our view on it.

2.	Discussion

During Rel-19 study on solutions for Ambient IoT in NR, the following three solutions are considered for conveying the A-IoT specific information in Topology 2 [3]:

- RRC-based solution: A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

- NAS-based solution: There is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

- UP-based solution: There is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

During the study phase, there was no evaluation and no down selection for T2 solutions. However, in SA#108, it was agreed in [4] to only support the RRC-based solution in Rel-20 considering the limited TUs as follows:

SP-250834 [4]: … The work tasks are: WT#1: Study the support of AIoT services in Topology 2, including: - Study and investigate how to support AIoT services under the RRC-based option for UE Reader connectivity. NOTE 3: Rel-19 TR solution and interim conclusion should be used as basis for rel-20 work. Solution submitted to rel-20 need to align with rel-19 TR interim conclusion. No rel-19 AIoT Device impact from this WT is expected. …

Based on the SA decision, RAN plenary also agreed to update the Rel-20 RAN WID [2] on A-IoT. Therefore, in this release, the RRC-based solution is only supported for Topology 2.

Observation 1: In Rel-20, the RRC-based solution is only supported for Topology 2.

As described above, in RRC-based solution, the A-IoT CN node provides the A-IoT specific information to the NG-RAN over NG interface. Then, this information is forwarded to the AIoT Device via Uu RRC. We think that the RRC-based solution in Topology 2 is similar with the overall mechanisms to be defined in Rel-19 for the support of Topology 1. In both solutions, the NG-RAN allocates and co-ordinates the usage of A-IoT radio resources based on the explicit information from the A-IoT CN node via the NGAP signaling. Only the difference between two solutions is that in RRC-based solution for Topology 2, the NG-RAN explicitly provides the resource allocation information for A-IoT radio resources to the UE Reader via the Uu RRC, while in Topology 1, this information is provided to the RAN Reader via the RAN internal signaling. From that point of view, we think that the architecture and signaling for Topology 1 can be also applicable to that for Topology 2.

In Rel-19, the direct connectivity and indirect connectivity for the NG-RAN are supported to communicate with the AIOTF in Topology 1. In addition, the NGAP signaling is enhanced to convey the A-IoT specific information and Inventory/Command Result. In Topology 2, after the registration, the UE can act as the Reader. That is, since the UE Reader has the UE functionality, it is reasonable to support the indirect connectivity with the AIOTF. Note also that SA2 has a clear assumption that the indirect connectivity is only applied for the support of Topology 2 according to TR 23.700-30 [5] as follows:

Clause 4.1 of TR 23.700-03 [5]: … ## 4.1 Architectural Assumptions The following traffic types for AIoT Device are to be studied: - DT: Device-terminated; - DO-DTT: Device-originated - device-terminated triggered; and - DO-A: Device-originated - autonomous. Common architectural assumptions: - Architecture reference models defined in TS 23.369 [3] are used as the baseline architecture for this study. - The AIoT Device is agnostic to topologies, i.e. Topology 1 and Topology 2. Architectural assumptions for support of Topology 2 for Key Issue#1: - No Rel-19 AIoT Device impact is expected. - The following topology as defined in TR 38.848 [10] is assumed: - BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control - Only Indirect Connectivity (i.e. NG-RAN <-> AMF <-> AIOTF) is applied. - The support of Topology 2 in this study is based on the RRC-based option and the interim conclusions of Rel-19 in TR 23.700-13 [7] is the baseline. Architectural assumptions for support of DO-A Capable AIoT Devices: - The AIoT Device is agnostic to Topology 1 architecture types, i.e. Direct Connectivity architecture and Indirect Connectivity architecture. NOTE 1: Coordination with RAN WGs is required to determine the AIoT Device capabilities in relation to system level of functionality (considering e.g. traffic scenarios, connectivity topologies, etc.). NOTE 2: The security aspects for Ambient IoT require coordination with SA WG3. NOTE 3: The charging aspects for Ambient IoT will be studied by SA WG5. …

Therefore, there is no need to consider the direct connectivity with the AIOTF for Topology 2. In addition, the A-IoT specific information needs to be provided to the NG-RAN via the NGAP signaling. However, whether this information for Topology 2 is conveyed via UE-associated signaling or non UE-associated needs to be further discussed in RAN3. As a result, the architecture and signaling for Topology 1 can be reused as a baseline for Topology 2.

Proposal 1: The indirect connectivity with AIOTF and its signaling for Topology 1 can be used as a baseline for Topology 2.

Proposal 1a: The direct connectivity with the AIOTF is not considered for Topology 2.

In TR 38.769 [3], the protocol stack for the RRC-based solution of Topology 2 is captured as follows:

Figure 1: RRC based solution of Topology 2 in TR 38.769 [3]

This protocol stack can be used as a baseline in Rel-20. In this figure, XXAP should be NGAP. In addition, considering the A-IoT NAS between A-IoT Device and the AIOTF, and provision of A-IoT specific information over NG interface as in Rel-19 Topology 1, the protocol stack for Topology 2 can be updated as follows:

Figure 2: RRC based solution of Topology 2

Proposal 2: It is proposed to use the protocol stack in Figure 2 as a baseline for the protocol stack of Topology 2.

In this release, the UE should be in RRC_CONNECTED state to act as the Reader according to the WID. Therefore, during the Registration procedure, the UE should indicate to the AMF whether it has the Reader capability. Since the subscription information stored in UDM indicates whether the UE is authorized to operate as the Reader, the AMF determines when/where the UE Reader can operate based on the subscription information and UE Reader capability. In other words, the AMF authorizes the UE Reader based on the subscription information and provides the Reader authorization indication to the NG-RAN over NGAP in the Registration procedure. The Reader authorization indication set to “authorized” enables the NG-RAN to allocate the A-IoT radio resources to the UE Reader. Therefore, a new indication (e.g., similar to “mobile IAB authorized” or “5G ProSe authorized” indication) needs to be introduced in the NGAP signaling.

Proposal 3: The AMF provides the Reader authorization indication (set to “authorized” or “not authorized”) to the NG-RAN via the NGAP signaling.

When the Reader authorization status changes (either “authorized”, or “not authorized”), the AMF informs the NG-RAN (i.e., the NG-RAN serving the UE Reader) of the new authorization accordingly. If the Reader authorization status changes from “authorized” to “not authorized”, the NG-RAN should stop allocating the A-IoT radio resources to the UE Reader because it cannot operate as the Reader (e.g. due to de-authorization). Once the Reader authorization status changes from “not authorized” to “authorized”, the NG-RAN can (re-)initiate the resource allocation for the UE Reader.

Proposal 4: The updated Reader authorization status should be provided to the NG-RAN.

Proposal 5: If the Reader authorization status changes from “authorized” to “not authorized”, the NG-RAN should stop allocating the A-IoT radio resources to the UE Reader.

Proposal 6: Once the Reader authorization status changes from “not authorized” to “authorized”, the NG-RAN can (re-)initiate the resource allocation for the UE Reader.

According to WID, the case where the UE Reader is served by the split RAN architecture should be also considered. In the CU-DU split case, our view is that the gNB-DU should be in charge of the A-IoT radio resource allocation. This is because the gNB-DU has the gNB-DU is generally responsible for performing the resource allocation based on the request of the gNB-CU. Therefore, upon the reception of the Reader authorization indication, the gNB-CU needs to forward it to the gNB-DU via F1AP signaling.

Proposal 7: In CU-DU split case, the gNB-DU performs the A-IoT radio resource allocation for the UE Reader.

Proposal 8: In CU-DU split case, the Reader authorization indication is also provided to the gNB-DU via F1AP signaling.

In Topology 2, the UE Reader may be moved to other NG-RAN. This mobility scenario is also considered in WID [2] as follows:

RP-252894 [2]: … RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling. RAN3 - Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility …

Therefore, it is required to enhance the XnAP/NGAP signaling for the support of the UE Reader mobility scenario.

Observation 2: During the HO, the Reader authorization indication needs to be provided to the target NG-RAN.

In NG-based HO, the AMF can provide the Reader authorization indication to the target NG-RAN by using the NGAP HANDOVER REQUEST message. In Xn-based HO, the source NG-RAN can provide this information to the target NG-RAN by using the XnAP HANDOVER REQUEST message. If the updated Reader authorization status needs to be provided to the target NG-RAN in Xn-based HO, the AMF can use the NGAP PATH SWITCH REQUEST ACKNOWLEDGE message. Then, when the target NG-RAN has the split RAN architecture, the gNB-CU of the target NG-RAN should provide this authorization information to its gNB-DU as described in Proposal 8.

Proposal 9: The Reader authorization indication is provided to the target NG-RAN by using the following signalings:

- XnAP HANDOVER REQUEST message,

- NGAP HANDOVER REQUEST message,

- NGAP PATH SWITCH REQUEST ACKNOWLEDGE message.

In Rel-19 Topology 1, the AIOTF is aware of the Reader related information (e.g., Reader ID, supported A-IoT Areas, Reader’s location) by means of OAM configuration. This is because the RAN Reader is fixed. However, in Rel-20 Topology 2, the UE Reader mobility is considered as mentioned above. Our understanding is that the A-IoT Area ID and the Reader ID of the UE Reader may need to be updated as it moves. Therefore, it seems to be difficult to preconfigure the Reader related information in AIOTF.

Observation 3: In Rel-20 Topology 2, it is difficult to preconfigure the Reader related information in AIOTF.

To alleviate this problem, the following potential options can be considered as a solution:

- Option 1: The A-IoT Area ID and the Reader ID of the UE Reader can be locally configured in the UE. The UE Reader ID should be globally unique to identify the UE Reader in the network. In this option, the supported A-IoT Areas of the NG-RAN are consisted of the A-IoT Areas for the UE Reader(s) belonging to the NG-RAN. Therefore, the NG-RAN may frequently update its supported A-IoT Areas whenever the list of the UE Reader belonging to the NG-RAN is changed. During the Registration procedure, the UE reports its A-IoT Area ID(s) and the Reader ID to the AMF. Then, the AMF can register to the ADM, NRF, or the AIOTF the information related to the UE Reader(s) it supports.

- Option 2: During the Registration procedure, the NG-RAN determines/allocates the A-IoT Area ID and the Reader ID of the UE Reader, and then provides this information to the UE and the AMF, respectively. The AMF can register to the ADM, NRF, or the AIOTF the information related to the UE Reader(s) it supports. Since the UE Reader ID is allocated by the NG-RAN, the Reader ID collision may be occurred in AMF and/or AIOTF. Therefore, in order to avoid this problem, the AMF and/or AIOTF may also take into account the NG-RAN node ID. In addition, during the HO, the target NG-RAN should reallocate the UE Reader ID to avoid the Reader ID collision.

- Option 3: During the Registration procedure, the AMF determines/allocates the A-IoT Area ID and the Reader ID of the UE Reader, and then provides this information to the UE and the AMF, respectively. Also, it can register to the ADM, NRF, or the AIOTF the information related to the UE Reader(s) it supports. In this option, the Reader ID collision may be occurred in AIOTF as well. In addition, in case where the AMF is changed during the NG-based HO, new AMF should reallocate the UE Reader ID to avoid the Reader ID collision.

- Option 4: The AIOTF can determine/allocate the A-IoT Area ID and the Reader ID of the UE Reader. To this end, the AMF should request the allocation for the A-IoT Area ID and the Reader ID of the UE Reader to the AIOTF during the Registration procedure. In this option, the UE Reader ID is globally unique in AIOTF. Because the service area of the AIOTF is generally wide, the UE Reader can be uniquely identified in wide coverage. However, frequent signaling exchange between AMF and AIOTF may be occurred to manage the Reader related information such as the A-IoT Area ID and UE Reader ID, e.g., due to authorization status change or UE RRC state change.

Based on these options, RAN3 needs to further discuss how to allocate/manage the Reader related information in Topology 2.

Proposal 10: It is proposed to discuss how to allocate/manage the Reader related information in Topology 2 based on the following options:

- Option 1: The A-IoT Area ID and the Reader ID of the UE Reader can be locally configured in the UE.

- Option 2: The A-IoT Area ID and the Reader ID of the UE Reader are allocated by the NG-RAN.

- Option 3: The A-IoT Area ID and the Reader ID of the UE Reader are allocated by the AMF.

- Option 4: The A-IoT Area ID and the Reader ID of the UE Reader are allocated by the AIOTF.

As mentioned earlier, the UE in RRC_CONNECTED state can act as the UE Reader. Therefore, since the AMF already knows the UE location on a serving NG-RAN node granularity and the serving cell ID can be additionally provided to the AMF by using the User Location Information, the AMF can determine the UE Reader’s location on a per cell level granularity. If needed, the AMF is also able to deduce the UE Reader’s location on a finer granularity with the help of the LMF. Therefore, the AIOTF can receive the UE Reader’s location from the AMF directly, or retrieve this information from the UDM (or ADM) when the AMF updates it in the UDM (or ADM). Upon the reception of the UE Reader ID in the INVENTORY REPORT message as in Rel-19 Topology 1, the AIOTF can determine the A-IoT Device location. As a result, the A-IoT Device localization based on Reader ID as specified in Rel-19 can be considered.

Proposal 11: The A-IoT Device localization based on Reader ID can be considered as a baseline for Topology 2.

For the Reader selection, the followings were agreed in Rel-19 Topology 1:

- Upon receiving neither the requested A-IoT Area nor the Requested Reader List in INVENTORY REQUEST message, the NG-RAN selects all the RAN Readers it serves.

- Upon the reception of only the requested A-IoT Area in INVENTORY REQUEST message, the NG-RAN selects the RAN Readers within the indicated area.

- Upon receiving only the Requested Reader List in INVENTORY REQUEST message, the NG-RAN shall take the Requested Reader List into account to perform inventory (within the Requested Reader List).

Our understanding is that these functionalities can be also applicable to Topology 2. As mentioned in Proposals 10 and 11, it is possible for the AIOTF to retrieve the Reader related information (e.g., Reader ID, supported A-IoT Areas, Reader’s location) for the UE Reader from the AMF or UDM (or ADM). Hence, based on this information, the AIOTF can perform the RAN node selection and optionally UE Reader selection as in Rel-19. In Topology 2, however, the AF may indicate a specific target UE ID (e.g., GPSI) to the AIOTF via NEF. In this case, the AIOTF just selects the target UE Reader based on the UE ID from the AF, and then includes the single UE Reader ID within the Requested Reader List. As a result, the NG-RAN just follows the AIOTF decision, and then allocates to this UE Reader the A-IoT radio resources for the inventory/command operation.

Proposal 12: The Reader selection mechanism in Rel-19 Topology 1 can be used as a baseline for Topology 2.

Proposal 13: Upon the reception of a specific UE Reader ID from the AF via the NEF, the AIOTF just provides this UE Reader ID to the NG-RAN.

3.	Conclusion

In this contribution, we focused on the RAN3 impacts on mechanisms to support the Topology 2, and then provided our views on it. The following proposals are kindly suggested to RAN3:

Proposal 1: The indirect connectivity with AIOTF and its signaling for Topology 1 can be used as a baseline for Topology 2.

Proposal 1a: The direct connectivity with the AIOTF is not considered for Topology 2.

Proposal 2: It is proposed to use the protocol stack in Figure 2 as a baseline for the protocol stack of Topology 2.

Proposal 3: The AMF provides the Reader authorization indication (set to “authorized” or “not authorized”) to the NG-RAN via the NGAP signaling.

Proposal 4: The updated Reader authorization status should be provided to the NG-RAN.

Proposal 5: If the Reader authorization status changes from “authorized” to “not authorized”, the NG-RAN should stop allocating the A-IoT radio resources to the UE Reader.

Proposal 6: Once the Reader authorization status changes from “not authorized” to “authorized”, the NG-RAN can (re-)initiate the resource allocation for the UE Reader.

Proposal 7: In CU-DU split case, the gNB-DU performs the A-IoT radio resource allocation for the UE Reader.

Proposal 8: In CU-DU split case, the Reader authorization indication is also provided to the gNB-DU via F1AP signaling.

Proposal 9: The Reader authorization indication is provided to the target NG-RAN by using the following signalings:

- XnAP HANDOVER REQUEST message,

- NGAP HANDOVER REQUEST message,

- NGAP PATH SWITCH REQUEST ACKNOWLEDGE message.

Proposal 10: It is proposed to discuss how to allocate/manage the Reader related information in Topology 2 based on the following options:

- Option 1: The A-IoT Area ID and the Reader ID of the UE Reader can be locally configured in the UE.

- Option 2: The A-IoT Area ID and the Reader ID of the UE Reader are allocated by the NG-RAN.

- Option 3: The A-IoT Area ID and the Reader ID of the UE Reader are allocated by the AMF.

- Option 4: The A-IoT Area ID and the Reader ID of the UE Reader are allocated by the AIOTF.

Proposal 11: The A-IoT Device localization based on Reader ID can be considered as a baseline for Topology 2.

Proposal 12: The Reader selection mechanism in Rel-19 Topology 1 can be used as a baseline for Topology 2.

Proposal 13: Upon the reception of a specific UE Reader ID from the AF via the NEF, the AIOTF just provides this UE Reader ID to the NG-RAN.

4.	Reference

- RP-251881, “WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2”, Huawei, June, 2025.

- RP-252894, “Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2”, Huawei, September, 2025.

- TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, v19.0.0.

- SP-250834, “SID on Architecture support of Ambient power-enabled Internet of Things - Phase 2”, SA2 (Huawei), June, 2025.

- TR 23.700-30, “Study on Architecture support of Ambient power-enabled Internet of Things (AIoT); Phase 2”, v0.1.0.