3GPP TSG-RAN WG3 Meeting #129bis	R3-257060

# Prague, Czech Republic, 13th - 17th October 2025

Agenda Item:	9.2.7

Source:	Ericsson

Title:	On security related open topics for Rel-19 AIoT

Document for:	Discussion and agreement

# 1	Introduction

We summarise the latest status of security related topics for Rel-19 AIoT and consequences for RAN3 work.

The main issue is on how to include security parameters in the Inventory Request message.

# 2	Discussion

Current structure of the Inventory Request message assumes that the device identification is included in the choice structure for group and individual paging:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Device Identification for Paging | M |  |  |  |
| >Single Device |  |  |  |  |
| >>Single Device Identity | M |  | BIT STRING (SIZE(1..1024, ...)) | Includes the AIoT device Identifier defined in TS 23.369 [60]. |
| >Group Devices |  |  |  |  |
| >>Group Device Identity | M |  | BIT STRING (SIZE(1..1024, ...)) | Includes the Filtering Information defined in TS 23.369 [60]. |
| >All Devices |  |  | NULL |  |

Addition of the security parameter RANDAIOT_n in the Inventory Request message, as specified in TS 33.369 [1] would leave us with two  possibility:

-	either include the RANDAIOT_n explicitly somewhere in the Inventory Request message

-	or assume the device identity and the security parameter to be provided in a NAS container, containing both information elements

Both approaches are feasible, but both options are dependent on progress in CT1 (for defining the NAS container) and RAN2 (defining the AIoT MAC protocol).

Independent of whether that new CT1 defined NAS container also includes an AIoT Identification type, the gNB would benefit from the knowledge whether individual or group paging needs to be performed. Respective assistance information on expected number of replies from the devices may not be always available and missing such information might be confused with individual paging. So that information would need to be retained in NGAP.

For paging "all" devices, we understand that, if authentication is applied, still RANDAIOT_n is provided. This needs to be reflected in the message/IE structure as well.

Regarding the size of the assumed NAS container, we could reconsider the type definition, given the fact that NAS containers are octet aligned. We could also consider information exchanged between SA2 and RAN2 on device identification length (see [2]), and information provided by SA3 (see [1]) in for the length of the security parameter, which results in 550 bits or 780 bits, dependent on the device identification length. Assuming additional protocol overhead, the assume maximum length of 1024 bits is deemed sufficient.

# 3	Conclusion and Proposals

We have discussed the current status of security related open topics for Rel-19 AIoT and propose the following:

Proposal 1:	We propose to wait for CT1 and RAN2 to decide how security related information is provided, either, together with device identification within a NAS container, or in a separated way.

# 4	References

[1]	TS 33.369 " Security aspects of Ambient Internet of Things (AIoT) services for isolated private networks ", latest Rel-19 version

[2]	S2-2507793 "LS on AIoT Device Permanent ID Length", LS out from SA2#170 Aug 2025