3GPP TSG-RAN WG3 Meeting # 129bis	R3-257167

Prague, Czech Republic, 13 – 17 October, 2025

Agenda Item:	9.2.12

Source:	ZTE Corporation

Title:	Discussion on  UL Rate Control on XR with the correction and draft LS

Document for:	Discussion and approval

1	Introduction

In previous meeting, RAN3 has made following progress for UL rate control on XR topic:

For UL Rate Control,

•	agree no coordination between MN and SN.

•	agree no need for gNB to inform SMF for the rate control information

•	agree to develop XnAP TP based on R3-255280, to add the missing behaviour text for Indication of Bitrate Adaptation IE.

•	no consensus on gNB-CU provides Recommended UL bit rate info per QoS flow to gNB-DU

In this contribution, we will further discuss the remaining issues based on the UL rate control progress so far and provide CR and draft LS to capture the potential agreements made this meeting.

- 2	Stage3 description in NGAP

- Currently, RAN3 has already introduced the Indication of Bitrate Adaptation IE in NGAP and F1AP for UL Bit Rate Control function. In previous meeting, companies agreed to add the stage 3 description for this indication in XnAP.

- But there is no stage3 description in TS 38.413. Similarly, RAN3 may add proper stage 3 description for NGAP procedures to further clarify how gNB handles the received information. An example is shown below:

- ===========example of stage3 description in NGAP ===========

For each QoS flow to be added or modified, if the Indication of Bitrate Adaptation IE is included in the QoS Flow Level QoS Parameters IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, store the received indication and use it for uplink rate control, as defined in TS 38.300 [8].

- ===========example of stage3 description in NGAP ===========

- Proposal 1: Add proper stage3 description in NGAP for the Indication of Bitrate Adaptation IE in the following NGAP messages:PDU SESSION RESOURCE SETUP REQUEST, PDU SESSION RESOURCE MODIFY REQUEST.

- A CR has been submitted in this meeting and can be treated as baseline.

- 3	Recommended UL bit rate info

- Based on previous agreement, DU can send the UL Bit Rate information to UE via MAC CE. Whether additional information may be informed from CU to DU is going to be further discussed this meeting. More specifically,

- Whether CU can send the recommended UL bit rate info per QoS flow to DU

- How CU gets the recommended bit rate info.

Figure 2.3.3-1

The figure above shows the solution which combined by alternatives provided by companies with the following procedures.

- CU sends the uplink bit rate indication for specific QoS flow to DU with/without recommended uplink bit rate information for this QoS flow.

- CU can send the recommended uplink bit rate information for a QoS flow to DU. Meanwhile,

  - If DU is aware of the uplink bit rate state for this QoS flow, DU may take the received uplink bit rate info into account and determine a recommended uplink bit rate info for this QoS flow. Then DU forwards this info to UE via MAC CE.

  - If DU is not aware of the uplink bit rate state for this QoS flow, and can not provide the recommended uplink bit rate info on its own. DU may directly forward the received uplink rate information for this QoS flow to UE via MAC CE.

By using this solution, RAN3 does not need to worried the DU capability on non-GBR QoS flow awareness. For gNB-DU that have such capacity, the DU itself can handle the uplink rate control function and may or may not take the received assistance info from CU into account. For gNB-DU that do not have such capacity, gNB-CU can provide the required uplink bit rate info for specific QoS flow, which this gNB-DU can directly forward to UE via MAC CE.

- Compared with the DU can fully decide the UL bit rate for a QoS flow, the combined solution above offers a flexible space for product implementation.

- Observation 1: Compared with the DU fully decided the UL bit rate for a QoS flow, CU can provide recommended UL bit rate method offers a flexible space for product implementation in case of different DUs have different capacity.

- Proposal 2: Introduce new IE(e.g. Recommended UL Bit Rate IE) which can be used by CU to provide recommended UL bit rate information for QoS flow to DU for the UL Rate Control function.

- In addition, before we further discussing how CU knows the detail on recommended UL bit rate info, we prefer to review why we need this UL Rate Control function.  Based on our understanding, the UL Rate control function is used to relieve the high gNB workload in some situations. Of course the UL rate adjusting can fully base on gNB implementation. But the drawback of this solution is also obvious. If the Rate control can be decide by gNB, gNB has no idea whether the adjusted data rate fulfills the application requirement. In other word, whether the UE application layer satisfy the new data rate can not be detected before this update.

- In another way, gNB can also ask UEs to give some assistance information on the rate control. But UE has no idea on how heavy the work load at gNB side. It is impossible for multiple UEs sync up with each other and provide assistance info on rate control info which may works at gNB side. Hence, the most straight forward way to solve this issue is to let CN provide some recommended UL rate info which may be confirmed/provided by XR application may let get the balance between gNB, UE, and XR application.

- Proposal 3: Introduce new IE(e.g. Recommended UL Bit Rate IE) which can be used by CN to provide recommended UL bit rate information for QoS flow to gNB for the UL Rate Control function.

- In previous received LS(R3-253014) from SA2, it is confirmed that SA2 has not decided yet on whether the existing or a new mechanism can be re-used for the UL rate control function. It is helpful for RAN3 to provide our understanding and progress to SA2 in our reply LS which may help SA2 further discussion on this topic in next a few meetings. A draft LS to SA2 on RAN3 understanding on CN related enhancement for UL rate control in annex can be treated as baseline.

- Proposal 4: LS to SA2 on RAN3 understanding on UL rate control.

- 3	Conclusion

- Proposal 1: Add proper stage3 description in NGAP for the Indication of Bitrate Adaptation IE in the following NGAP messages:PDU SESSION RESOURCE SETUP REQUEST, PDU SESSION RESOURCE MODIFY REQUEST.

- Observation 1: Compared with the DU fully decided the UL bit rate for a QoS flow, CU can provide recommended UL bit rate method offers a flexible space for product implementation in case of different DUs have different capacity.

- Proposal 2: Introduce new IE(e.g. Recommended UL Bit Rate IE) which can be used by CU to provide recommended UL bit rate information for QoS flow to DU for the UL Rate Control function.

- Proposal 3: Introduce new IE(e.g. Recommended UL Bit Rate IE) which can be used by CN to provide recommended UL bit rate information for QoS flow to gNB for the UL Rate Control function.

- Proposal 4: LS to SA2 on RAN3 understanding on UL rate control.

- Annex B LS to SA2 on UL Bit Rate Control

Title:	[Draft] Reply LS on uplink rate control

Response to:	R3-253014/S2-2504252

Release:	Rel-19

Work Item:	XRM_Ph2, NR_XR_Ph3-Core

Source:	ZTE Corporation [to be RAN3]

To:	SA2

Cc: 	SA4, RAN2

# Contact Person:

# Name:	Liu Yansheng

# Email:		liu.yansheng@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	[TBD]

# 1. Overall Description:

RAN3 thanks SA2 informing the progress on uplink rate control discussion. RAN3 has made the further discussion on uplink rate control and can not make consensus:

- CN may provide recommended UL rate info for QoS flow to gNB, enabkling coordinated rate adaptation among gNB, UE, and XR application.

- CU may send the recommended UL rate information for a specific QoS flow to DU. Then DU sends the recommended uplink bit rate information to UE via MAC CE. The rate info send from DU may either be determined by DU based on its local state and/or received recommended UL rate info from CU, or directly passed from CU.

- Some companies in RAN3 believed the recommended uplink bit rate can help gNB and gNB-CU to minimize the potential negative impact for the rate control. Some companies shown different view on this issue.

# 2. Actions:

# To SA2:

ACTION: RAN3 kindly asks SA2 to take the above RAN3 understanding into account and provide feedback if any.

# 3. Date of Next TSG RAN WG3 Meetings:

RAN3#130		17th Nov - 21st Nov 2025					Dallas, US

RAN3#131		09th Feb - 13th Feb 2026					Stor-Göteborg, SE