3GPP TSG-RAN WG3#130 	R3-258089

Dallas, USA, 17-21 November 2025

Agenda item:	12.2.2 Intra-CU LTM

Source:	Nokia

Title:	Discussion on AI/ML for Intra-CU LTM

Document for:	Discussion and Decision

# 1	Introduction

Rel-20 Study Item in RP-251868 introduces AI/ML with respect to L1/L2 Triggered Mobility (LTM) for both Intra-gNB and Inter-gNB scenarios. The objectives are as follows.

• The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts.

This paper discusses principles relevant for AI/ML applied to Intra-gNB LTM use cases with split gNB architecture.

Additionally, the following items and open items were captured at last RAN3#129bis meeting.

From RAN3#129bis: Prioritize normal intra-CU LTM scenarios. Cover both L1 and L3 Measurement Report based Intra-CU LTM? For CU-DU split architecture, the following two scenarios are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU. - AI/ML Model Training and Model Inference are both located in the gNB-CU. RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM. RAN3 to study AI/ML assisted candidate cell selection for LTM Handover Preparation RAN3 to study TA value and validity predication for Intra-CU LTM in spatial and/or temporal domain Beam prediction should also be considered? For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM. Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation. Further discuss whether to study addition of beam prediction in UE Trajectory prediction. Further discuss predicting TA value in the next meeting. Further discuss prediction of validity time of a measured TA value.

# 2	Intra-CU LTM

At previous RAN3#129bis meeting, agreement was reached that that AI/ML model inference is located at the CU. Thus, in our view the study of use cases following inference at CU approach should be taken with high priority. This is natural, as it also aligns with the agreed framework for AI/ML in NG-RAN in past releases. We see no need to deviate from this principle at advanced stages of 5G-Advanced.

Proposal 1: RAN3 to focus and prioritize AI/ML aided LTM use cases where inference is at CU.

Proposal 2: Study of use cases where inference is located at DU is down prioritized.

With the above as basis, we can also further consider that AI/ML usage can be applied mainly to L3 measurement based LTM.

Proposal 3: RAN3 to focus and prioritize AI/ML aided L3 measurement LTM use cases.

AI/ML may support decision making for LTM, e.g., at preparation phase to determine the appropriate candidate cells to become candidates, at decision making for early synchronization given radio conditions, as well as during the execution phase to select the most suitable target out of the available candidates.

## 2.1 LTM Candidate Cell Selection

The LTM framework relies on L3 measurements to initiate the LTM candidate cell selection. Based on received L3 measurements, the gNB-CU determines a candidate cell set to which LTM configuration will be requested.

The maximum number of LTM candidates to be configured for a UE is limited to 8. Thus, there is a trade-off between configuring likely targets early, and being late to do so. On the first case, many LTM cells could be included, on the expectation to avoid RRC Reconfiguration to the UE and fast mobility. However, this comes at the cost of over reserving resources at all candidates. One could say that a simpler approach is to add only extremely likely targets. However, that may also result in single target and no possibility for subsequent seamless mobility. Hence, would end up requiring RRC Reconfiguration for every additional candidate added. In that sense, there is a benefit in optimizing the number of candidates and maximize probability of subsequent LTM usage.

To this end we see both UE trajectory prediction regarding the cells that a given UE is expected to move through, as well as L3 measurement prediction as useful input for the AI/ML model to optimize the LTM candidate cell selection.

Proposal 4: UE trajectory prediction, and L3 measurement prediction can be inputs for AI/ML aided LTM preparation phase and candidate cell selection.

Further, given that resource reservation for early synchronization happens prior to early synchronization triggering, it is worth also investigating what additional information via AI/ML can help the gNB-CU avoid over reserving early synchronization resources based on past history. That is, optimize and determine which cells should be LTM cells with early synch and which should not, to reduce signaling and avoid admission control failures.

Proposal 5: UE past history can be inputs for AI/ML aided LTM preparation phase to determine to which cells to configure early synchronization resource reservation.

## 2.2 Early Synchronization

Each early synchronization event incurs additional signaling both at the network between nodes and toward the UE. Further, certain gNB-DU implementations may also incur interruption at the source gNB-DU to take an according action if it is pending receiving a TA value that was requested from target cell. Hence, for both cases, the optimization target is to trigger early synch only to the most likely candidates and minimize impact from early synchronization.

Proposal 6: RAN3 to study AI/ML assisted target cell selection for L3 measurement based LTM Early Synchronization.

Proposal 7: UE trajectory prediction, and L3 measurement prediction can be inputs for AI/ML aided LTM early synchronization phase to determine to which cells to carry out early TA acquisition.

Another possibility for early synchronization is to utilize AI/ML to predict a TA value or predict/determine whether a predicted/previously-measured TA value remains valid. In this respect it is important to note that provision of an inaccurate or wrong TA value can have drastic effects as the UE mobility will fail entirely. In other words, there is no fallback mechanism if a UE attempts mobility with a wrong TA and the call will fail. RAN3 has also determined that the validity of an acquired TA value is responsibility of the source cell. Similarly, RAN3 specifications do not force early TA synchronization to be carried out prior to mobility events even if resources have been configured at candidate cells. That is, the source gNB-DU/CU is flexibly already allowed to trigger a Cell Switch Command to a UE with a prior TA value (assumed to have been acquired at some point or determined to be “0” if on co-located deployment case) or with none if the source determines to do so.

In our view this flexibility already allows a given implementation to operate accordingly. Likewise, if TA prediction were to be used, that can remain an implementation choice and no impact is required to the existing specifications. In contrast, we see the task of specifying mechanisms for TA value prediction as risky and counter-productive, since the effect of a wrong value(s) will impact both UE and overall network KPIs. Therefore, any attempt to utilize AI/ML to predict TA values should remain an implementation choice and no specification impact should be pursued.

Observation 1: A wrong TA value provided to a UE will result in a call drop without possibility for fallback mechanism.

Proposal 8: RAN3 does NOT consider TA value prediction as part of Rel-20 AI-ML for NG-RAN Study Item, and if pursed should be based on implementation without any specification impacts.

With regard to TA validity prediction, we see this also as a task that can be fully implementation based as already allowed by Rel-18 LTM specifications, or, alternatively simply supported by signaling the TA Timer value from the candidate cell to the source cell when each TA is acquired. The former approach incurs no specification impact, while the latter approach can be enabled via introduction of a single new IE in DU-CU TA INFORMATION TRANSFER and CU-DU TA INFORMATION TRANSFER messages accordingly. However, we see this option as an over optimization.

Proposal 9: A source DU can determine whether a measured TA remains valid based on early RAN3 agreements without any specification changes.

Proposal 10: RAN3 to evaluate whether to optionally support a Candidate DU to signal the associated TA Timer for a candidate LTM cell within the DU-CU TA INFORMTION TRANSFER and CU-DU TA INFORMATION TRANSFER messages via the CU.

## 2.3 LTM Cell Switch

Even though LTM cell switch is possible on both L3 or L1 measurements in Rel-18 framework, a given candidate cell cannot be simultaneously configured for both. Further, in accordance to earlier RAN3 agreements for inference at CU, we see that AI/ML support focus should be on L3 measurement based LTM.

To this end, we foresee enhancements relevant for LTM. For instance, UE trajectory prediction while useful, it is currently limited as LTM operates on beam level. LTM as such relies on cell switch execution on beam level and indicates the TCI State ID both to the UE and to the candidate (target) cell when LTM mobility is triggered. Therefore, a finer granularity for UE trajectory prediction on beam level would benefit AI/ML for LTM. In a similar context, based on past UE history and prior mobility events a CU could learn cell beam relations to support its inference output.

Proposal 11: RAN3 to study AI/ML assisted L3 measurement based LTM cell switch execution.

Proposal 12: Introduce a finer granularity based on beam-level for UE trajectory prediction.

Proposal 13: UE trajectory/beam level prediction, L3 measurement prediction, beam-cell relations, and UE history are inputs for AI/ML aided LTM cell switch execution.

Proposal 14: Agree the TP provided in Annex capturing the above proposals.

# Annex	TP for TR 38.745

## 4.2	AI/ML assisted Intra-CU LTM

### 4.2.1	Use case description

Editor’s Note: Capture the description of use case

L1/L2 Triggered Mobility (LTM) is specified in TS 38.300 [2].

Intra-CU LTM is specified in TS38.401[3].

AI/ML can be used to optimise Intra-CU LTM procedures, e.g., to enhance Network and UE performance, optimize resource allocation and reduce mobility failures.

Use Case 1: AI/ML assisted intra-gNB LTM candidate cell selection

Description:

LTM relies on L3 measurements to initiate the LTM candidate cell selection. Based on received L3 measurements, the gNB-CU determines a candidate cell set to which LTM configuration will be requested. With assistance from AI/ML, the gNB-CU can optimize the candidate cell selection, to limit the candidate cells to those that are most likely to become target for mobility events (including subsequent LTM) and thus avoiding unnecessary resource reservation.

Expected benefits:

Optimized number of LTM candidate cells and maximize probability of subsequent LTM usage, reducing unnecessary resource reservation, lowering access control rejection and reducing network signaling.

Early determination of LTM candidate cells subject to early synch configuration, reducing network signaling.

Use case 2: AI/ML assisted LTM early synchronization triggering

Description:

In LTM early synchronization phase, each early synchronization event incurs additional signaling both at the network between nodes and toward the UE. Further, certain implementations may also incur interruption at the source cell to take an according action until a requested TA value has been received. Hence, with assistance from AI/ML, the gNB can optimize and trigger early synchronization only to the most likely candidate cells, reducing network signaling and speeding mobility events.

Expected benefits:

Optimized selection of candidate cells subject to early synchronization, reducing network signaling and speeding mobility events.

Use case 3: AI/ML assisted intra-gNB LTM cell switch execution

Description:

The LTM cell switch execution phase can be assisted via AI/ML to aid the target cell selection beyond currently received measurements alone, via supporting information such as UE and beam based trajectory prediction, and cell beam relations learnt via prior mobility events.

Expected benefits:

Optimized selection of target cells, improving handover success rate and reducing risk of radio link failures.

### 4.2.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

For CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2		Input data of AI/ML based Intra-CU LTM

Editor’s Note: To be updated

For LTM candidate selection phase

- UE trajectory prediction

- Measured/predicted L3 measurements

For LTM early synchronization phase

- UE trajectory prediction

- Measured/predicted L3 measurements

For LTM cell switch execution phase

- UE trajectory prediction

- Measured/predicted L3 measurements

- Beam based trajectory prediction

- Cell/beam relations

#### 4.1.2.3		Output data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as output data:

- Candidate cell for LTM HO Preparation

- Candidate cell for triggering LTM early synchronization

- Candidate cell and beam for LTM cell switch execution

<< End of Changes >>