3GPP TSG-RAN WG3 Meeting #130	R3-258209

Dallas, US, 17 – 21 November, 2025

Agenda item:	11.2.2

Source:	Nokia

Title:	(TP for SON BL CR for TS 38.300) MRO Enhancements for Conditional Intra-CU LTM

Document for:	Approval

# 1	Introduction

This is the contribution related to MRO enhancement for Conditional intra-CU LTM in Rel.20.

# 2	Discussion

## 2.1 MRO for Conditional intra-CU LTM

### 2.1.1 Agreements achieved in RAN3 #129bis

The agreements achieved in RAN3 # 129bis [1]:

Use cases:

- At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for Intra-CU C-LTM, i.e.: BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA (FFS whether both L1 and L3 apply to beam related selection optimization), LTM connection failure: too late and too early C-LTM cell switch execution, C-LTM cell switch execution to wrong cell;

- Near failure case: SHR successful HO

- Successful case: ping/pong

Solution options:

- Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO for intra-CU C-LTM, if applicable.

- For intra-CU C-LTM, the definition of connection failure cases for CHO defined in 38.300 are taken as base line.

- For intra-CU C-LTM, the detection mechanism of connection failure cases in 38.300 are taken as base line.

- For the connection failure cases, the last serving gNB-CU performs initial analysis.

## 2.1.2 Discussion

As part of the NR_SON_MDT_Ph4-Core WI in Rel. 19, MRO for Rel. 18 LTM was provided. The focus was related to enhancement of the RLF report with LTM relevant information. Apart of that RAN3 agreed network-based solution for so called case 2 “RACH less LTM cell switch failure due to wrong beam” and case 3 “LTM failure due to outdated TA”.

During the RAN3 # 129bis the following was agreed:

Use cases:

- At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for Intra-CU C-LTM, i.e.: BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA (FFS whether both L1 and L3 apply to beam related selection optimization), LTM connection failure: too late and too early C-LTM cell switch execution, C-LTM cell switch execution to wrong cell;

The intra CU C-LTM may be deployed with an L1 or L3 measurement-based condition. In first case with L1 measurements the execution conditions are provided to UE per candidate cell with a beam-based measurement criterion. In the second case with L3 measurements, the execution conditions are the conditional events known from CHO (e.g. condEventA3) provided to the UE per candidate cell with using a cell-based criterion. Once a target cell is selected based on these fulfilled cell-level conditions based on L3 measurements for a RACH-less attempt, the UE then selects a beam from the beams of the C-LTM target cell with an SS-RSRP value above the threshold cg-LTM-RSRP-ThresholdSSB and also being among the beams associated with the configured CG for that candidate cell. Both the threshold and configured CG are provided by the network by RRC Reconfiguration [3GPP TS 38.331]. Therefore, despite the L3 based C-LTM execution conditions not being beam-specific, the UE's beam selection is constrained by network configurations.

Observation 1: In both L1 and L3 measurement-based intra-CU C-LTM, the UE's beam selection for RACH-less cell switch is constrained by network configurations. Therefore, optimizing beam selection performance for case 2 in these contexts is a network-side responsibility.

Furthermore, compared to Rel. 18 LTM, because of the missing MAC CE Cell Switch Command for C-LTM the source DU may not be aware of the target cell selected by UE and, therefore, the source DU also may not be able to inform the target DU about the initiation of a cell switch command via the Cell Switch Notification message. The target DU may not be able to determine if the cell where a RACH-less C-LTM cell switch attempt failed is the same cell to which the UE successfully re-established or reconnected. Therefore, the NW-based approach from Rel-19 is no longer applicable for case 2, since the selected cell and beam are not known at target DU side, especially if there a several options possible being provided with configured CG.

Observation 2: The network-based solution adopted in Rel. 19 for case 2 is not applicable for RACH-less L1 and L3 measurement-based intra-CU C-LTM.

Proposal 1: RAN3 agrees on UE based solution for case 2 for RACH-less L1 and L3 measurement-based intra-CU C-LTM.

Regarding the case 3 as Early TA acquisition procedure is independent from a L1 or L3 based C-LTM, the Rel. 19 agreed network-based solution may be beneficial also for C-LTM.

Observation 3: For RACH-less L1 and L3 measurement-based intra-CU C-LTM the optimization for the case 3 is a network-side responsibility.

In contrast to Rel-18 LTM, C-LTM's lack of a MAC CE Cell Switch Command means the source DU may not know the UE's target cell selection, hindering its ability to send a Cell Switch Notification to the target DU. This also prevents the target DU from correlating a failed RACH-less C-LTM attempt with the cell where the UE eventually re-established or reconnected. Also, C-LTM may co-exist with MIMO multiple TAs in the case where the target DU has multi-TRP operations. In this case, the TAs are identified by a TAG ID (TA Group ID) and each TA may be configured with a separate TA validity timer. So, when a failure occurs due to invalid/outdated TA value, the network doesn’t know which TA was the culprit. Therefore, the Rel-19 network-based approach for case 3 is unsuitable for L1 and L3 measurement-based C-LTM scenarios, as the target DU remains unaware of the selected cell and TA in case of multi-TRP operations.

Observation 4: The network-based solution adopted in Rel. 19 for case 3 is not applicable for RACH-less L1 and L3 measurement-based intra-CU C-LTM.

Proposal 2: RAN3 agrees on UE based solution for case 3 for RACH-less L1 and L3 measurement-based intra-CU C-LTM.

As part of the NR_SON_MDT_Ph4-Core WI in Rel. the focus was also related to enhancement of the SHR report with LTM relevant information. Apart of that RAN3 agreed network-based solution for so called case 1 “BFR shortly after successful LTM cell switch to the wrong beam”.

Like case 2, the UE's beam selection for RACH-less cell switch in L1 and L3 measurement-based intra-CU C-LTM is constrained by network configurations in this scenario. Furthermore, considering the target DU can identify the cell where UE successfully completed RACH less C-LTM cell switch, i.e. target DU knows the selected cell and beam, and consequently the BFR, the network-based solution proposed in Rel. 19 shall be re-sued also for L1 and L3 measurement-based C-LTM.

Observation 5: The Rel. 19 network-based solution for case 1 is suitable also for RACH Less L1 and L3 measurement-based intra-CU C-LTM.

Proposal 3: RAN3 agrees to re-use the Rel. 19 network-based solution for case 1 for RACH Less L1 and L3 measurement-based intra-CU C-LTM.

With introduction of the Intra-CU C-LTM even the NW triggered UL synchronization when the UE performs early TA acquisition is s till applicable, the TA value is not sent to UE via Cell Switch Command.  Instead, the TA value if provided by network may be used later after the execution conditions for the given candidate target cell are fulfilled. The validity of the TA value is however limited by Time Alignment Validity Timer (TAT).  Too short TAT value may lead to RACH based LTM cell switch due to TAT expiry instead of RACH less despite the TA value received in RAR message is the same as the one previously received from network. On the other hand, inappropriately too long TAT value may lead to situation when the TA value is no longer valid but due to still running TAT the RACH less LTM switch fails, and UE consequently successfully recovers/re-establishes to the same cell with TA value received in RAR message different to one previously received from network. Almost outdated TA value when RACH less C-LTM cell switch is successful, but the time interval since successful execution and TAT expiry is below a configured threshold which may point out to near failure case.

Observation 6: Proper setting of the TAT may be crucial for proper working of the intra-CU C-LTM.

Proposal 4: RAN3 agrees to analyse potential solutions for optimizing the TAT for intra-CU C-LTM, specifically addressing use cases where TAT values are excessively short, long, or almost outdated.

During RAN3 # 129bis the following was agreed:

Solution options:

- For intra-CU C-LTM, the definition of connection failure cases for CHO defined in 38.300 are taken as base line.

For Rel. 18 LTM it was agreed in Rel. 19 to re-use the MRO connection failures defined for standard HO and CHO via adding appropriate text into TS 38.300, chapter 15.5.2.2.2. Similar approach seems to be applicable also for intra-CU C-LTM.

Proposal 5: RAN3 agrees to re-use the MRO connection failures defined for standard HO and CHO also for conditional intra-CU LTM via adding appropriate text into TS 38.300, chapter 15.5.2.2.2 as proposed in Annex 1.

# 3	Conclusions

We have the following observations and proposals:

Observation 1: In both L1 and L3 measurement-based C-LTM, the UE's beam selection for RACH-less cell switch is constrained by network configurations. Therefore, optimizing beam selection performance for case 2 in these contexts is a network-side responsibility.

Observation 2: The network-based solution adopted in Rel. 19 for case 2 is not applicable for RACH-less L1 and L3 measurement-based intra-CU C-LTM.

Proposal 1: RAN3 agrees on UE based solution for case 2 for RACH-less L1 and L3 measurement-based intra-CU C-LTM.

Observation 3: For RACH-less L1 and L3 measurement-based intra-CU C-LTM the optimization for the case 3 is a network-side responsibility.

Observation 4: The network-based solution adopted in Rel. 19 for case 3 is not applicable for RACH-less L1 and L3 measurement-based intra-CU C-LTM.

Proposal 2: RAN3 agrees on UE based solution for case 3 for RACH-less L1 and L3 measurement-based intra-CU C-LTM.

Observation 5: The Rel. 19 network-based solution for case 1 is suitable also for RACH Less L1 and L3 measurement-based intra-CU C-LTM.

Proposal 3: RAN3 agrees to re-use the Rel. 19 network-based solution for case 1 for RACH Less L1 and L3 measurement-based intra-CU C-LTM.

Observation 6: Proper setting of the TAT may be crucial for proper working of the intra-CU C-LTM.

Proposal 4: RAN3 agrees to analyse potential solutions for optimizing the TAT for intra-CU C-LTM, specifically addressing use cases where TAT values are excessively short, long, or almost outdated.

Proposal 5: RAN3 agrees to re-use the MRO connection failures defined for standard HO and CHO also for conditional intra-CU LTM via adding appropriate text into TS 38.300, chapter 15.5.2.2.2 as proposed in Annex 1.

# References

- RAN3 #129bis Meeting report

# Annex 1: Text proposal to TS 38.300 (MRO for Conditional Intra-CU LTM)

First change, ommited text not changed

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

In case of Conditional Intra-CU LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late Conditional Intra-CU LTM cell switch, Too Early Conditional Intra-CU LTM cell switch and Conditional Intra-CU LTM cell switch to Wrong Cell.

In case of CHO with candidate SCG(s), the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO with candidate SCG(s), Too Early CHO with candidate SCG and CHO Execution to Wrong Cell.

Next change, ommited text not changed