3GPP TSG-RAN WG3 Meeting #130	R3-258268

Dallas, USA, 17 – 21 November 2025

Agenda item:	10.2.1

Source:	Nokia

Title:	Discussion on 6G RAN general principles and requirements

WID/SID:	FS_6G_Radio

Document for:	pCR

# 1	Introduction

We here follow up on the discussion initiated at RAN3#129bis relative to overall requirements on the 6G RAN architecture. RAN3’s TR 38.760-3 [1] refers to TSG-RAN’s requirements for the 6G RAN, and we provide further discussion and clarification of some of these requirements in order to help definition of solutions.

# 2	Principles and requirements for the 6G architecture

## 2.1	Overall principles

RAN has provided guidance directly applicable for the 6G RAN architecture in the Study Item Description (SID) [2] as follows:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

[…]

- System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

Based on the above and other details provided in [1], the 6G RAN architecture shall support a single, non-backward compatible radio access technology named 6GR (6G Radio).

Furthermore, as per the justification section of the SID, concerns about implementation complexity, energy consumption, and UE burden are especially relevant given the expected wide adoption of 6G across both consumer and industrial verticals, where cost-efficiency and performance trade-offs are critical. The SID therefore provides guidance to e.g. avoid excessive configurations and minimizing multiple options for the same functionality. Based on this we propose:

Proposal 1: Add the following requirement to the TR: The 6G RAN architecture design shall avoid multiple options.

The following requirement is captured in TR 38.914 [3]:

-	The 6G RAN architecture shall support standalone RAN architecture.

A definition of “standalone RAN architecture” is therefore needed in RAN3’s TR. We propose to define the standalone 6G RAN architecture as an architecture supporting the 6GR RAT only, and which is connected to the 6G CN only.

Proposal 2: Add the following definition to the TR: A standalone 6G RAN supports the 6GR only and is connected to the 6G CN only.

## 2.2	Resiliency

Resiliency is a critical topic for networks in general and it impacts greatly costumer’s experience. The following requirement is captured in TR 38.914 [3]:

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

In 5G, SA2 introduced support for resiliency in the CN based on the adoption of NF sets whereby compute and storage resources were separated. This has proven highly beneficial in the CN from field experience of commercial 5GC resiliency. However, for RAN in 5G, resiliency solutions were only left to network implementation. Yet, given that RAN is a very critical component of the network infrastructure, it is crucial that a more detailed analysis is performed to assess whether more standardized solutions need to be adopted for enabling end-to-end resilient system in 6G, encompassing RAN.

Also, a definition of resilience to be used in RAN3’s study is needed for the TR. In our view, the resiliency in RAN should be aimed at minimizing interruption time of connectivity and increased service continuity in case of RAN node outages and maintenance as well as UE radio link failures as proposed in [4].

Proposal 3: Add the following definition to the TR: Resilience of 6G RAN corresponds to minimization of interruption time of connectivity and increase of service continuity in case of RAN node outages and maintenance as well as UE radio link failures.

## 2.3	Service Awareness in RAN

RAN plenary has captured the following requirement in TR 38.914:

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

The following open point was captured in the chair’s notes at last RAN3 meeting:

Study the impact of enhanced RAN-based service awareness on 6G RAN architecture.

As a first step, RAN3 needs to define the term “service awareness”. To this end, we believe that service awareness refers to methods for RAN obtaining relevant information to handle an application or service.

Proposal 4: Add the following definition to the TR: Service awareness in RAN refers to the methods for RAN to obtain relevant information to handle an application or service.

We provide further discussion on service awareness in our paper submitted to this meeting in [5].

## 2.4	Network optimization, support of AI/ML and data collection for 6G RAN

RAN has provided the following requirement for the 6G RAN framework [3]:

- System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

We believe that while this requirement is intended to be taken into account in the design of all RAN features, it also implies that dedicated features for network optimization are needed. Distributed SON features, under the responsibility of RAN3, have reached a high level of maturity in 5G. The above requirement implies in our view that at least a minimum set of distributed SON features is supported in the initial 6G release, and among them can be mentioned e.g. the ANR feature (Automatic Neighbour Relationships). The set of SON features to be supported may be studied in a later phase of the study item.

A further step for 5G network optimization came with the introduction of AI/ML for NG-RAN in Rel-18 and Rel-19, with support for several use cases related to automated network optimization where predictions enable preventive actions. AI/ML for RAN is handled under separate agenda item 10.5 for 6G study and we provide our detailed view about AI/ML for RAN use cases in [6], but here we describe the general principles and requirements.

In RAN study item description, it was agreed that:

- 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

Therefore, it is important that RAN Architecture is designed agnostic to the support of AI/ML use cases. That means that any specific architecture design for a particular AI/ML use case should be carefully studied and justified, and it should ensure smooth inter-operability with the baseline RAN Architecture.

In our view, data collection and signaling enhancements remain the main enabler for AI/ML in RAN. MDT was introduced in Rel-10 in order to collect radio measurement information for use by SON functions located in the OAM. To this end, the OAM system configures MDT measurements in the RAN, and the corresponding MDT measurement reports are sent by the RAN to the OAM system. From Rel-18, MDT may also be used in order to collect data for the purpose of AI/ML model training, e.g. for the use cases standardized as part of AI/ML for NG-RAN (cf. TR 37.817 (Rel-17), TR 38.743 (Rel-19)). However, as part of the emerging 6G AI/ML use cases it may be necessary to collect data in different entities in the RAN e.g., for AI/ML model training at the 6G Node B. Therefore, RAN3 should study how data should be collected and transported for the support of AI/ML (as well as SON) for 6G RAN. An efficient 6G RAN data collection framework shall exploit commonalities between SON, QoE and AI/ML for RAN use cases and enable availability of data in RAN or in OAM depending on where it is needed.

Proposal 5: Add the following requirement to the TR: The 6G RAN shall support efficient data collection for SON, QoE and AI/ML for RAN use cases. Data shall be available in RAN and/or OAM depending on where it is needed.

Proposal 6: Add the following requirement to the TR: 6G RAN data collection framework shall exploit commonalities between SON, QoE and AI/ML for RAN use cases.

# 3	Conclusion

This paper has discussed key principles for overall RAN architecture and has made the following observations and proposed the following requirements:

Proposal 1: Add the following requirement to the TR: The 6G RAN architecture design shall avoid multiple options.

Proposal 2: Add the following definition to the TR: A standalone 6G RAN supports the 6GR only and is connected to the 6G CN only.

Proposal 3: Add the following definition to the TR: Resilience of 6G RAN corresponds to minimization of interruption time of connectivity and increase of service continuity in case of RAN node outages and maintenance as well as UE radio link failures.

Proposal 4: Add the following definition to the TR: Service awareness in RAN refers to the methods for RAN to obtain relevant information to handle an application or service.

Proposal 5: Add the following requirement to the TR: The 6G RAN shall support efficient data collection for SON, QoE and AI/ML for RAN use cases. Data shall be available in RAN and/or OAM depending on where it is needed.

Proposal 6: Add the following requirement to the TR: 6G RAN data collection framework shall exploit commonalities between SON, QoE and AI/ML for RAN use cases.

A corresponding pCR to the TR is provided in annex of this paper.

# 4	References

- TR 38.760-3, Study on 6G Radio RAN3 aspects, v0.1.0

- RP-252912, 6G RAN study

- TR 38.914, Study on 6G Scenarios and requirements, v0.2.0

- R3-258168, Resilient 6G RAN Architecture, Nokia

- R3-258425, 6G Service Awareness in RAN, Nokia, T-Mobile USA, DT

- R3-258175, AI/ML Use Cases in 6G, Nokia

# Annex – pCR to TR 38.760-3

This pCR is based on [1].

# 4	Definitions of terms, symbols and abbreviations

## 4.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

Standalone 6G RAN: A standalone 6G RAN supports the 6GR only and is connected to the 6G CN only.

Resilience: Resilience of 6G RAN corresponds to minimization of interruption time of connectivity and increase of service continuity in case of RAN node outages and maintenance as well as UE radio link failures.

Service awareness: Service awareness in RAN refers to the methods for RAN to obtain relevant information to handle an application or service.

## 4.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 4.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 5	Objectives and requirements

Editor’s note: The detailed objectives of the study are:

Single technology framework based on a stand-alone architecture to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

## 5.1	General Principles

All requirements of TR 38.914 will serve as the basis for RAN3 architecture design principles.

The following requirements from TR 38.914 include terms defined in clause 4.1 for the purpose of the present study:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

Other requirements:

The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality. [FFS]

The 6G RAN architecture design shall avoid multiple options.

The 6G RAN shall support efficient data collection for SON, QoE and AI/ML for RAN use cases. Data shall be available in RAN and/or OAM depending on where it is needed.

6G RAN data collection framework shall exploit commonalities between SON, QoE and AI/ML for RAN use cases.

## 5.2	Deployment Scenarios

Editor’s note: This section may be used to describe the details/solutions related to deployment scenarios as per 38.914.

The 6G RAN architecture shall strive to support the deployment scenarios defined in TR 38.914.

-  FFS on the implications of this requirement on 6G RAN architecture.

-  FFS whether all deployment scenarios of TR 38.914 can be supported.

## 5.3	Overall RAN Architecture

Editor’s note: The logical architecture can be captured here and should include the entire architecture including possible influence from existing and new services.

### 5.3.1	RAN-CN Functional Split

Editor’s note: The aim of this section is to describe functions split between RAN and CN.

### 5.3.2	RAN Functions

Editor’s note: The aim of this section is to describe the functions supported in RAN.