3GPP TSG-RAN WG3 Meeting #130	R3-258286

Dallas, USA, 17th – 21st November, 2025

Agenda Item:	10.2.2

Source: 	OPPO

Title:  	Discussion on 6G RAN functionalities

Document for:	Discussion and Decision

- Introduction

In Rel-20 6G SID [1], the objectives led by RAN3 are captured as follows.

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

This contribution would like to provide our considerations on 6G RAN functionalities.

- Discussion

In 5G NR, the functional split between RAN node and CN is captured in 38300, as summarized as the following figure.

Figure 4.2-1: Functional Split between NG-RAN and 5GC

When we discuss RAN functionalities in 6G, one of the general principles is that we may take 5G NR functions as the baseline (other than some LTE-related functions), to determine each function of 6G RAN case by case. Note that for some functions cross-WGs coordination is needed.

- For 6G RAN functionalities, one of the general principles to be considered is to take 5G NR functionalities as the baseline for further study, in coordination with other WGs.

In details, as captured in 38300, for 5G NG-RAN, the gNB and ng-eNB host the functions as shown in the following table, as well as our comments for each bullet.

| 5G NG-RAN functions | Comments |
| --- | --- |
| - Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink and sidelink (scheduling); | Essential in 6G RAN (other than sidelink). Thus, suggest to capture in high level, i.e., “Functions for Radio Resource Management”. |
| - IP and Ethernet header compression, uplink data decompression, encryption and integrity protection of data; | Encryption and integrity protection of data is essential in 6G RAN, but the support of ROHC/EHC/UDC is up to RAN2 decision. Thus, suggest to capture “encryption and integrity protection of data”. |
| - Selection of an AMF at UE attachment when no routing to an AMF can be determined from the information provided by the UE; | Up to SA2 design. Thus, suggest to capture in high level, i.e., “Selection of an AMF” |
| - Routing of User Plane data towards UPF(s); | Essential in 6G RAN, and confirmed by RANP requirements in TR 38.914 [2]: - The 6G RAN architecture shall allow for control plane and user plane separation. |
| - Routing of Control Plane information towards AMF; | Essential in 6G RAN, and confirmed by RANP requirements in TR 38.914 [2]: - The 6G RAN architecture shall allow for control plane and user plane separation. |
| - Connection setup and release; | Essential in 6G RAN |
| - Scheduling and transmission of paging messages; | Essential in 6G RAN |
| - Scheduling and transmission of system broadcast information (originated from the AMF or OAM); | Essential in 6G RAN |
| - Measurement and measurement reporting configuration for mobility and scheduling; | Essential in 6G RAN |
| - Transport level packet marking in the uplink; | Essential in 6G RAN |
| - Session Management; | Essential in 6G RAN |
| - Support of Network Slicing; | Confirmed by RANP requirements in TR 38.914 [2]: - The 6G RAN architecture shall allow for the operation of network slicing. |
| - QoS Flow management and mapping to data radio bearers; | Essential in 6G RAN |
| - Support of UEs in RRC_INACTIVE state; | Up to RAN2 decision on RRC states in 6G [1] |
| - Distribution function for NAS messages; | Essential in 6G RAN |
| - Radio access network sharing; | Confirmed by RANP requirements in TR 38.914 [2]: - The 6G RAN architecture shall support sharing of the RAN between multiple operators. |
| - Dual Connectivity; | Up to RANP decision in Sept. 2026 [1] |
| - Tight interworking between NR and E-UTRA; | Suggest to capture “Interworking between 6G RAN and NR” in high level, based on RANP requirements in TR 38.914 [2]: - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. |
| - Maintain security and radio configuration for User Plane CIoT 5GS Optimisation, as defined in TS 23.501 [3] (ng-eNB only). | LTE-specific function |

Therefore, we understand that 6G RAN should support the following functionalities already supported in 5G:

-	Functions for Radio Resource Management;

-	Encryption and integrity protection of data;

-	Selection of an AMF;

-	Routing of User Plane data towards UPF(s);

-	Routing of Control Plane information towards AMF;

-	Connection setup and release;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the AMF or OAM);

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Transport level packet marking in the uplink;

-	Session Management;

-	Support of Network Slicing;

-	QoS Flow management and mapping to data radio bearers;

-	Distribution function for NAS messages;

-	Radio access network sharing;

-	Interworking between 6G RAN and NR.

- 6G RAN shall support the following functionalities already supported in 5G:
-	Functions for Radio Resource Management;
-	Encryption and integrity protection of data;
-	Selection of an AMF;
-	Routing of User Plane data towards UPF(s);
-	Routing of Control Plane information towards AMF;
-	Connection setup and release;
-	Scheduling and transmission of paging messages;
-	Scheduling and transmission of system broadcast information (originated from the AMF or OAM);
-	Measurement and measurement reporting configuration for mobility and scheduling;
-	Transport level packet marking in the uplink;
-	Session Management;
-	Support of Network Slicing;
-	QoS Flow management and mapping to data radio bearers;
-	Distribution function for NAS messages;
-	Radio access network sharing;
-	Interworking between 6G RAN and NR.

We propose to capture the 6G RAN functionalities already supported in 5G in section 5.3 of the 6G RAN3 TR, based on the pCR attached in Annex.

- For 6G RAN functionalities, capture the RAN functionalities already supported in 5G in section 5.3 of the 6G RAN3 TR, based on the pCR in Annex.

Besides, for new services and functionalities (e.g., AI/ML, sensing, etc) to be introduced in 6G, RAN3 may wait until there is any the progress in the leading WGs.

- For new services and functionalities (e.g., AI/ML, sensing, etc) to be introduced in 6G, RAN3 may wait until any progress made in the leading WGs.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- For 6G RAN functionalities, one of the general principles to be considered is to take 5G NR functionalities as the baseline for further study, in coordination with other WGs.

- 6G RAN shall support the following functionalities already supported in 5G:
-	Functions for Radio Resource Management;
-	Encryption and integrity protection of data;
-	Selection of an AMF;
-	Routing of User Plane data towards UPF(s);
-	Routing of Control Plane information towards AMF;
-	Connection setup and release;
-	Scheduling and transmission of paging messages;
-	Scheduling and transmission of system broadcast information (originated from the AMF or OAM);
-	Measurement and measurement reporting configuration for mobility and scheduling;
-	Transport level packet marking in the uplink;
-	Session Management;
-	Support of Network Slicing;
-	QoS Flow management and mapping to data radio bearers;
-	Distribution function for NAS messages;
-	Radio access network sharing;
-	Interworking between 6G RAN and NR.

- For 6G RAN functionalities, capture the RAN functionalities already supported in 5G in section 5.3 of the 6G RAN3 TR, based on the pCR in Annex.

- For new services and functionalities (e.g. AI/ML, sensing, etc) to be introduced in 6G, RAN3 may wait until any progress made in the leading WGs.

4. Reference

[1] RP-252912, New SID: Study on 6G Radio, NTT DOCOMO

[2] 3GPP TR 38.914 V0.2.0, Study on 6G Scenarios and Requirements, 2025-09.

5. Annex: pCR to 6G RAN3 TR

<<<<<<Next Change>>>>>>

5.3	Overall RAN architecture

Editor’s note: The logical architecture can be captured here and should include the entire architecture including possible influence from existing and new services.

5.3.1	RAN-CN Functional Split

Editor’s note: The aim of this section is to describe functions split between RAN and CN

5.3.2	RAN Functions

Editor’s note: The aim of this section is to describe the functions supported in RAN

The 6G RAN shall support the following functionalities:

-	Functions for Radio Resource Management;

-	Encryption and integrity protection of data;

-	Selection of an AMF;

-	Routing of User Plane data towards UPF(s);

-	Routing of Control Plane information towards AMF;

-	Connection setup and release;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the AMF or OAM);

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Transport level packet marking in the uplink;

-	Session Management;

-	Support of Network Slicing;

-	QoS Flow management and mapping to data radio bearers;

-	Distribution function for NAS messages;

-	Radio access network sharing;

-	Interworking between 6G RAN and NR.

<<<<<<Next Change>>>>>>