3GPP TSG-RAN WG3 Meeting #130	R3-258350

Dallas, US,  17th-21st Nov 2025

Title:	RAN-CN interface considerations

Source:				China Telecom

Agenda item:		10.3.2

Document for:		Discussion and Decision

1. Introduction

In RAN #108, a New 6G SID on Study on 6G Radio was agreed [1]. One of the objectives in this SID is as follows:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we provide our analysis on RAN-CN interface and protocol stack.

2. Discussion

2.1	RAN-CN interface options

In the New 6G SID [1], the following information can be found:

#### 4.1.1 Interim Milestone Interim results shall be delivered as per the milestones below, in coordination with the RAN Plenary 6G Study [RP-250810]. TSG#112 (June/2026): RAN1 to provide interim assessment on the following areas: - Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c) - Channel bandwidth (min and max), frame structure, numerology ((2) b, d) - Basic sync signal structure and associated periodicity(ies) ((2) h) For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026. RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split. RAN plenary to make a decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC. RAN plenary will task relevant RAN WGs for any specific technical analysis, as needed. NOTE: It is planned to decide on Release-21 timeline in June/2026.

According to the aforementioned information, following RAN-CN interface options can be considered:

- Option 1: Point-to-point (P2P) interface.

- Option 2: Service-based interface (SBI).

### 2.1.1 SBI or P2P

In 5G networks, the NG interface connects the RAN to the CN. The NG interface splits into NG-C for control plane and NG-U for user plane. It uses P2P protocols like NG-AP (for NG-C) and GTP-U (for NG-U). The NG interface, with its point-to-point design, offers deterministic latency and proven stability, making it well suited for critical signalling and user-plane procedures. These characteristics are crucial for time-sensitive procedures like handovers and connection management. Furthermore, evolving the existing NG interface provides significant benefits in terms of backward compatibility, reduced development cost, and faster deployment, as it builds on a mature and widely deployed foundation. However, its primary weakness is its rigidity. The NG interface is a monolithic, static link that hinders cloud-native principles like software modularity, independent scaling, and the flexible introduction of new services, ultimately preventing full end-to-end service-based automation.

In contrast, a Service-Based Interface (SBI) embodies the architectural philosophy of the 5G Core. Its strengths are flexibility, inherent cloud-nativity, and enhanced automation. SBI allows Network Functions (NFs) to be broken down into more granular, reusable microservices that can be discovered on-demand and scaled independently. This fosters rapid innovation and simplifies the integration of AI-based control loops. The main trade-offs are the potential overhead and complexity compared to a tight P2P link. HTTP/2 with JSON, while flexible, may not be as lean as a binary protocol, raising concerns about ultra-deterministic latency for the most critical RAN procedures.

In 6G, there is discussion on whether to inherit and evolve the 5G NG interface for continuity or define a fully new SBI for innovation. We have the following options:

- Option 1: Inherit and evolve the NG interface

- Option 2: Define a fully new SBI

- Option 3: Inherit and evolve the NG interface for legacy functions and define a fully new SBI for 6G new services

From our side, we need to discuss the following questions:

- Whether the option 1 can support the 6G new services?

- If we adopt option 2, how to design the access and mobility management function? Is it more efficient than option 1? Does it have backward compatibility issues with previous generations?

- If we adopt option 3, do we need to deploy and operate two kinds of interfaces in the network?

Therefore, we have following proposal:

Proposal 1: RAN3 needs take the following options into consideration

- Option 1: Inherit and evolve the NG interface

- Option 2: Define a fully new SBI

- Option 3: Inherit and evolve the NG interface for legacy functions and define a fully new SBI for 6G new services

and discuss the following questions

- Question 1: If we adopt option 1, how to design the access and mobility management function? Is it more efficient than P2P interface? Does it have backward compatibility issues with previous generations?

- Question 2: If we adopt option 2, use full P2P based interface for 6G RAN-CN, is it possible to achieve the direct connection

- Question 3: If we adopt option 3, do we need to deploy and operate two kinds of interfaces in the network?

### 2.1.2 Direct connection or indirect connection

As outlined in the 6G SID [1], 6G RAN architecture shall natively support various new services and functionalities, such as AI/ML and integrated sensing and communication (ISAC). For connection between 6G RAN and new 6G functions within 6G CN, there are two options:

- Direct connection: the 6G RAN could directly connect with the new 6G functions within 6G CN, and a new interface between 6G RAN and 6G CN needs to be defined. Signalling and data can be transferred through this new interface.

- Indirect connection: The 6G RAN could connect with the new 6G functions through AMF or UPF. In this case, the signalling is transferred between 6G RAN and new 6G services through AMF, and the data is transferred between 6G RAN and new 6G services through UPF.

The existing NG interface may pose limitations for the efficient transfer of data generated for new 6G services. For instance, transferring AI/ML training data or high-volume sensing information from the RAN to the CN might suffer from suboptimal latency and overhead due to the interface's rigid data models and procedures. It is efficient to transfer data through direct connection.

Observation 1: It is efficient to transfer data through direct connection between 6G RAN and new functions within 6G CN.

For control signalling transferring, in 5G, it is transmitted between RAN and CN through AMF. The AMF is the termination point for the N1 and N2 reference points. It receives all connection and registration requests from UEs via the gNB. It handles the initial registration and de-registration of UEs, managing their presence and reachability status in the network (e.g., whether they are idle or connected). In addition, the AMF is responsible for tracking the location of the UE. It manages cell and gNB changes, including handovers, ensuring seamless mobility for the user. It is the anchor point for the control plane during these movements. Therefore, it can be observed that it is necessary to keep the AMF to manage the access and mobility of UEs.

Observation 2: it is necessary to keep the AMF to manage the access and mobility of UEs.

Based on observations aforementioned, we prefer that the 6G RAN architecture could support direct connection for transferring data and indirect connection for transferring mobility related control signalling between 6G RAN and new functions within 6G CN. However, considering that transferring data and signalling is related to SA2, we think it is needed to coordinate with SA2 on the following connection options between 6G RAN and new functions within 6G CN:

- For transferring data between 6G RAN and new functions within 6G CN, the 6G RAN architecture could support direct connection.

- For transferring mobility related control signalling between 6G RAN and new functions within 6G CN, the 6G RAN architecture could support indirect connection.

Proposal 2: RAN3 needs to coordinate with SA2 on the following connection options between 6G RAN and new functions within 6G CN:

- For transferring data between 6G RAN and new functions within 6G CN, the 6G RAN architecture could support direct connection.

- For transferring mobility related control signalling between 6G RAN and new functions within 6G CN, the 6G RAN architecture could support indirect connection.

2.2	RAN-CN interface protocol stacks

### 2.2.1 Control plane

According to the TS 38.410 [2], the control plane protocol stack of the NG interface is shown on Figure 1. The transport network layer is built on IP transport. For the reliable transport of signalling messages, SCTP is added on top of IP. The application layer signalling protocol is referred to as NGAP (NG Application Protocol).

Figure 1: NG Interface Control Plane

In 6G, there are two potential solutions for the RAN-CN control plane interface:

- Solution 1: P2P-based interface

- Solution 2: SBI

For P2P based interface, there are two potential options for transfer layer protocol:

- Option 1: NGAP over SCTP

- Option 2: NGAP over QUIC

For SBI, there are two potential options for transfer layer protocol:

- Option 1: HTTP2 over TCP

- Option 2: HTTP3 over QUIC

Since there is no agreement on SBI and P2P based interface, so we have following proposal:

Proposal 3: For transport layer of 6G RAN-CN control plane interface, RAN3 needs need to wait for the agreement on the interface option (i.e. SBI or P2P based interface) and compare the pros and cons for SCTP and QUIC.

### 2.2.2 User plane

According to the TS 38.410 [2], the protocol stack for NG-U is shown in Figure 2. The NG user plane (NG-U) interface is defined between a NG-RAN node and a UPF. The NG-U interface provides non-guaranteed delivery of PDU Session/MBS session user plane PDUs between the NG-RAN node and the UPF.

Figure 2: NG-U protocol structure for PDU/MBS Session

From our understanding, we do not see the requirements for enhancing NG-U protocol structure. When discussing the RAN-CN user plane interface for 6G, the GTP-U tunnel over UDP/IP should be adopted. Therefore, we have following proposal:

Proposal 4: For the transport network layer of 6G RAN-CN user plane interface, GTP-U tunnel over UDP/IP can be used.

### 2.2.3 Data plane

Based on our proposal 2, we prefer to transfer the data generated for 6G new services through direct connection between 6G RAN and new functions within 6G CN (i.e. not through UPF). For P2P based interface, there are two potential options for transfer layer protocol:

- Option 1: GTP-U over UDP

- Option 2: GTP-U over QUIC

Since there is no agreement on data plane interface, so we have following proposal:

Proposal 5: For transport layer of 6G RAN-CN data plane interface, RAN3 needs wait for the agreement on the data plane interface and compare the pros and cons for UDP and QUIC.

3. Conclusion

In this contribution, we discuss RAN-CN interface. We have the following proposals:

Proposal 1: RAN3 needs take the following options into consideration

- Option 1: Inherit and evolve the NG interface

- Option 2: Define a fully new SBI

- Option 3: Inherit and evolve the NG interface for legacy functions and define a fully new SBI for 6G new services

and discuss the following questions

- Question 1: If we adopt option 1, how to design the access and mobility management function? Is it more efficient than P2P interface? Does it have backward compatibility issues with previous generations?

- Question 2: If we adopt option 2, use full P2P based interface for 6G RAN-CN, is it possible to achieve the direct connection

- Question 3: If we adopt option 3, do we need to deploy and operate two kinds of interfaces in the network?

Proposal 2: RAN3 needs to coordinate with SA2 on the following connection options between 6G RAN and new functions within 6G CN:

- For transferring data between 6G RAN and new functions within 6G CN, the 6G RAN architecture could support direct connection.

- For transferring mobility related control signalling between 6G RAN and new functions within 6G CN, the 6G RAN architecture could support indirect connection.

Proposal 3: For transport layer of 6G RAN-CN control plane interface, RAN3 needs need to wait for the agreement on the interface option (i.e. SBI or P2P based interface) and compare the pros and cons for SCTP and QUIC.

Proposal 4: For the transport network layer of 6G RAN-CN user plane interface, GTP-U tunnel over UDP/IP can be used.

Proposal 5: For transport layer of 6G RAN-CN data plane interface, RAN3 needs wait for the agreement on the data plane interface and compare the pros and cons for UDP and QUIC.

4. Reference

- RP-251881 New SID: Study on 6G Radio.

- 3GPP TS 38.410: "NG-RAN; NG general aspect and principles".