3GPP TSG-RAN WG3 Meeting #130	R3-258364

Dallas, US, 17 – 21 November 2025

Agenda item:	10.4

Source: 	Lenovo

Title: 	6G RAN Internal Functional Split and Interfaces

Document for:	Discussion and Approval

1. Introduction

The 6G RAN internal functional split and interfaces were discussed at the RAN3#129bis meeting with the following agreements [1].

Capture pain points and benefits (and potentially derived requirements) of 5G HLS To be continued...

In this contribution, we discuss the remaining issues of 6G RAN internal functional split and interfaces.

2. Discussion

In 5G, RAN architecture introduced CU-DU functional split and CU-CP/CU-UP functional split to improve scalability and flexibility. There was a long discussion on the detailed functional split between the CU and the DU, and 8 options were proposed. Depending on some factors related to radio network deployment scenarios, constraints and intended supported services, the option 2 was finally agreed, where the CU hosts the RRC and PDCP layers while the DU hosts the RLC, MAC and PHY layers. The benefits of the option 2 are [2]:

-	This option will allow traffic aggregation from NR and E-UTRA transmission points to be centralized. Additionally, it can facilitate the management of traffic load between NR and E-UTRA transmission points.

-	This option enables centralization of the PDCP layer, which may be predominantly affected by UP process and may scale with UP traffic load.

-	This option allows a separate U-plane while having a centralised RRC/RRM.

Furthermore, in 5G, the separation of CP and UP for the CU was agreed for the increasing use of very dense deployments where user terminals will be able to connect to multiple transmission points simultaneously. The benefits of the CP-UP separation are [2]:

-	A centralization of CP functions, controlling different transmission points, has the potential to achieve enhanced radio performance.

-	Flexibility to operate and manage complex networks, supporting different network topologies, resources and new service requirements.

-	Alignment with SDN concept that would result in a functional decomposition of the radio access, based on a partial de-coupled architecture, between user and control plane entities and on network abstractions [10].

-	For functions purely handling with CP or UP processes, independent scaling and realization for control and user plane functions operation.

-	Support of multi-vendor interoperability.

With the benefits shown as above, the CU-DU functional split and the separation of CU-CP and CU-UP can be supported in 6G. The 5G RAN internal function split and interfaces can be taken as a baseline for 6G.

Proposal 1: The 5G CU-DU functional split and CU-CP/CU-UP separation can be taken as a baseline for 6G RAN internal functional split.

However, the CU-DU split 5G RAN architecture did not fully realize its intended benefits, and challenges remained in efficiency supporting new services and minimizing operational complexity. As a result, there is no successful large-scale commercial 5G CU-DU split architecture deployment so far. Therefore, some enhancements of the CU-DU functional split can be taken in 6G.

The main drawback of the 5G CU-DU split architecture is that the RRC layer is located in the CU, which brings the following disadvantages:

- 	All procedures (e.g., mobility, UE context setup) cause extra control plane latency between the CU and the DU.

-	Reconfiguration of RRC parameters needs the involvement of the CU, even the RRC parameters related to radio resource are generated by the DU. For example, the DU related L1/L2 configurations needs to be configured via the CU, where the CU generates the RRC signalling sent to the UE via the DU transfer. Therefore, the signalling shall go through the path of DU -> CU -> DU -> UE, which results significant latency compared with direct transmission from the DU to the UE.

-	RRC pooling gains are not realized as the DU still owns most of lower layer functionalities.

-	Joint optimization of the CU and the DU procedures are not easily achievable due to too many inter-dependencies.

-	Unnecessary control information over MAC layer due to slow RRC procedure.

Proposal 2: The main drawbacks of the 5G HLS include:

- 	All procedures (e.g., mobility, UE context setup) cause extra control plane latency between the CU and the DU.

-	Reconfiguration of RRC parameters needs the involvement of the CU, even the RRC parameters related to radio resource are generated by the DU. For example, the DU related L1/L2 configurations needs to be configured via the CU, where the CU generates the RRC signalling sent to the UE via the DU transfer. Therefore, the signalling shall go through the path of DU -> CU -> DU -> UE, which results significant latency compared with direct transmission from the DU to the UE.

-	RRC pooling gains are not realized as the DU still owns most of lower layer functionalities.

-	Joint optimization of the CU and the DU procedures are not easily achievable due to too many inter-dependencies.

-	Unnecessary control information over MAC layer due to slow RRC procedure.

A possible approach is to reconsider the entity hosting the RRC layer, e.g., move the RRC layer from the CU to the DU. With this enhancement, the drawback of the legacy CU-DU split architecture can be avoided.

Proposal 3: RAN3 to discuss the 6G CU-DU functional split architecture where the DU hosts the (partial) RRC layer.

If the legacy CU-DU functional split in 5G is used, i.e., the DU hosts the RLC, MAC and PHY layers while the CU hosts the PDCP, RRC and SDAP layers, another possible enhancement is to enable direct interface between DUs. With this enhancement, the two DUs can coordinate with each other, especially for the L1/L2 coordination, without the involvement of the CU. Therefore, the signalling latency can be reduced.

Proposal 4: RAN3 to discuss the direct inter-DU interface to facilitate L1/L2 coordination.

In 6G, there are some proposals to revisit the L1/L2 functionalities provided by many companies. For example, the PDCP and RLC/MAC functionalities may be merged into one common protocol layer to provide one common SN and reduced processing. This will impact the CU-DU functional split architecture.

Proposal 5: The 6G RAN internal functional split architecture should take into account the L1/L2 functionalities revisited by RAN2.

Compared with the 5G RAN, the 6G RAN should support some new services, e.g., AI, sensing.

For the AI service, it typically requires high computing demand to guarantee the quality, low latency to support the real-time communication, and high security on the exposed AI data. To satisfy these requirements, the 6G RAN can provide a new function to support the AI service. For example, the new function supports the AI computing resource management, online AI model training, AI model inference, AI data collection and exposure, etc.

For the sensing service, it typically requires data privacy, low latency and flexible deployment (e.g., TRP mono-static, TRP-TRP bi-static, TRP-UE bi-static, UE-TRP bi-static, UE mono-static and UE-UE bi-static). To satisfy these requirements, the 6G RAN can provide a new function to support the sensing service. For example, the new function supports the sensing service initiation/termination to obtain sensing data, sensing data handling to obtain the cloud of points or sensing results, etc.

For the 6G RAN internal functional split architecture, the group should discuss how to support these new RAN functions, e.g., the new RAN functions are in the CU, the DU, or a separate node different from the CU and the DU.

Proposal 6: The 6G RAN internal functional split architecture should be designed to enable the new type of services, e.g., data collection, AI, and sensing.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: The 5G CU-DU functional split and CU-CP/CU-UP separation can be taken as a baseline for 6G RAN internal functional split.

Proposal 2: The main drawbacks of the 5G HLS include:

- 	All procedures (e.g., mobility, UE context setup) cause extra control plane latency between the CU and the DU.

-	Reconfiguration of RRC parameters needs the involvement of the CU, even the RRC parameters related to radio resource are generated by the DU. For example, the DU related L1/L2 configurations needs to be configured via the CU, where the CU generates the RRC signalling sent to the UE via the DU transfer. Therefore, the signalling shall go through the path of DU -> CU -> DU -> UE, which results significant latency compared with direct transmission from the DU to the UE.

-	RRC pooling gains are not realized as the DU still owns most of lower layer functionalities.

-	Joint optimization of the CU and the DU procedures are not easily achievable due to too many inter-dependencies.

-	Unnecessary control information over MAC layer due to slow RRC procedure.

Proposal 3: RAN3 to discuss the 6G CU-DU functional split architecture where the DU hosts the (partial) RRC layer.

Proposal 4: RAN3 to discuss the direct inter-DU interface to facilitate L1/L2 coordination.

Proposal 5: The 6G RAN internal functional split architecture should take into account the L1/L2 functionalities revisited by RAN2.

Proposal 6: The 6G RAN internal functional split architecture should be designed to enable the new type of services, e.g., data collection, AI, and sensing.

4. Reference

- RAN3#129bis meeting minutes.

- TR 38.801: Study on new radio access technology: radio access architecture and interfaces.