3GPP TSG-RAN WG3 #130	R3-258406

Dallas, USA, 17th – 21st Nov. 2025

Agenda Item:	9.2.14. Other

Source:	NTT DOCOMO INC.

Title:	Discussion on TNLA handling during Xn handover

Document for:	Discussion

Introduction

In the RAN3#111e meeting, TNL association handling of target gNB in Xn handover was discussed [1]. In the contribution [2], the ambiguity of the TS 23.502 4.2.7.2.2 was pointed out, and it was proposed that the expected target gNB behavior should be clearly defined in XnAP. RAN3 did not agree on the CR [3], but instead SA2 agreed on the CR for TS 23.502 [4], and the behavior was separately defined depending on whether the target 5G-AN had a TNL association to the TNL address received from the source 5G-AN.

However, the case where the target gNB does not have a TNL association to the TNL address received from the source gNB and cannot establish it is still not considered. In real deployments, because of this ambiguity, IOT issue happens, where the gNB responds with Xn Handover Preparation Failure in this case. We think the success of Xn handover should not be determined by whether the TNL association address received from the source gNB is available in the target gNB. Also, since the TNL address available to the gNB depends on the NW deployment of the operator, requiring that the TNL association used by the source gNB is always available in the target gNB in Xn handover greatly hinders operability.

Based on the above, this contribution proposes further clarification regarding the behavior of the target gNB during Xn handover regarding TNLA handling.

In the last meeting, we proposed same correction in [5], however, it was not opened due to lack of time, therefore, we resubmit this paper.

Discussion

TNLA handling during Xn handover/Retrieve UE Context

When executing the Xn Handover Request/Retrieve UE Context procedure, and both the source and target gNB are connected to the same AMF, the following cases can be assumed for the TNLA handlings.

1.	The target gNB has already established a TNL association for the TNL address which the source/old gNB bound to the UE.

2a.	The target gNB has not established a TNL association for the Signalling TNL Association Address at source NG-C side received by the source gNB, and it can.

2b.	The target gNB has not established a TNL association for the Signalling TNL Association Address at source NG-C side received from the source gNB, and it cannot.

In case 1, it is preferred to use the TNL Association Address received from the source gNB to create UE TNLA binding. In case 2a, a TNL association should be established for the received address and used for the binding. In the case of 2b, an available TNL association established with the same AMF should be used, even if the address is different from one received from source gNB.

According to the description below extracted from the current TS 23.502, the operation in case 2a is clear. On the other hand, the operations for case 1 and 2b are unclear. In our understanding, the behavior for case 1 is not explicitly specified, and the operation for case 2b is specified by "Otherwise,...". However, in the spec, "Otherwise,.. " seems to refer to both cases 1 and 2b, and the operation of the target gNB is unclear.

##### 4.2.7.2.2	Creating NGAP UE-TNLA-bindings during handovers

During an Xn-based inter NG-RAN node handover, the following applies

-	If an NGAP UE-TNLA-binding exists for a UE, the source 5G-AN node supplies the target 5G-AN node with the corresponding TNL address of the AMF for the currently used TNL association.

-	If the target 5G-AN does not have a TNL association towards the TNL address of the AMF received from the source 5G-AN node, the target 5G-AN node establishes a TNL association towards the TNL address received from the source 5G-AN node, creates an NGAP UE-TNLA-binding to this TNL association and sends the N2 Path Switch Request via this TNL association.

-	Otherwise, the target 5G-AN node creates an NGAP UE-TNLA-binding for the UE by selecting a TNL association from the available TNL associations towards the TNL address, permitted for the initial N2 message for the AMF identified by the UE's GUAMI.

-	The AMF may decide to use the TNL association selected by the 5G-AN or the AMF may modify the NGAP UE-TNLA-binding by triangular redirection.

Observation 1:	In the current 23.502, the behaviors are ambiguous 1) in case that the target gNB has already established a TNL association for the same TNL address as the Signalling TNL Association Address at source NG-C side IE received in the Handover Request/Retrieve UE Context Response message, and 2) in case that the target gNB has not established it and cannot establish a TNL association for that TNL address.

Fig. example deployment for case 2b.

In a real deployment, 2b is a possible scenario. As shown in the figure above, due to limited capacity of the GW to accommodate gNBs, Xn interfaces can be established between gNBs that are connected to the same AMF but cannot use the same AMF address. Xn handover should still be possible in this case, and the target gNB should create a UE-TNLA-binding using a different TNL Association Address from the one source gNB used.

Observation 2:	In a real deployment, Xn interfaces may be established between gNBs that are connected to the same AMF but cannot use the same AMF address, and Xn handover may be performed between the gNBs.

Clarification on the target gNB behavior on TNLA handling

The problem of the present specification is that the operation of 1 is not specified, and that TNLA establishment at 2a may be impossible is not considered. To fix this, one possible solution is to clarify the behavior of target gNB receiving Signalling TNL Association Address at source NG-C side IE in TS 38.423. The operation for all cases 1, 2a, and 2b is clarified as follows.

When the target NG-RAN node has an available TNL association towards the TNL address as indicated by the signalling TNL association address at source NG-C side IE, or when the target 5G-AN does not have a TNL association towards the TNL address of the AMF received from the source 5G-AN node and establishes a TNL association towards the TNL address received from the source 5G-AN node, the target NG-RAN node should select the TNL association to create an NGAP UE TNLA binding for the UE. Otherwise, the target NG-RAN node should select other available TNL association towards the same AMF or an AMF from the same AMF set to create an NGAP UE TNLA binding for the UE.

Proposal 1:	RAN3 to specify the behavior of the target/new gNB that receives the Signalling TNL Association Address at source NG-C side IE in the Handover Request/Retrieve UE Context procedure as follows, and agree the CR [6-9].

When the target NG-RAN node has an available TNL association towards the TNL address as indicated by the Signalling TNL association address at source NG-C side IE, or when the target NG-RAN node does not have a TNL association towards the TNL address of the AMF received from the source NG-RAN node and establishes a TNL association towards it, the target NG-RAN node should select the TNL association to create an NGAP UE TNLA binding for the UE. Otherwise, the target NG-RAN node should select other available TNL association towards the same AMF or an AMF from the same AMF set to create an NGAP UE TNLA binding for the UE.

Conclusions and proposals

Our proposals are summarized below.

Observation 1:	In the current 23.502, the behaviors are ambiguous 1) in case that the target gNB has already established a TNL association for the same TNL address as the Signalling TNL Association Address at source NG-C side IE received in the Handover Request/Retrieve UE Context Response message, and 2) in case that the target gNB has not established it and cannot establish a TNL association for that TNL address.

Observation 2:	In a real deployment, Xn interfaces may be established between gNBs that are connected to the same AMF but cannot use the same AMF address, and Xn handover may be performed between the gNBs.

Proposal 1:	RAN3 to specify the behavior of the target/new gNB that receives the Signalling TNL Association Address at source NG-C side IE in the Handover Request/Retrieve UE Context procedure as follows, and agree the CR [6-9].

When the target NG-RAN node has an available TNL association towards the TNL address as indicated by the Signalling TNL association address at source NG-C side IE, or when the target NG-RAN node does not have a TNL association towards the TNL address of the AMF received from the source NG-RAN node and establishes a TNL association towards it, the target NG-RAN node should select the TNL association to create an NGAP UE TNLA binding for the UE. Otherwise, the target NG-RAN node should select other available TNL association towards the same AMF or an AMF from the same AMF set to create an NGAP UE TNLA binding for the UE.

References

- “RAN3_111-e_agenda_with_Tdocs20210204_EOM.doc”, chair

- R3-210484, “Discussion on NG-RAN node behavior when it receives the Xn Signalling TNL association address at source NG-C side IE”, Nokia

- R3-210485, “Clarify NG-RAN node behavior upon the reception of the Signalling TNL association address at source NG-C side IE”, Nokia

- S2-2100651, “NGAP TNLA Binding correction”, Nokia

- R3-257133, “Discussion on TNLA handling during Xn handover”, NTT DOCOMO INC.

- R3-258408, “(R16 CR to TS38.423) Clarification on TNLA handling during Xn handover”, NTT DOCOMO INC.

- R3-258411, “(R17 CR to TS38.423) Clarification on TNLA handling during Xn handover”, NTT DOCOMO INC.

- R3-258413, “(R18 CR to TS38.423) Clarification on TNLA handling during Xn handover”, NTT DOCOMO INC.

- R3-258414, “(R19 CR to TS38.423) Clarification on TNLA handling during Xn handover”, NTT DOCOMO INC.