3GPP TSG-RAN WG3 #130	R3-258472

Dallas, USA, 17-21 November 2025

Agenda Item:	9.2.6

Source:	Ericsson, Aalyria, Thales, Airbus

Title:	Inter-Satellite Links and Transparent Payloads

Document for:	Discussion

# Introduction

Inter-Satellite Links (ISLs) have been introduced in Rel-19 as part of the 3GPP support for NTN with regenerative payloads. However, since ISL is a transport link, it can also connect one or two transparent payloads.

# Discussion

ISLs, transport links between payloads, have been introduced in Rel-19 in the scope of regenerative-payload NTN. Current stage 2 text reads:

“Inter-satellite link: wireless transport link between two regenerative NTN payloads.” (Sec. 3.2 of [1])

“The following connectivity is supported by the NTN regenerative payload: A regenerative NTN payload may terminate one or more inter-satellite links toward other regenerative payloads.” (Sec. 16.14.1 of [1])

Similar statements are in Secs. 3.1 and 4.12 of [2]: the IoT NTN discussion was aligned to the conclusions of the NR NTN discussion.

Observation 1: According to current stage 2 text, ISL is defined as a wireless transport link between two regenerative payloads.

In principle ISL could also connect a transparent payload, for at least the following use cases:

- Transporting NG between an AMF on the ground and a regenerative payload, via a transparent payload.

- Transporting Xn between regenerative payloads, via a transparent payload.

- Transporting Xn between a gNB on the ground and a regenerative payload, via a transparent payload.

The above 3 use cases only leverage the fact that ISL is a transport link and transport links are not part of the logical architecture. As such, they can be supported with no impact on other specified functionality. This applies to both NR NTN and IoT NTN.

Observation 2: ISL can also be used involving a transparent payload, to transport e.g. NG and/or Xn to a regenerative payload, with no impact on other specified functionality; this applies to both NR NTN and IoT NTN.

We therefore propose to remove this limitation on ISL usage from stage 2.

Proposal 1: Remove the limitation from stage 2, that ISL is only defined between two regenerative NTN payloads.

We welcome further discussion and we are providing two stage 2 CRs [3][4]. Notice that it seems necessary to only update the definition of ISL.

Proposal 2: Discuss, revise and if possible, agree the related CRs for NR NTN and IoT NTN.

# Conclusions and Proposals

Our observations and proposals are summarized below.

Observation 1: According to current stage 2 text, ISL is defined as a wireless transport link between two regenerative payloads.

Observation 2: ISL can also be used involving a transparent payload, to transport e.g. NG and/or Xn to a regenerative payload, with no impact on other specified functionality; this applies to both NR NTN and IoT NTN.

Proposal 1: Remove the limitation from stage 2, that ISL is only defined between two regenerative NTN payloads.

Proposal 2: Discuss, revise and if possible, agree the related CRs for NR NTN and IoT NTN.

# References

TS 38.300 v. 19.0.0.

TS 36.300 v. 19.0.0.

R3-258474	Inter-Satellite Links and Transparent Payloads – NR NTN, Ericsson, Aalyria, Thales, Airbus.

R3-258485	Inter-Satellite Links and Transparent Payloads – IoT NTN, Ericsson, Aalyria, Thales, Airbus.

Another potential use case might be transporting Uu between the gNB on the ground and a transparent payload via another transparent payload. This might have additional implications outside of RAN3 scope, due to e.g. timers between gNB and served UEs.