3GPP TSG-RAN WG3 Meeting #130	R3-258519

# Dallas, USA, 17 – 21 November 2025

Agenda Item:	10.4

Source:	Ericsson

Title:	About 6G RAN-internal architecture topics

Document for:	Discussion and decision

# 1	Introduction

6G RAN-internal architecture has been discussed during the last RAN3 meeting, and the discussion was mainly focused on the need for a Higher Layer Split in the 6G RAN. As a conclusion of this discussion, the following open issue was captured:

Capture pain points and benefits (and potentially derived requirements) of 5G HLS

To be continued...

In addition, the need for a “horizontal” interface between 6G RAN nodes was briefly discussed.

This paper will therefore continue the discussion on these 2 topics, consider the above objective for RAN3#130 and present the pain points, benefits and derived requirements of 5G HLS.

# 2	Discussion

HLS was discussed during the last RAN3 meeting, and while some companies see benefits in having this split in 6G, some other companies want first to discuss the pain points that 5G HLS introduced in the overall 5G RAN design and implementation before determining whether standardisation of HLS for 6G is required. This paper will address these points, as captured last meeting, but will also propose a way forward in order to progress on the RAN-internal architecture overall.

## 2.1 Pain points of 5G HLS

As captured during the last RAN3 meeting, the pain points that were identified in the 5G HLS are to be discussed in 3GPP, to move forward on the RAN-internal architecture design.

### 2.1.1. Split UE configuration and state management

The first difficulty to overcome when splitting the 5G RAN node into two logical entities (i.e., the gNB-CU and gNB-DU) is the handling of the UE configuration and state management, when these two logical entities are both involved in these tasks. With HLS, the UE context and state handling is split between gNB-CU and gNB-DU which complicates the application of optimised radio features and therefore leads to suboptimal performance and increased configuration delays. However, end-user performance should be regarded as a key requirement to favour considering the cost and scarcity of radio spectrum that operators need to deploy to provide communication services.

Splitting the responsibilities for a single UE is not straightforward. At the beginning of Rel-15, 3GPP needed to define a strict function and layer distribution between the gNB-CU and the gNB-DU. This was, and still is, a very time and effort consuming task that requires large amount resources for every feature that needs to be supported for the split architecture. In some cases, 3GPP has even come to the point where support of certain features for split architecture could not be achieved. For example, the AI/ML based UE performance feedback specified for AI/ML was supported for non-split architectures. However, it was not supported in Rel-18 for split architecture and has limited support in Rel-19, given its complexity when it comes to support in the HLS architecture option. 
Also, by having a fixed split standardised, and therefore only one option on how to distribute RAN functionalities, 3GPP limits the possible implementation freedom and innovation of providing more efficient function distribution solutions. This resulted in a compromised split of responsibilities, which is not always the best choice in terms of cost, performance, simplicity, etc...

In reality, the following issues were introduced by the HLS:

- The lack of knowledge of RAN nodes capabilities. A gNB-CU does not know the capabilities of a gNB-DU and vice versa. It is challenging to define capabilities to a reasonable level to be exchanged via an open interface. Many are implementation-specific and even if exchanged, at the cost of a non-negligeable bandwidth and memory cost, this will not be useful (or even comprehensible) for the peer node with a different implementation. This capability management also introduces a lot of complexity because a proper design needs to consider all possible capability combinations, including the error handling.

- Dependencies on the nodes forming the split architecture on processes that are implementation specific. For example, beam management/shaping, radio resource management, measurement gaps configuration, mobility decisions, scheduling. To make a split architecture work optimally, all nodes in such architecture should be fully aware of such processes’ designs. The latter is impossible to achieve in 3GPP as it would mean to standardise implementation aspects that are out of 3GPP scope.

There are many well-known examples of such sub-optimal design, but just to mention some:

- gNB-DU is responsible for the creation of the RRC: Cell Group Config IE, which contains a very high number of information concerning the cell level and UE level configuration needed at the UE. The gNB-CU is responsible to formulate the overall RRC reconfiguration towards the UE and many parts of such reconfiguration depend on the content of the Cell Group Config. This implies that the gNB-CU and the gNB-DU need to have compatible and tightly coordinated implementations that allow the decisions in the gNB-DU not to impact the gNB-CU (and vice versa) in providing an optimal configuration towards the UE. A proof of the need for such coordination is in the fact that RAN3 has decided to signal as explicit F1AP IEs information that are contained in the Cell Group Config, implying that:

  - Decisions in the gNB-DU on UE configuration impact decisions in the gNB-CU;

  - Lack of separation of concerns between gNB-DU and gNB-CU tasks;

- Duplication of information in F1 signalling messages. The gNB-CU is responsible for the configuration of measurements, i.e. it is responsible for the encoding of the RRC: MeasConfig IE, but the gNB-DU is responsible for allocation of resources for the configured measurements, i.e. the gNB-DU creates the RRC: MeasGapConfig. This brings some drawbacks such as:

  - The gNB-CU does not know if the gNB-DU can reserve resources to allocate measurement gaps. Bad decisions at the gNB-CU in configuring measurements could impact the performance at the UE due to gNB-DU being mandated to allocate gaps where the UE cannot be scheduled.

  - Again, lack of separation of concerns, where the measurement configuration process is split between two different entities, resulting in sub-optimal measurement configurations.

- Support of pre-configured measurement gaps (MG) for active bandwidth part (BWP) with NCD-SSB, with back-and-forth signalling, where gNB-CU needs to:

  - Get the list of encoded ServingCellMOs info with active BWP from gNB-DU;

  - Send the list of frequencies and BWP ID requiring preconfigured measurement gaps to gNB-DU;

  - Receives the updated content of CellGroupConfig from gNB-DU (and do not notify the UE) to decide whether to activate/deactivate the preconfigured measurement gaps;

  - Finally send the RRC message to UE with the Pre-MG to use when activated.

Therefore, the split complicated the decision logic and increased the overall complexity in gNB-CU and gNB-DU substantially, while leading to worse decisions, delayed actions and increased signalling and processing load.

Observation 1: HLS brought complexity and suboptimal performance by splitting UE Control and traffic handling between gNB-CU-CP, gNB-CU-UP and gNB-DU.

### 2.1.2. Delay in fronthaul

The (RRC) protocol shows unnecessary complexity because it needs to take CU/DU split into consideration. This complexity significantly impairs its efficiency. This problem is exacerbated by back-and-forth gNB-CU/gNB-DU signalling and the transport latency on the F1 Control Plane (CP), which delays communication between the gNB-CU and gNB-DU. This combination of protocol complexity, transmission delay and back-and-forth gNB-CU/gNB-DU signalling prevents the RRC from optimally fulfilling its primary task – the efficient and dynamic management of radio resources. The resulting delays in decision-making processes negatively impact the overall network performance.

As an example, consider the case of fast transitions of a UE between RRC_CONNECTED and RRC_INACTIVE, due to short bursts of data to be sent to the UE. Every time the UE has to be moved from RRC_INACTIVE to RRC_CONNECTED a full F1 UE context setup needs to be run. This procedure alone, takes the exchange of the following messages to produce a successful configuration towards the UE:

- INITIAL UL RRC MESSAGE TRANSFER

- UE CONTEXT SETUP REQUEST

- UE CONTEXT SETUP RESPONSE

- DL RRC MESSAGE TRANSFER

If we assume a typical one way delay of 10ms over transport connections between the gNB-DU and the gNB-CU, and if we consider a typical node internal message processing delay of 4ms, we can easily see that the F1 signalling alone adds 56ms delay to the process of moving a UE from RRC_INACTIVE to RRC_CONNECTED. This reduces the possibility to use RRC_INACTIVE as a dynamic way to place the UE in a state of low energy consumption and low signalling level.

The same observation can be done for UP latency. Transport network latency directly impacts end-user throughput primarily because TCP’s congestion control and reliability mechanisms are tightly coupled to round-trip time. It also hinders the deployment of new optimized features such as e.g. URLLC, XR. Also, round-trip time estimation influences retransmission timeouts. Spikes in latency can trigger bogus timeouts, leading to a new slow-start phase. Every millisecond of added latency reduces the speed of TCP’s window increase and the maximum number of in-flight packets, directly lowering the achievable throughput.

Observation 2: Fronthaul delays and back-and-forth signalling introduce latency that impacts performance (as seen by the end user) and hinders the deployments of new optimized features.

### 2.1.3. Interoperability

The design of the 5G HLS has been characterized by a reactive design approach that led to some interoperability issues. Instead of conceiving a holistic architecture from the outset, HLS functionalities were added incrementally across multiple 3GPP releases. This incremental process resulted in the F1 interface becoming increasingly complex and limited. The subsequent adjustments and extensions resulted in an architecture that shows many inconsistencies that hinder interoperability. 6G will likely run into the same issues as many releases, enhancements and new features are expected.

It has to be acknowledged that implementation details of complex entities such as a gNB-CU or a gNB-DU cannot be covered by the standard. This aspect has to do with the fact that proper interoperability over the F1 interface would have to take every implementation aspect into account down to very fine details of e.g., beam management/shaping, radio resource management, measurement gaps configuration, mobility decisions, scheduling, which are vendor specific. Obviously, without such detailed lock-in in the standard on implementation aspects the F1 became incomplete.

This has serious consequences for multi-vendor interoperability. Since essential implementation details were not in scope of 3GPP and therefore not defined, various network equipment vendors were forced to develop their own proprietary solutions for gNB-CU and gNB-DU functionalities. These vendor-specific implementations diverge significantly from one another, making the integration of components from different vendors considerably more difficult or even impossible. This systematically undermines the fundamental goal of standardized interfaces – namely vendor independence and flexible network architectures.

Observation 3: Inter vendor interoperability over an HLS interface is limited and not possible to achieve via standardisation due to implementation design aspects being out of 3GPP scope.

## 2.2. Requirements for 6G

Avoiding these pain points is essential to the 6G RAN success. RAN3 has been tasked by RAN to study HLS, and understanding requirements, pain points and benefits is part of this work. This study is ongoing and will continue. However, while important, this is not the only topic RAN3 needs to work on. Other topics, related to this Agenda Item or to other 6G Agenda Items need to progress. For example RAN3 needs to decide on a possible horizontal interface between 6G RAN nodes. And, if agreed, RAN3 needs to define the functionalities that this interface will support. These additional topics need to be discussed as early as possible to ensure the best conclusion possible at the end of the Study Item. And for that some definition of a 6G RAN node needs to be discussed first.

One architecture option that has always existed in 4G and 5G and has been widely deployed could easily be seen as a starting point: a RAN architecture based on a single logical RAN node supporting all 6G RAN protocols and functions. This 6G RAN logical entity shall be defined by RAN3, and functions and interfaces shall be developed taking this architecture option in mind. This of course does not preclude other architecture options, as discussed in the HLS topic. On the other hand, this would enable RAN3 to have a baseline architecture on which RAN3 can start working (e.g. agree on interfaces, termination points and functions supported by these interfaces).

Proposal 1: RAN3 agrees to support a 6G RAN- architecture with a single logical node supporting all 6G RAN protocols and functions. Other architecture options are not precluded.

Another important aspect, which was mentioned in previous sections is Uu communication protocol design. The Uu interface is key for increasing all performances that operators need to justify the deployment of 6G. And for these performances to be optimized as much as possible, Uu protocols such as RRC need to be constraint free. In the current 5G HLS approach, RRC responsibilities are divided so that some parameters are managed by the gNB-CU and some by the gNB-DU. This division makes it increasingly difficult to apply RRC optimizations. To ensure that the 6G Uu communication protocol design can accommodate future enhancements and remain easy to implement for both networks and devices, the architecture must be able to adapt flexibly to such changes. For that reason, fragmentation of Uu communication protocols handling shall not be taken into consideration for the initial Uu study. Any RAN-internal architecture option should enable the implementation of a flexible, modular and optimized 6G Uu communication protocol design. These RAN-internal architecture options also need to be invisible at UE and 6G Uu level.

Proposal 2: RAN3 shall specify the 6G RAN architecture so to avoid constraining the 6G Uu communication protocols design.

Looking at the pain points described in previous section, one of the main design blocks that we think a standalone 6G RAN should support is the ability to maintain at least functionalities for the configuration and handling of a UE in a single logical entity. We emphasised how complexity increases, when the configuration of a UE has to be negotiated between multiple logical nodes. The result is often a sub-optimal configuration, which impacts the overall performance. Coordination between different logical entities implies delays in exchanging information, processing, and converging to a viable configuration option. Such delays slow down the activation of optimised data transfer options. As a consequence, the following general principle can be derived and should be agreed. To enable a lean, efficient, and fast control plane for 6G, each UE should be controlled by a single RAN control entity function.

Proposal 3: UE-associated functionalities shall be controlled by a single logical RAN entity.

## 2.3 6G RAN node to 6G RAN node interface

A “horizontal” interface has always been part of 3GPP RAN generations. This is essential for interoperability between different vendors deployed in different neighbouring areas, or deployed in different node sizes (e.g. macro and small cells).

Proposal 4: Agree to standardise a 6G RAN node to 6G RAN node interface.

In order to achieve a common understanding of what functionalities should be supported by this new interface, we should initiate discussions on potential additional requirements that could warrant the inclusion of further functionalities that are truly needed for intervendor-interoperability.

We should therefore first focus on the functionalities which are essential for inter-vendor interoperability and needs to exist on the new interface from day one, such as:

- Mobility between 6G RAN nodes

- Interface Management and error handling

Proposal 5: Intra 6G-RAN Mobility and Interface Management are supported by the 6G RAN Node to 6G RAN node interface.

# 3	Conclusion

6G RAN-internal architecture was discussed, focusing on HLS analysis and proposing a way forward for other topics in the scope of this Agenda Item:

Observation 1: HLS brought complexity and suboptimal performance by splitting UE Control and traffic handling between gNB-CU-CP, gNB-CU-UP and gNB-DU.

Observation 2: Fronthaul delays and back-and-forth signalling introduce latency that impacts performance (as seen by the end user) and hinders the deployments of new optimized features.

Observation 3: Inter vendor interoperability over an HLS interface is limited and not possible to achieve via standardisation due to implementation design aspects being out of 3GPP scope.

Proposal 1: RAN3 agrees to support a 6G RAN architecture with a single logical node supporting all 6G RAN protocols and functions. Other architecture options are not precluded.

Proposal 2: RAN3 shall specify the 6G RAN architecture so to avoid constraining the 6G Uu communication protocols design.

Proposal 3: UE-associated functionalities shall be controlled by a single logical RAN entity.

Proposal 4: Agree to standardise a 6G RAN node to 6G RAN node interface.

Proposal 5: Intra 6G-RAN Mobility and Interface Management are supported by the 6G RAN Node to 6G RAN node interface.