3GPP TSG RAN WG3 Meeting #130	R3-25xxxx

Dallas, TX, USA, 17 – 21 November, 2025

Agenda Item:	8.1

Source: 	Ericsson

Title:  	On Restriction of PEIPS during emergency PDU session

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the SA2 reply on the RAN3 detected issue in [1] concerning usage of PEIPS during an emergency PDU session, which has been discussed extensively in previous meetings.

# Discussion

In the reply LS to CT1/RAN3 (S2-2509759) SA2 indicates that it has aligned the wording of the restriction to not use PEIPS during emergency in TS23.501 (stage 2) in Rel-17 with TS24.501 (stage 3). It is in fact captured that:

When an emergency PDU session is successfully established over 3GPP access, the UE and the network shall not use PEIPS assistance information, see details as described in TS 24.501 [47].

This is fully aligned with RAN current text (TS 38.300) and in CT1 specs. Therefore, no inter-operability problem is foreseen with this resolution. This was a very logical decision by SA2 to avoid any future complex handling between UE, gNB and AMF from different releases when PEIPS is used. We note that this alignment was also agreed by SA2 for Rel-18 and in Rel-19. These alignments for other releases were not communicated in the LS, but we refer them below for completeness:

| 4.1 | S2-2509758 | CR | Approval | 23.501 CR6453R2 (Rel-19, 'A'): PEIPS for emergency PDU session | ZTE | Rel-19 | NR_UE_pow_sav_enh-Core, TEI17 | Revision of S2-2509471. Agreed in parallel session. Block approved | Agreed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 4.1 | S2-2509757 | CR | Approval | 23.501 CR6468R1 (Rel-18, 'A'): PEIPS for emergency PDU session | ZTE | Rel-18 | NR_UE_pow_sav_enh-Core, TEI17 | Revision of S2-2509473. Agreed in parallel session. Block approved | Agreed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Observation 1: SA2 agreed to bring back the restriction to not use PEIPS during emergency PDU session, this means there is no backwards compatibility issue.

In current TS 38.300 it is stated that:

The AMF provides to the NG-RAN node the Core Network Assistance Information to assist the NG-RAN node's decision whether the UE can be sent to RRC_INACTIVE, and to assist UE configuration and paging in RRC_INACTIVE. The Core Network Assistance Information includes: […] The PEI with Paging Subgrouping assistance information, which may be taken into account by the NG-RAN node for subgroup paging in RRC_INACTIVE, except when the UE context contains an emergency PDU session in which case the PEI with Paging Subgrouping assistance information shall not be used according to TS 24.501 [28].

This clause from TS 38.300 refers to restriction of usage of PEIPS during RAN Paging when the UE context has an emergency PDU session. However, since it was agreed that the UE can also be released to RRC_IDLE (by implementation) during emergency PDU session, then the restriction should also be captured in the PAGING message case. Currently there is no such restriction in TS 38.413 Paging Procedure for PEIPS section copied below:

If the PEIPS Assistance Information IE is included in the PAGING message, the NG-RAN node shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [8].

We note also that TS 23.501 text did not agree on any behavior from AMF side to not include the PEIPS in the message towards RAN, as shown below.

If the AMF has determined AMF PEIPS Assistance Information, the AMF shall provide it to NG RAN when paging the UE. In addition, in order to support PEIPS for UEs in RRC_INACTIVE mode, the AMF shall provide the AMF PEIPS Assistance Information to NG-RAN as part of the RRC Inactive Assistance Information.

Therefore, it should be clarified that during RAN paging, the gNB does not use PEIPS in case the UE context has an emergency PDU session.

Observation 2: Since the UE can be released to RRC_IDLE (by implementation) during emergency PDU session, the PAGING message should also include the restriction to not use PEIPS during emergency PDU session, similar to RRC_INACTIVE case.

Proposal 1: Update procedural text in NGAP for Paging message that when PEIPS is included in the PAGING message, the NG-RAN node shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300, except when the UE context contains an emergency PDU session in which case the PEI with Paging Subgrouping assistance information shall not be used

CR to TS 38.413 is provided in [2]

# Summary

Observation 1: SA2 agreed to bring back the restriction to not use PEIPS during emergency PDU session, this means there is no backwards compatibility issue.

Observation 2: Since the UE can be released to RRC_IDLE (by implementation) during emergency PDU session, the PAGING message should also include the restriction to not use PEIPS during emergency PDU session, similar to RRC_INACTIVE case.

Proposal 1: Update procedural text in NGAP for Paging message that when PEIPS is included in the PAGING message, the NG-RAN node shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300, except when the UE context contains an emergency PDU session in which case the PEI with Paging Subgrouping assistance information shall not be used

# References

- R3-258007,LS Response on compatibility issue for PEI and emergency PDU session

- R3-258539-8541, NGAP CR on PEIPS