3GPP TSG-RAN WG3 Meeting #130	R3-258544

# Dallas, Texas, U.S., 17th - 21st November 2025

Agenda Item:	10.3.1

Source:	Ericsson

Title:	[pCR for TR 38.760-3] Definitions and Requirements for the 6G RAN-CN interface

Document for:	Discussions & Approval

# 1	Introduction

This TP follows discussions in R3-258543.

# 2	pseudo CR against latest version of TR 38.760-3

<<<<<<<<<<<<<<<<<<<< Begin of Changes >>>>>>>>>>>>>>>>>>>>

## 4.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Logical entity: an architectural item hosting a clearly defined set of functions.

Application Protocol: a set of rules defining the way how a logical entity communicates with another logical entity by means of protocol data units and how a logical entity behaves upon reception of a protocol data unit (e.g. execute a function, store data, reply with a respective defined protocol data unit).

Logical interface: an architectural item via which a logical entity interacts with another logical entity by means of exchanging protocol data.

Point-to-point interface: a logical, standardized control plane interface between two logical entities. It enables the establishment of a logical association and the exchange of protocol data between the logical entities constituting such point-to-point interface in a secure way. The realization of a logical point-to-point interface does not require a direct transport connection between the logical entities constituting it and allows flexible deployment scenarios including virtualized and cloud environments.

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

### 6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface.

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN interface assume the same RAN-CN functional split as for 5G for functionalities also supported in 6G

-	the 6G RAN-CN control plane interface is terminated at a single logical entity in the 6G RAN and the 6G CN.

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN;

-	from a logical point of view, the 6G RAN-CN interface is a Point-to-Point Interface between a 6G RAN logical entity and a 6G CN logical entity;

-	the 6G RAN-CN interface design shall enable realisation of 6G RAN and 6G CN modular functions in a cloud environment, while neither affecting inter-vendor operability nor requiring the realisation of modular functions to be visible outside logical entities;

-	how 6G RAN-CN interface traffic is distributed to internal modular functionalities shall be transparent to the peer entity;

-	the 6G RAN-CN control plane interface design shall consider expected 6G logical entity realisations applying modularisation by

-	defining dedicated procedures for different protocol functions,

-	minimising interactions between functions and

-	avoiding serving multiple functionalities with a single procedure as much as possible.

-	the 6G RAN-CN control plane interface design shall be decoupled from the signalling transport, i.e. it shall be "signalling TNL agnostic" and shall not expect any other function from the signalling transport than delivery of SDUs.

-	the 6G RAN-CN control plane interface transports information that enables efficient routing of AP messages supporting 6G logical entity realisations in a modularised cloud environment.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>