3GPP TSG-RAN WG3 Meeting #130	R3-258574

Dallas, US, 17-21 Nov, 2025

Title: 	Finalizing the compatibility issue for PEI and emergency PDU session

Source: 	Huawei

Agenda item:	8.1

Document Type:	Discussion

# 1. Introduction

- As the reply LS to [1], SA2#171 meeting agreed the LS on the allocation of CN assigned subgroup ID for LP-WUS, with the following contents [2].

1. Overall Description: SA2 thanks RAN3 for the LS in on the compatibility issue for PEI and emergency PDU session. SA2 has approved the attached Rel-17 CR. 2. Actions: To RAN3 and CT1: Action: SA2 kindly ask RAN3 and CT1 to take the above into account.

This contribution tries to finalize the issue which was initiated by RAN3.

# 2. Discussion

- Though the R17 CR was not attached in the incoming LS, we copy the changes as follows in [3].

#### 5.4.12.2 Core Network Assistance for PEIPS <Skip the irrelevance> When an emergency PDU session is successfully established over 3GPP access, the UE and the network shall not use PEIPS assistance information, see details as described in TS 24.501 [47].

- It can be observed that starting from R17, SA2 specifies that the UE and the network shall not use the PEIPS assistance information (i.e., CN subgroup ID) in case of emergency PDU session. Also, the mirror CRs for R18 and R19 were agreed. This means that SA2 has reverted their R19 CR.

- Observation 1: SA2 reverted the R19 CR so that the UE and the network shall not use the PEIPS assistance information in case of emergency PDU session.

- For CN paging, it is the CN control whether to use the PEIPSS. This means the CN can control not to send the PEIPS to the RAN in the NGAP Paging message, so that the RAN will not use it if the paging is triggered by the emergency PDU session.

- For RAN paging, it is the NG-RAN node to decide whether to use the PEIPS when the PEIPS is available. As excerpted below, the stage 2 texts already specify that the NG-RAN would not use the PEIPS for RRC inactive UE, when the UE context contains an emergency PDU session. Hence it can be concluded that there is no issue for PEIPS assistance information for the emergency PDU session for RRC inactive UE.

### 9.2.2 Mobility in RRC_INACTIVE #### 9.2.2.1 Overview <Skip the irrelevance> - The NG-RAN node takes into account the PEI with Paging Subgrouping assistance information for subgroup paging in RRC_INACTIVE except when the UE context contains an emergency PDU session in which case the PEI with Paging Subgrouping assistance information shall not be used according to TS 24.501 [28].

- Observation 2: In RAN specification, there is no issue for PEIPS assistance information for the emergency PDU session for RRC inactive UE.

- On the other hand, for UE-ID based subgrouping, however, there are no clear stage2 texts to specify whether the UE-ID based subgrouping could be used in case of emergency PDU session.  According to the LSs among different groups, e.g., in [4], it clearly replied that the UE-ID based PEI is not able to be restricted.

SA2 thanks RAN2 for informing the usage of UE-ID based PEI with emergency PDU session. SA2 would like to provide the following response: - SA2 discussed the issue and indicates that PEI based on CN sub-grouping (i.e. PEIPS-based PEI) is disabled in SA2 specs and CT1 specs when UE has an emergency PDU session. - In the case of CN triggered paging for a UE with an emergency PDU session, SA2 acknowledges that RAN does not know if there is emergency PDU session and UE-ID based PEI is not able to be restricted. SA2 discussed different options for the way forward (e.g., enforcing the restriction on the usage of PEI totally including the UE-ID based PEI, or completely removing the restriction on the usage of PEI including the PEIPS-based PEI) but could not yet reach any agreement. - However, SA2 also considers that the service impact is limited even without any change in the current specifications for this scenario. SA2 would like to point out that currently TS 23.167 indicates that emergency call back is handled like a normal MT call from service aspect. SA2 would also like to indicate that there are other features, such as not immediately paging in all cells of the registration area, that can cause MT call set up delays.

- Hence, especially for RAN paging, it is the NG-RAN to decide whether the UE-ID based subgrouping can be used or not. We propose to capture stage2 texts to specify that the NG-RAN node could use the UE-ID based subgrouping for CN paging and RAN paging, even in case of emergency PDU session, if CN controlled subgroup ID is not provided from AMF. With this text, the PEI issue (covering both the CN subgroup ID, and UE-ID based subgroup ID) in case of emergency PDU session can be finalized. Specifically, for RAN paging in case of emergency PDU session

- CN based subgrouping cannot be used (already covered)

- UE-ID based subgrouping can be used (needs to be specified).

For PEI, RAN3 to agree stage2 texts that the NG-RAN node could use the UE-ID based subgrouping for RAN paging, even in case of emergency PDU session, if CN controlled subgroup ID is not provided from AMF.

- For LP-WUS, SA2 already removed the restriction so that the LP-WUS could be used when an emergency PDU session is successfully established. Then newly added stage2 text can also applicable LP-WUS for UE-ID based subgrouping, i.e., the Texts for the Rel-17 can also be applicable for Rel-19 LP-WUS without any changes for UE-ID based subgrouping.

For LP-WUS, the above proposal is also applicable for UE-ID based subgrouping. The stage2 texts added for the Rel-17 PEI can also be applicable for Rel-19 LP-WUS without any changes.

- The CRs to reflect the above proposals could be found in [5-7].

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- Observation 1: SA2 reverted the R19 CR so that the UE and the network shall not use the PEIPS assistance information in case of emergency PDU session.

- Observation 2: In RAN specification, there is no issue for PEIPS assistance information for the emergency PDU session for RRC inactive UE.

- For PEI, RAN3 to agree stage2 texts that the NG-RAN node could use the UE-ID based subgrouping for RAN paging, even in case of emergency PDU session, if CN controlled subgroup ID is not provided from AMF.

- For LP-WUS, the above proposal is also applicable for UE-ID based subgrouping. The stage2 texts added for the Rel-17 PEI can also be applicable for Rel-19 LP-WUS without any changes.

# 4. Reference

- R3-255906		LS on compatibility issue for PEI and emergency PDU session, RAN3

- S2-2509759		LS Response on compatibility issue for PEI and emergency PDU session, SA2

- S2-2509756		PEIPS for emergency PDU session, ZTE

- R3-233727 		Reply LS on the use of PEI during an emergency PDU session (SA2(Ericsson))

- R3-258575		PEI usage in case of emergency PDU session	Huawei

- R3-258576		PEI usage in case of emergency PDU session	Huawei

- R3-258577		PEI usage in case of emergency PDU session	Huawei