3GPP TSG-RAN WG3 Meeting #130	R3-258631

Dallas, Texas, USA, November 17th – 21th, 2025

Agenda item:	13.3

Source: 	LG Electronics Inc.

Title: 	(TP to BL pCR to TR 38.765) Discussions for ISAC RAN-CN aspects

Document for:	Other

# Introduction

Rel-20 ISAC study (5G Advanced) for RAN WGs was agreed at RAN#108 and further revised in RAN#109 [1], with the following objectives:

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

For RAN-CN signalling aspects, the initial discussions on RAN3-129bis progressed as follows, with the first TP for the RAN-CN procedures and signalling captured in TR 38.765 [2]:

To continue the discussion on the necessary interface functions and the detailed signaling procedures for ISAC.

# 8 RAN-CN procedures and signalling Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing. ## 8.x Sensing Reporting Editor’s Note: the following may need further refinement. Figure 8.x-1: Message flow for sensing reporting 1. The SF sends sensing request to the gNB. 2. The gNB sends sensing response to the SF. 3. If requested in Step 1, the gNB sends sensing report to the SF.

Considering that SA2’s latest interim agreements [3], in this contribution, we discuss several aspects of RAN-CN signalling details to support ISAC operations in Rel-20, and propose the corresponding TP in Annex.

# Discussion

## Support for SE (gNB) discovery and selection

One important aspect for discussion is how a SE (gNB) is selected to perform sensing executions, which has not been discussed at the last RAN3 meeting.

In the meantime, SA2 has produced interim agreements in TR 23.700-14 [3], excerpted below:

### 7.1.2 Agreed Principles for KI#2: Authorization and Revocation to Support Sensing Service The following principles are agreed for KI#2: a) The Sensing Service authorization consists of two steps: 1) Authorisation of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9]; 2) Authorisation of the AF's Sensing Service Request performed by the Sensing authorization functionality; NOTE: In this clause, Sensing authorization functionality may refer to Sensing Function, Sensing Control Function or Sensing Gateway and the wording may need alignment according to the conclusion of key issue 1. b) Sensing authorization functionality determines whether the Sensing Service Request from the AF is authorised, considering the example of Sensing authorisation information described in Table 7.1.2-1. Additional parameters, if any and further details on Sensing authorization information can be determined during the normative phase. Table 7.1.2-1: Sensing Authorization information for Sensing Service AF Authorization Data; Description. AF ID; Identifier used to identify the AF.. Allowed/Not allowed area for sensing; Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations (NOTE 1).. Allowed/Not allowed time period for sensing; Indicate the allowed time period within which AF can trigger a particular sensing service operation.. (Allowed) sensing service type; Indicate the allowed sensing service type (e.g. object tracking, detection) for the indicated AF to trigger the sensing services operations.. NOTE 1: The Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed.. c) The Sensing Authorization information is pre-configured or configured by OAM and stored in Sensing authorization functionality for authorisation of the sensing service request. d) Sensing authorization functionality can revoke the Sensing Service Request when the authorisation criteria are no longer met. e) The AF can cancel the on-going Sensing Service Request when needed. f) There is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator. ### 7.1.3 Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection: - The Sensing Function selection is based on NEF's local configuration or NRF query. - The Sensing Function registers itself to the NRF, with its NF profile. - Sensing Function is selected based on at least its supported sensing area. - Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request. - Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area. NOTE: The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

While SA2 agreed that a gNB as SE does not require authorization (thus no authorization signalling needed), SA2 also agreed that an SE is selected based on its information, e.g. its supported sensing areas.

This implies that SF must be aware of the capabilities of SE-capable gNBs. One example is its supported sensing area (and there could be many more parameters). Such information should be exposed for SE’s  discovery and (re)selection.

Given that SA2 study is scheduled to conclude in this year, there is no need to define detailed procedures in RAN3 at this stage. For now, it would be sufficient for RAN3 to capture at a high level that a gNB authorized as SE provides the necessary information so it can be discovered and selected for sensing executions. The details can be discussed once SA2 makes conclusions.

Proposal 1: No need to capture the authorization of a gNB as SE, as agreed by SA2.

Proposal 2: Capture that a gNB (already authorized as SE) provides the necessary information to be discovered and selected for sensing executions, with details FFS (pending SA2 progress).

## RAN-CN sensing management

Another important aspect is how sensing operations are managed across SE (gNB) and SF (residing at 5GC). According to SA2 [3], it is evident that a SF responsible for orchestrating sensing services needs to request sensing executions from the selected SE (gNB) and receives the corresponding reports:

### 7.1.4 Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport The following principles are agreed for KI#4: - Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data. - Sensing Entity can support one time, periodical Sensing Data to the Sensing Function. Editor's note: The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase. - Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request. - The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s). NOTE: The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

To enable consistent life cycle management, it would be beneficial for RAN3 to introduce a sensing-task abstraction over the RAN-CN procedures for sensing service cycle (request, execution, report). A sensing task can represent a single job (requiring one or more sensing executions) and be uniquely identified by an associated ID (sensing task ID), so that this task ID can serve as the anchor for initiating, modifying, and terminating the task, and also for controlling the associated sensing parameters and reporting between them.

Proposal 3: Introduce a sensing “task”, as a basic unit of managing sensing service cycle between SF (residing at 5GC) and SE (gNB), which can be uniquely identifiable by a sensing task ID and can serve as the anchor for initiating, modifying, and terminating the task, and for controlling the associated sensing parameter configuration and reporting between them.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: No need to capture the authorization of a gNB as SE, as agreed by SA2.

Proposal 2: Capture that a gNB (already authorized as SE) provides the necessary information to be discovered and selected for sensing executions, with details FFS (pending SA2 progress).

Proposal 3: Introduce a sensing “task”, as a basic unit of managing sensing service cycle between SF (residing at 5GC) and SE (gNB), which can be uniquely identifiable by a sensing task ID and can serve as the anchor for initiating, modifying, and terminating the task, and for controlling the associated sensing parameter configuration and reporting between them.

Regarding Proposals 2 and 3, a text proposal for BL pCR to TR 38.765 [2] is provided in Annex.

# Reference

[1] RP-252819, New SI: Study on Integrated Sensing And Communication (ISAC) for NR (RAN1-led)

[2] TR 22.837, Study on Integrated Sensing and Communication (Rel-19), SA1

[3] TR 23.700-14, Study on Stage 2 for Integrated Sensing and Communication (Rel-20), SA2

# Annex (TP to BL pCR to TR 38.765 [2])

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

## 8.x	Sensing Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing reporting

1. The gNB authorized as Sensing Entity provides the sensing capability information to the Sensing CN node.

2. The Sensing CN node selects the gNB for sensing executions.

3.	The Sensing CN node sends sensing request to the gNB.

4.	The gNB sends sensing response to the Sensing CN node.

5.	If requested in Step 1, the gNB sends sensing report to the Sensing CN node.

NOTE: The message flow shown in Steps 3-5 corresponds to a single sensing task managed between the Sensing CN node and the gNB.