3GPP TSG-RAN WG3 Meeting #130	R3-258644

Dallas, US, November 17 - 21, 2025

Agenda Item:	9.1.1

Source:	ZTE Corporation

Title:	Discussion on paging in Store and Forward

Document for:	Discussion

- Introduction

In RAN2#131bis meeting, RAN2 has sent an LS to ask SA2 if any CN initiated paging is expected from MME to RAN during the time when the feeder link is not available [1].

According to RAN2 understanding, there will be no CN-initiated paging if there is no DL data buffered in MME when the feeder link is unavailable. RAN2 is discussing to have an indication in SIB to inform UEs that no paging (including CN initiated paging and RAN initiated paging) is expected in Store and Forward mode, if RAN confirms there will be no paging coming due to the unavailability of feeder link (so that the UEs may skip paging for power saving in Store and Forward mode). RAN2 would like to ask SA2 if any CN initiated paging is expected from MME to RAN during the time when the feeder link is not available.

Although RAN3 is in the cc list, the CN initiated paging from MME to RAN in Store and Forward mode may have impact on the S1 interface, in this contribution, we provide some considerations on this issue.

- Discussion

Based on the LS, RAN2 has reached the consensus that when the feeder link is unavailable, if there is no DL data buffered in the MME, there will be no CN-initiated paging. And RAN2 tries to introduce an indication in SIB to inform UEs that no paging is expected in Store and Forward mode, if RAN confirms there will be no paging coming due to the unavailability of feeder link.

In the Store and Forward mode, Either the Split MME architecture or Full CN on board architecture is supported. In this case, when the feeder link is unavailable, there should be no DL data transmitted from the ground the MME on board.

Observation: In the Store and Forward mode, when the feeder link is unavailable, there will be no DL data transmitted from ground to the MME.

After the feeder link becomes unavailable, the satellite can move into the serving area of the UEs, which means the service link is available. During the time when the service link is available, the eNB cannot predict in advance whether there will be DL data for the UEs, unless it receives the Paging message from the MME. However, since the MME is able to determine when to trigger the paging procedure based on the DRX cycle, the MME may send the Paging message to the eNB at any time before the satellite moves out of the serving area of the UEs.

Follow the scenario raised by RAN2, if the eNB has not received the Paging message from the MME for a long time after the feeder link becomes unavailable, it determines that there will be no more DL data for the UEs anymore. Then, the eNB shall broadcast the indication of no paging to the UEs in the coverage area at a specific time T1. After that, the UEs will stop monitoring the paging. However, it is possible that the eNB receives the Paging message from the MME shortly after T1, if there is some DL data buffered in the MME. In this case, the eNB has to broadcast another indication to inform the UEs to resume the paging monitoring. If the UE has already entered the power-saving mode after receiving the first indication, it cannot receive the second indication to resume the paging monitoring, which potentially causes the problem of paging and the packet loss.

In current specifications, there seems to be no corresponding description or clarification on whether there is DL data buffered in the MME when the feeder link is unavailable. And the eNB cannot know whether there is DL data buffered in the MME at that time. If so, the problem mentioned above exists. Therefore, before RAN2 decides to introduce the indication in SIB for no paging, whether there is DL data buffered in the MME when the feeder link is unavailable should be clarified, and the status should be known by the eNB.

Proposal: Whether there is DL data buffered in the MME when the feeder link is unavailable should be known by the eNB.

- Summary

Observation: In the Store and Forward mode, when the feeder link is unavailable, there will be no DL data transmitted from ground to the MME.

Proposal: Whether there is DL data buffered in the MME when the feeder link is unavailable should be known by the eNB.

4	Reference

- [1] R3-258091, LS to SA2 on the paging in Store and Forward, RAN2