3GPP TSG-RAN WG3 Meeting #130	R3-258656

Dallas, US, 17 – 21 November 2025

Agenda Item:		10.5.1

Source:		Ofinno

Title:	6G AI/ML Use Case on QoE

Document for:		Discussion and Decision

# Introduction

In last RAN3 #129bis meeting, the following agreements were achieved on AI/ML Use Cases for 6G:

The design of AI/ML algorithms and models for RAN3 led use cases are implementation-specific and out of RAN3 scope.

The following use cases will be studied in RAN3:

- AI/ML based Network energy saving

- AI/ML based mobility optimization

Other use cases can be discussed in future meetings based on contributions.

This contribution discusses further the potential AI/ML use case on QoE for 6G.

# Discussion

In 3GPP Release 17 and Rel-18, the work items on 5G QoE management for diverse services were introduced.

The existing 5QI (5G QoS identifier) service requirements are not enough to guarantee high-quality user experience for the various types of services. 5G QoE measurement collection was targeted to specify a mechanism to collect application layer measurements from the users. The users perform the application layer measurements according to user experience parameters/metrics for the services, e.g., streaming services, virtual reality (VR) services, extended reality (XR) services, multimedia telephony service for IMS (MTSI) services, multicast broadcast services (MBS).

For example, the user experience parameters/metrics for streaming services or VR services include average throughput, initial playout delay, buffer level, a list of playback periods, playout delay for media start-up and so on. Among them, several of them are RAN visible. Based on the collection of application layer measurements from the users, network can be enhanced to satisfy the user experience for the various types of services.

6G will be designed to provide a high-quality experience for the emerging services. Collection of application layer measurements from the users, i.e., QoE measurement, will be more important to realize the goal on top of QoS. The collected application layer measurements from the users can be input for AI/ML model training and inference. The user experience parameters/metrics can also be predicted based on the AI/ML model. That can be used by base station to predicate the resource assignment for users.

Especially RAN visible user experience parameters/metrics will be important for streaming services or XR services. The starting point may be from the average throughput, initial playout delay, buffer level, a list of playback periods, playout delay for media start-up and so on.

Proposal 1: To agree to include QoE use case for 6G AI/ML study in RAN3.

Proposal 2: To agree the TP in the Annex for AI/ML use case on QoE.

# Proposals

Proposal 1: To agree to include QoE use case for 6G AI/ML study in RAN3.

Proposal 2: To agree the TP in the Annex for AI/ML use case on QoE.

# Reference

[1] RP-251881 New SID: Study on 6G Radio.

[2] TR 38.843 AI/ML for NR air interface.

# Annex: TP for TR 38.760-3

# 7	AI/ML for RAN

## 7.1 	AI/ML use cases

## 7.1.x 	AI/ML based QoE

## 7.1.x.1 Use case description

QoE measurement collection is the scheme to specify a mechanism to collect application layer measurements from UE for emerging services, e.g., streaming services, virtual reality (VR) services, extended reality (XR) services, multimedia telephony service for IMS (MTSI) services, multicast broadcast services (MBS).

Based on the collection of application layer measurements from UE, network can be enhanced to satisfy the user experience for the services.

UE performs application layer measurements according to user experience metrics for the emerging services. For example, the user experience metrics for streaming services or XR services include average throughput, initial playout delay, buffer level, a list of playback periods, playout delay for media start-up and so on.

With AI/ML based QoE in 6G, the collected application layer measurements from UE can be input for AI/ML model training and inference. The user experience metrics can also be predicted based on the AI/ML model. Network/base station may determine resource assignment for UE based on the predication. The user experience for the services of a specific UE can be enhanced.